14 CFR Part 23 条款修订历史与背景分析报告

C 分部　结构

Subpart C - Structure

快照：Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier　｜　共 72 个条款 / 173 个历史版本

文档说明与编制依据 • 修订日期、修订案号、NPRM 与 Final Rule 编号：取自 FAA 动态法规系统（DRS）「Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier」条款修订版索引。 • 修订指令与讨论/评论处置段落：逐条引自本地已下载的 NPRM 与 Final Rule 官方文本，已译为中文；条文英文原文另附以供核对。 • 更正通告与 CFR 编纂校正：由官方 CFR 2016 版溯源注、federalregister.gov API 与govinfo 整期影印件三源交叉验证，详见 FAR23_更正通告权威台账.csv。 • 每个条款统一按「五节 + 一附录」编排：1 修订历史汇总 / 2 各阶段修订背景与原因深度剖析 / 3 条款原文演变对比 / 4 总结 / 5 参考文献 / 附录 DRS 条款版本记录 ID。 • 标题已写入 Word 大纲级别（1 级＝条款，2 级＝五节与附录，3 级＝小节，4 级＝子题），可在导航窗格或大纲视图中按级跳转。

# 本卷大纲

下按条款列出本卷结构。每一条款下统一设 1–5 节与附录；二级及以下标题可在 Word 导航窗格中展开。

| 条款 | 中文名称 | 英文标题（原文） | 版本数 | 更正件 |
| --- | --- | --- | --- | --- |
| § 23.301 | 载荷 | Loads. | 4 | 30 FR 258 |
| § 23.302 | 鸭式或串列翼构型 | Canard or tandem wing configurations. | 1 | — |
| § 23.303 | 安全系数 | Factor of safety. | 1 | — |
| § 23.305 | 强度与变形 | Strength and deformation. | 2 | — |
| § 23.307 | 结构验证 | Proof of structure. | 1 | — |
| § 23.321 | 总则（飞行载荷） | General. | 2 | — |
| § 23.331 | 对称飞行情况 | Symmetrical flight conditions. | 2 | 30 FR 258 |
| § 23.333 | 飞行包线 | Flight envelope. | 3 | — |
| § 23.335 | 设计空速 | Design airspeeds. | 5 | 52 FR 34745 |
| § 23.337 | 机动限制载荷因数 | Limit maneuvering load factors. | 4 | — |
| § 23.341 | 阵风载荷因数 | Gust load factors. | 5 | — |
| § 23.343 | 设计燃油载荷 | Design fuel loads. | 1 | — |
| § 23.345 | 高升力装置 | High lift devices. | 4 | — |
| § 23.347 | 不对称飞行情况 | Unsymmetrical flight conditions. | 2 | — |
| § 23.349 | 滚转情况 | Rolling conditions. | 4 | — |
| § 23.351 | 偏航情况 | Yawing conditions. | 2 | 30 FR 258 |
| § 23.361 | 发动机扭矩 | Engine torque. | 4 | — |
| § 23.363 | 发动机安装架侧向载荷 | Side load on engine mount. | 1 | — |
| § 23.365 | 增压座舱载荷 | Pressurized cabin loads. | 1 | — |
| § 23.367 | 发动机失效引起的不对称载荷 | Unsymmetrical loads due to engine failure. | 1 | — |
| § 23.369 | 后升力桁架的专用条件 | (Rear lift truss.) | 4 | 34 FR 17509 |
| § 23.371 | 陀螺载荷与气动载荷 | Gyroscopic loads. | 4 | — |
| § 23.373 | 减速装置（限速装置） | Speed control devices. | 1 | — |
| § 23.391 | 操纵面载荷 | Control surface loads. | 2 | — |
| § 23.393 | 平行于铰链线的载荷 | Loads parallel to hinge line. | 1 | — |
| § 23.395 | 操纵系统载荷 | Control system [loads.] | 2 | — |
| § 23.397 | 驾驶员限制操纵力 | Limit [control forces and torques.] | 5 | — |
| § 23.399 | 双套操纵系统 | Dual control system. | 2 | — |
| § 23.405 | 辅助操纵系统 | Secondary control system. | 1 | — |
| § 23.407 | 配平调整片效应 | Trim tab effects. | 1 | — |
| § 23.409 | 调整片 | Tabs. | 1 | — |
| § 23.415 | 地面阵风情况 | Ground gust conditions. | 4 | — |
| § 23.421 | 平衡载荷 | Balancing loads. | 3 | — |
| § 23.423 | 机动载荷（辅助翼面） | Maneuvering loads. | 3 | 56 FR 5455 |
| § 23.425 | 阵风载荷（辅助翼面） | Gust loads. | 3 | — |
| § 23.427 | 不对称载荷（辅助翼面） | Unsymmetrical loads. | 3 | — |
| § 23.441 | 机动载荷（尾翼） | Maneuvering loads. | 6 | — |
| § 23.443 | 阵风载荷（尾翼） | Gust loads. | 5 | 52 FR 34745、52 FR 7262 |
| § 23.445 | 外侧垂直安定面 | Outboard fins. | 3 | — |
| § 23.455 | 副翼 | Ailerons. | 4 | — |
| § 23.457 | 机翼襟翼 | Removed. | 2 | — |
| § 23.459 | 特殊装置 | Special devices. | 1 | — |
| § 23.471 | 总则（地面载荷） | General. | 1 | — |
| § 23.473 | 地面载荷情况与假设 | Ground load conditions and assumptions. | 5 | — |
| § 23.477 | 起落架布置 | Landing gear arrangement. | 1 | — |
| § 23.479 | 水平着陆情况 | Level landing conditions. | 3 | — |
| § 23.481 | 尾部下沉着陆情况 | Tail down landing conditions. | 1 | — |
| § 23.483 | 单轮着陆情况 | One-wheel landing conditions. | 1 | — |
| § 23.485 | 侧向载荷情况 | Side load conditions. | 2 | — |
| § 23.493 | 刹车滚滑情况 | Braked roll conditions. | 1 | — |
| § 23.497 | 尾轮的补充情况 | Supplementary conditions for tail wheels. | 2 | — |
| § 23.499 | 前轮的补充情况 | Supplementary conditions for nose wheels. | 2 | — |
| § 23.505 | 滑橇式飞机的补充情况 | Supplementary conditions for skiplanes. | 2 | — |
| § 23.507 | 顶升载荷 | Jacking loads. | 1 | — |
| § 23.509 | 牵引载荷 | Towing loads. | 1 | — |
| § 23.511 | 地面载荷；多轮装置的不对称载荷 | Ground load; unsymmetrical loads on multiple-wheel units. | 1 | — |
| § 23.521 | 水上载荷情况 | Water load conditions. | 3 | — |
| § 23.523 | 设计重量与重心位置 | Design weights and center of gravity positions. | 1 | — |
| § 23.525 | 载荷的施加 | (Application of loads.) | 1 | 58 FR 51970 |
| § 23.527 | 船身与主浮筒载荷因数 | (Hull and main float load factors.) | 1 | 58 FR 51970 |
| § 23.529 | 船身与主浮筒着陆情况 | (Hull and main float landing conditions.) | 1 | — |
| § 23.531 | 船身与主浮筒起飞情况 | (Hull and main float takeoff condition.) | 1 | — |
| § 23.533 | 船身与主浮筒底部压力 | (Hull and main float bottom pressures.) | 1 | 58 FR 51970 |
| § 23.535 | 辅助浮筒载荷 | (Auxiliary float loads.) | 1 | 58 FR 51970 |
| § 23.537 | 海翼载荷 | Seawing loads. | 1 | — |
| § 23.561 | 总则（应急着陆） | General. | 7 | 52 FR 34745、72 FR 59939、72 FR 72915 |
| § 23.562 | 应急着陆动力情况 | (Emergency landing dynamic conditions.) | 4 | — |
| § 23.571 | 增压座舱（疲劳评定） | Pressurized cabin. | 6 | 73 FR 19746 |
| § 23.572 | 机翼及相关结构 | Wing, empennage, and associated structures. | 6 | 73 FR 19746 |
| § 23.573 | 结构损伤容限与疲劳评定 | Damage tolerance and fatigue evaluation of structure. | 2 | 58 FR 51970、73 FR 19746 |
| § 23.574 | 通勤类飞机金属结构损伤容限与疲劳评定 | Metallic damage tolerance and fatigue evaluation of commuter category airplanes. | 1 | — |
| § 23.575 | 检查与其他程序 | Inspections and other procedures. | 1 | — |

# 目录

※ 在 Word/WPS 中按 Ctrl+A 后 F9（或右键→更新域）生成目录

# § 23.301　载荷（Loads.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 4 个历史版本

## 1　§23.301 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.301 |
| 条款标题（英文原文） | Loads. |
| 中文名称 | 载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 30 FR 258 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.301 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-28 | 1982-04-28 | Notice 80-23，1980-11-28 签发，45 FR 80450 | Docket 21129，1982-02-26 签发，47 FR 13312 | 删除内容 |
| 3 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 修订段落：(b) |
| 4 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1982 23-28　删除内容　→　1991 23-42　修订段落：(b)　→　1996 23-48　修订

## 2　§23.301 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：载荷

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-28（1982-04-28 生效）：通过修订第23.301(d)条第二句

▪ 修订沿革：本版由 NPRM　Notice 80-23，1980-11-28 签发，45 FR 80450　提出；最终规章 Docket 21129，1982-02-26 签发，47 FR 13312，1982-04-28 生效。

▪ 修订指令（中文译文）：通过修订第23.301(d)条第二句，删除“等效于第23.331条至第23.399条”字样，并在其位置插入“等效于第23.321条至第23.459条”字样。

修订指令英文原文（供核对）：

1. By amending the second sentence of Sec. 23.301(d) by removing the words "equivalent of Secs. 23.331 through 23.399" and inserting the words "equivalent of Secs. 23.321 through 23.459" in their place.

### 2.3　Amdt 23-42（1991-02-04 生效）：§23.301 条修订如下：修订 (b) 段

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：§23.301 条修订如下：修订 (b) 段，内容如下：　第 23 部新增 §23.302（置于 §23.301 之后），内容如下：

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案 8。本提案将制定 §23.301 标准，用于确定鸭式布局和串列翼布局飞机的载荷强度和分布。未收到关于本提案的评论，现按提案通过。

英文原文：

Proposal 8. This proposal would establish Sec. 23.301 criteria for determining loan intensities and distributions for airplanes with canard and tandem wing configurations. No comments were received on this proposal and it is adopted as proposed.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（2）：会议提案177部分建议将拟议的§23.302的要求作为新的(e)段添加到§23.301“载荷”中。然而，正如一位评论者指出的，除载荷外的其他要求也适用于鸭式布局。另一位评论者指出，具体项目不应属于一般载荷要求的一部分。美国联邦航空局同意这些意见，因此提议新增§23.302，以阐明适用于常规布局中执行某一功能的部件的所有要求，同样适用于这些非常规布局（鸭式和串翼）中执行相同功能的部件。

英文原文：

Conference proposal 177, in part, recommended that the requirements of proposed Sec. 23.302 be added as a new paragraph (e) to Sec. 23.301 Loads. However, as one commenter pointed out, requirements other than loads are applicable to canard configurations. Another commenter pointed out that specific items should not be part of the general loads requirements. The FAA agrees and therefore is proposing a new Sec. 23.302 to clarify that all requirements applicable to a component performing a function in a conventional configuration are applicable to the components of these unconventional configurations (canard and tandem wing) that perform the same function.

修订指令英文原文（供核对）：

8. Section 23.301 is amended by revising paragraph (b) to read as follows:

9. Part 23 is amended by adding a new Sec. 23.302 after Sec. 23.301 to read as follows:

▪ 本版重印条文段落：(b)

### 2.4　Amdt 23-48（1996-03-11 生效）：§23.301 条修订如下：修订 (d) 段

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：§23.301 条修订如下：修订 (d) 段，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议修订 §23.301(d) 段，将第 23 部附录 A 的适用范围限制为“单发（不含涡轮）”飞机，而不是现有的单发限制。拟议修订的影响将是消除涡轮发动机的备选附录 A 飞机设计要求，因为欧洲联合航空局（JAA）认定，且美国联邦航空局（FAA）同意，在引入附录 A 时，仅设想了单发（不含涡轮）飞机。涡轮飞机设计可继续通过证实符合第 23 部来由美国联邦航空局（FAA）取证。

英文原文：

The FAA proposed to amend Sec. 23.301(d) by limiting the applicability of Appendix A to part 23 to "single-engine, excluding turbines'' airplanes, rather than the current single-engine limitation. The effect of the proposed changes would be to eliminate alternative Appendix A airplane design requirements for turbine engines because the JAA determined, and the FAA agrees, that only single-engine airplanes, excluding turbines, were envisioned when Appendix A was introduced. Turbine airplane designs could continue to be FAA certificated by substantiation to part 23.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：为了将所有“襟翼”要求集中在一处，并与 JAR 23 实现协调统一，美国联邦航空局提议修订 §23.345 如下：（1）进行较小的组织性、非实质性的澄清性修改；（2）将术语“完全偏转”改为“完全放下”，因为后者更准确地描述襟翼状态和位置；（3）删除短语“导致限制载荷因数”，因为该要求已存在于 §23.301(a) 段；（4）将现行(c)段改列为(d)段，并修订该段以纳入 §23.457 的襟翼要求；（5）将现行(d)段改列为(c)段；（6）将 §23.457 的襟翼要求纳入经改列后的 §23.345(b) 段和 §23.345(d) 段，并删除冗余的(e)段。

英文原文：

To place all "flap" requirements in one location, and to harmonize the requirements with JAR 23, the FAA proposed to revise Sec. 23.345 as follows: (1) Make minor organizational, and non- substantive, clarifying changes; (2) Change the term "fully deflected" to "fully extended" because it more accurately describes flap conditions and positions; (3) Remove the phrase "resulting in limit load factors" because the requirement already exists in Sec. 23.301(a); (4) Redesignate current paragraph (c) as paragraph (d) and revise it to include the flap requirements of Sec. 23.457; (5) Redesignate current paragraph (d) as paragraph (c); and (6) Incorporate the flap requirements of Sec. 23.457 into Sec. 23.345(b) and Sec. 23.345(d), as redesignated, and delete paragraph (e), which is redundant.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（3）：本提案将修订 §23.301(d) 段，将附录 A 的适用性限制为"不包括涡轮发动机的单发"飞机，而非现行的单发限制。欧洲联合航空局（JAA）提议用"单发活塞式发动机"代替现行规章中的"单发"。美国联邦航空局（FAA）基于附录 A 前言中解释的理由，提议采用"不包括涡轮发动机的单发"。其效果将是取消针对涡轮发动机的附录 A 备选飞机设计要求，因为欧洲联合航空局（JAA）认定，且美国联邦航空局（FAA）同意，引入附录 A 时仅设想了不包括涡轮发动机的单发飞机。涡轮发动机飞机设计仍可通过符合第 23 部 C 分部的要求以及依据 §21.16 条规定的任何专用条件，继续取得美国联邦航空局（FAA）的型号合格证。对本节的拟议修订明确，附录 A 仅适用于除涡轮发动机以外的单发飞机。

英文原文：

This proposal would amend Sec. 23.301(d) by limiting the applicability of Appendix A to "single-engine, excluding turbines" airplanes rather than the current single-engine limitation. The JAA proposed "single reciprocating engine" instead of "single-engine," which appears in the current regulations. The FAA proposes, "single-engine, excluding turbines" for the reasons explained in the preamble to Appendix A. The effect would be to eliminate alternative Appendix A airplane design requirements for turbine engines because the JAA determined, and the FAA agrees, that only single-engine airplanes excluding turbines, were envisioned when Appendix A was introduced. Turbine airplane designs may continue to be FAA certificated by substantiation to part 23, Subpart C, requirements plus any special conditions as prescribed under Sec. 21.16. The proposed changes to this section clarify that Appendix A applies only to single-engine airplanes, except turbines.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（4）：修订后的 §23.345(a) 段将包含次要的、非实质性的澄清性修改。术语"完全偏转"将改为"完全展开"，因为它更准确地描述了襟翼状态和位置。短语"产生极限载荷因数"将被删除，因为该要求已存在于 §23.301(a) 段中。现行(d)段将重新指定为(c)段，不作更改。

英文原文：

Revised Sec. 23.345(a) would have minor, non-substantive, clarifying changes. The term "fully deflected" would be changed to "fully extended" because it more accurately describes flap conditions and positions. The phrase "resulting in limit load factors" would be removed because the requirement already exists in Sec. 23.301 (a). Current paragraph(d) would be redesignated as paragraph (c) without change.

修订指令英文原文（供核对）：

2. Section 23.301(d) is revised to read as follows:

▪ 本版重印条文段落：(d)

### 2.5　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.301 的实际改动 |
| --- | --- | --- | --- |
| 30 FR 258 | 1965-01-09 | 更正通告 (CFR 溯源注) | §23.365(d) 系数 1.3→1.33；§23.427 公式 'local'→'load' |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.301 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Strength requirements are specified in terms of limit loads (the maximum loads to be expected in service) and ultimate loads (limit loads multiplied by prescribed factors of safety). Unless otherwise provided, prescribed loads are limit loads. | 新增 |
| (b) | Unless otherwise provided, the air, ground, and water loads must be placed in equilibrium with inertia forces, considering each item of mass in the airplane. These loads must be distributed to conservatively approximate or closely represent actual conditions. | 新增 |
| (c) | If deflections under load would significantly change the distribution of external or internal loads, this redistribution must be taken into account. | 新增 |
| (d) | Simplified structural design criteria may be used if they result in design loads not less than those prescribed in Secs. 23.331 through 23.521. For conventional, single-engine airplanes with design weights of 6,000 pounds or less, the design criteria of Appendix A of this part are an approved equivalent of Secs. 23.331 through 23.399. If Appendix A is used, the entire Appendix must be substituted for the corresponding sections of this part. | 新增 |

### 3.2　版本2：Amdt 23-42（1991-02-04 生效）　来源：Docket 25811，1990-12-21 签发，56 FR 344

修订段落：(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | Unless otherwise provided, the air, ground, and water loads must be placed in equilibrium with inertia forces, considering each item of mass in the airplane. These loads must be distributed to conservatively approximate or closely represent actual conditions. Methods used to determine load intensities and distribution on canard and tandem wing configurations must be validated by flight test measurement unless the methods used for determining those loading conditions are shown to be reliable or conservative on the configuration under consideration. | 修订 |

### 3.3　版本3：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (d) | Simplified structural design criteria may be used if they result in design loads not less than those prescribed in Secs. 23.331 through 23.521. For airplane configurations described in appendix A, Sec. 23.1, the design criteria of appendix A of this part are an approved equivalent of Secs. 23.321 through 23.459. If appendix A of this part is used, the entire appendix must be substituted for the corresponding sections of this part. | 新增 |

### 3.4　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-42 (1991-02-04) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- |
| (a) | ● 新增 | ○ 无 | ○ 无 |
| (b) | ● 新增 | △ 修订 | ○ 无 |
| (c) | ● 新增 | ○ 无 | ○ 无 |
| (d) | ● 新增 | ○ 无 | ● 新增 |

## 4　§23.301 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 新技术 / 新构型 | ining loan intensities and distributions for airplanes with canard and tandem wing configurations. No comments were received on this proposal and it is adopted as proposed. Conference proposal 177, in part, recommended t… | Amdt 23-42 |
| 国际协调 | irplane design requirements for turbine engines because the JAA determined, and the FAA agrees, that only single-engine airplanes, excluding turbines, were envisioned when Appendix A was introduced. Turbine airplane desi… | Amdt 23-48 |

本条取证提示 共 4 个历史版本：初始版, Amdt 23-28, Amdt 23-42, Amdt 23-48。 其中 3 个版本在本地文献中检索到明确修订指令原文；3 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：30 FR 258，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 21129 | 47 FR 13312 | FinalRule_Docket 21129_Amdt23-28_25-55_47FR13312_1982-03-29.txt |
| [4] | NPRM（提案） | Notice 80-23 / Docket 21129 | 45 FR 80450 | NPRM_Notice 80-23_Docket21129_45FR80450_1980-12-04.txt |
| [5] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [6] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [7] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [8] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [9] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [10] | 更正通告 / 校正 | 30 FR 258 | 1965-01-09 | Correction_30FR258_1965-01-09_Amdt23-0.clean.txt |
| [11] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [12] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.301 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 779ae113-4c93-4818-8708-a6f6c830fa7d |
| Amdt 23-28 | 1982-04-28 | 0a5b7acc-2604-48de-bb19-4ef364d89485 |
| Amdt 23-42 | 1991-02-04 | ecdf97af-d35e-437c-83e6-3117a3209fba |
| Amdt 23-48 | 1996-03-11 | 474dfe0d-85b6-473a-839f-f4068c9299a6 |

# § 23.302　鸭式或串列翼构型（Canard or tandem wing configurations.）

1991-02-04 初版　｜　1991-02-04 Amdt 23-42　｜　共 1 个历史版本

## 1　§23.302 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.302 |
| 条款标题（英文原文） | Canard or tandem wing configurations. |
| 中文名称 | 鸭式或串列翼构型 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1991-02-04 初始版） |
| 时间跨度 | 1991-02-04 → 1991-02-04 （约 0 年） |
| 初版来源 | Docket 25811，1990-12-21 签发，56 FR 344 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.302 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 新增内容 |

### 1.3　修订脉络一览

1991 初版　新增内容

## 2　§23.302 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1991-02-04，Amdt 23-42）：鸭式或串列翼构型

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：第 23 部新增 §23.302（置于 §23.301 之后），内容如下：

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案 9。本提案将设立新的条 23.302，要求具有鸭翼或串

英文原文：

Proposal 9. This proposal would establish a new Sec. 23.302 to require that airplanes with canard or tandem wing configurations meet all requirements of subpart C and subpart D applicable to a wing. This proposal is necessary because the forward structure of a canard or a tandem wing configuration performs both a control function and a lifting surface function similar to a main wing, and, therefore, it should meet both the wing and control surface requirements.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（2）：会议提案177部分建议将拟议的§23.302的要求作为新的(e)段添加到§23.301“载荷”中。然而，正如一位评论者指出的，除载荷外的其他要求也适用于鸭式布局。另一位评论者指出，具体项目不应属于一般载荷要求的一部分。美国联邦航空局同意这些意见，因此提议新增§23.302，以阐明适用于常规布局中执行某一功能的部件的所有要求，同样适用于这些非常规布局（鸭式和串翼）中执行相同功能的部件。

英文原文：

Conference proposal 177, in part, recommended that the requirements of proposed Sec. 23.302 be added as a new paragraph (e) to Sec. 23.301 Loads. However, as one commenter pointed out, requirements other than loads are applicable to canard configurations. Another commenter pointed out that specific items should not be part of the general loads requirements. The FAA agrees and therefore is proposing a new Sec. 23.302 to clarify that all requirements applicable to a component performing a function in a conventional configuration are applicable to the components of these unconventional configurations (canard and tandem wing) that perform the same function.

修订指令英文原文（供核对）：

9. Part 23 is amended by adding a new Sec. 23.302 after Sec. 23.301 to read as follows:

## 3　§23.302 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1991-02-04 生效）　来源：Docket 25811，1990-12-21 签发，56 FR 344

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Meet all requirements of subpart C and subpart D of this part applicable to a wing; and | 新增 |
| (b) | Meet all requirements applicable to the function performed by these surfaces. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-42 (1991-02-04) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.302 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 新技术 / 新构型 | establish a new Sec. 23.302 to require that airplanes with canard or tandem wing configurations meet all requirements of subpart C and subpart D applicable to a wing. This proposal is necessary because the forward struc… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-42。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [2] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.302 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-42 | 1991-02-04 | 1f76fc5d-b65f-4b7e-a9ef-8d70d27c48f7 |

# § 23.303　安全系数（Factor of safety.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.303 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.303 |
| 条款标题（英文原文） | Factor of safety. |
| 中文名称 | 安全系数 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.303 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.303 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：安全系数

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.303 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.303 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.303 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | bf0b41ac-817f-4e04-9b52-246cb8edb45c |

# § 23.305　强度与变形（Strength and deformation.）

1965-02-01 初版　｜　1993-09-07 Amdt 23-45　｜　共 2 个历史版本

## 1　§23.305 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.305 |
| 条款标题（英文原文） | Strength and deformation. |
| 中文名称 | 强度与变形 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1993-09-07 （约 28 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.305 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订段落：(b) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1993 23-45　修订段落：(b)

## 2　§23.305 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：强度与变形

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-45（1993-09-07 生效）：§23.305 条修订如下：修订 (b) 段

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：§23.305 条修订如下：修订 (b) 段，内容如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订§23.305，以澄清静力极限载荷试验中失效的含义。
唯一发表评论的欧洲联合航空局（JAA）质疑，《规章制定提案通告》中提到的“宽松解释”为何出现在第23部规章的适用中，而未出现在第25部中。美国联邦航空局在《联邦航空规章》第23部机体适航性审查中处理了该问题，该审查发现极限载荷试验中存在失效定义不一致的情况。由该会议产生的咨询通告23-6回应了该评论者的关切。负责第25部的运输类飞机审定处已知悉第23部的规章制定行动。
美国联邦航空局按提案采纳§23.305。

英文原文：

The FAA proposed to revise Section 23.305 to clarify the meaning of failure during static ultimate load test.

The one commenter, the JAA, questions why the "liberal interpretation" mentioned in the NPRM occurs in applications of part 23 regulations and not in part 25. The FAA addressed this issue in the FAR part 23 Airframe Airworthiness Review, which identified inconsistent definitions of failure during ultimate load testing. Advisory Circular 23-6, which resulted from that meeting, addresses this commenter's concerns. The Transport Airplane Directorate, which is responsible for part 25, is aware of the part 23 regulatory action.

The FAA adopts Section 23.305 as proposed.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（2）：美国联邦航空局（FAA）按提案采纳第 23.305 条。

英文原文：

The FAA adopts Section 23.305 as proposed.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：说明：本提案澄清了美国联邦航空局（FAA）对静力极限载荷试验中失效的解释。按照现行§23.305条，在静力极限载荷试验中，如果零件或部件（如铆钉）在超过限制载荷但低于极限载荷时发生失效，则该试验即为失败。按照更宽松的解释，只要整个结构证明其具备承受极限载荷三秒钟的能力，则在限制载荷与极限载荷之间发生失效或结构失稳是可以接受的。本提案澄清了这一差异，但并不意图免除§23.301(c)条或§23.305(a)条所示的变形要求。本提案的意图在会议上未遭反对。

英文原文：

Explanation: This proposal clarifies the FAA's interpretation of failure during static ultimate load test. Using existing Section 23.305, the test is a failure if a part or component fails (e.g., a rivet) beyond limit load but below ultimate load during a static ultimate load test. Using a more liberal interpretation, a failure or structural instability between limit and ultimate load is acceptable as long as the entire structure demonstrates the capability to carry ultimate load for three seconds. This proposal clarifies this disparity but is not intended to relieve the requirement for deflection shown in Section 23.301(c) or Section 23.305(a). The intent of this proposal was unopposed at the conference.

修订指令英文原文（供核对）：

28. Section 23.305 is amended by revising paragraph (b) to read as follows:

▪ 本版重印条文段落：(b)

## 3　§23.305 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The structure must be able to support limit loads without detrimental, permanent deformation. At any load up to limit loads, the deformation may not interfere with safe operation. | 新增 |
| (b) | The structure must be able to support ultimate loads without failure for at least three seconds. However, when proof of strength is shown by dynamic tests simulating actual load conditions, the three second limit does not apply. | 新增 |

### 3.2　版本2：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

修订段落：(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | The structure must be able to support ultimate loads without failure for at least three seconds, except local failures or structural instabilities between limit and ultimate load are acceptable only if the structure can sustain the required ultimate load for at least three seconds. However when proof of strength is shown by dynamic tests simulating actual load conditions, the three second limit does not apply. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-45 (1993-09-07) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (b) | ● 新增 | △ 修订 |

## 4　§23.305 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | re during static ultimate load test. The one commenter, the JAA, questions why the "liberal interpretation" mentioned in the NPRM occurs in applications of part 23 regulations and not in part 25. The FAA addressed this i… | Amdt 23-45 |

本条取证提示 共 2 个历史版本：初始版, Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [4] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [5] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [6] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.305 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 76b216b4-a15f-4617-af2e-0a9979c4b28c |
| Amdt 23-45 | 1993-09-07 | 24d05ba2-1f5b-47da-865f-e00ad2610dd6 |

# § 23.307　结构验证（Proof of structure.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.307 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.307 |
| 条款标题（英文原文） | Proof of structure. |
| 中文名称 | 结构验证 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.307 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.307 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：结构验证

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.307 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Compliance with the strength and deformation requirements of Sec. 23.305 must be shown for each critical load condition. Structural analysis may be used only if the structure conforms to those for which experience has shown this method to be reliable. In other cases, substantiating load tests must be made. Dynamic tests, including structural flight tests, are acceptable if the design load conditions have been simulated. | 新增 |
| (b) | Certain parts of the structure must be tested as specified in Subpart D of this part. Flight Loads | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.307 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.307 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 1f967f90-554a-4bea-8108-26f75fdbbf7d |

# § 23.321　总则（飞行载荷）（General.）

1965-02-01 初版　｜　1993-09-07 Amdt 23-45　｜　共 2 个历史版本

## 1　§23.321 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.321 |
| 条款标题（英文原文） | General. |
| 中文名称 | 总则（飞行载荷） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1993-09-07 （约 28 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.321 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 新增段落：(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1993 23-45　新增段落：(c)

## 2　§23.321 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：总则（飞行载荷）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-45（1993-09-07 生效）：§23.321 条新增 (c) 段

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：§23.321 条新增 (c) 段，内容如下：　30. 对第23.321条进行修订，将(b)段的引言文本修订如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦

英文原文：

The FAA proposed to amend Section 23.321 so the effects of compressibility on flight loads will be considered at each speed within the flight envelope.

The GAMA recommends that Section 23.321(b) be rewritten to require consideration of compressibility effects above Mach 0.6. The GAMA argues that the effects of compressibility below Mach 0.6 are insignificant on flight loads. The JAA argues that compressibility needs to be taken into account only if significant and that compressibility is unlikely to be significant if the airplane Mach number is less than 0.5.

The FAA has reviewed the NPRM proposal and the comments received. To simplify certification procedures of lower performance airplanes, small compressibility effects may be neglected below a design dive speed of Mach (MD) 0.40. At Mach numbers above zero, theoretical compressibility effects cause an increase in an airfoil lift curve slope. This increase is proportional to the Prandtl-Glauert factor, 1/ü(1-M2), where M is the free stream Mach number. This theory correlates very well with wind tunnel tests of airfoils and wings.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（2）：美国联邦航空局（FAA）已决定，申请人必须考虑压缩性效应。由于风洞数据和试验方法、高度以及飞机设计的不同，压缩性阈值显著性会有所变化。由于这些原因，美国联邦航空局（FAA）未制定设计俯冲速度马赫数符合性阈值。原提案本应修订第 23.321 条的段(b)以规定压缩性效应。经重新评估，美国联邦航空局（FAA）得出结论，在新的段(c)中增加此要求会更清晰。美国联邦航空局（FAA）在采纳上述讨论的变更后通过第 23.321 条。

英文原文：

The FAA has decided that the effects of compressibility must be considered by the applicant. Compressibility threshold significance varies due to wind tunnel data and testing methods, altitudes, and airplane design. For these reasons, the FAA establishes no design dive speed Mach number compliance threshold. The original proposal would have revised paragraph (b) of Section 23.321 to provide for the effects of compressibility. Upon re-evaluation, the FAA has concluded that it would be clearer to add this requirement in a new paragraph (c). The FAA adopts Section 23.321 with the changes discussed above.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：说明：本提案建议，在包线内的每个速度下考虑压缩性对飞行载荷的影响。针对第23.321条共有三项会议提案。本提案源自几乎相同的会议提案183和185。

英文原文：

Explanation: This proposal recommends that the effects of compressibility on flight loads be considered at each speed within the envelope. There were three conference proposals directed toward Section 23.321. This proposal is developed from nearly identical conference proposals 183 and 185.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（4）：另一位评论者不反对修订第23.321条以考虑压缩性，但该评论者不同意会议提案183中的陈述，即要求考虑包线内每个速度（甚至低至慢速）的压缩性。

英文原文：

Another commenter did not object to revising Section 23.321 to consider compressibility, but that commenter did disagree with the statement in conference proposal 183 requiring consideration at every speed within the envelope, even down to slow speeds.

修订指令英文原文（供核对）：

29. Section 23.321 is amended by adding a new paragraph (c) to read as follows:

30. Section 23.321 is amended by revising paragraph (b) introductory text to read as follows:

▪ 本版重印条文段落：(c)

## 3　§23.321 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Flight load factors represent the ratio of the aerodynamic force component (acting normal to the assumed longitudinal axis of the airplane) to the weight of the airplane. A positive flight load factor is one in which the aerodynamic force acts upward, with respect to the airplane. | 新增 |
| (b) | Compliance with the flight load requirements of this subpart must be shown-- | 新增 |
| (1) | At each critical altitude within the range in which the airplane may be expected to operate; | 新增 |
| (2) | At each weight from the design minimum weight to the design maximum weight; and | 新增 |
| (3) | For each required altitude and weight, for any practicable distribution of disposable load within the operating limitations specified in Secs. 23.1583 through 23.1589. | 新增 |

### 3.2　版本2：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

新增段落：(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | When significant, the effects of compressibility must be taken into account. | 新增 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-45 (1993-09-07) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (b) | ● 新增 | ○ 无 |
| (1) | ● 新增 | ○ 无 |
| (2) | ● 新增 | ○ 无 |
| (3) | ● 新增 | ○ 无 |
| (c) | ○ 无 | ● 新增 |

## 4　§23.321 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | m Review (October 28, 1983), the FAA intended to initiate a study to define such a speed value. Another commenter did not object to revising Section 23.321 to consider compressibility, but that commenter did disagree wit… | Amdt 23-45 |

本条取证提示 共 2 个历史版本：初始版, Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [4] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [5] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [6] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.321 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 834026d0-fcc3-435d-a3e7-d28705abb0f6 |
| Amdt 23-45 | 1993-09-07 | e786a909-f86c-4d1d-a47f-acafd26de535 |

# § 23.331　对称飞行情况（Symmetrical flight conditions.）

1965-02-01 初版　｜　1991-02-04 Amdt 23-42　｜　共 2 个历史版本

## 1　§23.331 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.331 |
| 条款标题（英文原文） | Symmetrical flight conditions. |
| 中文名称 | 对称飞行情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1991-02-04 （约 26 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 30 FR 258 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.331 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 新增段落：(a)、(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1991 23-42　新增段落：(a)、(c)

## 2　§23.331 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：对称飞行情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-42（1991-02-04 生效）：第 23.331 条(a)段修改为：将“Sec. 23.331”替换

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：第 23.331 条(a)段修改为：将“Sec. 23.331”替换为“Sec. 23.333”，并新增(c)段如下：

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案10. 该提案将更正§23.331(a)段中的一个错误，把现有(a)段中对§23.331的引用改为§23.333。此外，将在§23.331中新增(c)段，以确保鸭式布局和串列翼布局中适用于水平翼面的飞行载荷在型号合格审定过程中得到评定。未收到针对该提案的评论，该提案按提案内容予以通过。

英文原文：

Proposal 10. This proposal would correct an error in Sec. 23.331(a) by changing the reference to Sec. 23.331 to Sec. 23.333 in existing paragraph (a). Also, a new paragraph (c) would be added to Sec. 23.331 to ensure that flight loads applicable to horizontal surfaces in canard and tandem wing configurations are evaluated during the type certification process. No comments were received on this proposal and it is adopted as proposed.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（2）：§23.331 条规定，对称飞行情况规定于 §23.331 至 §23.341 条。由于 §23.331 条中并未规定任何对称飞行情况，本提案纠正了自第 23 部重新编纂以来即存在的一个错误。

英文原文：

Section 23.331 states that symmetrical flight conditions are specified in Secs. 23.331 through 23.341. Since no symmetrical flight conditions are specified in Sec. 23.331, this proposal corrects an error existing since recodification of Part 23.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（3）：现行飞行载荷要求一般仅适用于水平安定面位于主机翼升力面之后的飞机。前置机翼的诱导速度和涡流会改变局部气流角，从而改变主机翼升力面的升力分布。鸭式或串列翼布局的飞机在垂直阵风中的响应与常规飞机不同。所关注的问题是，前置翼面先于主升力面进入阵风，从而增大飞机的有效迎角。当飞机飞出阵风时，情况则相反。因此，对于鸭式布局以及水平翼面位于主升力面前方的串列翼布局，其适用的飞行载荷需要在型号合格审定过程中进行评定。会议上，与会者一致支持该提案，认为其是对§23.331条的必要增补，美国联邦航空局（FAA）对此表示同意。

英文原文：

Present requirements for flight loads are generally applicable only to airplanes with the horizontal stabilizing surface located aft of the main wing lifting surface. The induced velocities and vortices of a forward wing change the local flow angles, and therefore, the lift distribution of the main wing lifting surface. An airplane with a canard or tandem wing configuration responds differently than a conventional airplane in vertical gusts. The concern is that the forward surface penetrates the gust ahead of the main lifting surface, thus, increasing the effective airplane angle of attack. The reverse occurs as the airplane exits the gust. Therefore, the flight loads applicable to canard configurations and tandem wing configuration in which a horizontal surface is located forward of the main lifting surface need to be evaluated during the type certification process. There was a consensus of support for the proposal at the conference as a needed addition to Sec. 23.331 and the FAA agrees.

修订指令英文原文（供核对）：

10. Section 23.331 is amended in paragraph (a) by replacing "Sec. 23.331" with "Sec. 23.333" and by adding a new paragraph (c) to read as follows:

▪ 本版重印条文段落：(c)

### 2.3　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.331 的实际改动 |
| --- | --- | --- | --- |
| 30 FR 258 | 1965-01-09 | 更正通告 (CFR 溯源注) | §23.365(d) 系数 1.3→1.33；§23.427 公式 'local'→'load' |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.331 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The appropriate balancing horizontal tail load must be accounted for in a rational or conservative manner when determining the wing loads and linear inertia loads corresponding to any of the symmetrical flight conditions specified in Secs. 23.331 through 23.341. | 新增 |
| (b) | The incremental horizontal tail loads due to maneuvering and gusts must be reacted by the angular inertia of the airplane in a rational or conservative manner. | 新增 |

### 3.2　版本2：Amdt 23-42（1991-02-04 生效）　来源：Docket 25811，1990-12-21 签发，56 FR 344

新增段落：(a)、(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | Mutual influence of the aerodynamic surfaces must be taken into account when determining flight loads. | 新增 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-42 (1991-02-04) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (b) | ● 新增 | ○ 无 |
| (c) | ○ 无 | ● 新增 |

## 4　§23.331 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 新技术 / 新构型 | sure that flight loads applicable to horizontal surfaces in canard and tandem wing configurations are evaluated during the type certification process. No comments were received on this proposal and it is adopted as propo… | Amdt 23-42 |

本条取证提示 共 2 个历史版本：初始版, Amdt 23-42。 其中 1 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：30 FR 258，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [4] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [5] | 更正通告 / 校正 | 30 FR 258 | 1965-01-09 | Correction_30FR258_1965-01-09_Amdt23-0.clean.txt |
| [6] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [7] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.331 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | ac248d53-ed8e-48f0-ae91-c7495bf919e1 |
| Amdt 23-42 | 1991-02-04 | 6e9c1d5c-881d-42f9-9c59-a87a6c11ca81 |

# § 23.333　飞行包线（Flight envelope.）

1965-02-01 初版　｜　1987-02-17 Amdt 23-34　｜　共 3 个历史版本

## 1　§23.333 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.333 |
| 条款标题（英文原文） | Flight envelope. |
| 中文名称 | 飞行包线 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 3 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1987-02-17 （约 22 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.333 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(c) |
| 3 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 新增段落：(b)、(c)、(d) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(c)　→　1987 23-34　新增段落：(b)、(c)、(d)

## 2　§23.333 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：飞行包线

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：17, 第23.333条经修订

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：17, 第23.333条经修订，将(c)段和(d)段修改为：　将对§23.333进行修订，删除(c)段，将(d)段的标题改为“阵风包线”，并按如下方式修订(d)段的包线，以与第25部§25.333(c) 段中的阵风包线一致：

来源：Final Rule｜34 FR 13078｜FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt

▪ 背景与评论处置（1）：本通告提议修订第23.333、23.341、23.345、23.425、23.443条及附录A23.3，要求基于飞机质量参数概念而非现行翼载荷概念进行阵风载荷研究。收到的评论反对更改现行第23.341条中的阵风载荷准则，因为新的阵风公式提高了载荷因数，从而给设计带来过重负担，且质量参数方法导致不必要的载荷和载荷因数增加。然而，美国联邦航空局认为，采用拟议的阵风公式对于更真实地反映更高速度和更高高度对阵风载荷的影响至关重要。推导新公式时使用了运输类飞机数据，因为所涉及的阵风强度同样适用于任何设计在与现行运输类飞机相同速度和高度范围内运行的飞机。
同时，新公式不会对低速、低空飞机的设计施加过度的负担，因为其设计通常机动起决定性作用。为清楚起见，提案的格式已作更改，以遵循第 23 部的现行格式，并且 §23.333(d) 段阵风包线图中代表限制组合包线的实线已被修订，以正确反映用于确定阵风包线的公式。

英文原文：

The notice proposed to amend Secs. 23.333, 23.341, 23.345, 23.425, 23.443, and Appendix A23.3 to require investigation of gust loads based on airplane mass parameter concepts rather than the present wing loading concept. Comments were received objecting to the proposed alteration of the gust load criteria in current Sec. 23.341 because the new gust formula raises the load factors, thereby placing an undue burden on design and the mass parameter method results in unnecessary increased loads and load factors. However, the FAA considers that the use of the proposed gust formulae is essential in order to more realistically reflect the effects of higher speeds and higher altitudes on gust loadings. Transport airplane data was used in deriving the new formulae since the gust intensities involved are equally applicable to any airplanes designed to operate in the same speed and altitude ranges as present transport category airplanes. At the same time, the new formulae do not impose an undue burden on the design of slow, low altitude airplanes because their designs are usually maneuver critical. For purposes of clarity the format of the proposal has been changed to follow the current format of Part 23, and the solid line representing the limit combined envelope in the gust envelope diagram in Sec. 23.333(d) has been revised to correctly reflect the formulae for determining the gust envelope.

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（2）：说明。目前的阵风载荷验证要求、规定的阵风量值以及相关的阵风载荷因数公式均基于飞机翼载概念，且未考虑高度对阵风载荷的影响。本提案下的若干变更将用基于飞机质量参数概念的阵风研究替代目前的翼载概念。在第
然而，当将拟议的值代入拟议的公式时，得出的载荷与目前基于翼载荷推导的标准下的载荷相似。

英文原文：

Explanation. The present gust load substantiation requirements, prescribed gusts magnitudes, and associated gust load factor formulae are based on airplane wing loading concepts, and do not account for the effect of altitude on gust loads. The several changes under this proposal would substitute gust investigation based on airplane mass parameter concepts in place of the present wing loading concepts. Experience with similar mass parameter derived requirements in Part 25 has shown these requirements to be more accurate than the present requirements at higher altitudes and more representative of actual gust loads sustained at low altitudes. The changes to the gust envelope in Sec. 23.333 (d) are proposed in order to represent graphically the variation of investigated gust velocities with altitude that is assumed under the mass approach. Several changes in prescribed values are proposed, such as the increased gust velocity from 15 to 25 fps in change No. 3. These changes appear to be large changes in substance. However, when the proposed values are placed in the proposed formulae, the resultant burdens are similar to those under the present wing loading derived standards.

修订指令英文原文（供核对）：

17, Section 23.333 is amended by amending paragraphs (c) and (d) to read as follows:

1. Section 23.333 would be amended by striking out paragraph (c), by changing the heading of paragraph (d) to "Gust envelope", and by amending the envelope of paragraph (d) as follows, consistent with the gust envelope in Sec. 25.333 (c) of Part 25:

▪ 本版重印条文段落：(c)、(i)、(ii)、(2)、(i)、(ii)、(d)

### 2.3　Amdt 23-34（1987-02-17 生效）：修订第23.333条

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 修订指令（中文译文）：20. 修订第23.333条，在(b)(3)段中“normal”一词后插入“and commuter”；新增(c)(1)(iii)段；并修订(d)段中的图表，增加额外的通勤类阵风参数，具体如下：

来源：Final Rule｜52 FR 1806｜FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt

▪ 背景与评论处置（1）：提案19——未收到对该提案的评论。因此，§23.333 条按提案修订。

英文原文：

Proposal 19 -- No comments were received to the proposal. Accordingly, Sec. 23.333 is amended as proposed.

修订指令英文原文（供核对）：

20. By amending Sec. 23.333 by inserting the words "and commuter" after the word "normal" in paragraph (b)(3); by adding a new paragraph (c)(1)(iii); and by revising the diagram in paragraph (d) to add additional commuter category gust parameters as follows:

## 3　§23.333 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | General. Compliance with the strength requirements of this subpart must be shown at any combination of airspeed and load factor on and within the boundaries of a flight envelope (similar to the one in paragraph (d) of this section) that represents the envelope of the flight loading conditions specified by the maneuvering and gust criteria of paragraphs (b) and (c) of this section respectively. | 新增 |
| (b) | Maneuvering envelope. Except where limited by maximum (static) lift coefficients, the airplane is assumed to be subjected to symmetrical maneuvers resulting in the following limit load factors: | 新增 |
| (1) | The positive maneuvering load factor specified in Sec. 23.337 at speeds up to VD; | 新增 |
| (2) | The negative maneuvering load factor specified in Sec. 23.337 at VC; and | 新增 |
| (3) | Factors varying linearly with speed from the specified value at VC to 0.0 at VD for the normal category, and -1.0 at VD for the acrobatic and utility categories. | 新增 |
| (c) | Gust envelope. Limit gust loads are the loads that would result when the airplane encounters the following symmetrical vertical gusts (assuming that gust load factors vary linearly between VC and VD): | 新增 |
| (1) | Positive (up) and negative (down) gusts of 30 feet per second nominal intensity at speeds up to VC. | 新增 |
| (2) | Positive and negative gusts of 15 feet per second at VD. | 新增 |
| (d) | Flight envelope. Note: Point G need not be investigated when the supplementary condition specified in Sec. 23.369 is investigated. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订：(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | Gust envelope. (1) The airplane is assumed to be subjected to symmetrical vertical gusts in level flight. The resulting limit load factors must correspond to the conditions determined as follows: | 修订 |
| (i) | Positive (up) and negative (down) gusts of 50 f.p.s. at VC must be considered at altitudes between sea level and 20,000 feet. The gust velocity may be reduced linearly from 50 f.p.s. at 20,000 feet to 25 f.p.s. at 50,000 feet. | 新增 |
| (ii) | Positive and negative gusts of 25 f.p.s. at VS must be considered at altitudes between sea level and 20,000 feet. The gust velocity may be reduced linearly from 25 f.p.s. at 20,000 feet to 12.5 f.p.s. at 50,000 feet. | 新增 |
| (2) | The following assumptions must be made: | 修订 |
| (i) | The shape of the gust is - =Distance penetrated into gust (ft.); =Mean geometric chord of wing (ft) and =Derived gust velocity referred to in subparagraph 1 of this section. | 新增 |
| (ii) | Gust load factors vary linearly with speed between and | 新增 |
| (d) | Flight envelope. NOTE: Point G need not be investigated when the supplementary condition specified in Sec. 23.369 is investigated. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 3 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (b) | ● 新增 | ○ 无 |
| (1) | ● 新增 | ○ 无 |
| (2) | ● 新增 | △ 修订 |
| (3) | ● 新增 | ○ 无 |
| (c) | ● 新增 | △ 修订 |
| (d) | ● 新增 | △ 修订 |
| (i) | ○ 无 | ● 新增 |
| (ii) | ○ 无 | ● 新增 |

## 4　§23.333 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | .341, 23.345, 23.425, 23.443, and Appendix A23.3 to require investigation of gust loads based on airplane mass parameter concepts rather than the present wing loading concept. Comments were received objecting to the prop… | Amdt 23-7 |

本条取证提示 共 3 个历史版本：初始版, Amdt 23-7, Amdt 23-34。 其中 2 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [6] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [7] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [8] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [9] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.333 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 1a50de54-f2e3-424d-938f-3e394d252c2d |
| Amdt 23-7 | 1969-09-14 | 3ddcba85-d624-464a-aca2-1b2e574df055 |
| Amdt 23-34 | 1987-02-17 | cccd9819-739a-4c6f-b57b-debc9fad843a |

# § 23.335　设计空速（Design airspeeds.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 5 个历史版本

## 1　§23.335 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.335 |
| 条款标题（英文原文） | Design airspeeds. |
| 中文名称 | 设计空速 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 5 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 52 FR 34745 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.335 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(a)、(2)、(1) |
| 3 | Amdt 23-16 | 1975-02-14 | Notice 74-33，1974-10-03 签发，39 FR 36595 | Docket 14052，1974-12-31 签发，40 FR 2576 | 修订 |
| 4 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 新增段落：(a)、(d to read as follows: Sec. 23.335 Design airspeeds. * * * * * (a) |
| 5 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增段落：(a)、(b)、(d) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(a)、(2)、(1)　→　1975 23-16　修订　→　1987 23-34　新增段落：(a)、(d to read as　→　1996 23-48　新增段落：(a)、(b)、(d)

## 2　§23.335 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：设计空速

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：修订第23.335条

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：18. 修订第23.335条，将其引导句和(a)、(b)段修改为如下内容：　2. 将新增§23.335(a)(3)条，内容如下：　3. 将修改第 23.335 条(b)(1)段，删除词语“带有所需最小设计巡航速度”。并将第 23.335 条(b)(1)(i)至(iii)段修改，将“”改为“”。　2. §23.629(a)(1) 将被重新指定为 (a) 段，并将通过在其末尾增加以下文字进行修订：“，包括直至按 §23.335(b)(1) 所选定的，或按 §23.335(b) 所选定值的 1.2 倍的所有速度。”　新增第23.335条第(b)(3)段，内容如下：

来源：Final Rule｜34 FR 13078｜FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt

▪ 背景与评论处置（1）：该通告提议修订 §23.253、§23.335、§23.1505 和 §23.1545 条的现行设计空速要求，以便为高性能飞机设计速度的选取提供更合理的依据，并纳入关于受压缩性限制的设计速度的规定。一条评论认为，拟议的新 §23.253(a) 条未要求在配平系统失控期间演示速度裕度和机动性，并建议增加此项要求。对 §23.253(a) 条的拟议修改仅限于非故意的速度增加，提案中列举了导致此类速度增加的情况示例。俯仰配平系统的故障和失控属于提案中所列情况及示例的一般性表述范围，美国联邦航空局（FAA）认为没有必要特别提及这些情况。对于拟议的设计空速要求存在反对意见，因为其删除了现行 §23.335(b) 条中的引用。
该通告
拟议的新 §23.1505(c) 段也已作出修改，将涡轮发动机飞机排除在现行 §23.1505(a) 段和 (b) 段的要求之外，从而要求此类飞机符合 §23.1505(c) 段。这一修改是必要的，以便适应因保留 §23.335(b) 段而对提案所作的修改。对提案的这些修改实现了通告中对高性能飞机所表明的意图，且不影响希望符合现行设计空速要求的申请人的设计空速要求。通告提议修订 §23.333、§23.341、§23.345、§23.425、§23.443 以及附录 A23.3，要求基于飞机质量参数概念（而非现行翼载荷概念）进行阵风载荷研究。已收到的评论反对对现行 §23.341 段中阵风载荷准则的拟议修改，因为新的阵风公式提高了载荷因数，从而给设计带来不必要的负担，而且质量参数方法导致载荷和载荷因数不必要的增加。
然而，美国联邦航空局认为，采用所提议的阵风公式对于更真实地反映更高速度和更高高度对阵风载荷的影响至关重要。推导新公式时采用了运输类飞机的数据，因为所涉及的阵风强度同样适用于任何设计在与当前运输类飞机相同速度和高度范围运行的飞机。与此同时，新公式不会对低速、低空飞机的设计造成不合理的负担，因为这些飞机的设计通常以机动载荷为临界。为清晰起见，提案的格式已作更改，以遵循第23部的现行格式；§23.333(d) 段阵风包线图中表示极限组合包线的实线也已修订，以正确反映确定阵风包线的公式。

英文原文：

The notice proposed to amend the current design airspeed requirements of Secs. 23.253, 23.335, 23.1505, and 23.1545 to provide a more rational basis for selecting design speeds for high performance airplanes, and to include provisions for design speeds limited by compressibility. One comment asserted that the proposed new Sec. 23.253(a) does not require demonstration of speed margin and maneuverability during trim system runaway, and suggested the addition of such a requirement. The proposed change to Sec. 23.253(a) is confined to inadvertent speed increases and examples of the conditions leading to such increases are set forth in the proposal. Malfunctions and runaways of pitch trimming systems come within the general statement of the conditions and examples set forth in the proposal and the FAA does not believe that it is necessary to specifically refer to these conditions. There were objectives to the proposed design airspeed requirement insofar as it deleted the reference to in present Sec. 23.335(b). The notice proposed in part that be not less than 1.4 where is the selected design cruise speed. The present requirement merely requires a not less than 1.4 where is specified in a formula, and in many cases the applicant selects a higher to accommodate the performance of the airplane without a restrictive operating limitation under Sec. 23.1505. In view of comments received, the FAA has determined that the proposal is unnecessarily severe and the present factors based on are being retained. However, to make the design speed provisions in Sec. 23.335 consistent with the operating limitations in Sec. 23.1505 (a) and (b), a provision has been added to Sec. 23.335(b)(1) specifying that may not be less than 1.25 . However, as an alternative, the applicant may use the "rational" speed margin in paragraph (b)(4). In addition, the proposed lead-in sentence of Sec. 23.335 has been editorially revised to reflect the fact that speed and Mach margins are not the same, and subdivision (b) (4) (ii) has been changed to make it clear that the cutoff is only intended for the Mach margin. Proposed new Sec. 23.1505(c) has also been changed to exclude turbine airplanes from the requirements of present Sec. 23.1505(a) and (b) and thus to require those airplanes to comply with Sec. 23.1505(c). This change is necessary in order to accommodate the changes to the proposal necessary because of the retention of in Sec. 23.335(b). These changes to the proposal accomplish the intent set forth in the notice for high performance aircraft without affecting the current design airspeed requirements for applicants who wish to comply with them.

The notice proposed to amend Secs. 23.333, 23.341, 23.345, 23.425, 23.443, and Appendix A23.3 to require investigation of gust loads based on airplane mass parameter concepts rather than the present wing loading concept. Comments were received objecting to the proposed alteration of the gust load criteria in current Sec. 23.341 because the new gust formula raises the load factors, thereby placing an undue burden on design and the mass parameter method results in unnecessary increased loads and load factors. However, the FAA considers that the use of the proposed gust formulae is essential in order to more realistically reflect the effects of higher speeds and higher altitudes on gust loadings. Transport airplane data was used in deriving the new formulae since the gust intensities involved are equally applicable to any airplanes designed to operate in the same speed and altitude ranges as present transport category airplanes. At the same time, the new formulae do not impose an undue burden on the design of slow, low altitude airplanes because their designs are usually maneuver critical. For purposes of clarity the format of the proposal has been changed to follow the current format of Part 23, and the solid line representing the limit combined envelope in the gust envelope diagram in Sec. 23.333(d) has been revised to correctly reflect the formulae for determining the gust envelope.

来源：Final Rule｜34 FR 13078｜FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt

▪ 背景与评论处置（2）：该通告
针对另一条评论
收到了反对该提案的评论，理由是它超出了运输类飞机的第 25 部要求，并且它会不当且不利地改变配平响应，使符合性试验极其危险。美国联邦航空局（FAA）不同意。配平要求仅仅是必须作为总体要求来考虑的若干控制系统要求之一。因此，在第 23 部和第 25 部的各个条款之间没有可比性。现有飞机的使用经验并未表明该提案会不当且不利地改变配平响应，或者符合性试验将是危险的。此外，为了符合该提案，并非必然需要等同于主飞行控制响应的配平响应。所要求的全部是具备足够的控制以实现安全飞行和着陆。因此，该修订按提案予以采纳。

英文原文：

The notice proposed to amend Sec. 23.629(a) to eliminate redundancies and to insure an adequate speed margin for flutter when a more rational speed spread between and is selected in place of the current arbitrary speed spread of Sec. 23.335(b)(1). One comment suggested that the flutter margin should be defined in terms of all of the flutter parameters rather than airspeed alone. However, since speed is the parameter which is used in establishing the operating limitations of the airplane, it is considered appropriate to express the required flutter margin in terms of speed. The FAA has determined that the proposed freedom from flutter speed for some designs could be unnecessarily severe. Therefore, the proposed amendment has been changed to provide for those cases where is the minimum value specified in current Sec. 23.335(a)(1) where is related to under Sec. 23.335(b)(2). This change also provides an adequate margin between and the flutter-proof speeds in those cases where intermediate values of are selected by the applicant. In response to another comment, a new paragraph (b) has been added to provide for a smaller speed margin when adequate damping is shown at . The notice also proposed to include in a new Sec. 23.629(d) additional flutter requirements applicable to turbopropeller powered airplanes. One comment suggested that the amendment should also apply to reciprocating engine installations, and that the "pivot turn ground case" should be accounted for. The FAA does not agree. The service experience of reciprocating engine installations does not show a safety need for the suggested applicability. The consideration of gyroscopic couple under the ground taxi turn is not a critical design case and is not needed, since the load proposed in the notice combines the gyroscopic couple with propeller thrust and limit maneuver flight loads.

The notice proposed to amend Sec. 23.677 to make it clear that operation of the trim system may not be dependent upon the primary control system and that failures of the primary control system may not prevent safe flight and landing. Comments were received objecting to the proposal on the ground that it exceeded the Part 25 requirements for transport category airplanes, and that it would improperly and undesirably change trim response and make compliance tests extremely hazardous. The FAA does not agree. The trim requirement is merely one of a number of control system requirements that must be considered as a total requirement. There is, therefore, no comparison between individual provisions in Parts 23 and 25. Service experience with existing airplanes does not indicate that the proposal will improperly and undesirably change trim response or that compliance testing will be hazardous. Furthermore, trim response equal to primary flight control response will not necessarily be needed to comply with the proposal. All that is required is that there be adequate control for safe flight and landing. The amendment is therefore adopted as proposed.

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（3）：提案 4. 设计巡航速度的确定（§23.335 条）。§23.335 条 (a) (2) 将被修订，(a) (3) 将被增加，且 (b) (2) 将被修订，修订后内容如下：

英文原文：

Proposal 4. Determination of design cruising speed (Sec. 23.335). Section 23.335 (a) (2) would be amended, (a) (3) added, and (b) (2) amended to read as follows:

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（4）：解释。拟议两项修改。第一项，修改规则以明确，确定设计巡航速度的倍乘系数的特定线性递减系允许但非强制。此项修改反映了该规则的长期既定意图。第二项，将设计巡航速度限制在 0.9 的许可扩展至所有飞机。对于翼载荷为 20 或更低，以及翼载荷大于 20 的飞机，若要求设计巡航速度仅依据翼载荷和§23.335(a)(1) 段规定的倍乘系数来确定，可能造成不必要的负担。由于该速度是额定转速和功率下的平飞最大速度，若设计巡航速度不必超过 0.9，安全性不会受到影响。

英文原文：

Explanation. Two changes are proposed. First, the rule would be changed to make it clear that the specified linear decreases in the multiplying factors determining design cruising speed are allowed but not required. This change reflects long standing intent with respect to the rule. Second, permission to limit design cruising speed to 0.9 would be extended to all airplanes. For airplanes with wing loadings of 20 or less, as well as those with wing loadings greater than 20, an unnecessary burden may result from requiring that the design cruising speed be determined solely with reference to wing loading and the multiplying factors specified in Sec. 23.335 (a) (1). Since is the maximum speed in level flight with rated r.p.m. and power, safety will not be compromised if the design cruising speed is not required to exceed 0.9

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（5）：说明。现行 §23.629(a)（第一句）与 (a)(1) 部分重复。拟议的合并将消除这一重复。对现行 §23.629(a)(1) 的拟议修改是必要的，以便在 与 之间选用更合理的速度范围来取代 §23.335(b)(1) 的任意速度范围时，确保足够的颤振速度裕度。

英文原文：

Explanation. Present Sec. 23.629 (a) (first sentence) is partially redundant with (a) (1). The proposed consolidation would eliminate this redundancy. The proposed change to current Sec. 23.629 (a) (1) is necessary in order to insure an adequate speed margin for flutter when a more rational speed spread between and is selected in place of the arbitrary speed spread of Sec. 23.335 (b) (1).

修订指令英文原文（供核对）：

18. Section 23.335 is amended by amending the lead-in sentence and paragraphs (a) and (b) to read as follows:

2. A new Sec. 23.335 (a) (3) would be added to read as follows:

3. Section 23.335 (b) (1) would be amended by deleting the words "with the required minimum design cruising speed". and Secs. 23.335 (b) (1) (i) through (iii) would be amended by changing "" to "".

2. Section 23.629 (a) (1) would be redesignated as (a) and would be amended by adding at the end thereof the words ", including all speeds up to selected under Sec. 23.335 (b) (1) or 1.2 times the value of selected under Sec. 23.335 (b)

4 A new Sec. 23.335 (b) (3) would be added to read as follows:

▪ 本版重印条文段落：(a)、(1)、(i)、(ii)、(2)、(3)、(4)、(b)、(1)、(2)、(i)、(ii)、(iii)、(3)、(4)、(i)、(ii)

### 2.3　Amdt 23-16（1975-02-14 生效）：修订§23.335(c)(1)

▪ 修订沿革：本版由 NPRM　Notice 74-33，1974-10-03 签发，39 FR 36595　提出；最终规章 Docket 14052，1974-12-31 签发，40 FR 2576，1975-02-14 生效。

▪ 修订指令（中文译文）：5. 修订§23.335(c)(1)，删去括号中的表述“（以英里每小时为单位）”。

来源：NPRM（提案）｜39 FR 36595｜NPRM_Notice 74-33_Docket14052_39FR36595_1974-10-11.txt

▪ 背景与评论处置（1）：1969年8月13日，第23-7号修正案《小型飞机型号合格审定要求》在《联邦公报》上公布（34 FR 13078）。经通过的第23-7号修正案，将第23部中所有使用"英里"和"英里/小时"之处分别改为"海里"和"节"。本通告拟对第23.335条和第23部附录A作出的修正案，将纳入在通过第23-7号修正案时被无意遗漏的澄清性修订。

英文原文：

On August 13, 1969, Amendment 23-7, Small Airplane Type Certification Requirements, was published in the FEDERAL REGISTER (34 FR 13078). Amendment 23-7, as adopted, changed all references to "miles" and "miles per hour" to "nautical miles" and "knots", respectively wherever used in Part 23. The amendments herein proposed to Sec. 23.335 and Appendix A to Part 23 would incorporate clarifying revisions that were inadvertently omitted when Amendment 23-7 was adopted.

修订指令英文原文（供核对）：

5. By amending Sec. 23.335(c)(1) by striking the parenthetical expression "(in miles per hour)".

### 2.4　Amdt 23-34（1987-02-17 生效）：通过修订§23.335

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 修订指令（中文译文）：21. 通过修订§23.335，修改(a)(1)(i)段和(b)(2)(i)段；并新增一段 (d，内容如下：　20. 通过修订第23.335条，将(a)(1)(i)段中的第一个“和”替换为逗号，并在“实用”一词后插入“，及通勤”；在(b)(2)(i)段中的“正常”一词后插入“及通勤”；并新增(d)段，内容如下：

来源：Final Rule｜52 FR 1806｜FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt

▪ 背景与评论处置（1）：提案 20——未收到对该提案的评论。因此，第 23.335 条按提案进行修订。

英文原文：

Proposal 20 -- No comments were received to the proposal. Accordingly, Sec. 23.335 is amended as proposed.

修订指令英文原文（供核对）：

21. By amending Sec. 23.335 by revising paragraphs (a)(1)(i) and (b)(2)(i); and by adding a new paragraph (d to read as follows:

20. By amending Sec. 23.335 by replacing the first "and" with a "," and inserting the words ", and commuter" after the word "utility" in paragraph (a)(1)(i); by inserting the words "and commuter" after the word "normal" in paragraph (b)(2)(i); and by adding a new paragraph (d) to as follows:

▪ 本版重印条文段落：(a)、(b)、(d)

### 2.5　Amdt 23-48（1996-03-11 生效）：第 23.335 条修订如下：修订(a)(1)段；删除(b)(4)(

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：第 23.335 条修订如下：修订(a)(1)段；删除(b)(4)(i)段末尾的句号，改为"; and either--"；修订(b)(4)(ii)段；新增(b)(4)(iii)段；修订(d)(1)段的引言文字如下：　3. 对第 23.335 条进行修订，修改(a)(1)段引导语、(a)(1)(i)段和(a)(1)(ii)段；删除(b)(4)(i)段末尾的句号，并添加"; and either"；修改(b)(4)(ii)段；新增(b)(4)(iii)段；修改(d)(1)段引导语，内容如下：　§23.335(b)(4) 段将通过增加新的 (b)(4)(iii) 段进行修订，该段包含针对通勤类飞机的新马赫数速度裕度 0.07M。由于通勤类飞机通常在比正常类、实用类和特技类飞机更高的高度运行，它们会遇到更大的大气变化，例如水平阵风以及穿越急流或冷锋。因此，需要更高的最小速度裕度。欧洲联合航空要求（JAR）提议增加这一马赫数速度裕度。原有的 0.05M 马赫数速度裕度保留用于正常类、实用类和特技类飞机。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订第23.335条的某些部分，以澄清并与JAR 23协调统一。美国联邦航空局提议修订(a)(1)段，增加短语“设计最大起飞重量下的机翼载荷”作为W/S的定义，并修订(a)(1)(i)和(a)(1)(ii)段，将设计巡航速度的方程从“33 W/S”更正为

英文原文：

The FAA proposed to revise portions of Sec. 23.335 for clarification and harmonization with JAR 23. The FAA proposed to revise paragraph (a)(1) by adding the phrase "wing loading at the design maximum takeoff weight" as a definition for W/S and by revising paragraphs (a)(1)(i) and (ii) to correct the equations for design cruise speed from "33 W/S" to

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局提议修订§23.335(b)(4)段，新增(b)(4)(iii)段，其中包含通勤类飞机的新马赫数速度裕度0.07M。由于通勤类飞机通常比正常类、实用类和特技类飞机在更高高度运行，因此会遇到更大的大气变化，例如水平阵风以及穿越急流或冷锋；因此，需要更高的最小速度裕度。欧洲联合航空要求（JAR）提议增加此马赫数速度裕度。正常类、实用类和特技类飞机将保留原有的0.05M马赫数速度裕度。

英文原文：

The FAA proposed to revise Sec. 23.335(b)(4) by adding a new paragraph (b)(4)(iii) that includes a new mach number speed margin, 0.07M, for commuter category airplanes. Because commuter category airplanes are normally operated at higher altitudes than normal, utility, and acrobatic category airplanes, they experience greater atmospheric variations, such as horizontal gusts and the penetration of jet streams or cold fronts; therefore, a higher minimum speed margin is required. The JAR proposed adding this mach number speed margin. The original mach number speed margin of 0.05M would be retained for normal, utility, and acrobatic category airplanes.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（3）：第 23.335 条的部分内容将进行修订，以澄清并与 JAR 23 协调统一。(a)(1) 段将修订，增加 W/S 的定义，即"设计最大起飞重量下的翼载荷"。(a)(1)(i) 和 (ii) 段将修订，将设计巡航速度的公式从"33 W/S"更正为"33 (W/S)"，从"36 W/S"更正为"36 (W/S)"。

英文原文：

Portions of Sec. 23.335 would be revised for clarification and harmonization with JAR 23. Paragraph (a)(1) would be revised by adding a definition for W/S as "wing loading at the design maximum takeoff weight." Paragraph (a)(1)(i) and (ii) would be revised to correct the equations for design cruise speed from "33 W/S" to "33 (W/S)" and from "36 W/S" to "36 (W/S). "

修订指令英文原文（供核对）：

3. Section 23.335 is amended by revising paragraph (a)(1); by removing the period and adding "; and either--" to the end of paragraph (b)(4)(i); by revising paragraph (b)(4)(ii); by adding a new paragraph (b)(4)(iii); and by revising the introductory text of paragraph (d)(1) to read as follows:

3. Section 23.335 is amended by revising paragraphs (a)(1) introductory text, (a)(1)(i), and (a)(1)(ii); by removing the period and adding"; and either" to the end of paragraph (b)(4)(i); by revising paragraph (b)(4)(ii); by adding a new paragraph (b)(4)(iii); and by revising the introductory text of paragraph (d)(1) to read as follows:

Section 23.335(b)(4) would be revised by adding a new paragraph (b)(4)(iii) that includes a new mach number speed margin, 0.07M, for commuter category airplanes. Because commuter category airplanes are normally operated at higher altitudes then normal, utility, and acrobatic category airplanes, they experience greater atmospheric variations, such as horizontal gusts and the penetration of jet streams or cold fronts. Therefore, a higher minimum speed margin is required. The JAR proposed adding this mach number speed margin. The original mach number speed margin of 0.05M is retained for normal, utility, and acrobatic category airplanes.

▪ 本版重印条文段落：(a)、(1)、(i)、(ii)、(b)、(4)、(ii)、(iii)、(d)、(1)

### 2.6　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.335 的实际改动 |
| --- | --- | --- | --- |
| 52 FR 34745 | 1987-09-14 | 更正通告 (CFR 溯源注) | 18 项印刷更正；其中第 9 项新增修订指令 25-1：§23.561(b)(2) 表格第一栏标题 'Normal and utility categories'→'Normal, utility, and commuter categories'。⚠ 本件 FR 抬头把 Amdt Nos. 误印为 23-24（正文通篇写 23-34），CFR 溯源注因此跟着写错 —— DRS 归 Amdt 23-34 是对的 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.335 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Design cruising speed, VC. For VC, the following apply: | 新增 |
| (1) | VC (in miles per hour) may not be less than-- | 新增 |
| (i) | 38 (for normal and utility category airplanes); and | 新增 |
| (ii) | 42 (for acrobatic category airplanes). | 新增 |
| (2) | For values of W/S more than 20, the numerical multiplying factors must be decreased linearly with W/S to a value of 33 where W/S =100. The required minimum value need not be more than 0.9 VH obtained at sea level. | 新增 |
| (b) | Design dive speed, VD. For VD, the following apply: | 新增 |
| (1) | With VC min the required minimum design cruising speed, VD (in miles per hour) may not be less than-- | 新增 |
| (i) | 1.40 VC min (for normal category airplanes); | 新增 |
| (ii) | 1.50 VC min (for utility category airplanes); and | 新增 |
| (iii) | 1.55 VC min (for acrobatic category airplanes). | 新增 |
| (2) | For values of W/S more than 20, the numerical multiplying factors must be decreased linearly with W/S to a value of 1.35 at W/S = 100. | 新增 |
| (c) | Design maneuvering speed VA. For VA, the following applies: | 新增 |
| (1) | VA (in miles per hour) may not be less than VS where-- | 新增 |
| (i) | VS is a computed stalling speed with flaps retracted at the design weight, normally based on the maximum airplane normal force coefficients, CNA; and | 新增 |
| (ii) | n is the limit maneuvering load factor used in design. | 新增 |
| (2) | The value of VA need not exceed the value of VC used in design. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订：(a)、(2)、(1)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Design cruising speed,. For the following apply: | 修订 |
| (1) | (in knots) may not be less than - | 修订 |
| (i) | (for normal and utility category airplanes); and ____ | 修订 |
| (ii) | (for acrobatic category airplanes). | 修订 |
| (2) | For values of W/S more than 20, the multiplying factors may be decreased linearly with W/S to a value of 28.6 where W/S=100. | 修订 |
| (3) | need not be more than 0.9 at sea level. | 新增 |
| (4) | At altitudes where anis established, a cruising speedlimited by compressibility may be selected. | 新增 |
| (b) | Design dive speed. For , the following apply: | 修订 |
| (1) | may not be less than 1.25 ; and | 修订 |
| (2) | With , the required minimum design cruising speed (in knots) may not be less than - | 修订 |
| (i) | 1.40 (for normal category airplanes); | 修订 |
| (ii) | 1.50 (for utility category airplanes); and | 修订 |
| (iii) | 1.55 (for acrobatic category airplanes). | 修订 |
| (3) | For values of W/S more than 20, the multiplying factors in subparagraph (2) of this paragraph may be decreased linearly with W/S to a value of 1.35 where W/S=100. | 新增 |
| (4) | Compliance with subparagraphs (1) and (2) of this paragraph need not be shown if is selected so that the minimum speed margin between and is the greater of the following: | 新增 |
| (i) | The speed increase resulting when, from the initial condition of stabilized flight at , the airplane is assumed to be upset, flown for 20 seconds along a flight path 7.5° below the initial path, and then pulled up with a load factor of 1.5 (0.5 g. acceleration increment). At least 75 percent maximum continuous power for reciprocating engines, and maximum cruising power for turbines, or, if less, the power required for for both kinds of engines, must be assumed until the pull-up is initiated, at which point power reduction and pilot-controlled drag devices may be used. | 修订 |
| (ii) | Mach 0.05 (at altitudes where an is established). | 修订 |

### 3.3　版本3：Amdt 23-34（1987-02-17 生效）　来源：Docket 23516，1987-01-08 签发，52 FR 1806

新增段落：(a)、(d to read as follows: Sec. 23.335 Design airspeeds. * * * * * (a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * (1) * * * (i) 33 W/S (for normal, utility, and commuter category airplanes); and | 修订 |
| (b) | * * * (2) * * * (i) 1.40 Vc min (for normal and commuter category airplanes); | 修订 |
| (d) | Design speed for maximum gust intensity, VB. For VB, the following apply: (1) VB may not be less than the speed determined by the intersection of the line representing the maximum positive lift CN MAR and the line representing the rough air gust velocity on the gust V-n diagram, or V (Ny) VS1, whichever is less, where: (i) Ng the positive airplane gust load factor due to gust, at speed VC, (in accordance with Sec. 23.341), and at the particular weight under consideration; and (ii) VS1 is the stalling speed with the flaps retracted at the particular weight under consideration. (2) VB need not be greater than VC. | 新增 |

### 3.4　版本4：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

新增段落：(a)、(b)、(d)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (1) | Where W/S'=wing loading at the design maximum takeoff weight, V c (in knots) may not be less than-- | 新增 |
| (i) | 33 (W/S) (for normal, utility, and commuter category airplanes); | 新增 |
| (ii) | 36 (W/S) (for acrobatic category airplanes). | 新增 |
| (b) | * * * | 修订 |
| (4) | * * * | 新增 |
| (ii) | Mach 0.05 for normal, utility, and acrobatic category airplanes (at altitudes where M D is established); or | 新增 |
| (iii) | Mach 0.07 for commuter category airplanes (at altitudes where M D is established) unless a rational analysis, including the effects of automatic systems, is used to determine a lower margin. If a rational analysis is used, the minimum speed margin must be enough to provide for atmospheric variations (such as horizontal gusts), and the penetration of jet streams or cold fronts), instrument errors, airframe production variations, and must not be less than Mach 0.05. | 新增 |
| (d) | * * * | 修订 |
| (1) | V B may not be less than the speed determined by the intersection of the line representing the maximum positive lift, C N MAX, and the line representing the rough air gust velocity on the gust V-n diagram, or V S1 n g, whichever is less, where: | 新增 |

### 3.5　段落结构演变矩阵（横向对照 5 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) | Amdt 23-34 (1987-02-17) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | △ 修订 | △ 修订 |
| (1) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (i) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (ii) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (2) | ● 新增 | △ 修订 | ○ 无 | ○ 无 |
| (b) | ● 新增 | △ 修订 | △ 修订 | △ 修订 |
| (iii) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (c) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (3) | ○ 无 | ● 新增 | ○ 无 | ○ 无 |
| (4) | ○ 无 | ● 新增 | ○ 无 | ● 新增 |
| (d) | ○ 无 | ○ 无 | ● 新增 | △ 修订 |

## 4　§23.335 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | .341, 23.345, 23.425, 23.443, and Appendix A23.3 to require investigation of gust loads based on airplane mass parameter concepts rather than the present wing loading concept. Comments were received objecting to the prop… | Amdt 23-7 |
| 澄清性与编辑性修订 | to Sec. 23.335 and Appendix A to Part 23 would incorporate clarifying revisions that were inadvertently omitted when Amendment 23-7 was adopted. 5. By amending Sec. 23.335(c)(1) by striking the parenthetical expression… | Amdt 23-16 |
| 国际协调 | sed to revise portions of Sec. 23.335 for clarification and harmonization with JAR 23. The FAA proposed to revise paragraph (a)(1) by adding the phrase "wing loading at the design maximum takeoff weight" as a definition… | Amdt 23-48 |

本条取证提示 共 5 个历史版本：初始版, Amdt 23-7, Amdt 23-16, Amdt 23-34, Amdt 23-48。 其中 4 个版本在本地文献中检索到明确修订指令原文；4 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：52 FR 34745，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 14052 | 40 FR 2576 | FinalRule_Docket 14052_Amdt21-43_23-16_25-37_40FR2576_1975-01-14_0002.txt |
| [6] | NPRM（提案） | Notice 74-33 / Docket 14052 | 39 FR 36595 | NPRM_Notice 74-33_Docket14052_39FR36595_1974-10-11.txt |
| [7] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [8] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [9] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [10] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [11] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [12] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [13] | 更正通告 / 校正 | 52 FR 34745 | 1987-09-14 | Correction_52FR34745_1987-09-14.clean.txt |
| [14] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [15] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.335 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 08ddb98d-cc9a-41c2-aef7-37a156c93015 |
| Amdt 23-7 | 1969-09-14 | 4f4b954a-7005-4a22-a5bb-704f455358ac |
| Amdt 23-16 | 1975-02-14 | 961a9316-4a09-44be-bf19-d4cdc850569a |
| Amdt 23-34 | 1987-02-17 | b9b54fca-9849-4c8d-bb20-6859a67b5225 |
| Amdt 23-48 | 1996-03-11 | 04e941b9-1f28-47e1-9488-fc4cc885dafb |

# § 23.337　机动限制载荷因数（Limit maneuvering load factors.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 4 个历史版本

## 1　§23.337 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.337 |
| 条款标题（英文原文） | Limit maneuvering load factors. |
| 中文名称 | 机动限制载荷因数 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.337 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(a)、(1) |
| 3 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 删除内容：(a)、(b) |
| 4 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(a)、(1)　→　1987 23-34　删除内容：(a)、(b)　→　1996 23-48　修订

## 2　§23.337 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：机动限制载荷因数

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：对第23.337条进行修订

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：19. 对第23.337条进行修订，修订(a)段，删除子段(1)中数字"3.8"之后的"且不得小于2.5"字样。

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案5. 正常类最小限制机动载荷因数（第23.337条）。第23.337条(a)(1)段将修订，删除数字"3.8"后面的文字。

解释。目前的最小限制机动载荷因数为2.5，低于根据第23部采用最大可能重量即12,500磅所能获得的最小值。由于该重量参数决定了在该公式下可能获得的最小限制机动载荷因数，因此无需规定最小值。

英文原文：

Proposal 5. Normal category minimum limit maneuvering load factor (Sec. 23.337). Section 23.337 (a) (1) would be amended by striking out the words following the number "3.8."

Explanation. The present minimum limit maneuvering load factor of 2.5 is below the minimum that can be obtained using the greatest possible weight under Part 23, namely 12,500 pounds. Since this weight parameter determines the minimum limit maneuvering load factor that can possibly be obtained under the formula, no minimum need be specified.

修订指令英文原文（供核对）：

19. Section 23.337 is amended by amending paragraph (a) by striking out the words "nor may it be less than 2.5" following the number "3.8" in subparagraph (1).

### 2.3　Amdt 23-34（1987-02-17 生效）：通过修订第23.337条

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 修订指令（中文译文）：通过修订第23.337条，在(a)(1)段中"category"一词前插入"and commuter"；并在(b)(1)段中删除"and"一词，在"categories"一词前插入", and commuter"。

来源：Final Rule｜52 FR 1806｜FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt

▪ 背景与评论处置（1）：提案21——未收到针对该提案的评论。因此，§23.337 条按提案予以修订。

英文原文：

Proposal 21 -- No comments were received to the proposal. Accordingly, Sec. 23.337 is amended as proposed.

修订指令英文原文（供核对）：

22. By amending Sec. 23.337, by inserting the words "and commuter" before the word "category" in paragraph (a)(1); and by removing the word "and" and by inserting ", and commuter" before the word "categories" in paragraph (b)(1).

### 2.4　Amdt 23-48（1996-03-11 生效）：第23.337条(a)(1)段修订如下

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：第23.337条(a)(1)段修订如下：　4. 修订第23.337条的(a)(1)段，内容如下：　将对第23.337条(a)(1)段进行修订，澄清公式并增加“W”的定义。该“W”（设计最大起飞重量）定义是应欧洲联合航空局（JAA）的要求提出的，以便与欧洲联合航空要求（JAR）第23部协调统一。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订 §23.337(a)(1) 段，通过澄清该等式并增加对“W”的定义。欧洲联合航空局（JAA）要求对“W”（设计最大起飞重量）作出这一定义，以与欧洲联合航空要求（JAR）第23部协调统一。

英文原文：

The FAA proposed to revise Sec. 23.337(a)(1) by clarifying the equation and by adding a definition for "W". This definition of "W," "design maximum takeoff weight," was requested by the JAA to harmonize with JAR 23.

修订指令英文原文（供核对）：

4. Section 23.337(a)(1) is revised to read as follows:

4. Section 23.337 is amended by revising paragraph (a)(1) to read as follows:

Section 23.337 (a)(1) would be revised by clarifying the equation and by adding a definition for "W." This definition of "W," design maximum takeoff weight," was requested by the JAA to harmonize with JAR 23.

▪ 本版重印条文段落：(a)、(1)

## 3　§23.337 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The positive limit maneuvering load factor n may not be less than-- | 新增 |
| (1) | 2.1+ for normal category airplanes, except that n need not be more than 3.8 nor may it be less than 2.5; | 新增 |
| (2) | 4.4 for utility category airplanes; or | 新增 |
| (3) | 6.0 for acrobatic category airplanes. | 新增 |
| (b) | The negative limit maneuvering load factor may not be less than-- | 新增 |
| (1) | 0.4 times the positive load factor for the normal and utility categories; or | 新增 |
| (2) | 0.5 times the positive load factor for the acrobatic category. | 新增 |
| (c) | Maneuvering load factors lower than those specified in this section may be used if the airplane has design features that make it impossible to exceed these values in flight. | 新增 |

### 3.2　版本2：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (1) | 2.1+(24,000 (W+10,000)) for normal and commuter category airplanes, where W=design maximum takeoff weight, except that n need not be more than 3.8; | 修订 |

### 3.3　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- |
| (a) | ● 新增 | △ 修订 |
| (1) | ● 新增 | △ 修订 |
| (2) | ● 新增 | ○ 无 |
| (3) | ● 新增 | ○ 无 |
| (b) | ● 新增 | ○ 无 |
| (c) | ● 新增 | ○ 无 |

## 4　§23.337 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | "W," "design maximum takeoff weight," was requested by the JAA to harmonize with JAR 23. 4. Section 23.337(a)(1) is revised to read as follows: Sec. 23.337 Limit maneuvering load factors. (a) * * * (1) 2.1+(24,000 (W+10… | Amdt 23-48 |

本条取证提示 共 4 个历史版本：初始版, Amdt 23-7, Amdt 23-34, Amdt 23-48。 其中 3 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [6] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [7] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [8] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [9] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [10] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [11] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [12] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.337 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 346e369d-29c7-412c-b48a-ae56ee30ecfe |
| Amdt 23-7 | 1969-09-14 | f4781a10-a15a-4365-a53c-89675ff7e68e |
| Amdt 23-34 | 1987-02-17 | fd02c81c-aee4-445f-81c1-f9bf6317b580 |
| Amdt 23-48 | 1996-03-11 | d7e22719-1ff5-44f2-92c0-13f9e26df9aa |

# § 23.341　阵风载荷因数（Gust load factors.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 5 个历史版本

## 1　§23.341 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.341 |
| 条款标题（英文原文） | Gust load factors. |
| 中文名称 | 阵风载荷因数 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 5 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.341 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订 |
| 3 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 条文未改动（本版为交叉引用或其他条款修订所致） |
| 4 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 新增段落：(b)、(a) |
| 5 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增段落：(a)、(b)、(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订　→　1987 23-34　条文未改动（本版为交叉引用或其他条款修订所致　→　1991 23-42　新增段落：(b)、(a)　→　1996 23-48　新增段落：(a)、(b)、(c)

## 2　§23.341 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：阵风载荷因数

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第23.341条修订如下

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：20. 第23.341条修订如下：　2. 第23.341条将修改如下：

来源：Final Rule｜34 FR 13078｜FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt

▪ 背景与评论处置（1）：本通告提议修订第23.333、23.341、23.345、23.425、23.443条及附录A23.3，要求基于飞机质量参数概念而非现行翼载荷概念进行阵风载荷研究。收到的评论反对更改现行第23.341条中的阵风载荷准则，因为新的阵风公式提高了载荷因数，从而给设计带来过重负担，且质量参数方法导致不必要的载荷和载荷因数增加。然而，美国联邦航空局认为，采用拟议的阵风公式对于更真实地反映更高速度和更高高度对阵风载荷的影响至关重要。推导新公式时使用了运输类飞机数据，因为所涉及的阵风强度同样适用于任何设计在与现行运输类飞机相同速度和高度范围内运行的飞机。
同时，新公式不会对低速、低空飞机的设计施加过度的负担，因为其设计通常机动起决定性作用。为清楚起见，提案的格式已作更改，以遵循第 23 部的现行格式，并且 §23.333(d) 段阵风包线图中代表限制组合包线的实线已被修订，以正确反映用于确定阵风包线的公式。

英文原文：

The notice proposed to amend Secs. 23.333, 23.341, 23.345, 23.425, 23.443, and Appendix A23.3 to require investigation of gust loads based on airplane mass parameter concepts rather than the present wing loading concept. Comments were received objecting to the proposed alteration of the gust load criteria in current Sec. 23.341 because the new gust formula raises the load factors, thereby placing an undue burden on design and the mass parameter method results in unnecessary increased loads and load factors. However, the FAA considers that the use of the proposed gust formulae is essential in order to more realistically reflect the effects of higher speeds and higher altitudes on gust loadings. Transport airplane data was used in deriving the new formulae since the gust intensities involved are equally applicable to any airplanes designed to operate in the same speed and altitude ranges as present transport category airplanes. At the same time, the new formulae do not impose an undue burden on the design of slow, low altitude airplanes because their designs are usually maneuver critical. For purposes of clarity the format of the proposal has been changed to follow the current format of Part 23, and the solid line representing the limit combined envelope in the gust envelope diagram in Sec. 23.333(d) has been revised to correctly reflect the formulae for determining the gust envelope.

修订指令英文原文（供核对）：

20. Section 23.341 is amended to read as follows:

2. Section 23.341 would be amended to read as follows:

▪ 本版重印条文段落：(a)、(1)、(2)、(b)、(1)、(2)、(c)

### 2.3　Amdt 23-34（1987-02-17 生效）：条文未改动（本版为交叉引用或其他条款修订所致）

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 本版对条文的处置：条文未改动（本版为交叉引用或其他条款修订所致）

### 2.4　Amdt 23-42（1991-02-04 生效）：第23.341条修订如下：将现有文本指定为(b)段；在新指定的(b)

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：11. 第23.341条修订如下：将现有文本指定为(b)段；在新指定的(b)段中，在“analysis”一词后增加“for conventional configurations”；并新增(a)段如下：　11. 对第 23.341 条进行修订，在现有试验中的“analysis”一词后增加“for conventional configurations”，并将经修订后的现有文本指定为(b)段；同时新增(a)段，内容如下：

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案 11. 本提案将在 §23.341 条中确立鸭翼或串列翼布局飞机必须满足的阵风载荷要求。

英文原文：

Proposal 11. This proposal would establish gust load requirements in Sec. 23.341 that must be met by an airplane with canard or tandem wing configurations.

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（2）：一位评论者就阵风载荷要求提供了以下分析。在多种常规飞机上已多次证明，机翼阵风载荷可以通过§23.341 条现行载荷因数公式的结果进行准确或保守的估算。这种近似的准确性取决于关于常规飞机对垂直阵风响应特性的成熟假设。对于鸭式布局飞机，其中一些基本假设并不成立。特别是，前翼在主翼进入阵风之前会使飞机产生相当大的上仰。这种情况很可能使以下假设失效，即飞机的响应可以仅用沉浮运动来充分表示。此外，前翼对主翼的下洗影响可能导致气动载荷沿翼展方向发生显著重新分布。
评论者还指出，现有第23.341条中的公式可能会低估鸭式布局飞机的惯性载荷因数。除了惯性因数的差异外，气动载荷出现的时间与峰值惯性因数出现的时间也不同。如果现有第23.341条中公式的假设是峰值气动载荷与峰值惯性载荷同时出现，则这种情况可能导致大幅低估主翼和前翼的净载荷。该假设仅对常规布局飞机有效。对于鸭式布局飞机，无论是主翼还是前翼，其惯性卸载都显著低于使用飞机重心处峰值加速度计算出的数值。美国联邦航空局同意评论者的意见，并修订了第23.341(a)条，以解决鸭式布局或串翼布局飞机中卸载惯性载荷与前翼载荷或主翼载荷不同相的问题。
增加“确定每个升力面上的阵风载荷”这些词语，以澄清阵风载荷分析必须分别考虑每个升力面。本提案按上述修改予以采纳。

英文原文：

One commenter provides the following analysis in regard to gust loads requirements. It has been shown many times, on a wide range of conventional airplanes, that wing gust loads can be accurately or conservatively estimated from the results of the current load factor formula of Sec. 23.341. The accuracy of this approximation is dependent upon well-proven assumptions concerning the nature of the response of a conventional airplane to a vertical gust. For a canard configured airplane, some of these basic assumptions are not valid. In particular, the forward wing can impart a considerable nose-up pitch to the airplane before the main wing becomes immersed in the gust. This condition is likely to nullify the assumption that the response can be considered to be adequately represented only by the plunge motion of the airplane. Also, the downwash influence of the forward wing on the main wing can lead to significant redistribution of the aerodynamic loading across the wing span.

The commenter also points out that the inertia load factor on the canard configured airplane can be underestimated by the formula in existing Sec. 23.341. In addition to the difference in inertia factors, the aerodynamic loads occur at different times than the peak inertia factor. This condition could result in substantially underestimating the net load on the main and forward wing if the formula assumption in existing Sec. 23.341 was that the peak aerodynamic load and peak inertia load occurred simultaneously. This assumption is valid only for conventional airplanes. For canard configured airplanes, for both the main wing and the forward wing, the inertia relief is significantly below the value that would be computed using the peak acceleration at the center of gravity of the airplane. The FAA agrees with the commenter and Sec. 23.341(a) is revised to address, for a canard or tandem wing configured airplane, the concern that the relieving inertia load is not in phase with the forward wing load or the main wing load. The words, "to develop the gust loading on each lifting surface," were added to clarify that the gust load analysis must be performed considering each surface separately. This proposal is adopted with the aforementioned changes.

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（3）：评论者还指出，现有第23.341条中的公式可能会低估鸭式布局飞机的惯性载荷因数。除了惯性因数的差异外，气动载荷出现的时间与峰值惯性因数出现的时间也不同。如果现有第23.341条中公式的假设是峰值气动载荷与峰值惯性载荷同时出现，则这种情况可能导致大幅低估主翼和前翼的净载荷。该假设仅对常规布局飞机有效。对于鸭式布局飞机，无论是主翼还是前翼，其惯性卸载都显著低于使用飞机重心处峰值加速度计算出的数值。美国联邦航空局同意评论者的意见，并修订了第23.341(a)条，以解决鸭式布局或串翼布局飞机中卸载惯性载荷与前翼载荷或主翼载荷不同相的问题。
增加“确定每个升力面上的阵风载荷”这些词语，以澄清阵风载荷分析必须分别考虑每个升力面。本提案按上述修改予以采纳。

英文原文：

The commenter also points out that the inertia load factor on the canard configured airplane can be underestimated by the formula in existing Sec. 23.341. In addition to the difference in inertia factors, the aerodynamic loads occur at different times than the peak inertia factor. This condition could result in substantially underestimating the net load on the main and forward wing if the formula assumption in existing Sec. 23.341 was that the peak aerodynamic load and peak inertia load occurred simultaneously. This assumption is valid only for conventional airplanes. For canard configured airplanes, for both the main wing and the forward wing, the inertia relief is significantly below the value that would be computed using the peak acceleration at the center of gravity of the airplane. The FAA agrees with the commenter and Sec. 23.341(a) is revised to address, for a canard or tandem wing configured airplane, the concern that the relieving inertia load is not in phase with the forward wing load or the main wing load. The words, "to develop the gust loading on each lifting surface," were added to clarify that the gust load analysis must be performed considering each surface separately. This proposal is adopted with the aforementioned changes.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（4）：本提案将为具有鸭翼或串列翼的飞机制定必要的阵风载荷要求。会议提案189建议在第23.341条中增加一条“注”，说明该方法不足以确定鸭翼或串列翼飞机的阵风载荷。会上有人表示反对，因为一位会议评论者指出，如果证明第23.341条的内容能产生保守或可接受的值，则该内容应可接受。会议参加者得出结论，美国联邦航空局也同意，如果申请人能够证明阵风载荷因数相对于第23.333(c)段的“1-余弦”阵风准则而言是保守的，那么在计算阵风载荷因数时使用第23.341条的现有内容是可以接受的。

英文原文：

This proposal would establish gust load requirements necessary for an airplane with a canard or tandem wing. Conference proposal 189 recommended a "Note" to Sec. 23.341 to state that the method was not adequate for determining gust loads for canard or tandem wing airplanes. There was opposition expressed at the conference because a conference commenter stated the material of Sec. 23.341 should be acceptable if it is demonstrated to produce conservative or acceptable values. It was concluded by the conference attendees, and the FAA agrees, that if an applicant can show gust load factors to be conservative with respect to the "1-Cosine" gust criteria of Sec. 23.333(c), it is acceptable to use the current material of Sec. 23.341 in computing the gust load factors.

修订指令英文原文（供核对）：

11. Section 23.341 is amended by designating the existing text as paragraph (b); by adding the words "for conventional configurations" after the word "analysis" in newly designated paragraph (b); and by adding a new paragraph (a) to read as follows:

11. Section 23.341 is amended by adding the words "for conventional configurations" after the word "analysis" in the existing test, and designating this existing text, as amended, as paragraph (b); and by adding a new paragraph (a) to read as follows:

▪ 本版重印条文段落：(a)

### 2.5　Amdt 23-48（1996-03-11 生效）：第23.341条经修订

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：第23.341条经修订，将现有(a)和(b)段分别重新编号为(b)和(c)段；新增(a)段；修订经重新编号的(b)段；并修订经重新编号的(c)段中的导语、公式及"W/S"定义，修订后内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议重组§23.341，以新增一个(a)段，明确每架飞机必须设计为能够承受§23.333(c)段所规定的阵风在每个升力面上产生的载荷。该局还提议按以下方式重组该条：(1) 将现有(a)段和(b)段分别重新指定为(b)段和(c)段；(2) 修订新(b)段的文本，删除“考虑§23.333(c)段的标准以确定每个升力面上的阵风载荷”这一表述，因为该要求将置于拟议的(a)段中；以及(3) 修订新(c)段，删除“对于常规构型”这一表述，因为它不再准确，并修订翼载荷(W/S)的定义。

英文原文：

The FAA proposed to reorganize Sec. 23.341 to provide a new paragraph (a), that would clarify that each airplane must be designed to withstand loads of each lifting surface that result from gusts specified in Sec. 23.333(c). It also proposed to reorganize the section as follows: (1) Redesignate existing paragraphs (a) and (b) as (b) and (c), respectively; (2) revise the text of new paragraph (b) to delete the phrase "considering the criteria of Sec. 23.333(c), to develop the gust loading on each lifting surface" since this requirement would be located in proposed paragraph (a); and, (3) revise new paragraph (c) to delete the phrase "for conventional configurations" because it is no longer accurate, and to revise the definition for wing loading (W/S).

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局（FAA）提议新增(c)段，以明确通勤类飞机除须符合第23.371(a)段的要求外，还必须符合第23.341条中的阵风条件。

英文原文：

The FAA proposed new paragraph (c) to clarify that commuter category airplanes must comply with the gust conditions in Sec. 23.341 in addition to the requirement of Sec. 23.371(a).

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（3）：第23.341条将重新组织，以规定一个新的(a)段，明确每架飞机必须设计成能够承受§23.333(c) 段规定的阵风在每个升力面上所产生的载荷。现有的(a)段和(b)段将分别重新指定为(b)段和(c)段。拟议的(b)段条文将予修订，删除“考虑§23.333(c) 段的标准，确定每个升力面上的阵风载荷”这一句，因为这一要求将纳入拟议的(a)段中。在重新指定的§23.341(b) 段中，对(b)段的引用改为对(c)段的引用，以保持一致。重新指定的(c)段条文将予修订，删除“对于常规构型”这一短语，因其不再准确，并修订翼载荷（W/S）的定义。这些修改是应欧洲联合航空局（JAA）的要求提议的，目的是与 JAR 23 协调统一。

英文原文：

Section 23.341 would be reorganized to provide a new paragraph(a) that clarifies that each airplane must be designed to withstand loads on each lifting surface that result from gusts specified in Sec. 23.333(c). Existing paragraphs (a) and (b) would be redesignated as (b) and (c), respectively. The text of the proposed paragraph (b) would be revised to eliminate the phrase, "considering the criteria of Sec. 23.333(c), to develop the gust loading on each lifting surface" since this requirement would be located in proposed paragraph(a). The reference to paragraph (b) in redesignated Sec. 23.341(b) is changed to paragraph(c) to conform. The text for the redesignated paragraph (c) would be revised to delete the phrase "for conventional configurations" because it is no longer accurate, and to revise the definition for wing loading (W/S). These changes are being proposed at the request of the JAA to harmonize with JAR 23.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（4）：拟议的新(c)段将澄清，除§23.371(a)段的要求外，通勤类飞机还必须符合§23.341中的阵风情况。这一澄清对于与欧洲联合航空要求（JAR）协调统一是必要的。

英文原文：

Proposed new paragraph (c) would clarify that commuter category airplanes must comply with the gust conditions in Sec. 23.341 in addition to the requirement of Sec. 23.371(a). This clarification is necessary to harmonize with the JAR.

修订指令英文原文（供核对）：

5. Section 23.341 is amended by redesignating existing paragraphs (a) and (b) as paragraphs (b) and (c), respectively; by adding a new paragraph (a); by revising the redesignated paragraph (b); and by revising the introductory text, the formula, and the definition of "W/ S" in the redesignated paragraph (c) to read as follows:

▪ 本版重印条文段落：(a)、(b)、(c)

## 3　§23.341 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The slope of the lift curve may be assumed to be that of the wing alone; and | 新增 |
| (b) | The gust load factors must be computed as follows: n = 1 + Where-- K = 1/2(W/S)1/4(for W/S 16 p.s.f.); U = nominal gust velocity, f.p.s. (Note that the "effective sharp-edged gust" equals KU); V = airplane speed, m.p.h.; m = slope of lift curve, CL per radian, corrected for aspect ratio; W/S = wing loading, p.s.f. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The airplane is assumed to be subjected to symmetrical vertical gusts in level flight. The resulting limit load factors must correspond to the conditions determined as follows: | 修订 |
| (1) | Positive (up) and negative (down) gusts of 50 fps at must be considered at altitudes between sea level and 20,000 feet. The gust velocity may be reduced linearly from 50 fps at 20,000 feet to 25 fps at 50,000 feet. | 新增 |
| (2) | Positive and negative gusts of 25 fps at must be considered at altitudes between sea level and 20,000 feet. The gust velocity may be reduced linearly from 25 fps at 20,000 feet to 12.5 fps at 50,000 feet. | 新增 |
| (b) | The following assumptions must be made: | 修订 |
| (1) | The shape of the gust is - where-- 8=distance penetrated into gust (ft); =mean geometric chord of wing (ft.); and =derived gust velocity referred to in paragraph (a) (fps). | 新增 |
| (2) | Gust load factors vary linearly with speed between and | 新增 |
| (c) | In the absence of a more rational analysis, the gust load factors must be computed as follows: Where-- Kg = = gust alleviation factor; = = airplane mass ratio; Ude = derived gust velocities referred to in paragraph (a) (fps); p = density of air (slugs/cu. ft.); W/S = wing loading (psf); = mean geometric chord (ft.); g = acceleration due to gravity (ft./sec.2); V = airplane equivalent speed (knots); and a = slope of the airplane normal force coefficient curve per radian if the gust loads are applied to the wings and horizontal tail surfaces simultaneously by a rational method. The wing lift curve slope CL per radian may be used when the gust load is applied to the wings only and the horizontal tail gust loads are treated as a separate condition. | 新增 |

### 3.3　版本3：Amdt 23-42（1991-02-04 生效）　来源：Docket 25811，1990-12-21 签发，56 FR 344

新增段落：(b)、(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The gust load for a canard or tandem wing configuration must be computed using a rational analysis, considering the criteria of Sec. 23.333(c), to develop the gust loading on each lifting surface or may be computed in accordance with paragraph (b) of this section provided that the resulting net loads are shown to be conservative with respect to the gust criteria on Sec. 23.333(c). | 修订 |

### 3.4　版本4：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

新增段落：(a)、(b)、(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each airplane must be designed to withstand loads on each lifting surface resulting from gusts specified in Sec. 23.333(c). | 修订 |
| (b) | The gust load for a canard or tandem wing configuration must be computed using a rational analysis, or may be computed in accordance with paragraph (c) of this section, provided that the resulting net loads are shown to be conservative with respect to the gust criteria of Sec. 23.333(c). | 新增 |
| (c) | In the absence of a more rational analysis, the gust load factors must be computed as follows-- [GRAPHIC] [TIFF OMMITTED] TR09FE96.010 W/S=Wing loading (p.s.f.) due to the applicable weight of the airplane in the particular load case. | 新增 |

### 3.5　段落结构演变矩阵（横向对照 5 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) | Amdt 23-42 (1991-02-04) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | △ 修订 | △ 修订 |
| (b) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (1) | ○ 无 | ● 新增 | ○ 无 | ○ 无 |
| (2) | ○ 无 | ● 新增 | ○ 无 | ○ 无 |
| (c) | ○ 无 | ● 新增 | ○ 无 | ● 新增 |

## 4　§23.341 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | .341, 23.345, 23.425, 23.443, and Appendix A23.3 to require investigation of gust loads based on airplane mass parameter concepts rather than the present wing loading concept. Comments were received objecting to the prop… | Amdt 23-7 |
| 新技术 / 新构型 | rements in Sec. 23.341 that must be met by an airplane with canard or tandem wing configurations. One commenter provides the following analysis in regard to gust loads requirements. It has been shown many times, on a wid… | Amdt 23-42 |
| 国际协调 | /S). These changes are being proposed at the request of the JAA to harmonize with JAR 23. Proposed new paragraph (c) would clarify that commuter category airplanes must comply with the gust conditions in Sec. 23.341 in a… | Amdt 23-48 |

本条取证提示 共 5 个历史版本：初始版, Amdt 23-7, Amdt 23-34, Amdt 23-42, Amdt 23-48。 其中 3 个版本在本地文献中检索到明确修订指令原文；4 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [6] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [7] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [8] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [9] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [10] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [11] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [12] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [13] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [14] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.341 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 5a4a5424-7b9c-4892-ad06-726b915ff320 |
| Amdt 23-7 | 1969-09-14 | e1a99bbf-bdc0-45ed-ad05-6f14f6203385 |
| Amdt 23-34 | 1987-02-17 | 519f5a84-f3ef-4957-8c78-1ed606704089 |
| Amdt 23-42 | 1991-02-04 | f92607a5-eb04-45f6-bfe2-5217c91f216f |
| Amdt 23-48 | 1996-03-11 | b0256a00-7f6b-4c2a-893c-f5e3f1fdd35e |

# § 23.343　设计燃油载荷（Design fuel loads.）

1996-03-11 初版　｜　1996-03-11 Amdt 23-48　｜　共 1 个历史版本

## 1　§23.343 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.343 |
| 条款标题（英文原文） | Design fuel loads. |
| 中文名称 | 设计燃油载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1996-03-11 初始版） |
| 时间跨度 | 1996-03-11 → 1996-03-11 （约 0 年） |
| 初版来源 | Docket 27805，1996-01-29 签发，61 FR 5138 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.343 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增条款：(c) |

### 1.3　修订脉络一览

1996 初版　新增条款：(c)

## 2　§23.343 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1996-03-11，Amdt 23-48）：设计燃油载荷

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：6. 新增第23.343条，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议新增§23.343条。该拟议要求将适用于第23部所有飞机类别，但（c）段除外，该段仅限于通勤类飞机。

英文原文：

The FAA proposed a new Sec. 23.343. The proposed requirement would apply to all part 23 airplane categories, except paragraph (c), which is limited to commuter category airplanes.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：拟议的新 §23.343 除 (c) 段外将与相应的欧洲联合航空要求（JAR）协调统一。该拟议要求是对涵盖运输类飞机的 §25.343 的修改版本，将适用于第 23 部所有飞机类别，但其中一段将仅限于通勤类飞机。

英文原文：

Proposed new Sec. 23.343 would harmonize with the corresponding JAR except for paragraph(c). This proposed requirement which is a modified version of Sec. 25.343 that covers transport category, would apply to all part 23 airplane categories, except one paragraph that would be limited to commuter category airplanes.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（3）：美国联邦航空局在1992年7月于维也纳举行的JAA/FAA协调统一研究组会议上同意，将要求分三段提出。欧洲联合航空局（JAA）将仅为JAR 23提出(a)段和(b)段，因为他们没有45分钟燃油储备的运行规则。此外，欧洲联合航空局（JAA）决定将(c)段纳入拟议措施通告（NPA），以等待必要运行规则的制定。1993年2月，同一工作组同意让(b)段涉及“最大零机翼燃油”重量，而非如上所述的“最大零燃油”重量。工作组同意不参照拟议的§23.1583(c)(6)段（如飞行协调统一规章制定提案通告（NPRM）中所述）所要求的《飞机飞行手册》（AFM）的“使用限制”部分，因为该部分已包含对§23.343的引用。

英文原文：

The FAA agreed, in a JAA/FAA Harmonization Study Group Meeting in Vienna, in July 1992, to propose the requirements in three paragraphs. The JAA would only propose paragraphs (a) and (b) for JAR 23 because they do not have a 45-minute fuel reserve operating rule. Also, the JAA decided to put paragraph (c) into a Notice of Proposed Action (NPA) to await the creation of the necessary operating rule. In February 1993, the same group agreed to have paragraph (b) address "maximum zero wing fuel" weight, instead of "maximum zero fuel" weight as mentioned above. The group agreed not to refer to the Operating Limitation Section of the Airplane Flight Manual (AFM) required by proposed Sec. 23.1583(c)(6) (as presented in the Flight Harmonization NPRM) since that section already contains a reference to Sec. 23.343.

修订指令英文原文（供核对）：

6. A new Sec. 23.343 is added to read as follows:

## 3　§23.343 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The disposable load combinations must include each fuel load in the range from zero fuel to the selected maximum fuel load. | 新增 |
| (b) | If fuel is carried in the wings, the maximum allowable weight of the airplane without any fuel in the wing tank(s) must be established as "maximum zero wing fuel weight," if it is less than the maximum weight. | 新增 |
| (c) | For commuter category airplanes, a structural reserve fuel condition, not exceeding fuel necessary for 45 minutes of operation at maximum continuous power, may be selected. If a structural reserve fuel condition is selected, it must be used as the minimum fuel weight condition for showing compliance with the flight load requirements prescribed in this part and-- | 新增 |
| (1) | The structure must be designed to withstand a condition of zero fuel in the wing at limit loads corresponding to: | 新增 |
| (i) | Ninety percent of the maneuvering load factors defined in Sec. 23.337, and | 新增 |
| (ii) | Gust velocities equal to 85 percent of the values prescribed in Sec. 23.333(c). | 新增 |
| (2) | The fatigue evaluation of the structure must account for any increase in operating stresses resulting from the design condition of paragraph (c)(1) of this section. | 新增 |
| (3) | The flutter, deformation, and vibration requirements must also be met with zero fuel in the wings. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-48 (1996-03-11) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |
| (1) | ● 新增 |
| (i) | ● 新增 |
| (ii) | ● 新增 |
| (2) | ● 新增 |
| (3) | ● 新增 |

## 4　§23.343 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | egory airplanes. The FAA agreed, in a JAA/FAA Harmonization Study Group Meeting in Vienna, in July 1992, to propose the requirements in three paragraphs. The JAA would only propose paragraphs (a) and (b) for JAR 23 becau… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [2] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [3] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.343 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-48 | 1996-03-11 | aa193159-74c1-47ee-b55e-99007790de6a |

# § 23.345　高升力装置（High lift devices.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 4 个历史版本

## 1　§23.345 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.345 |
| 条款标题（英文原文） | High lift devices. |
| 中文名称 | 高升力装置 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.345 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(a) |
| 3 | Amdt 23-23 | 1978-12-01 | Notice 75-10，1975-02-27 签发 Notice 75-19，1975-05-13 签发 Notice 75-23，1975-05-19 签发 Notice 75-26，1975-06-09 签发 Notice 75-31，1975-06-30 签发 | Docket 14324, 14606, 14625, 14685, 14779，1978-10-20 签发，43 FR 50578 | 修订 |
| 4 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订：(a) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(a)　→　1978 23-23　修订　→　1996 23-48　修订：(a)

## 2　§23.345 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：高升力装置

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：修订§23.345的(a)(2)段和(e)段

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：21. 修订§23.345的(a)(2)段和(e)段，内容如下：　3. 第23.345条(a)(2)段将被修订，将阵风速度值从15 fps改为25 fps。

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案6. 增升装置的验证（§23.345）。§23.345(e) 段将通过删除对 §23.175(c) 段的交叉引用进行修订。
说明。将飞行试验条件和气动载荷条件结合起来是不切实际的，而这正是对 §23.175(c) 段交叉引用的效果。

英文原文：

Proposal 6. Substantiation of high lift devices (Sec. 23.345). Section 23.345 (e) would be amended by striking out the cross reference to Sec. 23.175 (c).

Explanation. It is impractical to combine flight test conditions and airload conditions, which is the effect of the cross reference to Sec. 23.175 (c).

修订指令英文原文（供核对）：

21. Section 23.345 is amended by amending paragraphs (a)(2) and (e) to read as follows:

3. Section 23.345 (a) (2) would be amended by changing the gust velocity value from 15 fps to 25 fps.

▪ 本版重印条文段落：(a)、(2)、(e)

### 2.3　Amdt 23-23（1978-12-01 生效）：修订相应段落。

▪ 修订沿革：本版由 NPRM　Notice 75-10，1975-02-27 签发；Notice 75-19，1975-05-13 签发；Notice 75-23，1975-05-19 签发；Notice 75-26，1975-06-09 签发；Notice 75-31，1975-06-30 签发　提出；最终规章 Docket 14324, 14606, 14625, 14685, 14779，1978-10-20 签发，43 FR 50578，1978-12-01 生效。

▪ 修订指令（中文译文）：修订相应段落。

来源：Final Rule｜43 FR 50578｜FinalRule_Docket 14324_ 14606_ 14625_ 14685_ 14779_Amdt23-23_25-46_27-16_29_43FR50578_1978-10-30_0003.txt

▪ 背景与评论处置（1）：提案7-1。一位评论者建议删除拟议的§23.345(c)段中每秒25英尺的迎头阵风要求，因为没有记录显示任何襟翼或襟翼结构因迎头阵风而发生故障，且该提案会增加动力关闭时的襟翼载荷。美国联邦航空局认为，记录在案的襟翼及襟翼支撑结构故障大多数是由迎头阵风造成的。对§23.345(c)段的修订予以通过，未作实质性修改。

英文原文：

Proposal 7-1. One commentator recommended that the 25 feet per second head-on gust requirement in proposed Sec. 23.345(c) be deleted because there had been no recorded failures of any flaps or flap structure attributed to head-on gusts and the proposal would increase the power-off flap loads. The FAA believes that a majority of the recorded flap and flap support structural failures resulted from head-on gusts. The amendment to Sec. 23.345(c) is adopted without substantive change.

来源：Final Rule｜43 FR 50578｜FinalRule_Docket 14324_ 14606_ 14625_ 14685_ 14779_Amdt23-23_25-46_27-16_29_43FR50578_1978-10-30_0003.txt

▪ 背景与评论处置（2）：基于收到的评论以及美国联邦航空局内部的进一步审查，关于新增 §23.345(f) 段的提案被撤回。

英文原文：

Based on the comments received and upon further review within the FAA, the proposal for new Sec. 23.345(f) is withdrawn.

修订指令英文原文（供核对）：

1. By revising Sec. 23.345(c) to read as follows:

▪ 本版重印条文段落：(c)、(1)、(2)

### 2.4　Amdt 23-48（1996-03-11 生效）：修订§23.345如下

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：7. 修订§23.345如下：　现行(c)段将重新指定为(d)段，并通过纳入第23.457条的要求进行修订。(e)段将被删除，因为它仅引用第23.457条的要求；这些要求已移至第23.345条(d)段。这种安排将所有"襟翼"要求集中在一处，并将使这些要求与JAR 23协调统一。　§23.455 之前的标题将作修订，删除“机翼襟翼”一词，使标题变为“副翼和特殊装置”。此项修改反映了拟从 §23.457 中删除机翼襟翼要求并将其纳入 §23.345 的提案。 注：原文中 "Sec. 23.455" 按规则译为 "§23.455"，"Sec. 23.457" 译为 "§23.457"，"Sec. 23.345" 译为 "§23.345"。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：为了将所有“襟翼”要求集中在一处，并与 JAR 23 实现协调统一，美国联邦航空局提议修订 §23.345 如下：（1）进行较小的组织性、非实质性的澄清性修改；（2）将术语“完全偏转”改为“完全放下”，因为后者更准确地描述襟翼状态和位置；（3）删除短语“导致限制载荷因数”，因为该要求已存在于 §23.301(a) 段；（4）将现行(c)段改列为(d)段，并修订该段以纳入 §23.457 的襟翼要求；（5）将现行(d)段改列为(c)段；（6）将 §23.457 的襟翼要求纳入经改列后的 §23.345(b) 段和 §23.345(d) 段，并删除冗余的(e)段。

英文原文：

To place all "flap" requirements in one location, and to harmonize the requirements with JAR 23, the FAA proposed to revise Sec. 23.345 as follows: (1) Make minor organizational, and non- substantive, clarifying changes; (2) Change the term "fully deflected" to "fully extended" because it more accurately describes flap conditions and positions; (3) Remove the phrase "resulting in limit load factors" because the requirement already exists in Sec. 23.301(a); (4) Redesignate current paragraph (c) as paragraph (d) and revise it to include the flap requirements of Sec. 23.457; (5) Redesignate current paragraph (d) as paragraph (c); and (6) Incorporate the flap requirements of Sec. 23.457 into Sec. 23.345(b) and Sec. 23.345(d), as redesignated, and delete paragraph (e), which is redundant.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局提议修改§23.455之前的标题，删除"翼襟"一词，使标题变为"副翼和专用装置"。此项修改将反映从§23.457中删除翼襟要求并将其置于§23.345中的变化。

英文原文：

The FAA proposed to amend the heading the precedes Sec. 23.455 by deleting the term "Wing Flaps" so that the heading reads "AILERONS AND SPECIAL DEVICES". This change would reflect the deletion of the wing flap requirements from Sec. 23.457 and their placement in Sec. 23.345.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（3）：美国联邦航空局（FAA）提议删除本条。如上面 §23.345 所述，机翼襟翼要求已经修订并合并到 §23.345 中，以便将这些要求集中在一起。

英文原文：

The FAA proposed to delete this section. As discussed under Sec. 23.345, above, the wing flap requirements have been revised and consolidated in Sec. 23.345 to group these requirements together.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（4）：修订后的 §23.345(a) 段将包含次要的、非实质性的澄清性修改。术语"完全偏转"将改为"完全展开"，因为它更准确地描述了襟翼状态和位置。短语"产生极限载荷因数"将被删除，因为该要求已存在于 §23.301(a) 段中。现行(d)段将重新指定为(c)段，不作更改。

英文原文：

Revised Sec. 23.345(a) would have minor, non-substantive, clarifying changes. The term "fully deflected" would be changed to "fully extended" because it more accurately describes flap conditions and positions. The phrase "resulting in limit load factors" would be removed because the requirement already exists in Sec. 23.301 (a). Current paragraph(d) would be redesignated as paragraph (c) without change.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（5）：美国联邦航空局（FAA）提议删除本条。如上文 §23.345 条所述，襟翼要求已在拟议的 §23.345 条中修订并合并，以将这些要求归组在一起。

英文原文：

The FAA proposes to delete this section. As discussed under Sec. 23.345, above, the wing flap requirements have been revised and consolidated in proposed Sec. 23.345 to group these requirements together.

修订指令英文原文（供核对）：

7. Section 23.345 is revised to read as follows:

Current paragraph (c) would be redesignated as paragraph (d) and revised by including the requirements of Sec. 23.457. Paragraph (e) would be deleted since it merely references the requirements of sec. 23.457; which have been moved to Sec. 23.345 (d). This arrangement places all "flap" requirements in one location, and would harmonize the requirements with JAR 23.

The heading the precedes Sec. 23.455 would be amended by deleting the term "Wing Flaps" so that the heading reads "AILERONS AND SPECIAL DEVICES." This change reflects the proposed deletion of the wing flap requirements from Sec. 23.457 and their placement in Sec. 23.345.

▪ 本版重印条文段落：(a)、(1)、(2)、(b)、(1)、(2)、(3)、(c)、(d)、(1)、(2)

## 3　§23.345 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | If flaps or similar high lift devices to be used for takeoff, approach or landing are installed, the airplane, with the flaps fully deflected at VF, is assumed to be subjected to symmetrical maneuvers and gusts resulting in limit load factors within the range determined by-- | 新增 |
| (1) | Maneuvering, to a positive limit load factor of 2.0; and | 新增 |
| (2) | Positive and negative gusts of 15 feet per second acting normal to the flight path in level flight. | 新增 |
| (b) | VF must be assumed to be not less than 1.4 VS or 1.8 VSF, whichever is greater, where-- VS is the computed stalling speed with flaps retracted at the design weight; and VSF is the computed stalling speed with flaps fully extended at the design weight. However, if an automatic flap load limiting device is used, the airplane may be designed for the critical combinations of airspeed and flap position allowed by that device. | 新增 |
| (c) | In designing the flaps and supporting structures, slipstream effects must be accounted for, as specified in paragraph (b) of Sec. 23.457. | 新增 |
| (d) | In determining external loads on the airplane as a whole, thrust, slipstream, and pitching acceleration may be assumed to be zero. | 新增 |
| (e) | The requirements of Secs. 23.175(c), 23.457, and this section, may be complied with separately or in combination. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (2) | Positive and negative gust of 25 feet per second acting normal to the flight path in level flight. | 修订 |
| (e) | The requirements of Sec. 23.457, and this section may be complied with separately or in combination. | 修订 |

### 3.3　版本3：Amdt 23-23（1978-12-01 生效）　来源：Docket 14324, 14606, 14625, 14685, 14779，1978-10-20 签发，43 FR 50578

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | In designing the flaps and supporting structures, the following must be accounted for: | 新增 |
| (1) | A head-on gust having a velocity of 25 feet per second (EAS). | 新增 |
| (2) | The slipstream effects specified in Sec. 23.457(b). | 修订 |

### 3.4　版本4：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | If flaps or similar high lift devices are to be used for takeoff, approach or landing, the airplane, with the flaps fully extended at V F, is assumed to be subjected to symmetrical maneuvers and gusts within the range determined by-- | 新增 |
| (1) | Maneuvering, to a positive limit load factor of 2.0; and | 修订 |
| (2) | Positive and negative gust of 25 feet per second acting normal to the flight path in level flight. | 修订 |
| (b) | V F must be assumed to be not less than 1.4 V S or 1.8 V SF, whichever is greater, where-- | 新增 |
| (1) | V S is the computed stalling speed with flaps retracted at the design weight; and | 修订 |
| (2) | V SF is the computed stalling speed with flaps fully extended at the design weight. | 修订 |
| (3) | If an automatic flap load limiting device is used, the airplane may be designed for the critical combinations of airspeed and flap position allowed by that device. | 新增 |
| (c) | In determining external loads on the airplane as a whole, thrust, slipstream, and pitching acceleration may be assumed to be zero. | 修订 |
| (d) | The flaps, their operating mechanism, and their supporting structures, must be designed to withstand the conditions prescribed in paragraph (a) of this section. In addition, with the flaps fully extended at V F, the following conditions, taken separately, must be accounted for: | 新增 |
| (1) | A head-on gust having a velocity of 25 feet per second (EAS), combined with propeller slipstream corresponding to 75 percent of maximum continuous power; and | 修订 |
| (2) | The effects of propeller slipstream corresponding to maximum takeoff power. | 修订 |

### 3.5　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) | Amdt 23-23 (1978-12-01) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (1) | ● 新增 | ○ 无 | ● 新增 | △ 修订 |
| (2) | ● 新增 | △ 修订 | △ 修订 | △ 修订 |
| (b) | ● 新增 | ○ 无 | ○ 无 | ● 新增 |
| (c) | ● 新增 | ○ 无 | ● 新增 | △ 修订 |
| (d) | ● 新增 | ○ 无 | ○ 无 | ● 新增 |
| (e) | ● 新增 | △ 修订 | ○ 无 | ○ 无 |
| (3) | ○ 无 | ○ 无 | ○ 无 | ● 新增 |

## 4　§23.345 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | To place all "flap" requirements in one location, and to harmonize the requirements with JAR 23, the FAA proposed to revise Sec. 23.345 as follows: (1) Make minor organizational, and non- substantive, clarifying chang… | Amdt 23-48 |

本条取证提示 共 4 个历史版本：初始版, Amdt 23-7, Amdt 23-23, Amdt 23-48。 其中 3 个版本在本地文献中检索到明确修订指令原文；4 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 14324 | 43 FR 50578 | FinalRule_Docket 14324_ 14606_ 14625_ 14685_ 14779_Amdt23-23_25-46_27-16_29_43FR50578_1978-10-30_0003.txt |
| [6] | Final Rule（颁行） | Docket 14324 | 41 FR 55454 | FinalRule_Docket 14324_Amdt21-44_23-17_25-38_27_41FR55454_1976-12-20_0002.txt |
| [7] | NPRM（提案） | Notice 75-10 / Docket 14324 | （FR 引证未在文本中标注） | NPRM_Notice 75-10_Docket14324_1975-03-07.txt |
| [8] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [9] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [10] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [11] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [12] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.345 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 8575a353-1fd0-4eb5-bedb-130c34cb69d8 |
| Amdt 23-7 | 1969-09-14 | d7167098-a526-4d10-8e64-18f9e64505c4 |
| Amdt 23-23 | 1978-12-01 | 18e9579e-3b28-42ca-ad3d-fde257bbcae5 |
| Amdt 23-48 | 1996-03-11 | ef40c63f-056d-4c1e-8b5e-6ba510926f61 |

# § 23.347　不对称飞行情况（Unsymmetrical flight conditions.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 2 个历史版本

## 1　§23.347 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.347 |
| 条款标题（英文原文） | Unsymmetrical flight conditions. |
| 中文名称 | 不对称飞行情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.347 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增段落：(a)、(b) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1996 23-48　新增段落：(a)、(b)

## 2　§23.347 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：不对称飞行情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-48（1996-03-11 生效）：对 §23.347 条进行修订

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：8. 对 §23.347 条进行修订，将现有文本指定为(a)段，并新增(b)段，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议修订 §23.347，将现有内容重新指定为 (a) 段，并新增 (b) 段，以纳入快滚机动（snap roll）的要求（如针对特技类飞机提出此项要求）。

英文原文：

The FAA proposed to revise Sec. 23.347 to redesignate the existing text as paragraph (a) and to add a new paragraph (b) to include requirements for a flick maneuver (snap roll), if requested for acrobatic category airplanes.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：对第 23.347 条的拟议修订将把现有文本重新指定为 (a) 段，并新增 (b) 段，其中包含对急滚机动（快滚）的要求，如果为特技类飞机提出申请。进行此项更改是为了与欧洲联合航空要求（JAR）协调统一。

英文原文：

The proposed revision to Sec. 23.347 would redesignate the existing text as paragraph (a) and add a new paragraph (b) that includes requirements for a flick maneuver (snap roll), if requested for aerobatic category airplanes. This change is being made to harmonize with the JAR.

修订指令英文原文（供核对）：

8. Section 23.347 is amended by designating the existing text as paragraph (a) and by adding a new paragraph (b) to read as follows:

▪ 本版重印条文段落：(b)

## 3　§23.347 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

新增段落：(a)、(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | Acrobatic category airplanes certified for flick maneuvers (snap roll) must be designed for additional asymmetric loads acting on the wing and the horizontal tail. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | Amdt 23-48 (1996-03-11) |
| --- | --- |
| (b) | ● 新增 |

## 4　§23.347 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | aerobatic category airplanes. This change is being made to harmonize with the JAR. 8. Section 23.347 is amended by designating the existing text as paragraph (a) and by adding a new paragraph (b) to read as follows: Sec… | Amdt 23-48 |

本条取证提示 共 2 个历史版本：初始版, Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [4] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [5] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [6] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [7] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.347 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 68d32c34-0208-4de3-8da0-538f15f30e0e |
| Amdt 23-48 | 1996-03-11 | 438dadc3-8635-4c70-b4d5-6bb3aab3a238 |

# § 23.349　滚转情况（Rolling conditions.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 4 个历史版本

## 1　§23.349 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.349 |
| 条款标题（英文原文） | Rolling conditions. |
| 中文名称 | 滚转情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.349 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(a)、(1) |
| 3 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 修订段落：(a) |
| 4 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(a)、(1)　→　1987 23-34　修订段落：(a)　→　1996 23-48　修订

## 2　§23.349 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：滚转情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：对 §23.349 条进行修订

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：22. 对 §23.349 条进行修订，修订 (a) 段，删除子段 (1) 和 (2) 中的"机翼气动载荷"字样，并在原处插入"半翼展机翼气动载荷"字样。

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案 7. 滚转情况（§23.349）。§23.349(a)(1) 和 (2) 段中的"机翼气动载荷"一词将改为"半翼展机翼气动载荷"。

说明。规定百分比所适用的气动载荷是滚转轴两侧各自的机翼气动载荷，而不是机翼总气动载荷。因此，"机翼气动载荷"一词过于宽泛。"半翼展机翼气动载荷"更为精确。

英文原文：

Proposal 7. Rolling conditions (Sec. 23.349). The term "wing airload" in Sec. 23.349 (a) (1) and (2) would be changed to "semispan wing airload."

Explanation. The airloads to which the prescribed percentages apply are the wing airloads on each side of the roll axis, not the total wing airload. Therefore, "wing airload" is too broad. "Semi-span wing airload" is more precise.

修订指令英文原文（供核对）：

22. Section 23.349 is amended by amending paragraph (a) by striking out the words "wing airload" in subparagraphs (1) and (2) and by inserting the words "semispan wing airload" in place thereof.

### 2.3　Amdt 23-34（1987-02-17 生效）：通过修订 §23.349

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 修订指令（中文译文）：23. 通过修订 §23.349，将 (a)(2) 段修改为以下内容：

来源：Final Rule｜52 FR 1806｜FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt

▪ 背景与评论处置（1）：提案 22——未收到对该提案的评论。因此，第 23.349 条按提案予以修订。

英文原文：

Proposal 22 -- No comments were received to the proposal. Accordingly, Sec. 23.349 is amended as proposed.

修订指令英文原文（供核对）：

23. By amending Sec. 23.349 by revising paragraph (a)(2) to read as follows:

▪ 本版重印条文段落：(a)

### 2.4　Amdt 23-48（1996-03-11 生效）：第23.349条(a)(2)段修订为以下内容

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：9. 第23.349条(a)(2)段修订为以下内容：　§23.349(a)(2) 段将被修订，以将所有不超过 19,000 磅的设计重量的不对称半翼展载荷假设简化为 100% 和 75%。美国联邦航空局（FAA）曾建议将后一个百分比在 70% 和 77.5% 之间线性变化，以涵盖重量不超过 19,000 磅的飞机。在与欧洲联合航空局（JAA）讨论后，美国联邦航空局（FAA）同意，对于所有第 23 部飞机，75% 是一个适当的假设。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订第23.349(a)(2)条，以简化正常类、实用类和通勤类飞机的不对称半翼展载荷假设，即对于所有设计重量不超过19,000磅的飞机，一侧机翼半翼展取100%，另一侧机翼半翼展取75%。规章制定提案通告的前言部分未包含所提议的100%和75%载荷分布仅适用于正常类、实用类和通勤类飞机的说明。该提案未将特技类飞机纳入此项拟议要求。然而，第23.349(c)(2)条拟议的规章语言正确反映了美国联邦航空局的意图。在起草规章制定提案通告时，美国联邦航空局曾建议将后一百分比在70%至77.5%之间线性变化，以涵盖重量不超过19,000磅的飞机。经与欧洲联合航空局（JAA）讨论后，美国联邦航空局同意，除特技类飞机外，对所有第23部飞机采用75%作为适当假设。

英文原文：

The FAA proposed to revised Sec. 23.349(a)(2) to simplify the unsymmetric semispan load assumption for normal, utility, and commuter category airplanes to 100 percent on one wing semispan and 75 percent on the other wing semispan for all design weights up through 19,000 pounds. The preamble to the NPRM did not include the explanation that the proposed 100 percent and 75 percent load distribution applied only to normal, utility, and commuter category airplanes. The NPRM did not include acrobatic category airplanes in this proposed requirement. However, the proposed regulatory language for Sec. 23.349(c)(2) correctly reflects the FAA's intent. While preparing the NPRM, the FAA had suggested varying the latter percentage linearly between 70 percent and 77.5 percent to include aircraft weighing up to 19,000 pounds. After discussion with the JAA, the FAA agreed that 75 percent is an appropriate assumption for all part 23 airplanes except acrobatic category airplanes.

修订指令英文原文（供核对）：

9. Section 23.349(a)(2) is revised to read as follows:

Section 23.349(a)(2) would be revised to simplify the unsymmetric semispan load assumption to 100 percent and 75 percent for all design weights up through 19,000 pounds. The FAA had suggested varying the latter percentage linearly between 70 percent and 77.5 percent to include aircraft weighing up to 19,000 pounds. After discussion with the JAA, the FAA agrees that 75 percent is an appropriate assumption for all part 23 airplanes.

▪ 本版重印条文段落：(a)、(2)

## 3　§23.349 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Unsymmetrical wing loads appropriate to the category. Unless the following values result in unrealistic loads, the rolling accelerations may be obtained by modifying the symmetrical flight conditions in Sec. 23.333(d) as follows: | 新增 |
| (1) | For the acrobatic category, in conditions A and F, assume that 100 percent of the wing airload acts on one side of the plane of symmetry and 60 percent of this load acts on the other side. | 新增 |
| (2) | For the normal and utility categories, in condition A, assume that 100 percent of the wing airload acts on one side of the airplane and 70 percent of this load acts on the other side. For airplanes of more than 1,000 pounds design weight, the latter percentage may be increased linearly with weight up to 75 percent at 12,500 pounds. | 新增 |
| (b) | The loads resulting from the aileron deflections and speeds specified in Sec. 23.455, in combination with an airplane load factor of at least two thirds of the positive maneuvering load factor used for design. Unless the following values result in unrealistic loads, the effect of aileron displacement on wing torsion may be accounted for by adding the following increment to the basic airfoil moment coefficient over the aileron portion of the span in the critical condition determined in Sec. 23.333(d): cm=-0.01 where-- cm is the moment coefficient increment; and is the down aileron deflection in degrees in the critical condition. | 新增 |

### 3.2　版本2：Amdt 23-34（1987-02-17 生效）　来源：Docket 23516，1987-01-08 签发，52 FR 1806

修订段落：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * (2) For the normal, utility, and commuter categories, in Condition A, assume that 100 percent of the semispan wing airload acts on one side of the airplane and 70 percent of this load acts on the other side. For airplanes of more than 1,000 pounds design weight, the latter percentage may be increased linearly with weight up through 75 percent at 12,500 pounds to the maximum gross weight of the airplane. | 修订 |

### 3.3　版本3：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (2) | For normal, utility, and commuter categories, in Condition A, assume that 100 percent of the semispan wing airload acts on one side of the airplane and 75 percent of this load acts on the other side. | 新增 |

### 3.4　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-34 (1987-02-17) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | △ 修订 |
| (1) | ● 新增 | ○ 无 | ○ 无 |
| (2) | ● 新增 | ○ 无 | ● 新增 |
| (b) | ● 新增 | ○ 无 | ○ 无 |

## 4　§23.349 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | aft weighing up to 19,000 pounds. After discussion with the JAA, the FAA agreed that 75 percent is an appropriate assumption for all part 23 airplanes except acrobatic category airplanes. 9. Section 23.349(a)(2) is revis… | Amdt 23-48 |

本条取证提示 共 4 个历史版本：初始版, Amdt 23-7, Amdt 23-34, Amdt 23-48。 其中 3 个版本在本地文献中检索到明确修订指令原文；3 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [6] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [7] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [8] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [9] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [10] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [11] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [12] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.349 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 0e829781-b629-4aef-b060-83fc6938ba4a |
| Amdt 23-7 | 1969-09-14 | 42d0f8ce-58ef-41ce-b1ef-e12b724c2bdb |
| Amdt 23-34 | 1987-02-17 | 0975ed64-03da-421e-b27b-b91129056897 |
| Amdt 23-48 | 1996-03-11 | 594479b7-f296-4be4-8d9d-56fdc19221fa |

# § 23.351　偏航情况（Yawing conditions.）

1965-02-01 初版　｜　1991-02-04 Amdt 23-42　｜　共 2 个历史版本

## 1　§23.351 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.351 |
| 条款标题（英文原文） | Yawing conditions. |
| 中文名称 | 偏航情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1991-02-04 （约 26 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 30 FR 258 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.351 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 删除内容 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1991 23-42　删除内容

## 2　§23.351 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：偏航情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-42（1991-02-04 生效）：§23.351 经修订

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：12. §23.351 经修订，删除"tail"一词。

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案 12。该提案将把 §23.351 条中的偏航要求（目前仅限于垂直尾翼面）扩展至新飞机设计中的所有垂直表面，例如翼梢小翼。此项变更被认为是必要的，以便为所有垂直表面提供与常规垂直尾翼面所要求的相当的结构完整性。未收到关于该提案的评论，该提案按原提案予以采纳。

英文原文：

Proposal 12. This proposal would extend the yawing requirements in Sec. 23.351, currently limited to vertical tail surfaces, to all vertical surfaces, such as winglets, in new airplane designs. This change is considered necessary to provide structural integrity for all vertical surfaces equivalent to that required for conventional vertical tail surfaces. No comments were received on this proposal and it is adopted as proposed.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（2）：本提案将把目前仅限于垂直尾翼面的偏航情况要求扩展至所有垂直表面，例如新飞机设计中日益出现的翼梢小翼。会议没有针对修订本条的具体提案，本条修订也未在会议上讨论。然而，为以引用方式纳入 §23.351 的要求（特别是 §23.441 至 §23.445）制定建议修改的工作，引发了对本条的审查，并形成了本拟议修改。认为该修改对于提供所有垂直表面的结构完整性是必要的，使其等同于常规垂直尾翼面所要求的结构完整性。

英文原文：

This proposal would extend the yawing conditions requirements, that are now limited to vertical tail surfaces, to all vertical surfaces, such as winglets, that are now appearing in new airplane designs. There was not a specific conference proposal to amend this section and the amendment of this section was not discussed at the conference. However, development of recommended changes to requirements incorporated by reference in Sec. 23.351 specifically Secs. 23.441 through 23.445 caused a review of this section and resulted in this proposed change. This change is considered necessary to provide structural integrity of all vertical surfaces equivalent to that required for conventional vertical tail surfaces.

修订指令英文原文（供核对）：

12. Section 23.351 is amended by removing the word "tail".

### 2.3　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.351 的实际改动 |
| --- | --- | --- | --- |
| 30 FR 258 | 1965-01-09 | 更正通告 (CFR 溯源注) | §23.365(d) 系数 1.3→1.33；§23.427 公式 'local'→'load' |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.351 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 2 个版本）

（无可用全文，矩阵略。）

## 4　§23.351 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 2 个历史版本：初始版, Amdt 23-42。 其中 1 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：30 FR 258，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [4] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [5] | 更正通告 / 校正 | 30 FR 258 | 1965-01-09 | Correction_30FR258_1965-01-09_Amdt23-0.clean.txt |
| [6] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [7] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.351 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | fd819323-bd3d-4f50-98f6-c180c02b0caf |
| Amdt 23-42 | 1991-02-04 | a0edfe7a-1cf7-4a03-b7c6-e6692124d426 |

# § 23.361　发动机扭矩（Engine torque.）

1965-02-01 初版　｜　1993-09-07 Amdt 23-45　｜　共 4 个历史版本

## 1　§23.361 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.361 |
| 条款标题（英文原文） | Engine torque. |
| 中文名称 | 发动机扭矩 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1993-09-07 （约 28 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.361 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(a)、(1) |
| 3 | Amdt 23-26 | 1980-10-14 | Notice 74-5，1974-02-14 签发，39 FR 5785 Notice 74-5A，1974-05-22 签发，39 FR 5785 Notice 74-5B，1974-10-03 签发，39 FR 5785 Notice 74-33，1974-10-03 签发，39 FR 5785 Notice 74-5C，1974-11-25 签发，39 FR 5785 Notice 74-5D，1975-02-04 签发，39 FR 5785 Notice 75-10，1975-02-27 签发，39 FR 5785 Notice 75-19，1975-05-13 签发，39 FR 5785 Notice 75-26，1975-06-09 签发，39 FR 5785 Notice 75-31，1975-06-30 签发，39 FR 5785 | Docket 14779, 14324，1980-08-27 签发，45 FR 60154 | 修订：(a) |
| 4 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订：(a)、(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(a)、(1)　→　1980 23-26　修订：(a)　→　1993 23-45　修订：(a)、(c)

## 2　§23.361 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：发动机扭矩

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：修订第 23.361 条

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：23. 修订第 23.361 条，将(a)和(b)段修改为如下内容：　2. §23.361(b) 段的子段 (1) 和 (2) 将分别重新编号为子段 (2) 和 (3)，并新增子段 (1)，内容如下：

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案

英文原文：

Proposal 8. Engine torque (Sec. 23.361). 1. The word "and" following Sec. 23.361 (a) (1) would be moved to follow Sec 23.361 (a) (2), a semicolon would be inserted to replace the period following Sec. 23.361 (a) (2), and a new Sec. 23.361 (a) (3) would be added to read as follows:

修订指令英文原文（供核对）：

23. Section 23.361 is amended by amending paragraphs (a) and (b) to read as follows:

2. Subparagraphs (1) and (2) of Sec. 23.361 (b) would be redesignated as subparagraphs (2) and (3), respectively, and a new subparagraph (1) would be added to read as follows:

▪ 本版重印条文段落：(a)、(1)、(2)、(3)、(b)、(1)、(2)、(3)

### 2.3　Amdt 23-26（1980-10-14 生效）：修订相应段落。

▪ 修订沿革：本版由 NPRM　Notice 74-5，1974-02-14 签发，39 FR 5785；Notice 74-5A，1974-05-22 签发，39 FR 5785；Notice 74-5B，1974-10-03 签发，39 FR 5785；Notice 74-33，1974-10-03 签发，39 FR 5785；Notice 74-5C，1974-11-25 签发，39 FR 5785；Notice 74-5D，1975-02-04 签发，39 FR 5785；Notice 75-10，1975-02-27 签发，39 FR 5785；Notice 75-19，1975-05-13 签发，39 FR 5785；Notice 75-26，1975-06-09 签发，39 FR 5785；Notice 75-31，1975-06-30 签发，39 FR 5785　提出；最终规章 Docket 14779, 14324，1980-08-27 签发，45 FR 60154，1980-10-14 生效。

▪ 修订指令（中文译文）：修订相应段落。

来源：Final Rule｜45 FR 60154｜FinalRule_Docket 14779_Docket14779_ 14324_Amdt21-51_23-26_25-54_27_45FR60154_1980-09-11.txt

▪ 背景与评论处置（1）：提案 8-12。关于修订第 23.361 条以重新定义涡轮发动机安装需考虑的限制发动机扭矩载荷情况并作出其他澄清性修改的提案，未收到不利评论。因此，该提案获得通过，未作实质性修改。

英文原文：

Proposal 8-12. No unfavorable comments were received on the proposal to amend Sec. 23.361 to redefine the limit engine torque load conditions to be considered for turbine engine installations and to make other clarifying changes. Accordingly, the proposal is adopted without substantive change.

修订指令英文原文（供核对）：

11. By revising Sec. 23.361 to read as follows:

▪ 本版重印条文段落：(a)、(1)、(2)、(3)、(b)、(1)、(2)、(c)、(1)、(2)、(3)

### 2.4　Amdt 23-45（1993-09-07 生效）：对 §23.361 条进行修订

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：30. 对 §23.361 条进行修订，将 (a) 段的引言文字、(a)(2) 段以及 (c) 段的引言文字修改为如下内容：　31. 对 §23.361 条进行修订，将段 (a) 的引言、(a)(2) 和 (c) 的引言修改为如下内容：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议纠正由第 23-26 号修订案引入的 §23.361 条中的一处错误。该错误显著降低了起飞功率下飞行情况所要求的结构设计扭矩水平。其意图是 §23.361(c) 段的扭矩系数适用于 §23.361(a) 段的所有情况。

英文原文：

The FAA proposed to correct an error in Section 23.361 introduced by amendment 23-26. The error significantly reduced the structural design torque levels required for flight conditions at takeoff power. The intent is that the torque factors of Section 23.361(c) apply to all Section 23.361(a) conditions.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（2）：由于所收到的唯一评论——来自欧洲联合航空局（JAA）——同意所提议的修订，美国联邦航空局（FAA）按提案通过了 §23.361 条。

英文原文：

Since the only comment received, from the JAA, agrees with the proposed change, the FAA adopts Section 23.361 as proposed.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：说明：本提案修订了 §23.361 条，以纠正第 23-28 号修正案引入第 23 部的一项非预期变更，该变更显著降低了与起飞功率下的飞行情况结合考虑所需的结构设计扭矩水平。其意图是 (c) 段的扭矩系数适用于 (a) 段的全部。本提案基于会议提案 183，并在会议上无异议获得通过。

英文原文：

Explanation: This proposal revises Section 23.361 to correct an unintended change introduced to part 23 at amendment 23-28, which significantly reduced the structural design torque levels necessary to be considered in conjunction with flight conditions at takeoff power. The intent is that the torque factors of paragraph (c) apply to all of paragraph (a).

This proposal is based on conference proposal 183 and was accepted without objection at the conference.

修订指令英文原文（供核对）：

30. Section 23.361 is amended by revising the introductory text of paragraph (a), paragraph (a)(2)., and the introductory text of paragraph (c) to read as follows:

31. Section 23.361 is amended by revising paragraph (a) introductory text, (a)(2), and (c) introductory text to read as follows:

▪ 本版重印条文段落：(a)、(2)、(c)

## 3　§23.361 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each engine mount and its supporting structure must be designed for the effects of-- | 新增 |
| (1) | The limit torque corresponding to takeoff power and propeller speed acting simultaneously with 75 percent of the limit loads from flight condition A; and | 新增 |
| (2) | The limit torque corresponding to maximum continuous power and propeller speed acting simultaneously with the limit loads from flight condition A. | 新增 |
| (b) | The limit torque is obtained by multiplying the mean torque by a factor of-- | 新增 |
| (1) | 1.33 for engines with five or more cylinders; or | 新增 |
| (2) | Two, three, or four, for engines with four, three, or two cylinders, respectively. | 新增 |
| (c) | Engine torque effects need not be investigated for any other conditions. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订：(a)、(1)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each engine mount and its supporting structure must be designed for the effects of - | 修订 |
| (1) | The limit torque corresponding to takeoff power and propeller speed acting simultaneously with 75 percent of the limit loads from flight condition A of Sec. 23.333(d); | 修订 |
| (2) | The limit torque corresponding to the maximum continuous power and propeller speed, acting simultaneously with the limit loads from flight condition A of Sec. 23.333(d); and | 修订 |
| (3) | For turbopropeller installations, in addition to the condition specified in subparagraphs (1) and (2) of this paragraph, the limit engine torque corresponding to takeoff power and propeller speed, multiplied by a factor accounting for propeller control system malfunction, including quick feathering, acting simultaneously with 1g. level flight loads. In the absence of a rational analysis, a factor of 1.6 must be used. | 新增 |
| (b) | The limit torque is obtained by multiplying the mean torque by a factor of - | 修订 |
| (1) | 1.25 for turbopropeller installations; | 修订 |
| (2) | 1.33 for engines with five or more cylinders; and | 修订 |
| (3) | Two, three or four, for engines with four, three, or two cylinders, respectively. | 新增 |

### 3.3　版本3：Amdt 23-26（1980-10-14 生效）　来源：Docket 14779, 14324，1980-08-27 签发，45 FR 60154

修订：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each engine mount and its supporting structure must be designed for the effects of-- | 修订 |
| (1) | A limit engine torque corresponding to takeoff power and propeller speed acting simultaneously with 75 percent of the limit loads from flight condition A of Sec. 23.333(d); | —（继承上版） |
| (2) | The limit engine torque as specified in Sec. 23.361(c) acting simultaneously with the time loads from flight condition A of Sec. 23.333(d); and | —（继承上版） |
| (3) | For turbopropeller installations, in addition to the conditions specified in paragraphs (a)(1) and (a)(2) of this section, a limit engine torque corresponding to takeoff power and propeller speed, multiplied by a factor accounting for propeller control system malfunction, including quick feathering, acting simultaneously with 1g level flight loads. In the absence of a rational analysis, a factor of 1.6 must be used. | 修订 |
| (b) | For turbine engine installations, the engine mounts and supporting structure must be designed to withstand each of the following: | 修订 |
| (1) | A limit engine torque load imposed by sudden engine stoppage due to malfunction or structural failure (such as compressor jamming). | —（继承上版） |
| (2) | A limit engine torque load imposed by the maximum acceleration of the engine. | —（继承上版） |
| (c) | The limit engine torque to be considered under paragraph (a)(2) of this section must be obtained by multiplying the mean torque for maximum continuous power by a factor of-- | 新增 |
| (1) | 1.25 for turbopropeller installations; | —（继承上版） |
| (2) | 1.33 for engines with five or more cylinders; and | —（继承上版） |
| (3) | Two, three, or four, for engines with four, three, or two cylinders, respectively. | 修订 |

### 3.4　版本4：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

修订：(a)、(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each engine mount and its supporting structure must be designed for the effects of-- | —（继承上版） |
| (2) | A limit engine torque corresponding to maximum continuous power and propeller speed acting simultaneously with the limit loads from flight condition A of Section 23.333(d); and | 修订 |
| (c) | The limit engine torque to be considered under paragraph (a) of this section must be obtained by multiplying the mean torque by a factor of-- | 修订 |

### 3.5　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) | Amdt 23-26 (1980-10-14) | Amdt 23-45 (1993-09-07) |
| --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | △ 修订 | ● 有 |
| (1) | ● 新增 | △ 修订 | ● 有 | ○ 无 |
| (2) | ● 新增 | △ 修订 | ● 有 | △ 修订 |
| (b) | ● 新增 | △ 修订 | △ 修订 | ○ 无 |
| (c) | ● 新增 | ○ 无 | ● 新增 | △ 修订 |
| (3) | ○ 无 | ● 新增 | △ 修订 | ○ 无 |

## 4　§23.361 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | ents also fail to recognize that the smooth power impulse characteristics of turbine engines justify a multiplying factor as low as 1.25. This proposal would correct these deficiencies in a manner consistent with the req… | Amdt 23-7 |
| 澄清性与编辑性修订 | nsidered for turbine engine installations and to make other clarifying changes. Accordingly, the proposal is adopted without substantive change. 11. By revising Sec. 23.361 to read as follows: Sec. 23.361 Engine torque.… | Amdt 23-26 |
| 国际协调 | 61(a) conditions. Since the only comment received, from the JAA, agrees with the proposed change, the FAA adopts Section 23.361 as proposed. Explanation: This proposal revises Section 23.361 to correct an unintended chan… | Amdt 23-45 |

本条取证提示 共 4 个历史版本：初始版, Amdt 23-7, Amdt 23-26, Amdt 23-45。 其中 3 个版本在本地文献中检索到明确修订指令原文；4 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 14779 | 45 FR 60154 | FinalRule_Docket 14779_Docket14779_ 14324_Amdt21-51_23-26_25-54_27_45FR60154_1980-09-11.txt |
| [6] | Final Rule（颁行） | Docket 14779 | 45 FR 7750 | FinalRule_Docket 14779_ 14324_Amdt23-25_25-51_27-17_29_45FR7750_1980-02-04_0002.txt |
| [7] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 5785 | NPRM_Notice 74-5_Docket13542_39FR5785_1974-02-15.txt |
| [8] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 18663 | NPRM_Notice 74-5A_Docket13542_39FR18663_1974-05-29.txt |
| [9] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 41319 | NPRM_Notice 74-5C_Docket13542_39FR41319_1974-11-26.txt |
| [10] | NPRM（提案） | Notice 74-5 / Docket 13542 | 40 FR 5810 | NPRM_Notice 74-5D_Docket13542_40FR5810_1975-02-07.txt |
| [11] | NPRM（提案） | Notice 74-5 / Docket 13542 | （FR 引证未在文本中标注） | NPRM_Notice 74-5B_Docket13542_1974-10-11.txt |
| [12] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [13] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [14] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [15] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.361 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 3a950664-139a-4bc6-bc90-9c40cb4794a9 |
| Amdt 23-7 | 1969-09-14 | 58a215af-6399-4c71-b92b-5554f02708ac |
| Amdt 23-26 | 1980-10-14 | cab177cb-ba24-4af2-8366-e44737f4a1ae |
| Amdt 23-45 | 1993-09-07 | e6c57619-568a-4bb0-9242-2bbf65963706 |

# § 23.363　发动机安装架侧向载荷（Side load on engine mount.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.363 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.363 |
| 条款标题（英文原文） | Side load on engine mount. |
| 中文名称 | 发动机安装架侧向载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.363 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.363 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：发动机安装架侧向载荷

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.363 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each engine mount and its supporting structure must be designed for a limit load factor in a lateral direction, for the side load on the engine mount, of not less than-- | 新增 |
| (1) | 1.33, or | 新增 |
| (2) | One-third of the limit load factor for flight condition A. | 新增 |
| (b) | The side load prescribed in paragraph (a) of this section may be assumed to be independent of other flight conditions. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.363 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.363 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 98fdd4f2-dde2-4f06-bab7-9e8e5a71407f |

# § 23.365　增压座舱载荷（Pressurized cabin loads.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.365 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.365 |
| 条款标题（英文原文） | Pressurized cabin loads. |
| 中文名称 | 增压座舱载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.365 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.365 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：增压座舱载荷

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.365 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The airplane structure must be strong enough to withstand the flight loads combined with pressure differential loads from zero up to the maximum relief valve setting. | 新增 |
| (b) | The external pressure distribution in flight, and any stress concentrations, must be accounted for. | 新增 |
| (c) | If landings may be made, with the cabin pressurized, landing loads must be combined with pressure differential loads from zero up to the maximum allowed during landing. | 新增 |
| (d) | The airplane structure must be strong enough to withstand the pressure differential loads corresponding to the maximum relief valve setting multiplied by a factor of 1.33, omitting other loads. | 新增 |
| (e) | If a pressurized cabin has two or more compartments separated by bulkheads or a floor, the primary structure must be designed for the effects of sudden release of pressure in any compartment with external doors or windows. This condition must be investigated for the effects of failure of the largest opening in the compartment. The effects of intercompartmental venting may be considered. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |
| (d) | ● 新增 |
| (e) | ● 新增 |

## 4　§23.365 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.365 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 77795bc5-b2c3-4681-b7ca-0b3c3e1feb76 |

# § 23.367　发动机失效引起的不对称载荷（Unsymmetrical loads due to engine failure.）

1969-09-14 初版　｜　1969-09-14 Amdt 23-7　｜　共 1 个历史版本

## 1　§23.367 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.367 |
| 条款标题（英文原文） | Unsymmetrical loads due to engine failure. |
| 中文名称 | 发动机失效引起的不对称载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1969-09-14 初始版） |
| 时间跨度 | 1969-09-14 → 1969-09-14 （约 0 年） |
| 初版来源 | Docket 8083，1969-08-01 签发，34 FR 13078 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.367 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 新增条款 |

### 1.3　修订脉络一览

1969 初版　新增条款

## 2　§23.367 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1969-09-14，Amdt 23-7）：发动机失效引起的不对称载荷

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：24. 新增 §23.367 条，内容如下：

来源：Final Rule｜34 FR 13078｜FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt

▪ 背景与评论处置（1）：曾提议增加一条
有必要对建议允许的替代载荷作进一步研究，这将在未来的规章制定行动中予以考虑。该通告提议修订第 23.25、23.473 和 23.1583 条，以允许设计着陆重量等于设计最大重量减去总燃油容量 25% 的重量。此外，该提案将明确，除非申请人选择设计着陆重量选项，否则设计最大重量即为用于研究地面载荷的重量，并且设计着陆重量选项仅适用于实际代表着陆载荷的情况。收到了一些评论，建议应允许申请人使用根据该提案或现行要求确定的设计着陆重量。美国联邦航空局（FAA）同意。根据现行规章取得型号合格证的飞机，其设计着陆重量低至设计最大重量的 95%，这方面的经验表明，现行规定可作为替代方案予以保留。
在这方面，现在的修订
鉴于现行
由于疲劳失效与时间相关，预计老龄飞机的失效率会更高；然而，近年来取证的飞机也出现了疲劳问题。疲劳证实和更好的维修
这一要求将比运输类飞机所要求的更为严苛，且无法以使用经验为依据证明其合理。本修订按提案内容予以采纳，但采纳为新的 §23.572 条。

英文原文：

It was proposed to add a new Sec. 23.367 to require substantiation of turbopropeller airplanes for the vertical tail loads resulting from unsymmetrical power conditions due to engine failure in combination with windmilling propeller drag loads. In response to comments received, the introductory statement to paragraph (a) has been changed to make it clear that the requirement applies to turbopropeller airplanes rather than all airplanes. One comment stated that the rule should allow a conservative alternative load if time-history test data is not available. The Commentator suggested, as a conservative alternative to the proposed rational analysis of loads build-up after failure of the critical engine and subsequent to pilot corrective action, that the loads may be considered as a static yaw condition based on the maximum windmilling drag of the critical engine. The FAA does not agree that this is a conservative alternative. For example, there is no multiplying factor to account conservatively for likely dynamic airplane overswing yaw which can be expected after sudden engine failure. Further study of the suggested allowance of an alternative load is necessary and it will be considered in connection with future rule-making action.

The notice proposed to amend Secs. 23.25, 23.473, and 23.1583 to permit a design landing weight equal to the design maximum weight less the weight of 25 percent of the total fuel capacity. In addition, the proposal would make it clear that the design maximum weight is the weight to be used in investigating ground loads unless the design landing weight option is chosen by the applicant, and that the design landing weight option applies only to the conditions actually representing landing loads. Comments were received suggesting that an applicant should be permitted to use a design landing weight determined either in accordance with the proposal or the current requirements. The FAA agrees. Experience with airplanes that have been type certificated under the current rule with a design landing weight as low as 95 percent of design maximum weight indicates that the present provisions can be retained as an alternative. In this connection, the amendment now permits the design landing weight to be as low as 95 percent of the maximum weight if the minimum fuel capacity is enough for at least one-half hour of operation at maximum continuous power plus a capacity equal to a fuel weight which is the difference between the design maximum weight and the design landing weight or as low as the design maximum weight less the weight of 25 percent of the total fuel capacity. In addition, the current reference to Sec. 25.1001 in Sec. 23.473(c) has been changed to Sec. 23.1001. Section 25.1001 was amended subsequent to the issuance of Notice 67-14 containing these proposals and does not now reflect the appropriate requirement for Part 23. Therefore, the appropriate provisions of Sec. 25.1001 in effect on the date that Notice 67-14 was issued have been incorporated into a new Sec. 23.1001.

The notice proposed to amend Sec. 23.571 to require an effective fatigue evaluation of the wing and associated structures. Inasmuch as the title of current Sec. 23.571 might be misleading insofar as these requirements are concerned, the proposal has been adopted as new Sec. 23.572. Numerous comments were received objecting to this proposal. In this connection, it was stated that (1) the current strength requirements and design practices are conservative and adequate to prevent serious fatigue problems; (2) corrosion, prior abuse, and special purpose operations cause fatigue failure, rather then lack of design fatigue strength; (3) fatigue substantiation will not eliminate cracks, and a better maintenance program would be more effective; and (4) sufficient data is not available to establish load spectra for fatigue substantiation. The FAA does not agree. Service experience and discussions with industry of designs which have sustained fatigue failures indicate that present design practices do not adequately account for fatigue. Corrosion, prior abuse, and special purpose operations may contribute to fatigue failure, but the primary reason for such failures is lack of strength. Since fatigue failures are time-dependent, a higher failure rate among older airplanes is to be expected; however, fatigue problems have arisen in airplanes certificated in recent years. Neither fatigue substantiation nor better maintenance programs will eliminate all cracks. The purpose of the proposed rule is to prevent catastrophic failures. Furthermore, Part 23 airplanes are not designed on a redundant-structure, fail-safe basis, for which maintenance alone would be sufficient. Both fatigue substantiation and a good maintenance program are needed. Reasonable load spectra can and have been established from the extensive gust data which is available, and reasonable and acceptable methods of compliance have been established and widely publicized. However, while it has been determined that fatigue substantiation is necessary, the FAA does not agree with one comment which suggested that the proposed requirement should be expanded to a full limit load fail-safe requirement with a design objective of at least 10,000 hours and a test life of 30,000 flights. Such a requirement would be more severe than that required for transport category airplanes and cannot be justified on the basis of service experience. The amendment is adopted as proposed, except that it is adopted as new Sec. 23.572.

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（2）：提案 9. 涡轮发动机失效引起的非对称载荷（新增 §23.367 条）。将新增 §23.367 条，内容如下：

英文原文：

Proposal 9. Unsymmetrical loads due to turbine engine failure (new Sec. 23.367). A new Sec. 23.367 would be added to read as follows:

修订指令英文原文（供核对）：

24. A new Sec. 23.367 is added to read as follows:

## 3　§23.367 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Turbopropeller airplanes must be designed for the unsymmetrical loads resulting from the failure of the critical engine including the following conditions in combination with a single malfunction of the propeller drag limiting system, considering the probable pilot corrective action on the flight controls: | 新增 |
| (1) | At speeds between and , the loads resulting from power failure because of fuel flow interruption are considered to be limit loads. | 新增 |
| (2) | At speeds between and , the loads resulting from the disconnection of the engine compressor from the turbine or from loss of the turbine blades are considered to be ultimate loads. | 新增 |
| (3) | The time history of the thrust decay and drag buildup occurring as a result of the prescribed engine failures must be substantiated by test or other data applicable to the particular engine-propeller combination. | 新增 |
| (4) | The timing and magnitude of the probable pilot corrective action must be conservatively estimated, considering the characteristics of the particular engine-propeller-airplane combination. | 新增 |
| (b) | Pilot corrective action may be assumed to be initiated at the time maximum yawing velocity is reached, but not earlier than 2 seconds after the engine failure. The magnitude of the corrective action may be based on the limit pilot forces specified in Sec. 23.397 except that lower forces may be assumed where it is shown by analysis or test that these forces can control the yaw and roll resulting from the prescribed engine failure conditions. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-7 (1969-09-14) |
| --- | --- |
| (a) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (3) | ● 新增 |
| (4) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.367 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | hould allow a conservative alternative load if time-history test data is not available. The Commentator suggested, as a conservative alternative to the proposed rational analysis of loads build-up after failure of the cr… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-7。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [2] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.367 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-7 | 1969-09-14 | 105a7d7a-d26b-4092-bdf1-ee5d9e442055 |

# § 23.369　后升力桁架的专用条件（(Rear lift truss.)）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 4 个历史版本

## 1　§23.369 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.369 |
| 条款标题（英文原文） | (Rear lift truss.) |
| 中文名称 | 后升力桁架的专用条件 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 34 FR 17509 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.369 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订 |
| 3 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订 |
| 4 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订　→　1993 23-45　修订　→　1996 23-48　修订

## 2　§23.369 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：后升力桁架的专用条件

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：修订 §23.369(a) 条

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：25. 修订 §23.369(a) 条，删除公式

修订指令英文原文（供核对）：

25. Section 23.369(a) is amended by striking out the formula

### 2.3　Amdt 23-45（1993-09-07 生效）：通过修订标题对 23.369 条进行修订

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：31. 通过修订标题对 23.369 条进行修订，内容如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议修改第 23.369 条的标题，删除标题开头的短语“专用条件”。第 23.369 条的内容保持不变。唯一的评论者欧洲联合航空局（JAA）同意这一编辑性修改，并询问这是否是结构在反流情况下唯一需要特殊考虑的部分。基于 30 年的使用历史，美国联邦航空局（FAA）未了解到有任何额外的特殊考虑需要。美国联邦航空局（FAA）按提议采纳第 23.369 条。

英文原文：

The FAA proposed to change the heading of Section 23.369 by eliminating the phrase "Special conditions for" at the beginning of the heading. The content of Section 23.369 remains unchanged.

The one commenter, the JAA, agrees with the editorial change and asks whether this is the only part of the structure needing special consideration in reversed airflow conditions. The FAA is not aware of any additional need for special consideration based on 30 years of service history. The FAA adopts Section 23.369 as proposed.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（2）：唯一的评论者欧洲联合航空局（JAA）同意这一编辑性修改，并询问这是否是结构中唯一需要在逆向气流条件下予以特别考虑的部位。根据30

英文原文：

The one commenter, the JAA, agrees with the editorial change and asks whether this is the only part of the structure needing special consideration in reversed airflow conditions. The FAA is not aware of any additional need for special consideration based on 30 years of service history. The FAA adopts Section 23.369 as proposed.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：说明：本提案通过删除标题块开头的短语“专用条件”更改了第 23.369 条的标题。第 23.369 条的内容保持不变。

英文原文：

Explanation: This proposal changes the title of Section 23.369 by eliminating the phrase "Special conditions for" at the beginning of the title block. The content of Section 23.369 remains unchanged.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（4）：会议提案 196 提议全部删除第 23.369 条，因为提交者主张在过去 30 年中对带有后升力桁架的外撑式机翼几乎没有兴趣。一名评论者表示同意。
美国联邦航空局（FAA）已确定第 23.369 条的要求是有效的，并且继续适用于第 23 部。然而，美国联邦航空局（FAA）得出结论认为，“专用条件”一词应仅限于第 21.16 条中规定的设计特征，并且不应用于描述现有设计的要求。

英文原文：

Conference proposal 196 proposes to delete Section 23.369 in its entirely because the submitter contended that there has been little interest in externally braced wings with a rear lift truss for the past 30 years. One commenter agreed.

The FAA has determined that the requirements of Section 23.369 are valid and continue to be appropriate for part 23. However, the FAA has concluded that the term "special condition" should be limited to those design features set forth in Section 21.16 and should not be used to describe requirements for existing designs.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（5）：美国联邦航空局（FAA）已确定，第23.369条的要求是有效的，并且继续适用于第23部。然而，美国联邦航空局（FAA）得出结论认为，“专用条件”一词应仅限于第21.16条中规定的设计特征，不应被用来描述对现有设计的要求。

英文原文：

The FAA has determined that the requirements of Section 23.369 are valid and continue to be appropriate for part 23. However, the FAA has concluded that the term "special condition" should be limited to those design features set forth in Section 21.16 and should not be used to describe requirements for existing designs.

修订指令英文原文（供核对）：

31. Section 23.369 is amended by revising the heading to read as follows:

### 2.4　Amdt 23-48（1996-03-11 生效）：§23.369 条修订如下：修订 (a) 段

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：§23.369 条修订如下：修订 (a) 段，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议通过修订该方程并增加翼载荷（W/S）的定义来修订 §23.369 条，以澄清该规章。

英文原文：

The FAA proposed to amend Sec. 23.369 by amending the equation and by adding a definition for wing loading (W/S) to clarify the rule.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：本提案将通过修订方程式并增加翼载荷（W/S）的定义来修订 §23.369 条，以作澄清并与欧洲联合航空要求（JAR）23 相协调统一。

英文原文：

This proposal would amend Sec. 23.369 by amending the equation and by adding a definition for wing loading (W/S) for clarification and to harmonize with JAR 23.

修订指令英文原文（供核对）：

10. Section 23.369(a) is revised to read as follows:

▪ 本版重印条文段落：(a)

### 2.5　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.369 的实际改动 |
| --- | --- | --- | --- |
| 34 FR 17509 | 1969-10-30 | 更正通告 (CFR 溯源注) |  |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.369 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | If a rear lift truss is used, it must be designed for conditions of reversed airflow at a design speed of-- V = 10 + 10 (m.p.h.). | 新增 |
| (b) | Either aerodynamic data for the particular wing section used, or a value of CL equaling -0.8 with a chordwise distribution that is triangular between a peak at the trailing edge and zero at the leading edge, must be used. Control Surface and System Loads | 新增 |

### 3.2　版本2：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | If a rear lift truss is used, it must be designed to withstand conditions of reversed airflow at a design speed of-- V = 8.7 (W/S) + 8.7 (knots), where W/S = wing loading at design maximum takeoff weight. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- |
| (a) | ● 新增 | △ 修订 |
| (b) | ● 新增 | ○ 无 |

## 4　§23.369 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | of Section 23.369 remains unchanged. The one commenter, the JAA, agrees with the editorial change and asks whether this is the only part of the structure needing special consideration in reversed airflow conditions. The… | Amdt 23-45 |
| 国际协调 | definition for wing loading (W/S) for clarification and to harmonize with JAR 23. 10. Section 23.369(a) is revised to read as follows: Sec. 23.369 Rear lift truss. (a) If a rear lift truss is used, it must be designed t… | Amdt 23-48 |

本条取证提示 共 4 个历史版本：初始版, Amdt 23-7, Amdt 23-45, Amdt 23-48。 其中 3 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：34 FR 17509，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [6] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [7] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [8] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [9] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [10] | 更正通告 / 校正 | 34 FR 17509 | 1969-10-30 | Correction_34FR17509_1969-10-30.clean.txt |
| [11] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [12] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.369 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | a835d702-5ce5-439a-9ac5-298e8e879181 |
| Amdt 23-7 | 1969-09-14 | 222621bb-c7de-48d9-91c2-e4317c844e60 |
| Amdt 23-45 | 1993-09-07 | 117b2c53-100c-4838-885a-26f62e84cb67 |
| Amdt 23-48 | 1996-03-11 | f3012a78-f4b8-4eee-b25d-1b33d09f969b |

# § 23.371　陀螺载荷与气动载荷（Gyroscopic loads.）

1969-09-14 初版　｜　1996-03-11 Amdt 23-48　｜　共 4 个历史版本

## 1　§23.371 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.371 |
| 条款标题（英文原文） | Gyroscopic loads. |
| 中文名称 | 陀螺载荷与气动载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1969-09-14 初始版） |
| 时间跨度 | 1969-09-14 → 1996-03-11 （约 27 年） |
| 初版来源 | Docket 8083，1969-08-01 签发，34 FR 13078 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.371 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 新增条款 |
| 2 | Amdt 23-26 | 1980-10-14 | Notice 74-5，1974-02-14 签发，39 FR 5785 Notice 74-5A，1974-05-22 签发，39 FR 5785 Notice 74-5B，1974-10-03 签发，39 FR 5785 Notice 74-33，1974-10-03 签发，39 FR 5785 Notice 74-5C，1974-11-25 签发，39 FR 5785 Notice 74-5D，1975-02-04 签发，39 FR 5785 Notice 75-10，1975-02-27 签发，39 FR 5785 Notice 75-19，1975-05-13 签发，39 FR 5785 Notice 75-26，1975-06-09 签发，39 FR 5785 Notice 75-31，1975-06-30 签发，39 FR 5785 | Docket 14779, 14324，1980-08-27 签发，45 FR 60154 | 删除内容 |
| 3 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订 |
| 4 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订：(a) |

### 1.3　修订脉络一览

1969 初版　新增条款　→　1980 23-26　删除内容　→　1993 23-45　修订　→　1996 23-48　修订：(a)

## 2　§23.371 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1969-09-14，Amdt 23-7）：陀螺载荷与气动载荷

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：26. 新增 §23.371 条，内容如下：

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案 10。涡轮螺旋桨动力装置陀螺载荷（新增 §23.371 条）。将新增一条 §23.371，内容如下：

英文原文：

Proposal 10. Turbopropeller installation gyroscopic loads (new Sec. 23.371). A new Sec. 23.371 would be added to read as follows:

修订指令英文原文（供核对）：

26. A new Sec. 23.371 is added to read as follows:

### 2.2　Amdt 23-26（1980-10-14 生效）：删除 §23.371 条引导语中的“涡轮螺旋桨”一词

▪ 修订沿革：本版由 NPRM　Notice 74-5，1974-02-14 签发，39 FR 5785；Notice 74-5A，1974-05-22 签发，39 FR 5785；Notice 74-5B，1974-10-03 签发，39 FR 5785；Notice 74-33，1974-10-03 签发，39 FR 5785；Notice 74-5C，1974-11-25 签发，39 FR 5785；Notice 74-5D，1975-02-04 签发，39 FR 5785；Notice 75-10，1975-02-27 签发，39 FR 5785；Notice 75-19，1975-05-13 签发，39 FR 5785；Notice 75-26，1975-06-09 签发，39 FR 5785；Notice 75-31，1975-06-30 签发，39 FR 5785　提出；最终规章 Docket 14779, 14324，1980-08-27 签发，45 FR 60154，1980-10-14 生效。

▪ 修订指令（中文译文）：12. 删除 §23.371 条引导语中的“涡轮螺旋桨”一词，并在其位置上插入“涡轮”一词。

来源：Final Rule｜45 FR 60154｜FinalRule_Docket 14779_Docket14779_ 14324_Amdt21-51_23-26_25-54_27_45FR60154_1980-09-11.txt

▪ 背景与评论处置（1）：提案 8-13。美国联邦航空局（FAA）不同意一位评论者的建议，该评论者建议修改 §23.371 条的引导语，使陀螺载荷要求适用于活塞式发动机以及涡轮发动机。美国联邦航空局（FAA）没有信息表明本规章有必要涵盖活塞式发动机，评论者也没有提交任何此类信息。

英文原文：

Proposal 8-13. The FAA does not agree with a commenter who suggests that the lead-in of Sec. 23.371 be revised to make the gyroscopic load requirements applicable to piston as well as turbine engines. The FAA has no information to indicate a need for coverage of piston engines in this regulation, nor was any submitted by the commenter.

来源：Final Rule｜45 FR 60154｜FinalRule_Docket 14779_Docket14779_ 14324_Amdt21-51_23-26_25-54_27_45FR60154_1980-09-11.txt

▪ 背景与评论处置（2）：另一位评论者赞同 §23.371 条，认为根据 §23.371(a) 段对载荷进行的合理分析是 §23.371(b) 段中规定载荷的替代方法。这一假设是正确的。在这方面未对 §23.371 条提出修改。§23.371 条予以采纳，无实质性修改。

英文原文：

Another commenter concurs with Sec. 23.371, assuming that a rational analysis of loads under Sec. 23.371(a) is an alternate to the loads specified in Sec. 23.371(b). This assumption is correct. No change to Sec. 23.371 was proposed in this regard. Sec. 23.371 is adopted without substantive change.

修订指令英文原文（供核对）：

12. By deleting the word "turbopropeller" in the lead-in of Sec. 23.371 and inserting the word "turbine" in its place.

### 2.3　Amdt 23-45（1993-09-07 生效）：修订 §23.371 条

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：32. 修订 §23.371 条，将本条的标题和导言文字修改如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议将气动载荷与第 23.371 条要求的陀螺载荷一并纳入发动机架的设计。唯一一位评论者——欧洲联合航空局（JAA），建议增加"combined"一词，使引言表述为"按陀螺与气动组合载荷设计 * * *"。美国联邦航空局（FAA）同意并修订了该引言。提案中的第 23.371 条经上述讨论的修改后予以采纳。

英文原文：

The FAA proposed to include aerodynamic loads in the design of the engine mount with the gyroscopic loads required by Section 23.371. The one commenter, the JAA, suggests adding the word "combined" so the introductory statement reads "designed for the combined gyroscopic and aerodynamic loads * * *." The FAA agrees and revises the introductory statement. The proposed Section 23.371 is adopted with the change discussed above.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（2）：说明：本提案除第 23.371 条目前所要求的陀螺载荷外，还将气动载荷纳入发动机架设计。

英文原文：

Explanation: This proposal includes the aerodynamic loads in the design of the engine mount in addition to the gyroscopic loads currently required by Section 23.371.

修订指令英文原文（供核对）：

32. Section 23.371 is amended by revising the heading and the introductory text of this section to read as follows:

### 2.4　Amdt 23-48（1996-03-11 生效）：§23.371 条修订如下

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：11. §23.371 条修订如下：　§23.371(a) 段将被修订和重新组织，将现有文本指定为段 (a)，并新增段 (b) 和 )c)。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订并重组§23.371，将现有文本指定为(a)段，并新增(b)和(c)段。对提议的(a)段文本的修订将删除涡轮发动机的限制；增加惯性载荷；并将"发动机"替换为"发动机和螺旋桨（如适用）"。提议的变更澄清了这些要求适用于所有第23部飞机。美国联邦航空局提议新增(b)段，以澄清和区分经批准用于特技机动的飞机的要求。

英文原文：

The FAA proposed to revise and reorganize Sec. 23.371 by designating the existing text as paragraph (a) and adding new paragraphs (b) and (c).

The proposed revisions to the text of proposed paragraph (a) would delete the limitation for turbine powered engines; add inertial loads; and replace the word "engines" with "engine(s) and propeller(s), if applicable". The proposed changes clarify that these requirements apply to all part 23 airplanes.

The FAA proposed a new paragraph (b) to clarify and distinguish the requirements for airplanes approved for aerobatic maneuvers.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局（FAA）提议新增(c)段，以明确通勤类飞机除须符合第23.371(a)段的要求外，还必须符合第23.341条中的阵风条件。

英文原文：

The FAA proposed new paragraph (c) to clarify that commuter category airplanes must comply with the gust conditions in Sec. 23.341 in addition to the requirement of Sec. 23.371(a).

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（3）：评论：欧洲联合航空局（JAA）建议，JAR 23.371(b) 开头有而 §23.371(b) 没有的“此外”一词，可能导致对经批准可进行特技机动的飞机的要求产生误读。欧洲联合航空局（JAA）担心读者可能认为，对于经批准可进行特技机动的飞机，(b) 段的要求是替代而非补充 (a) 段的要求。

美国联邦航空局（FAA）回应：美国联邦航空局（FAA）知悉“此外”一词出现在欧洲联合航空要求（JAR）中，并理解欧洲联合航空局（JAA）认为该词对于防止将经批准可进行特技机动的飞机解读为仅需符合 (b) 段要求是必要的。

英文原文：

Comment: The JAA recommended that the words "In addition," which appear at the beginning of JAR 23.371(b) but not in Sec. 23.371(b), could result in misreading the requirements for airplanes approved for aerobatic maneuvers. The JAA's concern is that a reader might think that the requirements of paragraph (b) for airplanes approved for aerobatic maneuvers are in place of, rather than in addition to, the requirements of paragraph (a).

FAA Response: The FAA is aware that the words "in addition" appear in the JAR and understands that the JAA believes the words are necessary to prevent an interpretation that airplanes approved for aerobatic maneuvers need only comply with the requirements of paragraph (b).

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（4）：拟议的新(c)段将澄清，除§23.371(a)段的要求外，通勤类飞机还必须符合§23.341中的阵风情况。这一澄清对于与欧洲联合航空要求（JAR）协调统一是必要的。

英文原文：

Proposed new paragraph (c) would clarify that commuter category airplanes must comply with the gust conditions in Sec. 23.341 in addition to the requirement of Sec. 23.371(a). This clarification is necessary to harmonize with the JAR.

修订指令英文原文（供核对）：

11. Section 23.371 is revised to read as follows:

Section 23.371 (a) would be revised and reorganized by designating the existing text as paragraph (a) and adding new paragraphs (b) and )c).

▪ 本版重印条文段落：(a)、(1)、(2)、(i)、(ii)、(iii)、(iv)、(b)、(c)

## 3　§23.371 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The conditions prescribed in Secs. 23.351 and 23.423. | 新增 |
| (b) | All possible combinations of the following: | 新增 |
| (1) | A yaw velocity of 2.5 radians per second. | 新增 |
| (2) | A pitch velocity of 1 radian per second. | 新增 |
| (3) | A normal load factor of 2.5. | 新增 |
| (4) | Maximum continuous thrust. | 新增 |

### 3.2　版本2：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each engine mount and its supporting structure must be designed for the gyroscopic, inertial, and aerodynamic loads that result, with the engine(s) and propeller(s), if applicable, at maximum continuous r.p.m., under either: | 修订 |
| (1) | The conditions prescribed in Sec. 23.351 and Sec. 23.423; or | 修订 |
| (2) | All possible combinations of the following-- | 修订 |
| (i) | A yaw velocity of 2.5 radians per second; | 新增 |
| (ii) | A pitch velocity of 1.0 radian per second; | 新增 |
| (iii) | A normal load factor of 2.5; and | 新增 |
| (iv) | Maximum continuous thrust. | 新增 |
| (b) | For airplanes approved for aerobatic maneuvers, each engine mount and its supporting structure must meet the requirements of paragraph (a) of this section and be designed to withstand the load factors expected during combined maximum yaw and pitch velocities. | 修订 |
| (c) | For airplanes certificated in the commuter category, each engine mount and its supporting structure must meet the requirements of paragraph (a) of this section and the gust conditions specified in Sec. 23.341 of this part. | 新增 |

### 3.3　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | Amdt 23-7 (1969-09-14) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- |
| (a) | ● 新增 | △ 修订 |
| (b) | ● 新增 | △ 修订 |
| (1) | ● 新增 | △ 修订 |
| (2) | ● 新增 | △ 修订 |
| (3) | ● 新增 | ○ 无 |
| (4) | ● 新增 | ○ 无 |
| (i) | ○ 无 | ● 新增 |
| (ii) | ○ 无 | ● 新增 |
| (iii) | ○ 无 | ● 新增 |
| (iv) | ○ 无 | ● 新增 |
| (c) | ○ 无 | ● 新增 |

## 4　§23.371 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | ic loads required by Section 23.371. The one commenter, the JAA, suggests adding the word "combined" so the introductory statement reads "designed for the combined gyroscopic and aerodynamic loads * * *." The FAA agrees… | Amdt 23-45 |
| 国际协调 | addition to the requirement of Sec. 23.371(a). Comment: The JAA recommended that the words "In addition," which appear at the beginning of JAR 23.371(b) but not in Sec. 23.371(b), could result in misreading the requireme… | Amdt 23-48 |

本条取证提示 共 4 个历史版本：Amdt 23-7, Amdt 23-26, Amdt 23-45, Amdt 23-48。 其中 4 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [2] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [3] | Final Rule（颁行） | Docket 14779 | 45 FR 60154 | FinalRule_Docket 14779_Docket14779_ 14324_Amdt21-51_23-26_25-54_27_45FR60154_1980-09-11.txt |
| [4] | Final Rule（颁行） | Docket 14779 | 45 FR 7750 | FinalRule_Docket 14779_ 14324_Amdt23-25_25-51_27-17_29_45FR7750_1980-02-04_0002.txt |
| [5] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 5785 | NPRM_Notice 74-5_Docket13542_39FR5785_1974-02-15.txt |
| [6] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 18663 | NPRM_Notice 74-5A_Docket13542_39FR18663_1974-05-29.txt |
| [7] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 41319 | NPRM_Notice 74-5C_Docket13542_39FR41319_1974-11-26.txt |
| [8] | NPRM（提案） | Notice 74-5 / Docket 13542 | 40 FR 5810 | NPRM_Notice 74-5D_Docket13542_40FR5810_1975-02-07.txt |
| [9] | NPRM（提案） | Notice 74-5 / Docket 13542 | （FR 引证未在文本中标注） | NPRM_Notice 74-5B_Docket13542_1974-10-11.txt |
| [10] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [11] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [12] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [13] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [14] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.371 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-7 | 1969-09-14 | 67cc744d-666e-4a37-86c8-c7752a1ae989 |
| Amdt 23-26 | 1980-10-14 | ecaedf84-8d5f-4a82-9a09-124304b5bd6c |
| Amdt 23-45 | 1993-09-07 | 4c72c5a4-bf6d-41b2-9bb5-f1fa669f3618 |
| Amdt 23-48 | 1996-03-11 | efcc98ba-4b76-498a-9feb-1b010891d299 |

# § 23.373　减速装置（限速装置）（Speed control devices.）

1969-09-14 初版　｜　1969-09-14 Amdt 23-7　｜　共 1 个历史版本

## 1　§23.373 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.373 |
| 条款标题（英文原文） | Speed control devices. |
| 中文名称 | 减速装置（限速装置） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1969-09-14 初始版） |
| 时间跨度 | 1969-09-14 → 1969-09-14 （约 0 年） |
| 初版来源 | Docket 8083，1969-08-01 签发，34 FR 13078 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.373 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 新增条款：(a) |

### 1.3　修订脉络一览

1969 初版　新增条款：(a)

## 2　§23.373 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1969-09-14，Amdt 23-7）：减速装置（限速装置）

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：27. 新增一条 §23.373，内容如下：

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案11. 速度控制装置（新增§23.373条）。将新增§23.373条，内容如下：

英文原文：

Proposal 11. Speed control devices (New Sec. 23.373). A new Sec. 23.373 would be added to read as follows:

修订指令英文原文（供核对）：

27. A new Sec. 23.373 is added to read as follows:

## 3　§23.373 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The airplane must be designed for the symmetrical maneuvers and gusts prescribed in Secs. 23.333, 23.337, and 23.341, and the yawing maneuvers and lateral gusts in Secs. 23.441 and 23.443, with the device extended at speeds up to the placard device extended speed; and | 新增 |
| (b) | If the device has automatic operating or load limiting features, the airplane must be designed for the maneuver and gust conditions prescribed in paragraph (a) of this section at the speeds and corresponding device positions that the mechanism allows. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-7 (1969-09-14) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.373 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-7。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [2] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.373 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-7 | 1969-09-14 | 11fb3144-744c-495e-98cf-e4dde73a32ba |

# § 23.391　操纵面载荷（Control surface loads.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 2 个历史版本

## 1　§23.391 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.391 |
| 条款标题（英文原文） | Control surface loads. |
| 中文名称 | 操纵面载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.391 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 删除段落：(b) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1996 23-48　删除段落：(b)

## 2　§23.391 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：操纵面载荷

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-48（1996-03-11 生效）：修订§23.391

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：12. 修订§23.391，删除(b)段，并从剩余文本中删除'(a)'的标识。　12. 修订第 23.391 条，删除段 (b)，并从剩余文本中删除标号“(a)”。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订第23.391条，删除(b)段。(b)段引用了附录B，该附录已被第23-42号修正案（56 FR 344，1991年1月3日）删除。

英文原文：

The FAA proposed to revise Sec. 23.391 by deleting paragraph (b). Paragraph (b) references Appendix B, which was removed by Amendment No. 23-42 (56 FR 344, January 3, 1991).

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：本提案将修订 §23.391 条，删除段 并移除段的编号。现行段 是对附录 B 中操纵力替代值的引用。附录 B 此前已由第 23-42 号修正案 (56 FR 344, January 3, 1991) 移除。

英文原文：

This proposal would revise Sec. 23.391 by deleting paragraph (b) and removing the designation for paragraph(a). Current paragraph (b) is a reference to alternative values of control loading in Appendix B. Appendix B was previously removed by Amendment No. 23-42 (56 FR 344, January 3, 1991)

修订指令英文原文（供核对）：

12. Section 23.391 is amended by removing paragraph (b) and removing the designation of "(a)" from the remaining text.

12. Section 23.391 is amended by removing paragraph (b) and removing the designation "(a)" from the remaining text.

## 3　§23.391 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The control surface loads specified in Secs. 23.397 through 23.459 are assumed to occur in the conditions described in Secs. 23.331 through 23.351. | 新增 |
| (b) | If allowed by the following sections, the values of control surface loading in Appendix B of this part may be used, instead of particular control surface data, to determine the detailed rational requirements of Secs. 23.397 through 23.459, unless these values result in unrealistic loads. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.391 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 2 个历史版本：初始版, Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [4] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [5] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [6] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [7] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.391 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | b2045fb6-2c00-4a78-a75d-88249a9946ce |
| Amdt 23-48 | 1996-03-11 | 6d827a37-4f92-437d-ae4f-c409582e9a92 |

# § 23.393　平行于铰链线的载荷（Loads parallel to hinge line.）

1996-03-11 初版　｜　1996-03-11 Amdt 23-48　｜　共 1 个历史版本

## 1　§23.393 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.393 |
| 条款标题（英文原文） | Loads parallel to hinge line. |
| 中文名称 | 平行于铰链线的载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1996-03-11 初始版） |
| 时间跨度 | 1996-03-11 → 1996-03-11 （约 0 年） |
| 初版来源 | Docket 27805，1996-01-29 签发，61 FR 5138 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.393 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增条款 |

### 1.3　修订脉络一览

1996 初版　新增条款

## 2　§23.393 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1996-03-11，Amdt 23-48）：平行于铰链线的载荷

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：13. 新增第23.393条，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议新增一条§23.393。拟议的新§23.393将包含§23.657(c)段中关于平行于铰链线的载荷的要求的修改版本，这些载荷被提议从§23.657中删除。

英文原文：

The FAA proposed a new Sec. 23.393. Proposed new Sec. 23.393 would contain a modified version of the requirement of Sec. 23.657(c) concerning loads parallel to the hinge line, which were proposed to be deleted from Sec. 23.657.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局提议修订 §23.657，删除涵盖平行于铰链线的载荷的 (c) 段，因为该内容将包含在拟议的 §23.393 中。

英文原文：

The FAA proposed to amend Sec. 23.657 by deleting paragraph (c) that covers loads parallel to the hinge line because it would be covered in proposed Sec. 23.393.

修订指令英文原文（供核对）：

13. A new Sec. 23.393 is added to read as follows:

## 3　§23.393 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Control surfaces and supporting hinge brackets must be designed to withstand inertial loads acting parallel to the hinge line. | 新增 |
| (b) | In the absence of more rational data, the inertial loads may be assumed to be equal to KW, where-- | 新增 |
| (1) | K = 24 for vertical surfaces; | 新增 |
| (2) | K = 12 for horizontal surfaces; and | 新增 |
| (3) | W = weight of the movable surfaces. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-48 (1996-03-11) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (3) | ● 新增 |

## 4　§23.393 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [2] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [3] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.393 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-48 | 1996-03-11 | 40f54a07-9809-462f-be00-e972b932b88d |

# § 23.395　操纵系统载荷（Control system [loads.]）

1965-02-01 初版　｜　1969-09-14 Amdt 23-7　｜　共 2 个历史版本

## 1　§23.395 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.395 |
| 条款标题（英文原文） | Control system [loads.] |
| 中文名称 | 操纵系统载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1969-09-14 （约 4 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.395 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 新增内容：(1)、(a)、(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　新增内容：(1)、(a)、(c)

## 2　§23.395 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：操纵系统载荷

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第 23.395 条通过修订标题、在 (a) 段的子段 (1) 和

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：28. 第 23.395 条通过修订标题、在 (a) 段的子段 (1) 和 (2) 末尾增加一个新句子，并修订 (c) 段进行修订，内容如下：

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案 12. 操纵系统设计中使用规定的“最大”和“最小”飞行员力（§23.395 条）。1. 将通过在第 23.395 条(a)(1) 段末尾增加以下句子对其进行修订：“用于设计的飞行员力不必超过 §23.397(b) 条中规定的最大力。”

英文原文：

Proposal 12. Use of prescribed "maximum" and "minimum" pilot forces for control system design (Sec. 23.395). 1. Section 23.395 (a) (1) would be amended by adding the following sentence at the end thereof: "Pilot forces used for design need not exceed the maximum forces prescribed in Sec. 23.397 (b)."

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（2）：说明。在 §23.395 条中对 §23.397 条限制驾驶员力的唯一设计引用是 §23.395(c) 段中的一般性引用。这一引用仅仅是编辑性的，并未清楚说明这些驾驶员力的规章效力。本提案将纠正这一缺陷，而不作实质性变更。

英文原文：

Explanation. The only design reference in Sec. 23.395 to the limit pilot forces of Sec. 23.397 is the general reference in Sec. 23.395 (c). This reference is a mere editorial one that does not clearly state the regulatory effect of those pilot forces. This proposal would correct this deficiency without substantive change.

修订指令英文原文（供核对）：

28. Section 23.395 is amended by amending the title and adding a new sentence at the end of subparagraphs (1) and (2) of paragraph (a), and amending paragraph (c), to read as follows:

▪ 本版重印条文段落：(a)、(1)、(2)、(c)

## 3　§23.395 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each flight control system and its supporting structure must be designed for loads corresponding to at least 125 percent of the computed hinge moments of the movable control surface in the conditions prescribed in Secs. 23.391 through 23.459. In addition, the following apply: | 新增 |
| (1) | The system limit loads need not exceed the higher of the loads that can be produced by the pilot and automatic devices operating the controls. However, autopilot forces need not be added to pilot forces. The system must be designed for the maximum effort of the pilot or autopilot, whichever is higher. In addition, if the pilot and the autopilot act in opposition, the part of the system between them may be designed for the maximum effort of the one that imposes the lesser load. | 新增 |
| (2) | The design must, in any case, provide a rugged system for service use, considering jamming, ground gusts, taxiing downwind, control inertia, and friction. | 新增 |
| (b) | A 125 percent factor on computed hinge moments must be used to design elevator, aileron, and rudder systems. However, a factor as low as 1.0 may be used if hinge moments are based on accurate flight test data, the exact reduction depending upon the accuracy and reliability of the data. | 新增 |
| (c) | Acceptable maximum and minimum pilot forces for elevator, aileron, and rudder controls are shown in Sec. 23.397(b). These pilot forces are assumed to act at the appropriate control grips or pads as they would in flight, and to react at the attachments of the control system to the control surface horns. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

新增内容：(1)、(a)、(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (1) | * * * Pilot forces used for design need not exceed the maximum forces prescribed in Sec. 23.397(b). | 修订 |
| (2) | * * * Compliance with this subparagraph may be shown by designing for loads resulting from application of the minimum forces prescribed in Sec. 23.397(b). | 修订 |
| (c) | Pilot forces used for design are assumed to act at the appropriate control grips or pads as they would in flight, and to react at the attachments of the control system to the control surface horns. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) |
| --- | --- | --- |
| (a) | ● 新增 | △ 修订 |
| (1) | ● 新增 | △ 修订 |
| (2) | ● 新增 | △ 修订 |
| (b) | ● 新增 | ○ 无 |
| (c) | ● 新增 | △ 修订 |

## 4　§23.395 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 澄清性与编辑性修订 | eral reference in Sec. 23.395 (c). This reference is a mere editorial one that does not clearly state the regulatory effect of those pilot forces. This proposal would correct this deficiency without substantive change. 2… | Amdt 23-7 |

本条取证提示 共 2 个历史版本：初始版, Amdt 23-7。 其中 1 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [6] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.395 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 804541d7-9c3b-4513-8ffc-98bdb59e6a09 |
| Amdt 23-7 | 1969-09-14 | 83b82636-c5bf-4a8a-95b7-8ba472e36718 |

# § 23.397　驾驶员限制操纵力（Limit [control forces and torques.]）

1965-02-01 初版　｜　1993-09-07 Amdt 23-45　｜　共 5 个历史版本

## 1　§23.397 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.397 |
| 条款标题（英文原文） | Limit [control forces and torques.] |
| 中文名称 | 驾驶员限制操纵力 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 5 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1993-09-07 （约 28 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.397 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 删除内容：(a)、(b) |
| 3 | Amdt 23-17 | 1977-02-01 | Notice 74-5，1974-02-14 签发，39 FR 5785 Notice 74-5A，1974-05-22 签发，39 FR 5785 Notice 74-5B，1974-10-03 签发，39 FR 5785 Notice 74-33，1974-10-03 签发，39 FR 5785 Notice 74-5C，1974-11-25 签发，39 FR 5785 Notice 74-5D，1975-02-04 签发，39 FR 5785 Notice 75-10，1975-02-27 签发，39 FR 5785 Notice 75-19，1975-05-13 签发，39 FR 5785 Notice 75-20，1975-05-13 签发，39 FR 5785 Notice 75-23，1975-05-19 签发，39 FR 5785 Notice 75-25，1975-05-29 签发，39 FR 5785 Notice 75-26，1975-06-09 签发，39 FR 5785 Notice 75-31，1975-06-30 签发，39 FR 5785 | Docket 14324，1976-12-13 签发，41 FR 55454 | 新增内容 |
| 4 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 修订 |
| 5 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 删除内容 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　删除内容：(a)、(b)　→　1977 23-17　新增内容　→　1987 23-34　修订　→　1993 23-45　删除内容

## 2　§23.397 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：驾驶员限制操纵力

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：修订第 23.397 条

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：29. 修订第 23.397 条，将标题更改为“限制驾驶员力”，删除段(a)的标题，并修订段(b)如下：　2. §23.397(b) 段中的脚注4将修改为如下内容：

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案 12. 操纵系统设计中使用规定的“最大”和“最小”飞行员力（§23.395 条）。1. 将通过在第 23.395 条(a)(1) 段末尾增加以下句子对其进行修订：“用于设计的飞行员力不必超过 §23.397(b) 条中规定的最大力。”

英文原文：

Proposal 12. Use of prescribed "maximum" and "minimum" pilot forces for control system design (Sec. 23.395). 1. Section 23.395 (a) (1) would be amended by adding the following sentence at the end thereof: "Pilot forces used for design need not exceed the maximum forces prescribed in Sec. 23.397 (b)."

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（2）：说明。在 §23.395 条中对 §23.397 条限制驾驶员力的唯一设计引用是 §23.395(c) 段中的一般性引用。这一引用仅仅是编辑性的，并未清楚说明这些驾驶员力的规章效力。本提案将纠正这一缺陷，而不作实质性变更。

英文原文：

Explanation. The only design reference in Sec. 23.395 to the limit pilot forces of Sec. 23.397 is the general reference in Sec. 23.395 (c). This reference is a mere editorial one that does not clearly state the regulatory effect of those pilot forces. This proposal would correct this deficiency without substantive change.

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（3）：提案 13. 驾驶盘操纵力（副翼）的澄清（§23.397）。1. §23.397(b) 段中的数值"53D 英寸-磅"和"40D 英寸-磅"将分别修订为"50 磅"和"40 磅"。

英文原文：

Proposal 13. Clarification of wheel control force (aileron) (Sec. 23.397). 1. The values "53D in-pounds" and "40D in-pounds" in Sec. 23.397 (b) would be amended to read "50 pounds" and "40 pounds", respectively.

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（4）：提案31。操纵系统操作试验（§23.683）。§23.683(b)(1)段将予以修订，在该段末尾增加“或§23.397(b)段中的极限驾驶员作用力，取其较小者”等字样。
说明：本提案旨在保持与§23.397条的一致性，该条规定，操纵面上的气动载荷不必超过施加§23.397(b)段中的作用力所产生的载荷。

英文原文：

Proposal 31. Control system operation tests (Sec. 23.683). Section 23.683 (b) (1) would be amended by adding, at the end thereof, the words "or the limit pilot forces in Sec. 23.397 (b), whichever are less".

Explanation. This proposal would preserve consistency with Sec. 23.397 which provides that the airloads on the surfaces need not exceed those that would result from application of the forces in Sec. 23.397 (b).

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（5）：说明。本提案旨在保持与第23.397条的一致性，该条规定，各翼面上的气动载荷无需超过依据第23.397条(b)段所施加载荷而产生的载荷。 说明。本提案旨在保持与第23.397条的一致性，该条规定，各翼面上的气动载荷无需超过依据第23.397条(b)段所施加载荷而产生的载荷。

英文原文：

Explanation. This proposal would preserve consistency with Sec. 23.397 which provides that the airloads on the surfaces need not exceed those that would result from application of the forces in Sec. 23.397 (b).

修订指令英文原文（供核对）：

29. Section 23.397 is amended by changing the title to read "Limit pilot forces", by deleting the title of paragraph (a), and by amending paragraph (b) as follows:

2. Footnote 4 in Sec. 23.397 (b) would be amended to read as follows:

### 2.3　Amdt 23-17（1977-02-01 生效）：修订 §23.397 的标题

▪ 修订沿革：本版由 NPRM　Notice 74-5，1974-02-14 签发，39 FR 5785；Notice 74-5A，1974-05-22 签发，39 FR 5785；Notice 74-5B，1974-10-03 签发，39 FR 5785；Notice 74-33，1974-10-03 签发，39 FR 5785；Notice 74-5C，1974-11-25 签发，39 FR 5785；Notice 74-5D，1975-02-04 签发，39 FR 5785；Notice 75-10，1975-02-27 签发，39 FR 5785；Notice 75-19，1975-05-13 签发，39 FR 5785；Notice 75-20，1975-05-13 签发，39 FR 5785；Notice 75-23，1975-05-19 签发，39 FR 5785；Notice 75-25，1975-05-29 签发，39 FR 5785；Notice 75-26，1975-06-09 签发，39 FR 5785；Notice 75-31，1975-06-30 签发，39 FR 5785　提出；最终规章 Docket 14324，1976-12-13 签发，41 FR 55454，1977-02-01 生效。

▪ 修订指令（中文译文）：7. 修订 §23.397 的标题，修订 §23.397(b) 的引导语、表格及脚注2和4，并在 §23.397(b) 中新增脚注5，内容如下：

来源：Final Rule｜41 FR 55454｜FinalRule_Docket 14324_Amdt21-44_23-17_25-38_27_41FR55454_1976-12-20_0002.txt

▪ 背景与评论处置（1）：提案 2-9。未收到关于修订第 23.397 条提案的不利评论。因此，该提案被采纳，无实质性更改。

英文原文：

Proposal 2-9. No unfavorable comments were received on the proposal to amend Sec. 23.397. Accordingly, the proposal is adopted without substantive change.

修订指令英文原文（供核对）：

7. By amending the heading of Sec. 23.397, revising the lead-in, table and footnotes 2 and 4 of Sec. 23.397(b), and adding footnote 5 to Sec. 23.397(b) to read as follows:

▪ 本版重印条文段落：(b)

### 2.4　Amdt 23-34（1987-02-17 生效）：修订 §23.397(b) 段中的表格

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 修订指令（中文译文）：24. 修订 §23.397(b) 段中的表格，修订脚注1，删除句子末尾的句号，并插入以下文字："并且对于通勤类飞机，规定值必须随重量线性增加，在设计重量为 19,000 磅时达到规定值的 1.35 倍。"

来源：Final Rule｜52 FR 1806｜FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt

▪ 背景与评论处置（1）：对第23.397条的评论。对于通勤类飞机，一位评论者认为，第23.397条中力值表格的脚注1应修改为如下内容：“……规定的最大值必须随重量线性增加，在19,000磅设计重量时达到规定值的1.35倍。”该评论者建议审查CAR 3.212以了解该脚注的意图。经审查CAR 3.212及更早的要求，美国联邦航空局认为，该脚注是在确立小型飞机12,500磅重量限制之前纳入的，其要求是将5,000磅时提供的最大操纵力随重量线性增加，在5,000磅时系数为1.0，在25,000磅时系数为1.5。在通过第3-10号修正案采用12,500磅重量限制（自1953年5月16日起生效）之后，该脚注在CAR 3中继续沿用相同表述。
当CAR 3被重新编纂为第23部时，该注释被修订，要求最大力随重量线性增加，在设计重量为12,500磅时达到规定值的1.18倍，该系数正是CAR 3脚注中要求的线性增加所将获得的系数。对CAR 3.212的审查表明，建议的修改仅会识别并延续先前适航标准中对此类尺寸飞机已作要求的条款。由于该建议修改与先前适用的适航标准一致，并且通过提供新的最大设计重量19,000磅处线性增加系数的数值而具有澄清作用，美国联邦航空局同意该评论者的主张。§23.397的脚注1修订为：线性增加必须在通勤类飞机最大允许重量19,000磅时达到规定值的1.35倍。

英文原文：

Comment to Sec. 23.397. For commuter category airplanes, one commenter contends that the Footnote 1 to the table of forces in Sec. 23.397 should be changed to read as follows: "...the specified maximum values must be increased linearly with weight to 1.35 times the specified values at a design weight of 19,000 pounds." This commenter suggests a review of CAR 3.212 for the intent of the footnote. Upon review of CAR 3.212 and earlier requirements, the FAA finds that it was included prior to establishment of a 12,500 pound weight limit for small airplanes and called for the provided maximum control forces at 5,000 pounds to be increased linearly with weight by a factor of 1.0 at 5,000 pounds to a factor of 1.5 at 25,000 pounds. This footnote continued to read the same in CAR 3 after the adoption of the 12,500 weight limitation by Amendment 3-10, effective May 16, 1953. When CAR 3 was recodified into Part 23, this note was revised to call for the maximum forces to be linearly increased with weight to 1.18 times the specified values at a design weight of 12,500 pounds which is the factor that would have been obtained from the linear increase called for in the CAR 3 footnote.

The review of CAR 3.212 has shown that the recommended changes would only identify and continue provisions which have been required for airplanes of this size under previous airworthiness standards. Since this recommended change is consistent with previously applied airworthiness standards and will be clarifying by providing the value of the linearly increased factor at the new maximum design weight of 19,000 pounds, the FAA agrees with the contention of this commenter. Footnote 1 to Sec. 23.397 is amended to the extent that the linear increase must be to 1.35 times the specified values at the maximum permissible weight of 19,000 pounds for the commuter category airplane.

修订指令英文原文（供核对）：

24. By amending the table in Sec. 23.397(b) by revising Footnote 1 by removing the period at the end of the sentence and inserting the phrase "and for commuter category airplanes, the specified values must be increased linearly with weight to 1.35 times the specified values at a design weight of 19,000 pounds."

### 2.5　Amdt 23-45（1993-09-07 生效）：修订§23.397(b) 段

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：33. 修订§23.397(b) 段，删除表格最后一行中的“130磅”字样，并在其位置插入“150磅”字样。

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议将 §23.397(b) 段表格最后一行中的最小方向舵力从 130 磅提高至 150 磅，以使其与 §23.143 段中所示的“飞行员力量”限制相一致。由于收到的唯一评论（来自欧洲联合航空局（JAA））同意所提议的修改，§23.397(b) 段按提议通过。

英文原文：

The FAA proposed to increase the minimum rudder force, from 130 pounds to 150 pounds, in the last line of the table of Section 23.397(b) to make it compatible with the "strength of pilots" limits shown in Section 23.143. Since the only comment received, from the JAA, agrees with the proposed change, Section 23.397(b) is adopted as proposed.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（2）：说明：本提案将 §23.397(b) 段表格最后一行所示的最小方向舵力从 130 磅提高到 150 磅，以使其与 §23.143 条所示的"驾驶员体力"限制相容。

英文原文：

Explanation: This proposal increases the minimum rudder force shown in the last line of the table of Section 23.397(b) from 130 pounds to 150 pounds to make it compatible with the "strength of pilots" limits shown in Section 23.143.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：共有两项会议提案针对 §23.397 条。本提案由会议提案 200 发展而来。会议提案 199 在会议上被撤回。会议提案 200 在会议上未经评论或反对即被接受。

英文原文：

There are two conference proposals directed at Section 23.397. This proposal was developed from conference proposal 200. Conference proposal 199 was withdrawn at the conference. Conference proposal 200 was accepted at the conference without comment or opposition.,

修订指令英文原文（供核对）：

33. Section 23.397(b) is amended by removing the words "130 pounds" in the last line of the table and inserting the words "150 pounds" in its place.

## 3　§23.397 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | General. In the control surface flight loading condition, the airloads on movable surfaces and the corresponding deflections need not exceed those that would result in flight from the application of any pilot force within the ranges specified in paragraph (b) of this section. In applying this criterion, the effects of control system boost and servo-mechanisms, and the effects of tabs must be considered. The automatic pilot effort must be used for design if it alone can produce higher control surface loads than the human pilot. | 新增 |
| (b) | The limit pilot forces. The limit pilot forces are as follows: Control Maximum forces for design weight W equal to or less than 5,000 pounds1 Minimum forces2 Aileron: Stick----------------------------- 67 pounds------------------------ 40 pounds. Wheel3-------------------------- 53 D in-pounds4-------------- 40 D in-pounds4. Elevator: Stick------------------------------ 167 pounds---------------------- 100 pounds. Wheel---------------------------- 200 pounds---------------------- 100 pounds. Rudder-------------------------- 200 pounds---------------------- 130 pounds. 1 For design weight (W) more than 5,000 pounds, the specified maximum values must be increased linearly with weight to 1.18 times the specified values at a design weight of 12,500 pounds. 2 If the design of any individual set of control systems or surfaces makes these specified minimum forces inapplicable, values corresponding to the pertinent hinge moments obtained under Sec. 23.415, but not less than 0.6 of the specified minimum forces, may be used. 3 The critical parts of the aileron control system must also be designed for a single tangential force with a limit value of 1.25 times the couple force determined from the above criteria. 4 D=wheel diameter. | 新增 |

### 3.2　版本2：Amdt 23-17（1977-02-01 生效）　来源：Docket 14324，1976-12-13 签发，41 FR 55454

新增内容

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | The limit pilot forces and torques are as follows: Control Maximum forces or torques for design weight, weight equal to or less than 5,000 pounds1 Minimum forces or torques 3 Aileron: Stick----------------------------------------- Wheel3------------------------------------ 67 lbs.------------------------------- 50 D in.-lbs.4---------------------- 40 lbs. 40 D in.-lbs.4 Elevator: Stick----------------------------------------- Wheel (symmetrical).---------------- Wheel (unsymmetrical).5----------- 167 lbs.----------------------------- 200 lbs.---------------------------- --------------- 100 lbs. 100 lbs. 100 lbs. Rudder------------------------------------- 200 lbs.----------------------------- 130 lbs. 1 * * * 2 If the design of any individual set of control systems or surfaces makes these specified minimum forces or torques inapplicable, values corresponding to the present hinge moments obtained under sec. 23.415, but not less than 0.6 of the specified minimum forces or torques, may be used. 3 * * * 4 D=wheel diameter (inches). 5 The unsymmetrical force must be applied at one of the normal handgrip points on the control wheel. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 5 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-17 (1977-02-01) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (b) | ● 新增 | △ 修订 |

## 4　§23.397 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 澄清性与编辑性修订 | eral reference in Sec. 23.395 (c). This reference is a mere editorial one that does not clearly state the regulatory effect of those pilot forces. This proposal would correct this deficiency without substantive change. P… | Amdt 23-7 |
| 澄清性与编辑性修订 | with previously applied airworthiness standards and will be clarifying by providing the value of the linearly increased factor at the new maximum design weight of 19,000 pounds, the FAA agrees with the contention of this… | Amdt 23-34 |
| 国际协调 | n Section 23.143. Since the only comment received, from the JAA, agrees with the proposed change, Section 23.397(b) is adopted as proposed. Explanation: This proposal increases the minimum rudder force shown in the last… | Amdt 23-45 |

本条取证提示 共 5 个历史版本：初始版, Amdt 23-7, Amdt 23-17, Amdt 23-34, Amdt 23-45。 其中 4 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 14324 | 41 FR 55454 | FinalRule_Docket 14324_Amdt21-44_23-17_25-38_27_41FR55454_1976-12-20_0002.txt |
| [6] | Final Rule（颁行） | Docket 14324 | 43 FR 50578 | FinalRule_Docket 14324_ 14606_ 14625_ 14685_ 14779_Amdt23-23_25-46_27-16_29_43FR50578_1978-10-30_0003.txt |
| [7] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 5785 | NPRM_Notice 74-5_Docket13542_39FR5785_1974-02-15.txt |
| [8] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 18663 | NPRM_Notice 74-5A_Docket13542_39FR18663_1974-05-29.txt |
| [9] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 41319 | NPRM_Notice 74-5C_Docket13542_39FR41319_1974-11-26.txt |
| [10] | NPRM（提案） | Notice 74-5 / Docket 13542 | 40 FR 5810 | NPRM_Notice 74-5D_Docket13542_40FR5810_1975-02-07.txt |
| [11] | NPRM（提案） | Notice 74-5 / Docket 13542 | （FR 引证未在文本中标注） | NPRM_Notice 74-5B_Docket13542_1974-10-11.txt |
| [12] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [13] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [14] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [15] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [16] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [17] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [18] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.397 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | b7881b69-3b88-4c60-9bed-4fd75a61ac2a |
| Amdt 23-7 | 1969-09-14 | a67f20c3-74ac-4b16-9a3e-e7ecef063a02 |
| Amdt 23-17 | 1977-02-01 | 4711b733-58c7-42d5-9635-ae1a6ab756bb |
| Amdt 23-34 | 1987-02-17 | d70b6c2f-4fba-416e-839e-1a81803831ba |
| Amdt 23-45 | 1993-09-07 | dbd86521-c15a-4974-afe2-26d528d1d073 |

# § 23.399　双套操纵系统（Dual control system.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 2 个历史版本

## 1　§23.399 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.399 |
| 条款标题（英文原文） | Dual control system. |
| 中文名称 | 双套操纵系统 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.399 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1996 23-48　修订

## 2　§23.399 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：双套操纵系统

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-48（1996-03-11 生效）：第 23.399 条修订后如下

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：14. 第 23.399 条修订后如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议将 §23.399 条的正文重新编号为段(a)，并增加一个新的段(b)，以涉及当两名驾驶员共同操纵时作用在双重操纵系统上的力。这将明确适用力中较大者。

英文原文：

The FAA proposed to redesignate the text of Sec. 23.399 as paragraph (a), and to add a new paragraph (b) that addresses the forces exerted on a dual control system when both pilots act together. This would clarify that it is the greater of the forces that apply.

修订指令英文原文（供核对）：

14. Section 23.399 is revised to read as follows:

▪ 本版重印条文段落：(a)、(1)、(2)、(b)

## 3　§23.399 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | 0.75 times those obtained under Sec. 23.395; or | 新增 |
| (b) | The minimum forces specified in Sec. 23.397(b). | 新增 |

### 3.2　版本2：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each dual control system must be designed to withstand the force of the pilots operating in opposition, using individual pilot forces not less than the greater of-- | 修订 |
| (1) | 0.75 times those obtained under Sec. 23.395; or | 新增 |
| (2) | The minimum forces specified in Sec. 23.397(b). | 新增 |
| (b) | Each dual control system must be designed to withstand the force of the pilots applied together, in the same direction, using individual pilot forces not less than 0.75 times those obtained under Sec. 23.395. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- |
| (a) | ● 新增 | △ 修订 |
| (b) | ● 新增 | △ 修订 |
| (1) | ○ 无 | ● 新增 |
| (2) | ○ 无 | ● 新增 |

## 4　§23.399 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 澄清性与编辑性修订 | The FAA proposed to redesignate the text of Sec. 23.399 as paragraph (a), and to add a new paragraph (b) that addresses the forces exerted on a dual control system when both pilots… | Amdt 23-48 |

本条取证提示 共 2 个历史版本：初始版, Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [4] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [5] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [6] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [7] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.399 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 325e04be-8528-46ba-900d-8c07217f75af |
| Amdt 23-48 | 1996-03-11 | ed5d1e34-9c1a-4071-a475-ee5d5b145568 |

# § 23.405　辅助操纵系统（Secondary control system.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.405 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.405 |
| 条款标题（英文原文） | Secondary control system. |
| 中文名称 | 辅助操纵系统 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.405 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.405 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：辅助操纵系统

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.405 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.405 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.405 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 0c0c51bc-444b-49f5-b869-0c346322b6d1 |

# § 23.407　配平调整片效应（Trim tab effects.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.407 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.407 |
| 条款标题（英文原文） | Trim tab effects. |
| 中文名称 | 配平调整片效应 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.407 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.407 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：配平调整片效应

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.407 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.407 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.407 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 558304ad-684f-449b-89e9-195afa579ac6 |

# § 23.409　调整片（Tabs.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.409 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.409 |
| 条款标题（英文原文） | Tabs. |
| 中文名称 | 调整片 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.409 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.409 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：调整片

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.409 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.409 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.409 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | db96dc85-8c21-4b78-a43c-686e393b6351 |

# § 23.415　地面阵风情况（Ground gust conditions.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 4 个历史版本

## 1　§23.415 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.415 |
| 条款标题（英文原文） | Ground gust conditions. |
| 中文名称 | 地面阵风情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.415 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订 |
| 3 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 新增段落：(c) |
| 4 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | FR 29 FR 17955，29 FR 17955; 34 FR 13089; 58 FR 42160; 61 FR 5145 Docket 27805，1996-01-29 签发，61 FR 5138 | 修订段落：(a)、(b) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订　→　1993 23-45　新增段落：(c)　→　1996 23-48　修订段落：(a)、(b)

## 2　§23.415 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：地面阵风情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第 23.415(a)(2) 条通过修订 "q" 的定义修改如下

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：30. 第 23.415(a)(2) 条通过修订 "q" 的定义修改如下：

修订指令英文原文（供核对）：

30. Section 23.415(a)(2) is amended by amending the definition of "q" to read as follows:

▪ 本版重印条文段落：(a)、(2)

### 2.3　Amdt 23-45（1993-09-07 生效）：§23.415 条新增 (c) 段

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：§23.415 条新增 (c) 段，内容如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦

英文原文：

The FAA proposed to revise Section 23.415 to add requirements defining airplane tie-down loads and to include design criteria for attachment fittings and surrounding structure.

The one commenter, the JAA, submits a recommended "more comprehensive" regulatory paragraph for consideration, and interpretative material based on JAR 23.415(c). The JAA proposes considering all weights between the empty weight and the maximum weight declared for tie-down limit load conditions. The JAA believes the following areas should be included in the consideration: tie-down points stated in the appropriate manual, surrounding structure, control system surfaces, and associated gust locks.

The FAA agrees that all weights should be considered for tie-down points and that structure surrounding the tie-down points should be substantiated for adequate strength. The recommendation to consider these weights is beyond the scope of the notice. The recommendation is retained for a future rulemaking notice. The FAA adopts Section 23.415 as proposed.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（2）：美国联邦航空局（FAA）同意，对于系留点，应考虑所有重量，且系留点周围的结构应经验证具有足够的强度。考虑这些重量的建议超出了该通告的范围。该建议将予以保留，供未来的规章制定通告使用。美国联邦航空局按提案内容采纳 §23.415 条。

英文原文：

The FAA agrees that all weights should be considered for tie-down points and that structure surrounding the tie-down points should be substantiated for adequate strength. The recommendation to consider these weights is beyond the scope of the notice. The recommendation is retained for a future rulemaking notice. The FAA adopts Section 23.415 as proposed.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：说明：本提案修订了第 23.415 条，以增加规定飞机系留载荷的要求，包含连接接头和周围结构的设计准则，并且基于会议提案 202。

英文原文：

Explanation: This proposal revises Section 23.415 to add requirements defining airplane tie-down loads, includes design criteria for attachment fittings and surrounding structure and is based on conference proposal 202.

修订指令英文原文（供核对）：

34. Section 23.415 is amended by adding a new paragraph (c) to read as follows:

▪ 本版重印条文段落：(c)

### 2.4　Amdt 23-48（1996-03-11 生效）：第 23.415 条通过修订段 (a)(2) 和 (c) 修订如下

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 FR 29 FR 17955，29 FR 17955; 34 FR 13089; 58 FR 42160; 61 FR 5145；Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：15. 第 23.415 条通过修订段 (a)(2) 和 (c) 修订如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议修订 §23.415 条，通过修订段 (a)(2) 来增加翼载（W/S）的定义。美国联邦航空局（FAA）还提议修订段 (c)，该段是在第 23-45 号修正案（58 FR 42136, August 6, 1993）中增加的，以纳入更全面的系留准则。

英文原文：

The FAA proposed to amend Sec. 23.415 by revising paragraph (a)(2) to add a definition for wing loading (W/S). The FAA also proposed to revise paragraph (c), which was added in Amendment No. 23-45 (58 FR 42136, August 6, 1993), to incorporate a more comprehensive tie-down criteria.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：本提案将通过修订段 (a)(2) 来修订 §23.415 条，以增加翼载荷（W/S）的定义，从而与欧洲联合航空要求（JAR）23 协调统一。它还将修订段 (c)。在第 23-45 号修正案增加段 (c) 之前，美国联邦航空局（FAA）同意了由欧洲联合航空局（JAA）建议的更全面版本的系留准则。本修正案将落实该协议并协调统一规章。

英文原文：

This proposal would amend Sec. 23.415 by revising paragraph (a)(2) to add a definition for wing loading (W/S) to harmonize with JAR 23. It would also revise paragraph (c). Before paragraph (c) was added in Amendment No. 23-45, the FAA agreed to a more comprehensive version of the tie-down criteria that was suggested by the JAA. This amendment would implement that agreement and harmonize the rules.

修订指令英文原文（供核对）：

15. Section 23.415 is amended by revising paragraphs (a)(2) and (c) to read as follows:

▪ 本版重印条文段落：(a)、(2)、(c)

## 3　§23.415 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The control system must be investigated as follows for control surface loads due to ground gusts and taxiing downwind: | 新增 |
| (1) | If an investigation of the control system for ground gust loads is not required by subparagraph (2) of this paragraph, but the applicant elects to design a part of the control system for these loads, these loads need only be carried from control surface horns through the nearest stops or gust locks and their supporting structures. | 新增 |
| (2) | If pilot forces less than the minimums specified in Sec. 23.397(b) are used for design, the effects of surface loads due to ground gusts and taxiing downwind must be investigated for the entire control system according to the formula: H = K c S q where-- H = limit hinge moment (ft.-lbs.); c = mean chord of the control surface aft of the hinge line (ft.); S = area of control surface aft of the hinge line (sq. ft.); q = dynamic pressure (p.s.f.) based on a design speed not less than 10 +10 (m.p.h.) except that the design speed need not exceed 60 m.p.h.; and K = limit hinge moment factor for ground gusts derived in paragraph (b) of this section. (For ailerons and elevators, a positive value of K indicates a moment tending to depress the surface and a negative value of K indicates a moment tending to raise the surface). | 新增 |
| (b) | The limit hinge moment factor K for ground gusts must be derived as follows: Surface K Position of controls | 新增 |
| (a) | Aileron---------------------------------- 0.75 Control column locked or lashed in mid-position. | 新增 |
| (b) | Aileron---------------------------------- ±0.50 Ailerons at full throw; + moment on one aileron, - moment on the other. | 新增 |
| (c) | Elevator-------------------------------- ±0.75 | 新增 |
| (c) | Elevator full up (-). | 新增 |
| (d) | Elevator-------------------------------- -------- (d) Elevator full down (+). | 新增 |
| (e) | Rudder--------------------------------- ±0.75 | 新增 |
| (e) | Rudder in neutral. | 新增 |
| (f) | Rudder---------------------------------- -------- | 新增 |
| (f) | Rudder at full throw. Horizontal Tail Surfaces | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (2) | * * * q = Dynamic pressure (p.s.f.) based on a design speed not less than 14.6+ 14.6 (f.p.s.) except that the design speed need not exceed 88 (f.p.s.); and | 修订 |

### 3.3　版本3：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

新增段落：(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | The tie-down attachment fittings and the surrounding structure must be designed for limit load conditions resulting from wind speeds up to 65 knots horizontally from any direction for the weight determined to be critical for tie-down. | 新增 |

### 3.4　版本4：Amdt 23-48（1996-03-11 生效）　来源：FR 29 FR 17955，29 FR 17955; 34 FR 13089; 58 FR 42160; 61 FR 5145；Docket 27805，1996-01-29 签发，61 FR 5138

修订段落：(a)、(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 新增 |
| (2) | If pilot forces less than the minimums specified in Sec. 23.397(b) are used for design, the effects of surface loads due to ground gusts and taxiing downwind must be investigated for the entire control system according to the formula: H = K c S q where--H = limit hinge moment (ft.-lbs.); c = mean chord of the control surface aft of the hinge line (ft.); S = area of control surface aft of the hinge line (sq. ft.); q = dynamic pressure (p.s.f.) based on a design speed not less than 14.6 (W/S) + 14.6 (f.p.s.) where W/S = wing loading at design maximum weight, except that the design speed need not exceed 88 (f.p.s.); K = limit hinge moment factor for ground gusts derived in paragraph (b) of this section. (For ailerons and elevators, a positive value of K indicates a moment tending to depress the surface and a negative value of K indicates a moment tending to raise the surface). | 新增 |
| (c) | At all weights between the empty weight and the maximum weight declared for tie-down stated in the appropriate manual, any declared tie-down points and surrounding structure, control system, surfaces and associated gust locks, must be designed to withstand the limit load conditions that exist when the airplane is tied down and that result from wind speeds of up to 65 knots horizontally from any direction. | 修订 |

### 3.5　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) | Amdt 23-45 (1993-09-07) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (1) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (2) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (b) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (c) | ● 新增 | ○ 无 | ● 新增 | △ 修订 |
| (d) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (e) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (f) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |

## 4　§23.415 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | fittings and surrounding structure. The one commenter, the JAA, submits a recommended "more comprehensive" regulatory paragraph for consideration, and interpretative material based on JAR 23.415(c). The JAA proposes con… | Amdt 23-45 |
| 国际协调 | agraph (a)(2) to add a definition for wing loading (W/S) to harmonize with JAR 23. It would also revise paragraph (c). Before paragraph (c) was added in Amendment No. 23-45, the FAA agreed to a more comprehensive version… | Amdt 23-48 |

本条取证提示 共 4 个历史版本：初始版, Amdt 23-7, Amdt 23-45, Amdt 23-48。 其中 3 个版本在本地文献中检索到明确修订指令原文；4 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [6] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [7] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [8] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [9] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [10] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [11] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.415 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 18068dda-476c-44d5-b209-5c107ed80a8e |
| Amdt 23-7 | 1969-09-14 | 058a034d-5de0-4cca-bc60-a46ee3da0c7c |
| Amdt 23-45 | 1993-09-07 | 29b49f79-2bac-4204-b6ba-6f0488abd25f |
| Amdt 23-48 | 1996-03-11 | d2a9ea63-5f30-43b3-96be-b3ac4eeb537c |

# § 23.421　平衡载荷（Balancing loads.）

1965-02-01 初版　｜　1991-02-04 Amdt 23-42　｜　共 3 个历史版本

## 1　§23.421 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.421 |
| 条款标题（英文原文） | Balancing loads. |
| 中文名称 | 平衡载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 3 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1991-02-04 （约 26 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.421 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(b) |
| 3 | Amdt 23-42 | 1991-02-04 | — | — | 条文未改动（本版为交叉引用或其他条款修订所致） |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(b)　→　1991 23-42　条文未改动（本版为交叉引用或其他条款修订所致

## 2　§23.421 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：平衡载荷

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：修订§23.421的(b)段

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：31. 修订§23.421的(b)段，删除图6引用中的数字“6”，并替换为数字“B6”。

修订指令英文原文（供核对）：

31. Section 23.421 is amended by amending paragraph (b) by striking out the number "6" in the reference to figure 6 and inserting in place thereof the number "B6".

### 2.3　Amdt 23-42（1991-02-04 生效）：条文未改动（本版为交叉引用或其他条款修订所致）

▪ 修订沿革：本版由 NPRM　—　提出；最终规章 —，1991-02-04 生效。

▪ 本版对条文的处置：条文未改动（本版为交叉引用或其他条款修订所致）

## 3　§23.421 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | A horizontal tail balancing load is a load necessary to maintain equilibrium in any specified flight condition with no pitching acceleration. | 新增 |
| (b) | Horizontal tail surfaces must be designed for the balancing loads occurring at any point on the limit maneuvering envelope and in the flap conditions specified in Sec. 23.345. The distribution in figure 6 of Appendix B may be used. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 3 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.421 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 3 个历史版本：初始版, Amdt 23-7, Amdt 23-42。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [6] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.421 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 5431f3cc-3dd0-42a7-b99b-2991be41ceda |
| Amdt 23-7 | 1969-09-14 | f825b0c0-9e32-43c9-b627-3ba09fe15fd5 |
| Amdt 23-42 | 1991-02-04 | 9f81b512-2169-4b97-8066-51faefe0b81a |

# § 23.423　机动载荷（辅助翼面）（Maneuvering loads.）

1965-02-01 初版　｜　1991-02-04 Amdt 23-42　｜　共 3 个历史版本

## 1　§23.423 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.423 |
| 条款标题（英文原文） | Maneuvering loads. |
| 中文名称 | 机动载荷（辅助翼面） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 3 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1991-02-04 （约 26 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 56 FR 5455 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.423 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订 |
| 3 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 修订 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订　→　1991 23-42　修订

## 2　§23.423 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：机动载荷（辅助翼面）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第23.423条修订如下

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：32. 第23.423条修订如下：

修订指令英文原文（供核对）：

32. Section 23.423 is amended as follows:

### 2.3　Amdt 23-42（1991-02-04 生效）：第 23.423 条修订为以下内容

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：15. 第 23.423 条修订为以下内容：

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案15。本提案拟将§23.423 条中针对常规构型飞机的现行机动载荷要求扩展至鸭翼构型和串列翼构型，并禁止使用附录B的方法表明符合性。在现行要求涉及操纵面偏转以及向上和向下载荷之处，提议改为将操纵动作表述为飞机的抬头和低头俯仰。禁止使用附录B的理由已在提案14的说明中详细讨论。

英文原文：

Proposal 15. This proposal would extend the current maneuvering loads requirements of Sec. 23.423 for conventional type airplanes to canard and tandem wing configurations and prohibit the use of appendix B methods for demonstrating compliance. Where the current requirements refer to control deflections and up and down loads, it is proposed to refer to the control movements as nose-up and nose-down pitching of the airplane. The reasons for prohibiting the use of appendix B are discussed in detail in the explanation for proposal 14.

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局（FAA）同意该评论意见，即规章制定提案通告中所述的提案可被解释为不适用于鸭翼或串列翼构型飞机的主机翼。在规章制定提案通告中，"the main wing of a canard or tandem wing configuration" 一语被加入 Sec. 23.423 第一句。
评论者还指出，Sec. 23.423 的适用范围可被解释为排除水平面的支撑结构。美国联邦航空局同意该评论意见，并在提案第一句中加入了 "and its supporting structure" 一语。本提案的其余部分涉及三翼面构型飞机（例如带常规尾翼的机翼、鸭翼构型）前翼面上的机动载荷。该三翼面构型可具有不带俯仰操纵的鸭翼面。本提案在上述修改后予以采纳。

英文原文：

The FAA agrees with the comment that the proposal, as written in the NPRM, could be interpreted as not being applicable to the main wing of an airplane with a canard or tandem wing configuration. In the NPRM, the words "the main wing of a canard or tandem wing configuration" were added to the first sentence of Sec. 23.423.

The commenter also states that the applicability of Sec. 23.423 could be interpreted to exclude the supporting structure of the horizontal surface. The FAA agrees with this comment and the words "and its supporting structure" have been added to the first sentence of the proposal. The balance of this proposal addresses the maneuvering loads on the forward surface of a three-surface configuration airplane, such as a wing, canard configuration, with a conventional tail. This three-surface configuration could have a canard surface without pitch control. This proposal is adopted with the aforementioned changes.

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（3）：评论者还指出，对§23.423适用性的解释可能将水平面的支撑结构排除在外。美国联邦航空局同意该评论意见，并已在提案的第一句中增加了"及其支撑结构"字样。本提案的其余部分涉及三翼面布局飞机（如带常规尾翼的机翼、鸭式布局）前翼面上的机动载荷。这种三翼面布局可能带有无俯仰控制的鸭翼面。本提案按上述修改通过。

英文原文：

The commenter also states that the applicability of Sec. 23.423 could be interpreted to exclude the supporting structure of the horizontal surface. The FAA agrees with this comment and the words "and its supporting structure" have been added to the first sentence of the proposal. The balance of this proposal addresses the maneuvering loads on the forward surface of a three-surface configuration airplane, such as a wing, canard configuration, with a conventional tail. This three-surface configuration could have a canard surface without pitch control. This proposal is adopted with the aforementioned changes.

修订指令英文原文（供核对）：

15. Section 23.423 is revised to read as follows:

▪ 本版重印条文段落：(a)、(b)、(n)、(1)、(2)

### 2.4　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.423 的实际改动 |
| --- | --- | --- | --- |
| 56 FR 5455 | 1991-02-11 | 更正通告 (CFR 溯源注) |  |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.423 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | A sudden deflection of the elevator control, at VA, to (1) the maximum upward deflection, and (2) the maximum downward deflection, as limited by the control stops, or pilot effort, whichever is critical. The average loading of B23.11 of Appendix B and the distribution in figure 7 of Appendix B may be used. | 新增 |
| (b) | A sudden upward deflection of the elevator, at speeds above VA, followed by a downward deflection of the elevator, resulting in the following combinations of normal and angular acceleration: Condition Normal acceleration (n) Angular acceleration (radian/sec.2) Down load------------------------ 1.0---------------------------------- nm (nm-1.5) Up load---------------------------- nm--------------------------------- nm(nm-1.5) where-- | 新增 |
| (1) | nm=positive limit maneuvering load factor used in the design of the airplane; and | 新增 |
| (2) | V=initial speed in miles per hour. The conditions in this paragraph involve loads corresponding to the loads that may occur in a "checked maneuver" (a maneuver in which the pitching control is suddenly displaced in one direction and then suddenly moved in the opposite direction), the deflections and timing avoiding exceeding the limit maneuvering load factor. The total tail load for both down and up load conditions is the sum of the balancing tail loads at V and the specified value of the normal load factor n, plus the maneuvering load increment due to the specified value of the angular acceleration. The maneuvering load increment in figure 2 of Appendix B and the distributions in figure 7 (for down loads) and in figure 8 (for up loads) of Appendix B may be used. | 新增 |

### 3.2　版本2：Amdt 23-42（1991-02-04 生效）　来源：Docket 25811，1990-12-21 签发，56 FR 344

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | A sudden movement of the pitching control, at the speed VA, to the maximum aft movement, and the maximum forward movement, as limited by the control stops, or pilot effort, whichever is critical. | 修订 |
| (b) | A sudden aft movement of the pitching control at speeds above VA, followed by a forward movement of the pitching control resulting in the following combinations of normal and angular acceleration: Condition Normal acceleration | 修订 |
| (n) | Angular acceleration (radian/sec2) Nose-up pitching 1.0 +39nm + V x (nm-1.5) Nose-down pitching nm -39nm + V x (nm-1.5) where -- | 新增 |
| (1) | nm = positive limit maneuvering load factor used in the design of the airplane; and | 修订 |
| (2) | V = initial speed in knots. The condition in this paragraph involve loads corresponding to the loads that may occur in a "checked maneuver" (a maneuver in which the pitching control is suddenly displaced in one direction and then suddenly moved in the opposite direction). The deflections and timing of the "checked maneuver" must avoid exceeding the limit maneuvering load factor. The total horizontal surface load for both nose-up and nose-down pitching conditions is the sum of the balancing loads at V and the specified value of the normal load factor n, plus the maneuvering load increment due to the specified value of the angular acceleration. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 3 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-42 (1991-02-04) |
| --- | --- | --- |
| (a) | ● 新增 | △ 修订 |
| (b) | ● 新增 | △ 修订 |
| (1) | ● 新增 | △ 修订 |
| (2) | ● 新增 | △ 修订 |
| (n) | ○ 无 | ● 新增 |

## 4　§23.423 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 新技术 / 新构型 | uirements of Sec. 23.423 for conventional type airplanes to canard and tandem wing configurations and prohibit the use of appendix B methods for demonstrating compliance. Where the current requirements refer to control d… | Amdt 23-42 |

本条取证提示 共 3 个历史版本：初始版, Amdt 23-7, Amdt 23-42。 其中 2 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：56 FR 5455，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [6] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [7] | 更正通告 / 校正 | 56 FR 5455 | 1991-02-11 | Correction_56FR5455_1991-02-11.clean.txt |
| [8] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [9] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.423 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 2ca279ef-10ca-4759-9204-2c53223ca1c7 |
| Amdt 23-7 | 1969-09-14 | 4eb13001-80e3-4cb9-af55-554ce76a09c7 |
| Amdt 23-42 | 1991-02-04 | 301501d8-c4e7-40a5-a597-85e394cfc51f |

# § 23.425　阵风载荷（辅助翼面）（Gust loads.）

1965-02-01 初版　｜　1991-02-04 Amdt 23-42　｜　共 3 个历史版本

## 1　§23.425 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.425 |
| 条款标题（英文原文） | Gust loads. |
| 中文名称 | 阵风载荷（辅助翼面） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 3 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1991-02-04 （约 26 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.425 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(1)、(a)、(b) |
| 3 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 修订段落：(b)、(a)、(d) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(1)、(a)、(b)　→　1991 23-42　修订段落：(b)、(a)、(d)

## 2　§23.425 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：阵风载荷（辅助翼面）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第23.425条经修订

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：33. 第23.425条经修订，对(a)段第(1)和(2)子段以及(b)段和(d)段进行修订，内容如下：　4. 第23.425条(a)(1)和(2)以及(d)段将修改如下：

修订指令英文原文（供核对）：

33. Section 23.425 is amended by amending subparagraphs (1) and (2) of paragraph (a) and paragraphs (b) and (d) to read as follows:

4. Section 23.425 (a) (1) and (2) and (d) would be amended to read as follows:

▪ 本版重印条文段落：(a)、(1)、(2)、(b)、(d)

### 2.3　Amdt 23-42（1991-02-04 生效）：第 23.425 条通过以下方式修订：删除当前段 (b) 的文本并将

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：16. 第 23.425 条通过以下方式修订：删除当前段 (b) 的文本并将其标记为"[预留]"；修订段 (a)、(c) 和 (d) 的引言文本如下所述；并将段 (d) 后公式中 aht 和 Sht 的定义由 "aht = 水平尾翼升力曲线斜率（每弧度）" 修订为 "aht = 后部水平升力面升力曲线斜率（每弧度）"，由 "Sht = 水平尾翼面积（ft2）；及" 修订为 "Sht = 后部水平升力面面积（ft2）；及"。　16. 第 23.425 条通过删除当前段 b) 的文本并将段 指定为"[预留]"，以及通过修订段 的引言文本、段 和段 的引言文本使其内容如下所述而修订。此外，段 后公式中和的定义通过将"水平尾翼升力曲线斜率（每弧度）"更改为"后部水平升力曲线斜率（每弧度）"，以及通过将"水平尾翼面积（ft3）；和"更改为"后部水平升力面面积（ft2）；和"而修订。

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案 16。本提案将修订 §23.425 条，将当前针对水平尾翼面的阵风载荷要求扩展至具有鸭翼和串列翼构型的飞机，并如提案 14 中详细讨论的那样禁止使用附录 B。未收到关于本提案的评论，并按提案予以采纳。

英文原文：

Proposal 16. This proposal would amend Sec. 23.425 by extending the current gust load requirements for the horizontal tail surface to airplanes with a canard and tandem wing configuration and prohibit the use of appendix B, as discussed in detail in proposal 14. No comments were received on this proposal and it is adopted as proposed.

修订指令英文原文（供核对）：

16. Section 23.425 is amended by removing the text of current paragraph (b) and marking it "[Reserved]"; by revising paragraphs (a), (c), and (d) introductory text to read as set forth below; and by revising definitions of aht and Sht in the formula following paragraph (d) from "aht = Slope of horizontal tail lift curve (per-radian)" to "aht = Slope of aft horizontal lift curve (per radian)" and "Sht = Area of horizontal tail (ft2); and" to "Sht = Area of aft horizontal lift surface (ft2); and".

16. Section 23.425 is amended by removing the text of current paragraph b) and designating paragraph (b) as "Reserved"; and by revising the introductory text of paragraph (a), paragraph (c), and the introductory text of paragraph (d) to read as set forth below. In addition, the definitions of and in the formula following paragraph (d) is revised by changing fromSlope of horizontal tail lift curve (per radian)" to Slope of aft horizontal lift curve (per radian)" and by changing from Area of horizontal tail (ft3); and" to Area of aft horizontal lift surface (ft2); and".

## 3　§23.425 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each horizontal tail surface must be designed for loads resulting from-- | 新增 |
| (1) | Positive and negative gusts of 30 feet per second nominal intensity at VC, corresponding to the flight condition specified in Sec. 23.333(c), with flaps retracted; and | 新增 |
| (2) | Positive and negative gusts of 15 feet per second nominal intensity at VF , corresponding to the flight condition specified in Sec. 23.345(a)(2), with flaps extended and at VD corresponding with the flight conditions specified in Sec. 23.333(c)(2) with flaps retracted. | 新增 |
| (b) | The average loadings in figures 3 and 4 of Appendix B and the distribution in figure 8 of Appendix B may be used instead of the requirements of subparagraph (a)(1). | 新增 |
| (c) | When determining the total load on the horizontal tail for the conditions specified in paragraph (a) of this section, the initial balancing tail loads for steady unaccelerated flight at the pertinent design speeds VF, VC, and VD must first be determined. The incremental tail load resulting from the gusts must be added to the initial balancing tail load to obtain the total tail load. | 新增 |
| (d) | The incremental tail load due to the gust may be computed by the formula t = 0.1KUVStat where-- t= the limit gust load increment on the tail in pounds; K = gust coefficient K derived from Sec. 23.341; U = nominal gust intensity in feet per second; V= airplane speed in miles per hour; St = tail surface area in square feet; at = slope of lift curve of tail surface, CL per degree, corrected for aspect ratio; aw = slope of lift curve of wing, CL per degree; and Rw = aspect ratio of the wing. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订：(1)、(a)、(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (1) | Gust velocities specified in Sec. 23.333(c) with flaps retracted; and | 修订 |
| (2) | Positive and negative gusts of 25 f.p.s. nominal intensity at Vcorresponding to the flight conditions specified in Sec. 23.345(a)(2). | 修订 |
| (b) | The average loadings in figures B3 and B4 of Appendix B and the distribution in figure B8 of Appendix B may be used instead of the requirements of paragraph (a)(1) of this section. | 修订 |
| (d) | In the absence of a more rational analysis, the incremental tall load due to the gust must be computed as follows: where-- = Incremental horizontal tail load (lbs.); Kg = Gust alleviation factor defined in Sec. 23.341; Ude = Derived gust velocity (f.p.s.); V= Airplane equivalent speed (knots); aht = Slope of aft horizontal tail lift curve (per radian); Sht = Area of horizontal tail (ft2); and = Downwash factor.] | 修订 |

### 3.3　段落结构演变矩阵（横向对照 3 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) |
| --- | --- | --- |
| (a) | ● 新增 | △ 修订 |
| (1) | ● 新增 | △ 修订 |
| (2) | ● 新增 | △ 修订 |
| (b) | ● 新增 | △ 修订 |
| (c) | ● 新增 | ○ 无 |
| (d) | ● 新增 | △ 修订 |

## 4　§23.425 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 新技术 / 新构型 | rements for the horizontal tail surface to airplanes with a canard and tandem wing configuration and prohibit the use of appendix B, as discussed in detail in proposal 14. No comments were received on this proposal and i… | Amdt 23-42 |

本条取证提示 共 3 个历史版本：初始版, Amdt 23-7, Amdt 23-42。 其中 2 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [6] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [7] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [8] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.425 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 9186ed7b-6fb2-4f04-b6bc-27fdaaee85ef |
| Amdt 23-7 | 1969-09-14 | cfd9007c-315d-4735-81f5-d5dc79233b4a |
| Amdt 23-42 | 1991-02-04 | 177980ac-e159-4c3f-8105-4b233addae36 |

# § 23.427　不对称载荷（辅助翼面）（Unsymmetrical loads.）

1965-02-01 初版　｜　1991-02-04 Amdt 23-42　｜　共 3 个历史版本

## 1　§23.427 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.427 |
| 条款标题（英文原文） | Unsymmetrical loads. |
| 中文名称 | 不对称载荷（辅助翼面） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 3 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1991-02-04 （约 26 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.427 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-14 | 1973-12-20 | Notice 71-13，1971-04-27 签发，36 FR 8398 | Docket 11011，1973-11-07 签发，38 FR 31816 | 修订 |
| 3 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 删除内容：(a)、(b)、(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1973 23-14　修订　→　1991 23-42　删除内容：(a)、(b)、(c)

## 2　§23.427 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：不对称载荷（辅助翼面）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-14（1973-12-20 生效）：§23.427 条修订为如下内容

▪ 修订沿革：本版由 NPRM　Notice 71-13，1971-04-27 签发，36 FR 8398　提出；最终规章 Docket 11011，1973-11-07 签发，38 FR 31816，1973-12-20 生效。

▪ 修订指令（中文译文）：12. §23.427 条修订为如下内容：　13. §23.427 条将修订如下：

来源：Final Rule｜38 FR 31816｜FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt

▪ 背景与评论处置（1）：一位评论

英文原文：

One commentator requested that the prescribed flight conditions of proposed Sec. 23.427(a) be detailed as in present Sec. 23.427. The FAA agrees and the section has been so revised. In addition, proposed Sec. 23.427(b)(2) has been redesignated Sec. 23.427(c) in order to clarify the requirement of that subsection. The commentator also suggested that yawing and slipstream effects should not be taken into consideration in complying with proposed Sec. 23.427(a). The FAA does not agree and no reason for the suggested change was presented by the commentator.

来源：NPRM（提案）｜36 FR 8398｜NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt

▪ 背景与评论处置（2）：解释：对第 23.427 条的本次修订被认为是必要的，以涵盖关于尾翼面布置的新设计。

英文原文：

Explanation: This amendment to Sec. 23.427 is considered necessary in order to cover the new designs with respect to tail surface arrangements.

修订指令英文原文（供核对）：

12. Section 23.427 is amended to read as follows:

13. Sec. 23.427 would be amended to read as follows:

▪ 本版重印条文段落：(a)、(b)、(1)、(i)、(ii)、(2)

### 2.3　Amdt 23-42（1991-02-04 生效）：对第 23.427 条进行修订：删除 (a) 段中的"尾翼"一词

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：17. 对第 23.427 条进行修订：删除 (a) 段中的"尾翼"一词，并在"水平面"之后插入"主翼以外的"短语；删除 (b) 段中的"尾翼面，"短语，并在其位置插入"主翼以外的水平面，"短语；删除 (c) 段中的"尾翼"一词，并在"水平面"短语之后插入"主翼以外的"短语。　17. 修订第 23.427 条，删除段 和 中的“尾”一词；删除段 中的“尾”一词并在原处插入“水平”一词；并在段、 和 中读作“水平翼面”的修订短语之后添加“除主机翼外”字样。

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案17。本提案将现行§23.427中针对常规布局水平尾翼的不对称载荷要求扩展至鸭式布局和串列翼布局的飞机。未收到针对本提案的评论，因此按提案原样采纳。

英文原文：

Proposal 17. This proposal would extend the current Sec. 23.427 unsymmetrical loads requirements for horizontal tail surfaces of conventional configurations to airplanes with canard and tandem wing configurations. No comments were received on this proposal and it is adopted as proposed.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（2）：本提案将把常规布局水平尾翼面的现行不对称载荷要求扩展至鸭翼和串列翼布局。会议上的评论者表示担忧，认为会议提案 208 会将要求扩展至主翼。这并非意图所在，本提案将明确排除主翼，但涉及所有其他水平翼面。会议提案 209 将限制 §23.427(c) 段对 V 型尾翼面的适用性。该提案遭到美国联邦航空局（FAA）和与会业界代表的反对，因为他们认为现行规章对 V 型尾翼面及其支撑结构是充分的。因此，提案 209 未被纳入本通告。

英文原文：

This proposal would extend the current unsymmetrical loads requirements for horizontal tail surfaces of conventional configurations to canard and tandem wing configurations. Commenters at the conference expressed concern that conference proposal 208 would extend the requirements to the main wing. This was not the intend and this proposal would specifically exclude the main wing but involve all other horizontal surfaces. Conference proposal 209 would limit the applicability of Sec. 23.427(c) relative to V-tail surfaces. This proposal was opposed by FAA and industry representatives at the conference because they considered the current rule adequate for V-tail surfaces and their supporting structure. Accordingly, proposal 209 has not been included in this notice.

修订指令英文原文（供核对）：

17. Section 23.427 is amended by removing the word "tail" in paragraph (a) and inserting the phrase "other than main wing" after the words "horizontal surfaces"; by removing the phrase "tail surfaces," in paragraph (b) and inserting the phrase "horizontal surfaces other than main wing," in its place; and by removing the word "tail" in paragraph (c) and inserting the phrase "other than main wing" after the phrase "horizontal surfaces".

17. Section 23.427 is amended by removing the word "tail" from paragraphs (a) and (c); by removing the word "tail" from paragraph (b) and inserting in its place the word "horizontal"; and by adding the words "other than main wing" after the amended phrase which reads "horizontal surfaces" in paragraphs (a), (b), and (c).

## 3　§23.427 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：Amdt 23-14（1973-12-20 生效）　来源：Docket 11011，1973-11-07 签发，38 FR 31816

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Horizontal tail surfaces and their supporting structure must be designed for unsymmetrical loads arising from yawing and slipstream effects, in combination with the prescribed flight conditions. | 新增 |
| (b) | In the absence of more rational data, the following apply: | 新增 |
| (1) | For airplanes that are conventional in regard to location of engines, wings, tail surfaces, and fuselage shape- | 新增 |
| (i) | 100 percent of the maximum loading from the symmetrical flight conditions may be assumed on the surface on one side of the plane of symmetry; and | 新增 |
| (ii) | The following percentage of that loading must be applied to the opposite side: %=100-10 (n-1), where n is the specified positive maneuvering load factor, but this value may not be more than 80 percent; or | 新增 |
| (2) | For airplanes that are not conventional (such as airplanes with horizontal tail surfaces having appreciable dihedral or supported by the vertical tail surfaces) the surfaces and supporting structures must be designed for combined vertical and horizontal surface loads resulting from each prescribed flight condition taken separately. Explanation: This amendment to Sec. 23.427 is considered necessary in order to cover the new designs with respect to tail surface arrangements. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 3 个版本）

| 段落 | Amdt 23-14 (1973-12-20) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |
| (1) | ● 新增 |
| (i) | ● 新增 |
| (ii) | ● 新增 |
| (2) | ● 新增 |

## 4　§23.427 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 澄清性与编辑性修订 | o revised. In addition, proposed Sec. 23.427(b)(2) has been redesignated Sec. 23.427(c) in order to clarify the requirement of that subsection. The commentator also suggested that yawing and slipstream effects should not… | Amdt 23-14 |
| 产业豁免压力倒逼 | ve to V-tail surfaces. This proposal was opposed by FAA and industry representatives at the conference because they considered the current rule adequate for V-tail surfaces and their supporting structure. Accordingly, pr… | Amdt 23-42 |

本条取证提示 共 3 个历史版本：初始版, Amdt 23-14, Amdt 23-42。 其中 2 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 11011 | 38 FR 31816 | FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt |
| [4] | NPRM（提案） | Notice 71-13 / Docket 11011 | 36 FR 8398 | NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt |
| [5] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [6] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [7] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [8] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.427 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 86620649-6dc6-4de9-b45f-797423e84dde |
| Amdt 23-14 | 1973-12-20 | ac698eb5-f3c0-4725-8ce8-e87de068859a |
| Amdt 23-42 | 1991-02-04 | 42d728da-8e8a-40a8-b5f9-c5cbb86ed524 |

# § 23.441　机动载荷（尾翼）（Maneuvering loads.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 6 个历史版本

## 1　§23.441 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.441 |
| 条款标题（英文原文） | Maneuvering loads. |
| 中文名称 | 机动载荷（尾翼） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 6 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.441 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(b) |
| 3 | Amdt 23-14 | 1973-12-20 | Notice 71-13，1971-04-27 签发，36 FR 8398 | Docket 11011，1973-11-07 签发，38 FR 31816 | 修订：(a) |
| 4 | Amdt 23-28 | 1982-04-28 | Notice 80-23，1980-11-28 签发，45 FR 80450 | Docket 21129，1982-02-26 签发，47 FR 13312 | 删除内容 |
| 5 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 修订 |
| 6 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增内容：(a)、(b) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(b)　→　1973 23-14　修订：(a)　→　1982 23-28　删除内容　→　1991 23-42　修订　→　1996 23-48　新增内容：(a)、(b)

## 2　§23.441 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：机动载荷（尾翼）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：修订 §23.441 条

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：34. 修订 §23.441 条，方式是修订 (b) 段，删除对图 1、6、7 和 8 的引用中的数字“1”、“6”、“7”和“8”，并分别将其替换为数字“B1”、“B6”、“B7”和“B8”。

修订指令英文原文（供核对）：

34. Section 23.441 is amended by amending paragraph (b) by striking out the numbers "1", "6", "7", and "8" in the references to figures 1, 6, 7, and 8, and inserting in place thereof the numbers "B1", "B6", "B7", and "B8", respectively.

### 2.3　Amdt 23-14（1973-12-20 生效）：第 23.441 条的 (a) 段修订为如下内容

▪ 修订沿革：本版由 NPRM　Notice 71-13，1971-04-27 签发，36 FR 8398　提出；最终规章 Docket 11011，1973-11-07 签发，38 FR 31816，1973-12-20 生效。

▪ 修订指令（中文译文）：13. 第 23.441 条的 (a) 段修订为如下内容：　14. §23.441 的 (a) 段将修订为如下内容：

来源：NPRM（提案）｜36 FR 8398｜NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt

▪ 背景与评论处置（1）：说明：该拟议修订落实了对 §23.441 条的拟议修订。

英文原文：

Explanation: The proposed amendment implements the proposed amendment to Sec. 23.441.

修订指令英文原文（供核对）：

13. Paragraph (a) of Sec. 23.441 is amended to read as follows:

14. Paragraph (a) of Sec. 23.441 would be amended to read as follows:

▪ 本版重印条文段落：(a)、(1)、(2)、(3)

### 2.4　Amdt 23-28（1982-04-28 生效）：通过修订 §23.441(b) 段

▪ 修订沿革：本版由 NPRM　Notice 80-23，1980-11-28 签发，45 FR 80450　提出；最终规章 Docket 21129，1982-02-26 签发，47 FR 13312，1982-04-28 生效。

▪ 修订指令（中文译文）：2. 通过修订 §23.441(b) 段，删除短语“(a)(1)、(a)(2) 和 (a)(3) 分别，”，并在其位置上插入短语“本条的 (a)(2)、(a)(1) 和 (a)(3) 分别。”。　4. 通过修订 §23.441(b) 段，删除短语"分别为(a)(1)、(a)(2)和(a)(3)"，并在其位置插入短语"分别为本条的(a)(2)、(a)(1)和(a)(3)"。

来源：Final Rule｜47 FR 13312｜FinalRule_Docket 21129_Amdt23-28_25-55_47FR13312_1982-03-29.txt

▪ 背景与评论处置（1）：这个

英文原文：

Proposal 4. This proposal would rearrange paragraphs (a)(1), (a)(2), and (a)(3) of Sec. 23.441 to ensure that the correct tail load distribution is imposed for the flight condition. One commenter points out that the desired correct correlation between load specifying figures in Appendix B and the alternate load requirements of Secs. 23.441(a), (b), and (c) could also be accomplished by leaving (a)(1), (a)(2), and (a)(3) in their present order while changing B6, B7, and B8, to B7, B6, and B8. Inasmuch as the interchange of the numbers 6 and 7 occurred initially when the prefix letter B was added in Amendment No. 23-7, August 13, 1969, and because other printings of Part 23 use the order B7, B6, and B8, the FAA disagrees with the commenter. Accordingly, the proposal is adopted without substantive change.

修订指令英文原文（供核对）：

2. By amending Sec. 23.441(b) by removing the phrase "(a)(1), (a)(2), and (a)(3), respectively," and by inserting the phrase "(a)(2), (a)(1), and (a)(3), respectively, of this section." in its place.

4. By amending Sec. 23.441(b) by deleting the phrase "(a)(1), (a)(2), and (a)(3), respectively" and inserting the phrase "(a)(2), (a)(1), and (a)(3), respectively, of this section," in its place.

### 2.5　Amdt 23-42（1991-02-04 生效）：对 C 分部进行修订

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：18. 对 C 分部进行修订，将 §23.441 之前的标题修改为如下内容：　19. 第 23.441 条的修订方式为：删除段(a)中两处的"tail"一词；删除段(b)的正文并将段(b)标为"[预留]"。　18. 修订 C 分部，将 §23.441 之前的引言标题修改如下：

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案

英文原文：

Proposal 18. The proposal would remove the word "tail" from the heading preceding Sec. 23.441 because the present heading implies that the sections following it are limited to tail surfaces of conventional airplane designs. The affected sections, as amended, would be applicable to design features of airplanes utilizing vertical surfaces at locations other than the tail of the airplane. No comments were received on this proposal and it is adopted as proposed.

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（2）：提案 19。该提案将把目前仅限于垂直尾翼面的 §23.441 条机动载荷要求扩展至新飞机设计中的所有垂直翼面（如翼梢小翼）。该提案还将禁止使用附录 B，这一点已在提案 14 中详细讨论。未收到针对该提案的任何评论，该提案按原提案予以采纳。

英文原文：

Proposal 19. This proposal would extend the maneuvering loads requirements of Sec. 23.441, which are currently limited to vertical tail surfaces, to all vertical surfaces, such as winglets, in new airplane designs. It also would prohibit the use of appendix B, as discussed in detail in proposal 14. No comments were received on this proposal and it is adopted as proposed.

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（3）：提案 21。本提案将修订 §23.445 条中的外侧垂尾要求，以纳入可能同时发生的所有载荷。它将要求合理分析包括可能施加于水平面的所有载荷，以及在 §23.441 条规定的机动情况下的 1g 非加速正常水平面载荷。它还将把要求扩展至安装在水平面上的所有垂直面，包括机翼。未收到关于本提案的任何评论，且按提案予以采纳。

英文原文：

Proposal 21. This proposal would amend the outboard fin requirements in Sec. 23.445 to include all loads that are likely to occur simultaneously. It would require that the rational analysis include all loads likely to be applied to horizontal surfaces, and the 1g unaccelerated normal horizontal surface loads during the maneuvering conditions specified in Sec. 23.441. It also would extend the requirements to all vertical surfaces that are mounted on horizontal surfaces, including wings. No comments were received on this proposal and it is adopted as proposed.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（4）：要求合理分析包括在 §23.441 条规定的机动情况期间，除 1g 非加速正常水平面载荷外，可能施加于水平面的所有载荷；
        这里语序调整符合中文习惯，意思准确。

        "be deleting the last sentence of paragraph (a)" -> "be deleting" 应该是 "by deleting" 的笔误？原文写的是 "be deleting"。翻译时按原意处理：“通过删除段 (a) 的最后一句”或“即删除段 (a) 的最后一句”。译为“即删除段 (a) 的最后一句”更通顺。

        "extend the proposed revised requirements to all vertical surfaces that are mounted on horizontal surfaces including wings."
        译文：并将拟议的修订要求扩展至安装在包括机翼在内的水平面上的所有垂直面。

        "Other recommended changes in conference proposal 211 were accepted without discussion."
        译文：会议提案 211 中的其他建议修改未经讨论即被接受。

4.  最终输出确认：
    本提案将修订 §23.445 条中的外侧鳍片要求，以包括可能同时发生的所有载荷；要求合理分析包括在 §23.441 条规定的机动情况期间，除 1g 非加速正常水平面载荷外，可能施加于水平面的所有载荷；并将拟议的修订要求扩展至安装在包括机翼在内的水平面上的所有垂直面。会议提案 211 建议修订要求以包括可能同时发生的所有载荷，即删除段 (a) 的最后
在制定本提案期间，通过会议提案和会议讨论确定了需要提出新的(d)段，其依据是过去的取证实践，即接受国家分析，其中垂直面上的机动载荷通过机身同时施加到机翼上，并伴有1g水平平衡载荷。

英文原文：

This proposal would amend the outboard fins requirements in Sec. 23.445 to include all loads that are likely to occur simultaneously; require that rational analysis include all loads likely to be applied to horizontal surfaces, in addition to the one-g unaccelerated normal horizontal surface loads, during the maneuvering conditions specified in Sec. 23.441; and extend the proposed revised requirements to all vertical surfaces that are mounted on horizontal surfaces including wings. Conference proposal 211 recommended amending the requirements to include all loads that are likely to occur simultaneously be deleting the last sentence of paragraph (a); however, it did not clearly recommend extending the requirements to vertical surfaces during a rational analysis. Discussions at the conference clarified that conference proposal 211 was intended to include vertical surfaces on wings. Other recommended changes in conference proposal 211 were accepted without discussion. The need to propose a new paragraph (d) was identified during development of this proposal from conference proposals and conference discussions and is based on past certification practices of accepting national analysis where maneuvering loads on vertical surfaces are applied to the wings through the fuselage simultaneously with one-g horizontal balancing load.

修订指令英文原文（供核对）：

18. Subpart C is amended by revising the heading preceding Sec. 23.441 to read as follows:

19. Section 23.441 is amended by removing the word "tail" in two places in paragraph (a); and by removing the text of paragraph (b) and designating paragraph (b) as "Reserved."

18. Subpart C is amended by revising the introductory title preceding Sec. 23.441 to read as follows:

### 2.6　Amdt 23-48（1996-03-11 生效）：修订 §23.441

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：16. 修订 §23.441，修改 (a)(2) 段，并新增 (b) 段，内容如下。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议修订 §23.441(b) 段，纳入一项关于通勤类飞机垂直尾翼的新设计要求。

英文原文：

The FAA proposed to revise Sec. 23.441(b) to include a new design requirement for the vertical tail of a commuter category airplane.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：欧洲联合航空
预计在 §23.441(b) 段中增加拟议的要求，将减少通勤类飞机中的此类失效。

英文原文：

The JAA suggested that Sec. 23.441 (b) be revised to include a new design requirement for the vertical tail of a commuter category airplane. The JAA determined that the vertical tail structure must be shown to be adequate for the loads imposed when the airplane is yawed by rudder deflection to the maximum attainable angle and is suddenly allowed to return by neutralizing the rudder. The maximum yaw condition is governed by any of several constraining conditions; for example, control surface stops, maximum available booster effect, or the various maximum pilot rudder forces that may be imposed. The JAA stressed that the design yaw excursions need to be examined throughout the full range of speeds of the flight envelope. The FAA agrees. Although this is a significant departure from the structural design philosophy depicted in part 23 (full use of all controls at maneuvering speed) the addition of a similar requirement to part 25 has served to reduce the static overload failures in part 25 airplanes. It is expected that the addition of the proposed requirement to Sec. 23.441(b) would reduce this type failure in commuter category airplanes.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（3）：此外，在 Sec. 23.441(a)(2) 下可假设的允许过摆角将从段(a)(3)静力侧滑角的1.3倍改为1.5倍。欧洲联合航空局（JAA）建议1.5这一数值更接近实际情况。美国联邦航空局（FAA）表示同意，并对规章作出修改以与欧洲联合航空要求（JAR）协调统一。最后，为澄清起见，将段(a)(2)第一句中的"resulting"一词改为"overswing"。

英文原文：

In addition, the permissible overswing angle that may be assumed under Sec. 23.441 (a)(2) would be changed from 1.3 to 1.5 times the static sideslip angle of paragraph (a)(3). The JAA suggested that the 1.5 figure more closely represents reality. The FAA agrees and the rule is changed to harmonize with the JAR. Finally, for clarification, the word "resulting" is changed to "overswing" in the first sentence of paragraph (a)(2).

修订指令英文原文（供核对）：

16. Section 23.441 is amended by revising paragraph (a)(2) and adding a new paragraph (b) to read as follows.

▪ 本版重印条文段落：(a)、(2)、(b)、(1)、(i)、(ii)、(iii)、(2)

## 3　§23.441 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | At speeds up to VA, the vertical tail surfaces must be designed to withstand- | 新增 |
| (1) | A sudden displacement of the rudder control (with the airplane in unaccelerated flight with zero yaw) to the maximum deflection allowed by the control stops or by pilot strength, whichever is critical; | 新增 |
| (2) | A yaw angle of 15 degrees with the rudder fully deflected (except as limited by pilot strength) in the direction tending to increase the slip; and | 新增 |
| (3) | A yaw angle of 15 degrees with the rudder control maintained in the neutral position (except as limited by pilot strength). | 新增 |
| (b) | The average loading of B23.11 and figure 1 of Appendix B and the distribution in figures 7, 6, and 8 of Appendix B may be used instead of the requirements of subparagraphs (a)(1), (a)(2), and (a)(3), respectively. | 新增 |
| (c) | The yaw angles specified in paragraph (a)(3) of this section may be reduced if the yaw angle chosen for a particular speed cannot be exceeded in-- | 新增 |
| (1) | Steady slip conditions; | 新增 |
| (2) | Uncoordinated rolls from steep banks; or | 新增 |
| (3) | Sudden failure of the critical engine with delayed corrective action. | 新增 |

### 3.2　版本2：Amdt 23-14（1973-12-20 生效）　来源：Docket 11011，1973-11-07 签发，38 FR 31816

修订：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | As speeds up to VA, the vertical tail surfaces must be designed to withstand the following conditions. In computing the tail loads, the yawing velocity may be assumed to be zero; | 修订 |
| (1) | With the airplane in unaccelerated flight at zero yaw, it is assumed that the rudder control is suddenly displaced to the maximum deflection, as limited by the control stops or by limit pilot forces. | 修订 |
| (2) | With the rudder deflected as specified in paragraph (a)(1) of this section, it is assumed that the airplane yaws to the resulting sideslip angle. In lieu of a rational analysis, an overswing angle equal to 1.3 times the static sideslip angle of paragraph (a)(3) of this section may be assumed. | 修订 |
| (3) | A yaw angle of 15 degrees with the rudder control maintained in the neutral position (except as limited by pilot strength). | 修订 |

### 3.3　版本3：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

新增内容：(a)、(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (2) | With the rudder deflected as specified in paragraph (a)(1) of this section, it is assumed that the airplane yaws to the overswing sideslip angle. In lieu of a rational analysis, an overswing angle equal to 1.5 times the static sideslip angle of paragraph (a)(3) of this section may be assumed. | 修订 |
| (b) | For commuter category airplanes, the loads imposed by the following additional maneuver must be substantiated at speeds from V A to V D/M D. When computing the tail loads-- | 新增 |
| (1) | The airplane must be yawed to the largest attainable steady state sideslip angle, with the rudder at maximum deflection caused by any one of the following: | 修订 |
| (i) | Control surface stops; | 新增 |
| (ii) | Maximum available booster effort; | 新增 |
| (iii) | Maximum pilot rudder force as shown below: BILLING CODE 4910-13-M [GRAPHIC] [TIFF OMMITTED] TR09FE96.006 BILLING CODE 4910-13-C | 新增 |
| (2) | The rudder must be suddenly displaced from the maximum deflection to the neutral position. | 修订 |

### 3.4　段落结构演变矩阵（横向对照 6 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-14 (1973-12-20) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | △ 修订 |
| (1) | ● 新增 | △ 修订 | △ 修订 |
| (2) | ● 新增 | △ 修订 | △ 修订 |
| (3) | ● 新增 | △ 修订 | ○ 无 |
| (b) | ● 新增 | ○ 无 | ● 新增 |
| (c) | ● 新增 | ○ 无 | ○ 无 |
| (i) | ○ 无 | ○ 无 | ● 新增 |
| (ii) | ○ 无 | ○ 无 | ● 新增 |
| (iii) | ○ 无 | ○ 无 | ● 新增 |

## 4　§23.441 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 澄清性与编辑性修订 | s during a rational analysis. Discussions at the conference clarified that conference proposal 211 was intended to include vertical surfaces on wings. Other recommended changes in conference proposal 211 were accepted wi… | Amdt 23-42 |
| 国际协调 | for the vertical tail of a commuter category airplane. The JAA suggested that Sec. 23.441 (b) be revised to include a new design requirement for the vertical tail of a commuter category airplane. The JAA determined that… | Amdt 23-48 |

本条取证提示 共 6 个历史版本：初始版, Amdt 23-7, Amdt 23-14, Amdt 23-28, Amdt 23-42, Amdt 23-48。 其中 5 个版本在本地文献中检索到明确修订指令原文；3 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 11011 | 38 FR 31816 | FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt |
| [6] | NPRM（提案） | Notice 71-13 / Docket 11011 | 36 FR 8398 | NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt |
| [7] | Final Rule（颁行） | Docket 21129 | 47 FR 13312 | FinalRule_Docket 21129_Amdt23-28_25-55_47FR13312_1982-03-29.txt |
| [8] | NPRM（提案） | Notice 80-23 / Docket 21129 | 45 FR 80450 | NPRM_Notice 80-23_Docket21129_45FR80450_1980-12-04.txt |
| [9] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [10] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [11] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [12] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [13] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [14] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [15] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.441 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | f6db247e-bef8-4b9a-bb04-8e46e5876be6 |
| Amdt 23-7 | 1969-09-14 | f9690316-22ec-440a-a0d8-7216ba35f44e |
| Amdt 23-14 | 1973-12-20 | 2dd57d31-1d46-4fce-b975-ae1d392b36a3 |
| Amdt 23-28 | 1982-04-28 | b83e8561-337b-462c-893d-5107e92b8351 |
| Amdt 23-42 | 1991-02-04 | a64dc02d-43e2-42ff-8ce4-cc16266ba2d3 |
| Amdt 23-48 | 1996-03-11 | e9206756-6bbf-43cf-aa04-a34d985640ee |

# § 23.443　阵风载荷（尾翼）（Gust loads.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 5 个历史版本

## 1　§23.443 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.443 |
| 条款标题（英文原文） | Gust loads. |
| 中文名称 | 阵风载荷（尾翼） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 5 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 52 FR 34745、52 FR 7262 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.443 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订 |
| 3 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 新增段落：(b)、(c) |
| 4 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 新增内容：(a)、(c)、(d) |
| 5 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订　→　1987 23-34　新增段落：(b)、(c)　→　1991 23-42　新增内容：(a)、(c)、(d)　→　1996 23-48　修订

## 2　§23.443 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：阵风载荷（尾翼）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第 23.443 条修订如下

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：35. 第 23.443 条修订如下：

修订指令英文原文（供核对）：

35. Section 23.443 is amended to read as follows:

▪ 本版重印条文段落：(a)、(b)、(c)

### 2.3　Amdt 23-34（1987-02-17 生效）：通过修订 §23.443 条

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 修订指令（中文译文）：25. 通过修订 §23.443 条，将段和段分别重新编号为段和段；并增加一个新段，内容如下：　23. 通过修订 §23.443 条，分别将段 (b) 和 (c) 以及段 (c) 和 (d) 重新编号；并增加新的段 (b)，内容如下：

来源：Final Rule｜52 FR 1806｜FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt

▪ 背景与评论处置（1）：提案 23 —— 未收到对该提案的评论。因此，§23.443 条按提案修订。

英文原文：

Proposal 23 -- No comments were received to the proposal. Accordingly, Sec. 23.443 is amended as proposed.

修订指令英文原文（供核对）：

25. By amending Sec. 23.443 by redesignating paragraphs (b) and (c) as paragraphs (c) and (d) respectively; and by adding a new paragraph (b) to read as follows:

23. By amending Sec. 23.443 by redesignating paragraphs (b) and (c) and paragraphs (c) and (d) respectively; and by adding a new paragraph (b) to read as follows:

▪ 本版重印条文段落：(b)

### 2.4　Amdt 23-42（1991-02-04 生效）：第 23.443 条的修订方式为：删除段(a)中的"tail"一词

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：20. 第 23.443 条的修订方式为：删除段(a)中的"tail"一词；删除段(c)定义中三处出现的"tail"一词，并在原处加入"surface"一词；以及删除段(d)。　20. 对第23.443条进行修订，删除(a)段中的“tail”一词；删除(c)段中三处“tail”一词，并在其位置加入“surface”一词；并删除(d)段。

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案 20。本提案将扩展 §23.443 条针对常规飞机的阵风载荷要求，以涵盖鸭翼和串列翼构型，并禁止使用附录 B，如提案 14 中详细讨论的那样。未收到关于本提案的评论，并按提案予以采纳。

英文原文：

Proposal 20. This proposal would extend the gust load requirements of Sec. 23.443 for conventional airplanes to include the canard and tandem wing configuration and prohibit the use of appendix B, as discussed in detail in proposal 14. No comments were received on this proposal and it is adopted as proposed.

修订指令英文原文（供核对）：

20. Section 23.443 is amended by removing the word "tail" from paragraph (a); by removing in three places the word "tail" in the definitions in paragraph (c) and adding in its place the word "surface"; and by removing paragraph (d).

20. Section 23.443 is amended by removing the word "tail" from paragraph (a); by removing in three places the word "tail" in paragraph (c) and adding in its place the word "surface"; and by removing paragraph (d).

### 2.5　Amdt 23-48（1996-03-11 生效）：§23.443 条修订如下：修订 (c) 段

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：§23.443 条修订如下：修订 (c) 段，内容如下：　§23.443(c) 段将通过更改公式的格式、修订重量“W,”的定义以及更正到升力中心距离的下标进行修订，当前定义的内容为“W=飞机重量（lbs.）。”拟议定义的内容为“W = 飞机在特定载荷情况下的适用重量（lbs.）。”拟议的更改是为了清晰以及与欧洲联合航空要求（JAR）23 协调统一。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦

英文原文：

The FAA proposed to revise Sec. 23.443(c) by changing the format of the formula, revising the definition of weight ("W"), and correcting the subscripts of the distance to the lift center, ("l

修订指令英文原文（供核对）：

17. Section 23.443(c) is revised to read as follows:

Section 23.443(c) would be revised by changing the format of the formula, revising the definition of weight, "W," and correcting the subscripts of the distance to the lift center, The current definition reads " W=airplane weight (lbs.)." The proposed definition reads " W = the applicable weight of the airplane in the particular load case (lbs.)." The proposed changes are for clarity and harmonization with JAR 23.

▪ 本版重印条文段落：(c)、(2)、(2)

### 2.6　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.443 的实际改动 |
| --- | --- | --- | --- |
| 52 FR 34745 | 1987-09-14 | 更正通告 (CFR 溯源注) | 18 项印刷更正；其中第 9 项新增修订指令 25-1：§23.561(b)(2) 表格第一栏标题 'Normal and utility categories'→'Normal, utility, and commuter categories'。⚠ 本件 FR 抬头把 Amdt Nos. 误印为 23-24（正文通篇写 23-34），CFR 溯源注因此跟着写错 —— DRS 归 Amdt 23-34 是对的 ¦ 23 项更正；涉条文的是 §23.67(e)(3) 'Vs4'→'Vs1'、§23.443(b) 'the'→'The'、§23.1201(a) 'Material'→'No material'，其余为前言拼写错误 |
| 52 FR 7262 | 1987-03-09 | 更正通告 (CFR 溯源注) | 18 项印刷更正；其中第 9 项新增修订指令 25-1：§23.561(b)(2) 表格第一栏标题 'Normal and utility categories'→'Normal, utility, and commuter categories'。⚠ 本件 FR 抬头把 Amdt Nos. 误印为 23-24（正文通篇写 23-34），CFR 溯源注因此跟着写错 —— DRS 归 Amdt 23-34 是对的 ¦ 23 项更正；涉条文的是 §23.67(e)(3) 'Vs4'→'Vs1'、§23.443(b) 'the'→'The'、§23.1201(a) 'Material'→'No material'，其余为前言拼写错误 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.443 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Vertical tail surfaces must be designed to withstand, in unaccelerated flight at VC, a gust of 30 feet per second nominal intensity normal to the plane of symmetry. | 新增 |
| (b) | The gust loading for that part of a vertical tail surface with a well defined leading edge must be computed by the formula Where-- =average limit unit pressure in pounds per square foot; K= , except that K may not be less than 1.0. U=nominal gust intensity in feet per second; V=airplane speed in miles per hour; m=slope of lift curve of vertical surface, CL per radian, corrected for aspect ratio; W=design weight in pounds; and SV=vertical surface area in square feet. A value of K obtained by rational determination may be used. | 新增 |
| (c) | The average loading in figure 5 and the distribution in figure 8 of Appendix B may be used. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Vertical tail surfaces must be designed to withstand, in unaccelerated flight at speed VC, lateral gusts of the values prescribed for VC in Sec. 23.333(c). | 修订 |
| (b) | In the absence of a more rational analysis, the gust load must be computed as follows: Where-- Lvt = Vertical tail loads (lbs); Kgt = = gust alleviation factor; = = lateral mass ratio; Ude= Derived gust velocity (f.p.s.); = Air density (slugs/cu. ft); W = Airplane weight (lbs.); Svt = Area of vertical tail (ft.2); = Mean geometric chord or vertical surface (ft.); avt = Lift curve slope of vertical tail (per radian); K = Radius of gyration in yaw (ft.); 1t = Distance from airplane c.g. to lift center of vertical surface (ft.); g = Acceleration due to gravity (ft./sec.2); and V = Airplane equivalent speed (knots). | 修订 |
| (c) | The average loading in figure B5 and the distribution in figure B8 of Appendix B may be used. | 修订 |

### 3.3　版本3：Amdt 23-34（1987-02-17 生效）　来源：Docket 23516，1987-01-08 签发，52 FR 1806

新增段落：(b)、(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | In addition, for commuter category airplanes, the airplane is assumed to encounter derived gusts normal to the plane of symmetry while in unaccelerated flight at VB, VC, VD, and VF. The derived gusts and airplane speeds corresponding to these conditions, as determined by Secs. 23.341 and 23.345, must be investigated. The shape of the gust must be as specified in Sec. 23.333(c)(2)(i). | 修订 |

### 3.4　版本4：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | In the absence of a more rational analysis, the gust load must be computed as follows: [GRAPHIC] [TIFF OMMITTED] TR09FE96.000 Where--L vt=Vertical surface loads (lbs.); [GRAPHIC] [TIFF OMMITTED] TR09FE96.001 [GRAPHIC] [TIFF OMMITTED] TR09FE96.002 U de=Derived gust velocity (f.p.s.); =Air density (slugs/cu.ft.); W=the applicable weight of the airplane in the particular load case (lbs.); S vt=Area of vertical surface (ft. | 新增 |
| (2) | ; c t=Mean geometric chord of vertical surface (ft.); a vt=Lift curve slope of vertical surface (per radian); K=Radius of gyration in yaw (ft.); l vt=Distance from airplane c.g. to lift center of vertical surface (ft.); g=Acceleration due to gravity (ft./sec. | 新增 |
| (2) | ; and V=Equivalent airspeed (knots). | 新增 |

### 3.5　段落结构演变矩阵（横向对照 5 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) | Amdt 23-34 (1987-02-17) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | ○ 无 | ○ 无 |
| (b) | ● 新增 | △ 修订 | △ 修订 | ○ 无 |
| (c) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (2) | ○ 无 | ○ 无 | ○ 无 | ● 新增 |

## 4　§23.443 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 澄清性与编辑性修订 | .443 is amended as proposed. 25. By amending Sec. 23.443 by redesignating paragraphs (b) and (c) as paragraphs (c) and (d) respectively; and by adding a new paragraph (b) to read as follows: Sec. 23.443 Gust loads. * * *… | Amdt 23-34 |
| 新技术 / 新构型 | ts of Sec. 23.443 for conventional airplanes to include the canard and tandem wing configuration and prohibit the use of appendix B, as discussed in detail in proposal 14. No comments were received on this proposal and i… | Amdt 23-42 |
| 国际协调 | load case (lbs.)." The proposed changes are for clarity and harmonization with JAR 23.… | Amdt 23-48 |

本条取证提示 共 5 个历史版本：初始版, Amdt 23-7, Amdt 23-34, Amdt 23-42, Amdt 23-48。 其中 4 个版本在本地文献中检索到明确修订指令原文；4 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：52 FR 34745、52 FR 7262，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [6] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [7] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [8] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [9] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [10] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [11] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [12] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [13] | 更正通告 / 校正 | 52 FR 34745 | 1987-09-14 | Correction_52FR34745_1987-09-14.clean.txt |
| [14] | 更正通告 / 校正 | 52 FR 7262 | 1987-03-09 | Correction_52FR7262_1987-03-09.clean.txt |
| [15] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [16] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.443 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 707c955d-9f20-4932-8801-ca1135ef8139 |
| Amdt 23-7 | 1969-09-14 | cc759d72-17ec-43a4-a8d3-c795af22e6f8 |
| Amdt 23-34 | 1987-02-17 | fdb89547-b285-4216-8605-027832057b51 |
| Amdt 23-42 | 1991-02-04 | 5a12fafb-0441-4b66-82e8-ed3d7fdf0107 |
| Amdt 23-48 | 1996-03-11 | 369786c8-c84e-4abb-b505-a9689f7136ec |

# § 23.445　外侧垂直安定面（Outboard fins.）

1965-02-01 初版　｜　1991-02-04 Amdt 23-42　｜　共 3 个历史版本

## 1　§23.445 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.445 |
| 条款标题（英文原文） | Outboard fins. |
| 中文名称 | 外侧垂直安定面 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 3 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1991-02-04 （约 26 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.445 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-14 | 1973-12-20 | Notice 71-13，1971-04-27 签发，36 FR 8398 | Docket 11011，1973-11-07 签发，38 FR 31816 | 新增段落：(c)、(b) |
| 3 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 新增段落：(a)、(b)、(d) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1973 23-14　新增段落：(c)、(b)　→　1991 23-42　新增段落：(a)、(b)、(d)

## 2　§23.445 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：外侧垂直安定面

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-14（1973-12-20 生效）：§23.445 条新增 (c) 段

▪ 修订沿革：本版由 NPRM　Notice 71-13，1971-04-27 签发，36 FR 8398　提出；最终规章 Docket 11011，1973-11-07 签发，38 FR 31816，1973-12-20 生效。

▪ 修订指令（中文译文）：§23.445 条新增 (c) 段，内容如下：　15. 修订§23.445，增加一个新的段(c)，内容如下：

来源：Final Rule｜38 FR 31816｜FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt

▪ 背景与评论处置（1）：一条评论对拟
一位评论者建议对该条进行修订，要求在疲劳关键区域采用破损安全结构。美国联邦航空局（FAA）不同意该意见。该评论超出了通告的范围，而且按建议对该条进行修订将对航空器设计施加不必要的限制。

英文原文：

A comment questioned the reference in proposed Sec. 23.445(c) to "vertical surfaces in paragraph (b)." The proposed subsection is clear and the reference is correct.

In response to a comment received, Secs. 23.507(a) and 23.509(c)(1) have been editorially revised for greater clarity. In addition, proposed Sec. 23.507(b) has been revised to state more clearly the requirement of that paragraph. One commentator objected to proposed Sec. 23.507(a)(2)(ii) as being too severe and suggested that 0.2 times the vertical static reactions be used for the fore, aft, and lateral load factors. The FAA does not agree. The commentator has not submitted, and the FAA is unaware of, any data to substantiate the claim of severity.

The notice explained that the purpose of the proposed amendment to Sec. 23.572 (a)(1) was to make it clear that, for a fatigue evaluation, the use of analysis alone is acceptable only under certain specified circumstances. The word "only" has accordingly been inserted in that section to make the requirement clear. One commentator suggested that the section be revised to require fail safe structure in fatigue critical areas. The FAA does not agree. The comment is beyond the scope of the notice and a revision of the section, as suggested, would place an unnecessary restriction on aircraft design.

修订指令英文原文（供核对）：

14. Section 23.445 is amended by adding a new paragraph (c) to read as follows:

15. By amending Sec. 23.445 by adding a new paragraph (c) to read as follows:

▪ 本版重印条文段落：(c)

### 2.3　Amdt 23-42（1991-02-04 生效）：对第 23.445 条进行修订：修订该条标题；修订 (a) 段；在

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：21. 对第 23.445 条进行修订：修订该条标题；修订 (a) 段；在 (b) 段和 (c) 段中"outboard fins"一词之后增加"or winglets"（翼梢小翼）一词；并增加新的 (d) 段，内容如下：　21. 第 23.445 条通过以下方式修订：修订条标题；修订段(a)；在段(b)和(c)中“外侧鳍板”一词之后增加“或翼梢小翼”一词；并增加新的段(d)，内容如下：

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案 21。本提案将修订 §23.445 条中的外侧垂尾要求，以纳入可能同时发生的所有载荷。它将要求合理分析包括可能施加于水平面的所有载荷，以及在 §23.441 条规定的机动情况下的 1g 非加速正常水平面载荷。它还将把要求扩展至安装在水平面上的所有垂直面，包括机翼。未收到关于本提案的任何评论，且按提案予以采纳。

英文原文：

Proposal 21. This proposal would amend the outboard fin requirements in Sec. 23.445 to include all loads that are likely to occur simultaneously. It would require that the rational analysis include all loads likely to be applied to horizontal surfaces, and the 1g unaccelerated normal horizontal surface loads during the maneuvering conditions specified in Sec. 23.441. It also would extend the requirements to all vertical surfaces that are mounted on horizontal surfaces, including wings. No comments were received on this proposal and it is adopted as proposed.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（2）：本提案将目前仅适用于垂直尾翼面的机动载荷要求扩展至所有垂直面，例如新飞机设计中出现的翼梢小翼。此前并无按本文所提方式修订本条的具体会议提案，会议期间也未对此进行讨论。但是，对 §23.445 条（该条以引用方式纳入本条）要求的建议修改内容的制定工作，促使对本条进行了审查，并由此形成了本提案的修改。此项修改被认为是必要的，以确保受影响表面的结构完整性等同于对常规垂直尾翼面所要求的结构完整性。

英文原文：

This proposal would extend the maneuvering loads requirements that are now limited to vertical tail surfaces to all vertical surfaces such as winglets that are appearing in new airplane designs. There was not a specific conference proposal to amend this section as proposed herein and it was not discussed at the conference. However, development of recommended changes to requirements in Sec. 23.445, which incorporates this section by reference, caused a review of this section and resulted in this proposed change. This change is considered necessary to assure structural integrity of the affected surfaces equivalent to that required for conventional vertical tail surfaces.

来源：NPRM（提案）｜54 FR 9276｜NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt

▪ 背景与评论处置（3）：要求合理分析包括在 §23.441 条规定的机动情况期间，除 1g 非加速正常水平面载荷外，可能施加于水平面的所有载荷；
        这里语序调整符合中文习惯，意思准确。

        "be deleting the last sentence of paragraph (a)" -> "be deleting" 应该是 "by deleting" 的笔误？原文写的是 "be deleting"。翻译时按原意处理：“通过删除段 (a) 的最后一句”或“即删除段 (a) 的最后一句”。译为“即删除段 (a) 的最后一句”更通顺。

        "extend the proposed revised requirements to all vertical surfaces that are mounted on horizontal surfaces including wings."
        译文：并将拟议的修订要求扩展至安装在包括机翼在内的水平面上的所有垂直面。

        "Other recommended changes in conference proposal 211 were accepted without discussion."
        译文：会议提案 211 中的其他建议修改未经讨论即被接受。

4.  最终输出确认：
    本提案将修订 §23.445 条中的外侧鳍片要求，以包括可能同时发生的所有载荷；要求合理分析包括在 §23.441 条规定的机动情况期间，除 1g 非加速正常水平面载荷外，可能施加于水平面的所有载荷；并将拟议的修订要求扩展至安装在包括机翼在内的水平面上的所有垂直面。会议提案 211 建议修订要求以包括可能同时发生的所有载荷，即删除段 (a) 的最后
在制定本提案期间，通过会议提案和会议讨论确定了需要提出新的(d)段，其依据是过去的取证实践，即接受国家分析，其中垂直面上的机动载荷通过机身同时施加到机翼上，并伴有1g水平平衡载荷。

英文原文：

This proposal would amend the outboard fins requirements in Sec. 23.445 to include all loads that are likely to occur simultaneously; require that rational analysis include all loads likely to be applied to horizontal surfaces, in addition to the one-g unaccelerated normal horizontal surface loads, during the maneuvering conditions specified in Sec. 23.441; and extend the proposed revised requirements to all vertical surfaces that are mounted on horizontal surfaces including wings. Conference proposal 211 recommended amending the requirements to include all loads that are likely to occur simultaneously be deleting the last sentence of paragraph (a); however, it did not clearly recommend extending the requirements to vertical surfaces during a rational analysis. Discussions at the conference clarified that conference proposal 211 was intended to include vertical surfaces on wings. Other recommended changes in conference proposal 211 were accepted without discussion. The need to propose a new paragraph (d) was identified during development of this proposal from conference proposals and conference discussions and is based on past certification practices of accepting national analysis where maneuvering loads on vertical surfaces are applied to the wings through the fuselage simultaneously with one-g horizontal balancing load.

修订指令英文原文（供核对）：

21. Section 23.445 is amended by revising the section heading; by revising paragraph (a); by adding the words "or winglets" after the words "outboard fins" in paragraphs (b) and (c); and by adding a new paragraph (d) to read as follows:

21. Section 23.445 is amended by revising the section title; by revising paragraph (a); by adding the words "or winglets" after the words "outboard fins" in paragraphs (b) and (c); and by adding a new paragraph (d) to read as follows:

▪ 本版重印条文段落：(a)、(d)

## 3　§23.445 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | If outboard fins are on the horizontal tail surface, the tail surfaces must be designed for the maximum horizontal surface load in combination with the corresponding loads induced on the vertical surfaces by endplate effects. These induced effects need not be combined with other vertical surface loads. | 新增 |
| (b) | If outboard fins extend above and below the horizontal surface, the critical vertical surface loading (the load per unit area as determined under Secs. 23.441 and 23.443) must be applied to-- | 新增 |
| (1) | The part of the vertical surfaces above the horizontal surface with 80 percent of that loading applied to the part below the horizontal surface; and | 新增 |
| (2) | The part of the vertical surfaces below the horizontal surface with 80 percent of that loading applied to the part above the horizontal surface. Ailerons, Wing Flaps, and Special Devices | 新增 |

### 3.2　版本2：Amdt 23-14（1973-12-20 生效）　来源：Docket 11011，1973-11-07 签发，38 FR 31816

新增段落：(c)、(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | The end plate effects of outboard fins must be taken into account in applying the yawing conditions of Sec. 23.441 and Sec. 23.443 to the vertical surfaces in paragraph (b) of this section. | 新增 |

### 3.3　版本3：Amdt 23-42（1991-02-04 生效）　来源：Docket 25811，1990-12-21 签发，56 FR 344

新增段落：(a)、(b)、(d)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | If outboard fins or winglets are included on the horizontal surfaces or wings, the horizontal surfaces or wings must be designed for their maximum load in combination with loads induced by the fins or winglets and moments or forces exerted on the horizontal surfaces or wings by the fins or winglets. | 新增 |
| (d) | When rational methods are used for computing loads, the maneuvering loads of Sec. 23.441 on the vertical surfaces and the one g horizontal surface load, including induced loads on the horizontal surface and moments or forces exerted on the horizontal surfaces by the vertical surfaces, must be applied simultaneously for the structural loading condition. | 新增 |

### 3.4　段落结构演变矩阵（横向对照 3 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-14 (1973-12-20) | Amdt 23-42 (1991-02-04) |
| --- | --- | --- | --- |
| (a) | ● 新增 | ○ 无 | ● 新增 |
| (b) | ● 新增 | ○ 无 | ○ 无 |
| (1) | ● 新增 | ○ 无 | ○ 无 |
| (2) | ● 新增 | ○ 无 | ○ 无 |
| (c) | ○ 无 | ● 新增 | ○ 无 |
| (d) | ○ 无 | ○ 无 | ● 新增 |

## 4　§23.445 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 澄清性与编辑性修订 | omment received, Secs. 23.507(a) and 23.509(c)(1) have been editorially revised for greater clarity. In addition, proposed Sec. 23.507(b) has been revised to state more clearly the requirement of that paragraph. One comm… | Amdt 23-14 |
| 澄清性与编辑性修订 | s during a rational analysis. Discussions at the conference clarified that conference proposal 211 was intended to include vertical surfaces on wings. Other recommended changes in conference proposal 211 were accepted wi… | Amdt 23-42 |

本条取证提示 共 3 个历史版本：初始版, Amdt 23-14, Amdt 23-42。 其中 2 个版本在本地文献中检索到明确修订指令原文；3 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 11011 | 38 FR 31816 | FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt |
| [4] | NPRM（提案） | Notice 71-13 / Docket 11011 | 36 FR 8398 | NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt |
| [5] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [6] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [7] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [8] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.445 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 569f8b7a-6096-4379-b28b-61ab7c87fd35 |
| Amdt 23-14 | 1973-12-20 | fdf0d417-a879-43ae-a429-356cf39d43be |
| Amdt 23-42 | 1991-02-04 | 71d29947-c1bb-4212-aae1-e747e0ee3723 |

# § 23.455　副翼（Ailerons.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 4 个历史版本

## 1　§23.455 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.455 |
| 条款标题（英文原文） | Ailerons. |
| 中文名称 | 副翼 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.455 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(b) |
| 3 | Amdt 23-42 | 1991-02-04 | Notice 89-5，1989-02-23 签发，54 FR 9276 | Docket 25811，1990-12-21 签发，56 FR 344 | 删除内容：(b) |
| 4 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 删除内容 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(b)　→　1991 23-42　删除内容：(b)　→　1996 23-48　删除内容

## 2　§23.455 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：副翼

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第23.455条经修订

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：36. 第23.455条经修订，将(b)段修改为如下内容：

修订指令英文原文（供核对）：

36. Section 23.455 is amended by amending paragraph (b) to read as follows:

▪ 本版重印条文段落：(b)

### 2.3　Amdt 23-42（1991-02-04 生效）：修订第23.455条

▪ 修订沿革：本版由 NPRM　Notice 89-5，1989-02-23 签发，54 FR 9276　提出；最终规章 Docket 25811，1990-12-21 签发，56 FR 344，1991-02-04 生效。

▪ 修订指令（中文译文）：22. 修订第23.455条，删除(b)段的文本，并将其标记为“[预留]”。　22. 第23.455条经修订，删除(b)段的内容，并将(b)段指定为“预留”。

来源：Final Rule｜56 FR 344｜FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt

▪ 背景与评论处置（1）：提案22。本提案将禁止在§23.455中使用附录B，如提案14中详细讨论的那样。未收到关于本提案的评论意见，现按提案原样予以采纳。

英文原文：

Proposal 22. This proposal would prohibit the use of appendix B in Sec. 23.455, as discussed in detail in proposal 14. No comments were received on this proposal and it is adopted as proposed.

修订指令英文原文（供核对）：

22. Section 23.455 is amended by removing the text of paragraph (b) and marking it "[Reserved]".

22. Section 23.455 is amended by removing the text of paragraph (b) and designating paragraph (b) as "Reserved."

### 2.4　Amdt 23-48（1996-03-11 生效）：§23.455 之前的标题将作修订

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：§23.455 之前的标题将作修订，删除“机翼襟翼”一词，使标题变为“副翼和特殊装置”。此项修改反映了拟从 §23.457 中删除机翼襟翼要求并将其纳入 §23.345 的提案。 注：原文中 "Sec. 23.455" 按规则译为 "§23.455"，"Sec. 23.457" 译为 "§23.457"，"Sec. 23.345" 译为 "§23.345"。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修改§23.455之前的标题，删除"翼襟"一词，使标题变为"副翼和专用装置"。此项修改将反映从§23.457中删除翼襟要求并将其置于§23.345中的变化。

英文原文：

The FAA proposed to amend the heading the precedes Sec. 23.455 by deleting the term "Wing Flaps" so that the heading reads "AILERONS AND SPECIAL DEVICES". This change would reflect the deletion of the wing flap requirements from Sec. 23.457 and their placement in Sec. 23.345.

修订指令英文原文（供核对）：

The heading the precedes Sec. 23.455 would be amended by deleting the term "Wing Flaps" so that the heading reads "AILERONS AND SPECIAL DEVICES." This change reflects the proposed deletion of the wing flap requirements from Sec. 23.457 and their placement in Sec. 23.345.

## 3　§23.455 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The ailerons must be designed for the loads to which they are subjected-- | 新增 |
| (1) | In the neutral position during symmetrical flight conditions; and | 新增 |
| (2) | By the following deflections (except as limited by pilot effort), during unsymmetrical flight conditions: | 新增 |
| (i) | Sudden maximum displacement of the aileron control at VA. Suitable allowance may be made for control system deflections. | 新增 |
| (ii) | Sufficient deflection at VC, where VC is more than VA, to produce a rate of roll not less than obtained in subparagraph (2)(i). | 新增 |
| (iii) | Sufficient deflection at VD to produce a rate of roll not less than one-third of that obtained in subparagraph (2)(i). | 新增 |
| (b) | The average loading in B23.11 and figure 1 of Appendix B and the distribution in figure 9 of Appendix B may be used. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订：(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | The average loading in B23.11 and figure B1 of Appendix B and the distribution in figure B9 of Appendix B may be used. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (1) | ● 新增 | ○ 无 |
| (2) | ● 新增 | ○ 无 |
| (i) | ● 新增 | ○ 无 |
| (ii) | ● 新增 | ○ 无 |
| (iii) | ● 新增 | ○ 无 |
| (b) | ● 新增 | △ 修订 |

## 4　§23.455 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 4 个历史版本：初始版, Amdt 23-7, Amdt 23-42, Amdt 23-48。 其中 3 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 25811 | 56 FR 344 | FinalRule_Docket 25811_Amdt1-37_23-42_56FR344_1991-01-03_0002.txt |
| [6] | NPRM（提案） | Notice 89-5 / Docket 25811 | 54 FR 9276 | NPRM_Notice 89-5_Docket25811_54FR9276_1989-03-06.txt |
| [7] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [8] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [9] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [10] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [11] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.455 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | c0d3ab68-2dfb-4d68-a9cd-5405a8f2c5b6 |
| Amdt 23-7 | 1969-09-14 | 8ffa58d9-c069-4077-9b59-4afd8bc22d9e |
| Amdt 23-42 | 1991-02-04 | 82a1e09a-a7fc-40f0-84b7-be36aff1597a |
| Amdt 23-48 | 1996-03-11 | b98f0c24-8aa6-43fd-a2cc-fb9e59eddb3b |

# § 23.457　机翼襟翼（Removed.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 2 个历史版本

## 1　§23.457 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.457 |
| 条款标题（英文原文） | Removed. |
| 中文名称 | 机翼襟翼 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.457 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 删除段落 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1996 23-48　删除段落

## 2　§23.457 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：机翼襟翼

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-48（1996-03-11 生效）：§23.457 条予以删除。

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：19. §23.457 条予以删除。　现行(c)段将重新指定为(d)段，并通过纳入第23.457条的要求进行修订。(e)段将被删除，因为它仅引用第23.457条的要求；这些要求已移至第23.345条(d)段。这种安排将所有"襟翼"要求集中在一处，并将使这些要求与JAR 23协调统一。　§23.455 之前的标题将作修订，删除“机翼襟翼”一词，使标题变为“副翼和特殊装置”。此项修改反映了拟从 §23.457 中删除机翼襟翼要求并将其纳入 §23.345 的提案。 注：原文中 "Sec. 23.455" 按规则译为 "§23.455"，"Sec. 23.457" 译为 "§23.457"，"Sec. 23.345" 译为 "§23.345"。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：为了将所有“襟翼”要求集中在一处，并与 JAR 23 实现协调统一，美国联邦航空局提议修订 §23.345 如下：（1）进行较小的组织性、非实质性的澄清性修改；（2）将术语“完全偏转”改为“完全放下”，因为后者更准确地描述襟翼状态和位置；（3）删除短语“导致限制载荷因数”，因为该要求已存在于 §23.301(a) 段；（4）将现行(c)段改列为(d)段，并修订该段以纳入 §23.457 的襟翼要求；（5）将现行(d)段改列为(c)段；（6）将 §23.457 的襟翼要求纳入经改列后的 §23.345(b) 段和 §23.345(d) 段，并删除冗余的(e)段。

英文原文：

To place all "flap" requirements in one location, and to harmonize the requirements with JAR 23, the FAA proposed to revise Sec. 23.345 as follows: (1) Make minor organizational, and non- substantive, clarifying changes; (2) Change the term "fully deflected" to "fully extended" because it more accurately describes flap conditions and positions; (3) Remove the phrase "resulting in limit load factors" because the requirement already exists in Sec. 23.301(a); (4) Redesignate current paragraph (c) as paragraph (d) and revise it to include the flap requirements of Sec. 23.457; (5) Redesignate current paragraph (d) as paragraph (c); and (6) Incorporate the flap requirements of Sec. 23.457 into Sec. 23.345(b) and Sec. 23.345(d), as redesignated, and delete paragraph (e), which is redundant.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局提议修改§23.455之前的标题，删除"翼襟"一词，使标题变为"副翼和专用装置"。此项修改将反映从§23.457中删除翼襟要求并将其置于§23.345中的变化。

英文原文：

The FAA proposed to amend the heading the precedes Sec. 23.455 by deleting the term "Wing Flaps" so that the heading reads "AILERONS AND SPECIAL DEVICES". This change would reflect the deletion of the wing flap requirements from Sec. 23.457 and their placement in Sec. 23.345.

修订指令英文原文（供核对）：

19. Section 23.457 is removed.

Current paragraph (c) would be redesignated as paragraph (d) and revised by including the requirements of Sec. 23.457. Paragraph (e) would be deleted since it merely references the requirements of sec. 23.457; which have been moved to Sec. 23.345 (d). This arrangement places all "flap" requirements in one location, and would harmonize the requirements with JAR 23.

The heading the precedes Sec. 23.455 would be amended by deleting the term "Wing Flaps" so that the heading reads "AILERONS AND SPECIAL DEVICES." This change reflects the proposed deletion of the wing flap requirements from Sec. 23.457 and their placement in Sec. 23.345.

## 3　§23.457 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The wing flaps, their operating mechanisms, and their supporting structures must be designed for critical loads occurring in the flaps-extended flight conditions with the flaps in any position. However, if an automatic flap load limiting device is used, these components may be designed for the critical combinations of airspeed and flap position allowed by that device. | 新增 |
| (b) | The effects of propeller slipstream, corresponding to takeoff power, must be taken into account at not less than 1.4 VS, where VS is the computed stalling speed with flaps fully retracted at the design weight. For the investigation of slipstream effects, the load factor may be assumed to be 1.0. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.457 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | To place all "flap" requirements in one location, and to harmonize the requirements with JAR 23, the FAA proposed to revise Sec. 23.345 as follows: (1) Make minor organizational, and non- substantive, clarifying chang… | Amdt 23-48 |

本条取证提示 共 2 个历史版本：初始版, Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [4] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [5] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [6] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [7] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.457 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 348a1c4b-f8e9-4dbe-b070-d735f22e3fa3 |
| Amdt 23-48 | 1996-03-11 | c3eb1496-478a-4e4a-b426-02fb3ecc7d40 |

# § 23.459　特殊装置（Special devices.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.459 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.459 |
| 条款标题（英文原文） | Special devices. |
| 中文名称 | 特殊装置 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.459 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.459 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：特殊装置

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.459 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.459 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.459 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 313f4876-a484-4882-8931-3b9aa8c95528 |

# § 23.471　总则（地面载荷）（General.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.471 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.471 |
| 条款标题（英文原文） | General. |
| 中文名称 | 总则（地面载荷） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.471 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.471 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：总则（地面载荷）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.471 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.471 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.471 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | d1a73526-4bd1-4bfd-8540-f24ec760967b |

# § 23.473　地面载荷情况与假设（Ground load conditions and assumptions.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 5 个历史版本

## 1　§23.473 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.473 |
| 条款标题（英文原文） | Ground load conditions and assumptions. |
| 中文名称 | 地面载荷情况与假设 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 5 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.473 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(a)、(b) |
| 3 | Amdt 23-28 | 1982-04-28 | Notice 80-23，1980-11-28 签发，45 FR 80450 | Docket 21129，1982-02-26 签发，47 FR 13312 | 删除内容 |
| 4 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订段落：(f) |
| 5 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订段落：(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(a)、(b)　→　1982 23-28　删除内容　→　1993 23-45　修订段落：(f)　→　1996 23-48　修订段落：(c)

## 2　§23.473 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：地面载荷情况与假设

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第23.473条修订

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：第23.473条修订，将(a)、(b)、(c)和(g)段修改为如下内容：　2. 第23.473条(a)、(b)段以及(c)段的引言文字将修改为如下内容：　3. 条 23.473(c) (1) 将通过删除交叉引用“§23.67 条”并插入该交叉　4. 第23.473条(g)段将进行修订，在数字“2.0”之后插入“在最大设计重量”。

来源：Final Rule｜34 FR 13078｜FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt

▪ 背景与评论处置（1）：该通告提议修订第 23.25、23.473 和 23.1583 条，以允许设计着陆重量等于设计最大重量减去总燃油容量 25% 的重量。此外，该提案将明确，除非申请人选择设计着陆重量选项，否则设计最大重量为用于研究地面载荷的重量，且设计着陆重量选项仅适用于实际代表着陆载荷的情况。收到的评论建议，应允许申请人使用根据该提案或现行要求确定的设计着陆重量。美国联邦航空局（FAA）同意。在现行规章下取得型号合格证且设计着陆重量低至设计最大重量 95% 的飞机的经验表明，现有规定可作为备选方案予以保留。
在这方面，现在的修订
鉴于现行
由于疲劳失效与时间相关，预计老龄飞机的失效率会更高；然而，近年来取证的飞机也出现了疲劳问题。疲劳证实和更好的维修
这样的要求将比对

英文原文：

The notice proposed to amend Secs. 23.25, 23.473, and 23.1583 to permit a design landing weight equal to the design maximum weight less the weight of 25 percent of the total fuel capacity. In addition, the proposal would make it clear that the design maximum weight is the weight to be used in investigating ground loads unless the design landing weight option is chosen by the applicant, and that the design landing weight option applies only to the conditions actually representing landing loads. Comments were received suggesting that an applicant should be permitted to use a design landing weight determined either in accordance with the proposal or the current requirements. The FAA agrees. Experience with airplanes that have been type certificated under the current rule with a design landing weight as low as 95 percent of design maximum weight indicates that the present provisions can be retained as an alternative. In this connection, the amendment now permits the design landing weight to be as low as 95 percent of the maximum weight if the minimum fuel capacity is enough for at least one-half hour of operation at maximum continuous power plus a capacity equal to a fuel weight which is the difference between the design maximum weight and the design landing weight or as low as the design maximum weight less the weight of 25 percent of the total fuel capacity. In addition, the current reference to Sec. 25.1001 in Sec. 23.473(c) has been changed to Sec. 23.1001. Section 25.1001 was amended subsequent to the issuance of Notice 67-14 containing these proposals and does not now reflect the appropriate requirement for Part 23. Therefore, the appropriate provisions of Sec. 25.1001 in effect on the date that Notice 67-14 was issued have been incorporated into a new Sec. 23.1001.

The notice proposed to amend Sec. 23.571 to require an effective fatigue evaluation of the wing and associated structures. Inasmuch as the title of current Sec. 23.571 might be misleading insofar as these requirements are concerned, the proposal has been adopted as new Sec. 23.572. Numerous comments were received objecting to this proposal. In this connection, it was stated that (1) the current strength requirements and design practices are conservative and adequate to prevent serious fatigue problems; (2) corrosion, prior abuse, and special purpose operations cause fatigue failure, rather then lack of design fatigue strength; (3) fatigue substantiation will not eliminate cracks, and a better maintenance program would be more effective; and (4) sufficient data is not available to establish load spectra for fatigue substantiation. The FAA does not agree. Service experience and discussions with industry of designs which have sustained fatigue failures indicate that present design practices do not adequately account for fatigue. Corrosion, prior abuse, and special purpose operations may contribute to fatigue failure, but the primary reason for such failures is lack of strength. Since fatigue failures are time-dependent, a higher failure rate among older airplanes is to be expected; however, fatigue problems have arisen in airplanes certificated in recent years. Neither fatigue substantiation nor better maintenance programs will eliminate all cracks. The purpose of the proposed rule is to prevent catastrophic failures. Furthermore, Part 23 airplanes are not designed on a redundant-structure, fail-safe basis, for which maintenance alone would be sufficient. Both fatigue substantiation and a good maintenance program are needed. Reasonable load spectra can and have been established from the extensive gust data which is available, and reasonable and acceptable methods of compliance have been established and widely publicized. However, while it has been determined that fatigue substantiation is necessary, the FAA does not agree with one comment which suggested that the proposed requirement should be expanded to a full limit load fail-safe requirement with a design objective of at least 10,000 hours and a test life of 30,000 flights. Such a requirement would be more severe than that required for transport category airplanes and cannot be justified on the basis of service experience. The amendment is adopted as proposed, except that it is adopted as new Sec. 23.572.

The notice proposed to amend Sec. 23.607 to permit the use of self-locking nuts on bolts subject to rotation in operation if additional locking means were provided. Subsequent to the issuance of the notice, Amendments 27-4 and 29-5 have been adopted which permit self-locking nuts to be used on rotorcraft only if the additional locking means is a nonfriction locking device. Those amendments also added requirements for removable bolts, screws, nuts, pins, and other fasteners whose loss could jeopardize the safe operation of the rotorcraft. Similar requirements have been proposed for transport category airplanes in Notice 68-18. In view of comments received in connection with Amendments 27-4 and 29-5 and the proposal in Notice 68-18, the proposed amendment to Sec. 23.607 is withdrawn pending further study.

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（2）：说明。
这两项澄清均反映现行做法，不涉及实质性变更。可在小于最大重量的设计着陆重量下使用的条件规定于 §23.473(b)(1)、(2) 和 (3) 段。(b)(1) 段中的结构条件因本提案第 6 次修改在 §23.473(g) 段中增加"设计最大重量"一词而成为多余，将被删除。(b)(2) 段中的半小时最低燃油容量要求因采用 25% 燃油数值而成为多余。对 §23.473(c)(1) 段交叉引用的修改仅为精确性修改，无实质性变更。§23.473(c)(2)(i) 和 (ii) 段将被删除，因为自通过 §23.473 以来，第 23 部中已增加了地面载荷和起落架要求，无需再纳入第 25 部标准。

英文原文：

Explanation. Section 23.473 (b) now allows a design landing weight as low as 95 percent of the maximum weight. FAA studies have shown that allowing a design landing weight equal to the design maximum weight less 25 percent of the total fuel capacity would be more rational and would not compromise strength for the landing conditions. The 25 percent figure is based on the increasing average time per flight, increasing fuel capacities, increasing fuel consumption rates, and the typical flight missions of modern small airplanes, including IFR alternate field fuel needs, especially for turbine engine powered airplanes. Proposed new Sec. 23.473 (a) would make it clear that the design maximum weight is the weight to be used in investigating ground loads unless the design landing weight option is chosen by the applicant. That paragraph would also make it clear that the design landing weight option applies only to the conditions actually representing landing loads, namely the conditions in Secs. 23.479, 23.481, and 23.483. Both of these clarifications reflect present practice and would involve no substantive change.

The conditions under which a design landing weight, less than the maximum weight, may be used are specified in Sec. 23.473(b) (1), (2), and (3). The structural condition in (b) (1) would be surplus and would be deleted because of the addition of the words "design maximum weight" to Sec. 23.473 (g) in change 6 of this proposal. The one-half hour minimum fuel capacity requirement in (b) (2) would be surplus since the 25 percent fuel figure would be used.

The change to the cross-reference in Sc. 23.473 (c) (1) is one of precision only, with no substantive change. Section 23.473 (c ) (2) (i) and (ii) would be deleted because there is no need to incorporate Part 25 standards in addition to ground load and landing gear requirements that have been added to Part 23 since the adoption of Sec. 23.473.

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（3）：允许使用小于最大

英文原文：

The conditions under which a design landing weight, less than the maximum weight, may be used are specified in Sec. 23.473(b) (1), (2), and (3). The structural condition in (b) (1) would be surplus and would be deleted because of the addition of the words "design maximum weight" to Sec. 23.473 (g) in change 6 of this proposal. The one-half hour minimum fuel capacity requirement in (b) (2) would be surplus since the 25 percent fuel figure would be used.

The change to the cross-reference in Sc. 23.473 (c) (1) is one of precision only, with no substantive change. Section 23.473 (c ) (2) (i) and (ii) would be deleted because there is no need to incorporate Part 25 standards in addition to ground load and landing gear requirements that have been added to Part 23 since the adoption of Sec. 23.473.

The addition of "design maximum weight" to Sec. 23.473 (g) is made because (1) load factors without prescribed weights are meaningless, and (2) the intent of (g) is to prescribe load factors at the highest weight at which compliance with each applicable structural loading condition is shown.

修订指令英文原文（供核对）：

37. Section 23.473 is amended by amending paragraphs (a), (b), (c), and (g) to read as follows:

2. Section 23.473 (a), (b), and the introductory text of (c) would be amended to read as follows:

3. Section 23.473(c) (1) would be amended by striking out the cross reference "Sec. 23.67" and inserting the cross

4. Section 23.473 (g) would be amended by inserting the words "at design maximum weight" after the number "2.0."

▪ 本版重印条文段落：(a)、(b)、(1)、(2)、(c)、(1)、(2)、(g)

### 2.3　Amdt 23-28（1982-04-28 生效）：修订第 23.473 条(f)段

▪ 修订沿革：本版由 NPRM　Notice 80-23，1980-11-28 签发，45 FR 80450　提出；最终规章 Docket 21129，1982-02-26 签发，47 FR 13312，1982-04-28 生效。

▪ 修订指令（中文译文）：3. 修订第 23.473 条(f)段，删除其中引用的“第 23.725 条”，并代之以“第 23.723(a) 条”。

来源：NPRM（提案）｜45 FR 80450｜NPRM_Notice 80-23_Docket21129_45FR80450_1980-12-04.txt

▪ 背景与评论处置（1）：§23.473(f) 段要求依据 §23.725 段通过自由落震试验进行能量吸收试验。美国联邦航空局（FAA）提议修订该要求，改为引用 §23.723(a) 段。该变更将允许进行自由落震试验以外的落震试验，并将使该要求与第 25 部、第 27 部和第 29 部的相应条保持一致。

英文原文：

Sec. 23.473(f) requires energy absorption tests to be made by free drop tests under Sec. 23.725. The FAA proposes to amend the requirement to reference Sec. 23.723(a) instead. This change would permit drop tests other than free drop tests and would make the requirement consistent with the corresponding sections of Parts 25, 27, and 29.

修订指令英文原文（供核对）：

3. By amending Sec. 23.473(f) by removing the reference "Sec. 23.725" and inserting the reference "Sec. 23.723(a)" in its place.

### 2.4　Amdt 23-45（1993-09-07 生效）：§23.473 条修订如下：修订 (f) 段

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：§23.473 条修订如下：修订 (f) 段，内容如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局提议澄清第 23.473(f) 条何时要求进行地面载荷能量吸收试验。美国联邦航空局收到了关于该提案的一条评论。该评论者提议进行一项措辞修改，这将修订第 23.473(f) 条并改变该要求的含义。由于美国联邦航空局认为该评论者提议的修改超出了该通告的范围，美国联邦航空局按提案采纳了第 23.473(f) 条。

英文原文：

The FAA proposed to clarify when Section 23.473(f) requires a ground load energy absorption test. The FAA received one comment on this proposal. The commenter proposed a wording change that would revise Section 23.473(f) and would change the meaning of this requirement. Since the FAA considers the commenter's proposed change beyond the scope of the notice, the FAA adopts Section 23.473(f) as proposed.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（2）：说明：本提案修订了第 23.473(f) 条，以明确何时需要进行能量吸收试验。第 23.723(a) 条在某些情况下免除了进行能量吸收试验的需要，并允许通过分析表明符合性。目前，第 23.473(f) 条规定必须按照第 23.723(a) 条进行试验。本提案基于从会议提案 212 收到的意见。至少有一名评论者表示，在某些情况下不需要进行试验，尽管第 23.473 条中未具体定义这些情况。美国联邦航空局（FAA）已确定，拟议的修改将明确第 23.473 条的意图。

英文原文：

Explanation: This proposal revises Section 23.473(f) to clarify when an energy absorption test is required. Section 23.723(a) exempts the need for an energy absorption test under certain circumstances and allows for compliance by analysis. Currently, Section 23.473(f) states that tests must be made under Section 23.723(a).

This proposal is based on comments received from conference proposal 212. At least one commenter stated that there were circumstances when testing was not required even though not specifically defined in Section 23.473. The FAA has determined that the proposed change will clarify the intent of Section 23.473.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：本提案基于会议提案 212 所收到的意见。至少有一位评论者指出，存在无需进行试验的情况，尽管这些情况未在 §23.473 条中具体界定。美国联邦航空局（FAA）已认定，拟议的修改将澄清 §23.473 条的意图。

英文原文：

This proposal is based on comments received from conference proposal 212. At least one commenter stated that there were circumstances when testing was not required even though not specifically defined in Section 23.473. The FAA has determined that the proposed change will clarify the intent of Section 23.473.

修订指令英文原文（供核对）：

36. Section 23.473 is amended by revising paragraph (f) to read as follows:

▪ 本版重印条文段落：(f)

### 2.5　Amdt 23-48（1996-03-11 生效）：第 23.473 条通过修订 (c)(1) 段和 (f) 段

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：20. 第 23.473 条通过修订 (c)(1) 段和 (f) 段，内容如下：　20. 第 23.473 条通过修订 (c)(1) 和 (f) 段修改如下：　§23.473(c)(1) 段中的引用将被修订。在第 23-42 号修订（1991 年 1 月 3 日，56 FR 344）中，§23.473(c)(1) 段错误地继续引用了“§23.67(a) 或 (b)(1) 段。”§23.473(c)(1) 段中的引用本应更改为“§23.67(b)(1) 段。”

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订第23.473(c)(1)条，将错误引用的“第23.67(a)或(b)(1)条”改为“第23.67(b)(1)或(c)条”。

英文原文：

The FAA proposed to revise Sec. 23.473(c)(1) to change the incorrect reference to "Sec. 23.67 (a) or (b)(1)" to "Sec. 23.67 (b)(1) or (c)".

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：最初，美国联邦

英文原文：

Originally, the FAA intended to harmonize Sec. 23.473(c)(1) by citing only Sec. 23.67. However, after considering the two issues noted above, the FAA has determined that the intent described is lost unless Sec. 23.473(c)(1) specifically includes "Sec. 23.67(b)(1) or (c)."

修订指令英文原文（供核对）：

20. Section 23.473 is amended by revising paragraphs (c)(1) and (f) to read as follows:

20. Section 23.473 is amended by revising paragraph (c)(1) and (f) to read as follows:

The reference in Sec. 23.473(c)(1) would be revised. In amendment No. 23-42 (January 3, 1991, 56 FR 344), Sec. 23.473 (c)(1) incorrectly continued to reference "Sec. 23.67 (a) or (b)(1)." The reference in Sec. 23.473(c)(1) should have been changed to "Sec. 23.67(b)(1)."

▪ 本版重印条文段落：(c)、(1)、(f)

## 3　§23.473 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The design landing weight (the maximum weight for landing conditions at the maximum descent velocity) may be used for structural design purposes only. Except as provided in paragraphs (b) and (c) of this section, this weight may not be less than the maximum weight. | 新增 |
| (b) | The design landing weight may be as low as 95 percent of the maximum weight if-- | 新增 |
| (1) | The structural limit load values at the maximum weight are not exceeded at speeds up to takeoff speed over terrain as rough as that expected in service; | 新增 |
| (2) | The minimum fuel capacity is enough for at least one-half hour of operation at maximum continuous power plus the capacity equal to a fuel weight equal to the difference between the maximum weight and the design landing weight; and | 新增 |
| (3) | The operating limitations limit the takeoff weight to ensure that landing weights in normal operation do not exceed the design landing weight. | 新增 |
| (c) | The design landing weight of a multiengine airplane may be less than 95 percent of the maximum weight if-- | 新增 |
| (1) | The airplane meets the one-engine-inoperative climb requirements of Sec. 23.67; and | 新增 |
| (2) | Instead of the corresponding requirements of this part, compliance is shown with the following requirements of Part 25 [New]: | 新增 |
| (i) | The ground load requirements of Secs. 25.471 and 25.473. | 新增 |
| (ii) | The landing gear requirements of Secs. 25.721 through 25.733. | 新增 |
| (iii) | The fuel jettisoning system requirements of Sec. 25.1001. | 新增 |
| (d) | The selected limit vertical inertia load factor at the center of gravity of the airplane for the ground load conditions prescribed in this subpart may not be less than that which would be obtained when landing with a descent velocity (V), in feet per second, equal to 4.4 , except that this velocity need not be more than 10 feet per second and may not be less than seven feet per second. | 新增 |
| (e) | Wing lift not exceeding two-thirds of the weight of the airplane may be assumed to exist throughout the landing impact and to act through the center of gravity. The ground reaction load factor may be equal to the inertia load factor minus the ratio of the above assumed wing lift to the airplane weight. | 新增 |
| (f) | Energy absorption tests (to determine the limit load factor corresponding to the required limit descent velocities) must be made under Sec. 23.725. | 新增 |
| (g) | No inertia load factor used for design purposes may be less than 2.67, nor may the limit ground reaction load factor be less than 2.0, unless these lower values will not be exceeded in taxiing at speeds up to takeoff speed over terrain as rough as that expected in service. | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订：(a)、(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The ground load requirements of this subpart must be complied with at the design maximum weight except that Secs. 23.479, 23.481, and 23.483 may be complied with at a design landing weight (the highest weight for landing conditions at the maximum descent velocity) allowed under paragraphs (b) and (c) of this section. | 修订 |
| (b) | The design landing weight may be as low as - | 修订 |
| (1) | 95 percent of the maximum weight if the minimum fuel capacity is enough for at least one-half hour of operation at maximum continuous power plus a capacity equal to a fuel weight which is the difference between the design maximum weight and the design landing weight; or | 修订 |
| (2) | The design maximum weight less the weight of 25 percent of the total fuel capacity. | 修订 |
| (c) | The design landing weight of a multiengine airplane may be less than that allowed under paragraph (b) of this section if - | 修订 |
| (1) | The airplane meets the one-engine-inoperative climb requirements of Sec. 23.67 (a) or (b)(1); and | 修订 |
| (2) | Compliance is shown with the fuel jettisoning system requirements of Sec. 23.1001. | 修订 |
| (g) | No inertia load factor used for design purposes may be less than 2.67, nor may the limit ground reaction load factor be less than 2.0 at design maximum weight, unless these lower values will not be exceeded in taxiing at speeds up to takeoff speed over terrain as rough as that expected in service. | 修订 |

### 3.3　版本3：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

修订段落：(f)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (f) | Energy absorption tests (to determine the limit load factor corresponding to the required limit descent velocities) must be made under Section 23.723(a) unless specifically exempted by that section. Explanation: This proposal revises Section 23.473(f) to clarify when an energy absorption test is required. Section 23.723(a) exempts the need for an energy absorption test under certain circumstances and allows for compliance by analysis. Currently, Section 23.473(f) states that tests must be made under Section 23.723(a). This proposal is based on comments received from conference proposal 212. At least one commenter stated that there were circumstances when testing was not required even though not specifically defined in Section 23.473. The FAA has determined that the proposed change will clarify the intent of Section 23.473. Reference: Conference proposal 212. | 新增 |

### 3.4　版本4：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订段落：(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | * * * | 新增 |
| (1) | The airplane meets the one-engine-inoperative climb requirements of Sec. 23.67(b)(1) or (c); and | 新增 |
| (f) | If energy absorption tests are made to determine the limit load factor corresponding to the required limit descent velocities, these tests must be made under Sec. 23.723(a). | 修订 |

### 3.5　段落结构演变矩阵（横向对照 5 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) | Amdt 23-45 (1993-09-07) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | ○ 无 | ○ 无 |
| (b) | ● 新增 | △ 修订 | ○ 无 | ○ 无 |
| (1) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (2) | ● 新增 | △ 修订 | ○ 无 | ○ 无 |
| (3) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (c) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (i) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (ii) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (iii) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (d) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (e) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (f) | ● 新增 | ○ 无 | ● 新增 | △ 修订 |
| (g) | ● 新增 | △ 修订 | ○ 无 | ○ 无 |

## 4　§23.473 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | posed amendment to Sec. 23.607 is withdrawn pending further study. Explanation. Section 23.473 (b) now allows a design landing weight as low as 95 percent of the maximum weight. FAA studies have shown that allowing a des… | Amdt 23-7 |
| 澄清性与编辑性修订 | The FAA proposed to clarify when Section 23.473(f) requires a ground load energy absorption test. The FAA received one comment on this proposal. The commenter proposed a wording ch… | Amdt 23-45 |
| 国际协调 | "Sec. 23.67 (b)(1) or (c)". Originally, the FAA intended to harmonize Sec. 23.473(c)(1) by citing only Sec. 23.67. However, after considering the two issues noted above, the FAA has determined that the intent described i… | Amdt 23-48 |

本条取证提示 共 5 个历史版本：初始版, Amdt 23-7, Amdt 23-28, Amdt 23-45, Amdt 23-48。 其中 4 个版本在本地文献中检索到明确修订指令原文；4 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 21129 | 47 FR 13312 | FinalRule_Docket 21129_Amdt23-28_25-55_47FR13312_1982-03-29.txt |
| [6] | NPRM（提案） | Notice 80-23 / Docket 21129 | 45 FR 80450 | NPRM_Notice 80-23_Docket21129_45FR80450_1980-12-04.txt |
| [7] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [8] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [9] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [10] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [11] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [12] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [13] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.473 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | e616706d-173e-4226-84e8-db3ca2e3f39f |
| Amdt 23-7 | 1969-09-14 | d38669ed-ccb6-4fd1-b98b-ad432fd096e1 |
| Amdt 23-28 | 1982-04-28 | f3188edf-fb76-42c9-a617-5b98431c84c1 |
| Amdt 23-45 | 1993-09-07 | 1fe654c2-836f-41a1-8ed7-3d1a026d1aac |
| Amdt 23-48 | 1996-03-11 | e998509c-093d-4280-965c-620e2e14e3ce |

# § 23.477　起落架布置（Landing gear arrangement.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.477 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.477 |
| 条款标题（英文原文） | Landing gear arrangement. |
| 中文名称 | 起落架布置 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.477 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.477 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：起落架布置

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.477 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.477 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.477 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 964c5bb4-687f-430a-b7d6-b51f09ec3bd9 |

# § 23.479　水平着陆情况（Level landing conditions.）

1965-02-01 初版　｜　1993-09-07 Amdt 23-45　｜　共 3 个历史版本

## 1　§23.479 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.479 |
| 条款标题（英文原文） | Level landing conditions. |
| 中文名称 | 水平着陆情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 3 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1993-09-07 （约 28 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.479 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-17 | 1977-02-01 | Notice 74-5，1974-02-14 签发，39 FR 5785 Notice 74-5A，1974-05-22 签发，39 FR 5785 Notice 74-5B，1974-10-03 签发，39 FR 5785 Notice 74-33，1974-10-03 签发，39 FR 5785 Notice 74-5C，1974-11-25 签发，39 FR 5785 Notice 74-5D，1975-02-04 签发，39 FR 5785 Notice 75-10，1975-02-27 签发，39 FR 5785 Notice 75-19，1975-05-13 签发，39 FR 5785 Notice 75-20，1975-05-13 签发，39 FR 5785 Notice 75-23，1975-05-19 签发，39 FR 5785 Notice 75-25，1975-05-29 签发，39 FR 5785 Notice 75-26，1975-06-09 签发，39 FR 5785 Notice 75-31，1975-06-30 签发，39 FR 5785 | Docket 14324，1976-12-13 签发，41 FR 55454 | 新增内容：(a) |
| 3 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订段落：(b) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1977 23-17　新增内容：(a)　→　1993 23-45　修订段落：(b)

## 2　§23.479 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：水平着陆情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-17（1977-02-01 生效）：增加新的 §23.479(d) 段

▪ 修订沿革：本版由 NPRM　Notice 74-5，1974-02-14 签发，39 FR 5785；Notice 74-5A，1974-05-22 签发，39 FR 5785；Notice 74-5B，1974-10-03 签发，39 FR 5785；Notice 74-33，1974-10-03 签发，39 FR 5785；Notice 74-5C，1974-11-25 签发，39 FR 5785；Notice 74-5D，1975-02-04 签发，39 FR 5785；Notice 75-10，1975-02-27 签发，39 FR 5785；Notice 75-19，1975-05-13 签发，39 FR 5785；Notice 75-20，1975-05-13 签发，39 FR 5785；Notice 75-23，1975-05-19 签发，39 FR 5785；Notice 75-25，1975-05-29 签发，39 FR 5785；Notice 75-26，1975-06-09 签发，39 FR 5785；Notice 75-31，1975-06-30 签发，39 FR 5785　提出；最终规章 Docket 14324，1976-12-13 签发，41 FR 55454，1977-02-01 生效。

▪ 修订指令（中文译文）：8. 增加新的 §23.479(d) 段，内容如下：

来源：Final Rule｜41 FR 55454｜FinalRule_Docket 14324_Amdt21-44_23-17_25-38_27_41FR55454_1976-12-20_0002.txt

▪ 背景与评论处置（1）：提案 2-10。关于新增 §23.479(d) 段的提案，未收到不利评论。因此，该提案在不作实质性修改的情况下予以采纳。

英文原文：

Proposal 2-10. No unfavorable comments were received on the proposal to add a new Sec. 23.479(d). Accordingly, the proposal is adopted without substantive change.

修订指令英文原文（供核对）：

8. By adding a new Sec. 23.479(d) to read as follows:

▪ 本版重印条文段落：(d)

### 2.3　Amdt 23-45（1993-09-07 生效）：修订第 23.479 条

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：36. 修订第 23.479 条，将段 (b) 和段 (c) 修改为如下内容：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局

英文原文：

The FAA proposed to revise Section 23.479(c) to add a new requirement for landing gear spring-back loads. Additionally, this proposal allows for loads development based on testing or rational analyses other than that referenced in appendix D. This proposal also restricts the minimum values of the drag component when using the method referenced in appendix D. Since the only comment received, from the JAA, agrees with the proposed change, Section 23.479(c) is adopted as proposed.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（2）：美国联邦航空局（FAA）提议修订附录 D，增加一个新的段 (c)，该段支持 §23.479(c) 条中关于起落架动态回弹的一项新要求。

英文原文：

The FAA proposed to amend appendix D by adding a new paragraph (c), which supports a new requirement in Section 23.479(c) concerning dynamic spring-back of the landing gear.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：说明：本提案修订第 23.479(c) 条，增加一项新要求以处理地面载荷制定期间的回弹载荷。此外，本提案允许基于试验或基于附录 D 中引用之外的合理分析来制定载荷。如果使用附录 D 中引用的方法，本提案还限制阻力分量的最小值。

英文原文：

Explanation: This proposal revises Section 23.479(c) to add a new requirement to address spring-back loads during the development of ground loads. Additionally, this proposal allows for loads development based on testing or based on a rational analysis other than that referenced in appendix D. This proposal also restricts the minimum values of the drag component if the method referenced in appendix D is used.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（4）：一名评论者反对会议提案213，转而支持会议提案513。该评论者也是会议提案513的提交者，其主张会议提案513更合适且更具澄清性，因为它涉及回弹载荷。该评论者主张回弹载荷在CAR 3中有所涉及，但在第23部重新编纂期间被遗漏。该评论者正确地指出，现行第23.479条仅涉及起转载荷，未涉及回弹情况。

美国联邦航空局（FAA）对该提案的分析表明，在正常着陆期间，起落架产生向后载荷，这是由机轮和轮胎从飞行中的某个初始旋转速度（通常为零）加速至地面着陆速度下滚动轮胎的旋转速度所引起的。在初始撞击期间，能量以起落架结构变形的形式以及机轮和轮胎动能的形式储存。由此产生的向后载荷通常被称为起转载荷。

英文原文：

One commenter opposed conference proposal 213 in favor of conference proposal 513. That commenter, who also was the submitter of conference proposal 513 , contended that conference proposal 513 is more appropriate and more clarifying because it addresses spring-back loads. That commenter contended that spring-back loads were addressed in CAR 3 but were omitted during part 23 recodification. That commenter correctly pointed out that current Section 23.479 addresses only spin-up loads and does not address the springback condition.

FAA analysis of this proposal indicates that during normal landings, the landing gear develops aft loads caused by the acceleration of the wheel and tire from some initial rotational velocity in flight (usually zero) to the rotational velocity of the rolling tire, on the ground, at landing speed. During initial impact, energy is stored as deflection in the structure of the landing gear and also as kinetic energy of the wheel and tire. The resulting aft load is usually referred to as the spin-up load.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（5）：说明：有关说明，参见§23.479。

英文原文：

Explanation: For explanation, see Section 23.479.

修订指令英文原文（供核对）：

36. Section 23.479 is amended by revising paragraphs (b) and (c) to read as follows:

▪ 本版重印条文段落：(b)、(c)

## 3　§23.479 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | For a level landing, the airplane is assumed to be in the following attitudes: | 新增 |
| (1) | For airplanes with tail wheels, a normal level flight attitude. | 新增 |
| (2) | For airplanes with nose wheels, attitudes in which-- | 新增 |
| (i) | The nose and main wheels contact the ground simultaneously; and | 新增 |
| (ii) | The main wheels contact the ground and the nose wheel is just clear of the ground. The attitude used in subdivision (i) of this subparagraph may be used in the analysis required under subdivision (ii) of this subparagraph. | 新增 |
| (b) | When investigating landing conditions, the drag components simulating the forces required to accelerate the tires and wheels up to the landing speed must be properly combined with the corresponding instantaneous vertical ground reactions, assuming wing lift and a tire-sliding coefficient of friction of 0.8. However, the drag loads may not be less than 25 percent of the maximum vertical ground reactions (neglecting wing lift). | 新增 |
| (c) | In determining the wheel spin-up loads for landing conditions, the method set forth in Appendix D or the arbitrary drag components in Appendix C must be used. However, if Appendix D is used, the 25 percent value for the minimum drag component must be used. | 新增 |

### 3.2　版本2：Amdt 23-17（1977-02-01 生效）　来源：Docket 14324，1976-12-13 签发，41 FR 55454

新增内容：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (d) | For airplanes with tip tanks or large overhung masses (such as turbopropeller or jet engines) supported by the wing, the tip tanks and the structure supporting the tanks or overhung masses must be designed for the effects of dynamic responses under the level landing conditions of either paragraph (a)(1) or (a)(2)(ii) of this section. In evaluating the effects of dynamic response, an airplane lift equal to the weight of the airplane may be assumed. | 新增 |

### 3.3　版本3：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

修订段落：(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | When investigating landing conditions, the drag components simulating the forces required to accelerate the tires and wheels up to the landing speed (spin-up) must be properly combined with the corresponding instantaneous vertical ground reactions, and the forward-acting horizontal loads resulting from rapid reduction of the spin-up drag loads (spring-back) must be combined with vertical ground reactions at the instant of the peak forward load, assuming wing lift and a tire-sliding coefficient of friction of 0.8. However, the drag loads may not be less than 25 percent of the maximum vertical ground reactions (neglecting wing lift). | 新增 |
| (c) | In the absence of specific tests or a more rational analysis for determining the wheel spin-up and spring-back loads for landing conditions, the method set forth in appendix D of this part must be used. If appendix D of this part is used, the drag components used for design must not be less than those given by appendix C of this part. | 新增 |

### 3.4　段落结构演变矩阵（横向对照 3 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-17 (1977-02-01) | Amdt 23-45 (1993-09-07) |
| --- | --- | --- | --- |
| (a) | ● 新增 | ○ 无 | ○ 无 |
| (1) | ● 新增 | ○ 无 | ○ 无 |
| (2) | ● 新增 | ○ 无 | ○ 无 |
| (i) | ● 新增 | ○ 无 | ○ 无 |
| (ii) | ● 新增 | ○ 无 | ○ 无 |
| (b) | ● 新增 | ○ 无 | ● 新增 |
| (c) | ● 新增 | ○ 无 | ● 新增 |
| (d) | ○ 无 | ● 新增 | ○ 无 |

## 4　§23.479 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | ed in appendix D. Since the only comment received, from the JAA, agrees with the proposed change, Section 23.479(c) is adopted as proposed. The FAA proposed to amend appendix D by adding a new paragraph (c), which suppor… | Amdt 23-45 |

本条取证提示 共 3 个历史版本：初始版, Amdt 23-17, Amdt 23-45。 其中 2 个版本在本地文献中检索到明确修订指令原文；3 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 14324 | 41 FR 55454 | FinalRule_Docket 14324_Amdt21-44_23-17_25-38_27_41FR55454_1976-12-20_0002.txt |
| [4] | Final Rule（颁行） | Docket 14324 | 43 FR 50578 | FinalRule_Docket 14324_ 14606_ 14625_ 14685_ 14779_Amdt23-23_25-46_27-16_29_43FR50578_1978-10-30_0003.txt |
| [5] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 5785 | NPRM_Notice 74-5_Docket13542_39FR5785_1974-02-15.txt |
| [6] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 18663 | NPRM_Notice 74-5A_Docket13542_39FR18663_1974-05-29.txt |
| [7] | NPRM（提案） | Notice 74-5 / Docket 13542 | 39 FR 41319 | NPRM_Notice 74-5C_Docket13542_39FR41319_1974-11-26.txt |
| [8] | NPRM（提案） | Notice 74-5 / Docket 13542 | 40 FR 5810 | NPRM_Notice 74-5D_Docket13542_40FR5810_1975-02-07.txt |
| [9] | NPRM（提案） | Notice 74-5 / Docket 13542 | （FR 引证未在文本中标注） | NPRM_Notice 74-5B_Docket13542_1974-10-11.txt |
| [10] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [11] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [12] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [13] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.479 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 2f09a6e3-0435-4c63-bf03-7bf7e9d90400 |
| Amdt 23-17 | 1977-02-01 | fd4e307a-3aa0-491a-9c99-0b66c8b0baf9 |
| Amdt 23-45 | 1993-09-07 | a4ce9ed1-27e8-4cea-ae1a-c019f03344d8 |

# § 23.481　尾部下沉着陆情况（Tail down landing conditions.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.481 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.481 |
| 条款标题（英文原文） | Tail down landing conditions. |
| 中文名称 | 尾部下沉着陆情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.481 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.481 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：尾部下沉着陆情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.481 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | For a tail down landing, the airplane is assumed to be in the following attitudes: | 新增 |
| (1) | For airplanes with tail wheels, an attitude in which the main and tail wheels contact the ground simultaneously. | 新增 |
| (2) | For airplanes with nose wheels, a stalling attitude, or the maximum angle allowing ground clearance by each part of the airplane, whichever is less. | 新增 |
| (b) | For airplanes with either tail or nose wheels, ground reactions are assumed to be vertical, with the wheels up to speed before the maximum vertical load is attained. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.481 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.481 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 51bb6c28-0d63-43e0-901e-260d85927829 |

# § 23.483　单轮着陆情况（One-wheel landing conditions.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.483 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.483 |
| 条款标题（英文原文） | One-wheel landing conditions. |
| 中文名称 | 单轮着陆情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.483 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.483 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：单轮着陆情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.483 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.483 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.483 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | c0863eee-89a6-4bfc-8805-6b6d76f4c198 |

# § 23.485　侧向载荷情况（Side load conditions.）

1965-02-01 初版　｜　1993-09-07 Amdt 23-45　｜　共 2 个历史版本

## 1　§23.485 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.485 |
| 条款标题（英文原文） | Side load conditions. |
| 中文名称 | 侧向载荷情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1993-09-07 （约 28 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.485 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 新增段落：(d)、(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1993 23-45　新增段落：(d)、(c)

## 2　§23.485 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：侧向载荷情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-45（1993-09-07 生效）：§23.485 条新增 (d) 段

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：§23.485 条新增 (d) 段，内容如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议明确第 23.485 条中载荷的位置和组合。由于收到的唯一评论（来自欧洲联合航空局（JAA））同意提议的修改，美国联邦航空局（FAA）按提议修订第 23.485 条。

英文原文：

The FAA proposed to clarify the location and combination of loads in Section 23.485. Since the only comment received, from the JAA, agrees with the proposed change, the FAA is amending Section 23.485, as proposed.

修订指令英文原文（供核对）：

37. Section 23.485 is amended by adding a new paragraph (d) to read as follows:

▪ 本版重印条文段落：(d)

## 3　§23.485 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | For the side load condition, the airplane is assumed to be in a level attitude with only the main wheels contacting the ground and with the shock absorbers and tires in their static positions. | 新增 |
| (b) | The limit vertical load factor must be 1.33, with the vertical ground reaction divided equally between the main wheels. | 新增 |
| (c) | The limit side inertia factor must be 0.83, with the side ground reaction divided between the main wheels so that-- | 新增 |
| (1) | 0.5 (W) is acting inboard on one side; and | 新增 |
| (2) | 0.33 (W) is acting outboard on the other side. | 新增 |

### 3.2　版本2：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

新增段落：(d)、(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (d) | The side loads prescribed in paragraph (c) of this section are assumed to be applied at the ground contact point and the drag loads may be assumed to be zero. | 新增 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-45 (1993-09-07) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (b) | ● 新增 | ○ 无 |
| (c) | ● 新增 | ○ 无 |
| (1) | ● 新增 | ○ 无 |
| (2) | ● 新增 | ○ 无 |
| (d) | ○ 无 | ● 新增 |

## 4　§23.485 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | n Section 23.485. Since the only comment received, from the JAA, agrees with the proposed change, the FAA is amending Section 23.485, as proposed. 37. Section 23.485 is amended by adding a new paragraph (d) to read as fo… | Amdt 23-45 |

本条取证提示 共 2 个历史版本：初始版, Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [4] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [5] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [6] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.485 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | b50101ec-e528-4fda-908f-504644b1d481 |
| Amdt 23-45 | 1993-09-07 | 1ff9b35e-33bc-4bef-adf3-536f306ed5ea |

# § 23.493　刹车滚滑情况（Braked roll conditions.）

1965-02-01 初版　｜　1965-02-01 初始版　｜　共 1 个历史版本

## 1　§23.493 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.493 |
| 条款标题（英文原文） | Braked roll conditions. |
| 中文名称 | 刹车滚滑情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1965-02-01 （约 0 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.493 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文

## 2　§23.493 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：刹车滚滑情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

## 3　§23.493 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The limit vertical load factor must be 1.33. | 新增 |
| (b) | The attitudes and ground contacts must be those described in Sec. 23.479 for level landings. | 新增 |
| (c) | A drag reaction equal to the vertical reaction at the wheel multiplied by a coefficient of friction of 0.8 must be applied at the ground contact point of each wheel with brakes, except that the drag reaction need not exceed the maximum value based on limiting brake torque. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |

## 4　§23.493 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：初始版。 其中 0 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [4] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.493 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 4326ac12-9578-4ac7-ba5e-6e830462be80 |

# § 23.497　尾轮的补充情况（Supplementary conditions for tail wheels.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 2 个历史版本

## 1　§23.497 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.497 |
| 条款标题（英文原文） | Supplementary conditions for tail wheels. |
| 中文名称 | 尾轮的补充情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.497 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增段落：(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1996 23-48　新增段落：(c)

## 2　§23.497 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：尾轮的补充情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-48（1996-03-11 生效）：§23.497 条新增 (c) 段

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：§23.497 条新增 (c) 段，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议了新的 Sec. 23.497(c) 条，以迁移装有后置螺旋桨的飞机的尾轮、缓冲器或能量吸收装置设计标准。这些要求目前存在于 Sec. 23.925(b) 条中。它们被迁移是因为美国联邦航空局（FAA）认定，这些装置设计标准的某些部分更适合归属于 C 分部——结构。

英文原文：

The FAA proposed a new Sec. 23.497(c) to relocate tail wheel, bumper, or energy absorption device design standards for airplanes with aft-mounted propellers. These requirements currently exist in Sec. 23.925(b). They are being moved because the FAA determined that certain portions of the design standards for these devices more properly belong in Subpart C--Structure.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦

英文原文：

The FAA proposed to amend Sec. 23.925(b), Aft mounted propellers, by removing the requirements on tail wheels, bumpers, and energy absorption devices and moving them to Sec. 23.497, Supplementary conditions for tail wheels, as discussed as discussed above. The FAA also proposed to delete the inspection and replacement criteria for tail wheel, bumper, and energy absorption devices because the inspection and replacement requirements are stated in Sec. 23.1529.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（3）：拟新增的 §23.497(c) 段将为 §23.925(b) 段所涉及的后置螺旋桨规定设计标准。美国联邦航空局（FAA）已认定，后置螺旋桨设计标准中的某些部分置于有关结构的 C 分部更为恰当。其余标准将保留在 E 分部。

英文原文：

Proposed new Sec. 23.497(c) would establish design standards for the aft-mounted propellers of Sec. 23.925(b). The FAA has determined that certain portions of the design standards for aft-mounted propellers more properly being in subpart C on structure. The remainder of the standards would remain in subpart E.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（4）：本提案将修订 §23.925(b) 段（后置螺旋桨），删除关于尾轮、缓冲器和能量吸收装置的要求，并将其移至 §23.497 段（尾轮补充条件），如上文所述。尾轮、缓冲器和能量吸收装置的检查/更换标准将被删除，因为检查/更换已在 §23.1529 段中要求，无需在此重复。

英文原文：

This proposal would amend Sec. 23.925(b), Aft mounted propellers, by removing the requirements on tail wheels, bumpers, and energy absorption devices and moving them to Sec. 23.497, Supplementary conditions for tail wheels, as discussed above. The inspection/replacement criteria for tail wheel, bumper, and energy absorption device would be deleted because the inspection/replacement is required in Sec. 23.1529 and does not need to be repeated here.

修订指令英文原文（供核对）：

21. Section 23.497 is amended by adding a new paragraph (c) to read as follows:

▪ 本版重印条文段落：(c)、(1)、(2)

## 3　§23.497 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | For the obstruction load, the limit ground reaction obtained in the tail down landing condition is assumed to act up and aft through the axle at 45 degrees. The shock absorber and tire may be assumed to be in their static positions. | 新增 |
| (b) | For the side load, a limit vertical ground reaction equal to the static load on the tail wheel, in combination with a side component of equal magnitude, is assumed. In addition-- | 新增 |
| (1) | If a swivel is used, the tail wheel is assumed to be swiveled 90 degrees to the airplane longitudinal axis with the resultant ground load passing through the axle; | 新增 |
| (2) | If a lock, steering device, or shimmy damper is used, the tail wheel is also assumed to be in the trailing position with the side load acting at the ground contact point; and | 新增 |
| (3) | The shock absorber and tire are assumed to be in their static positions. | 新增 |

### 3.2　版本2：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

新增段落：(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | If a tail wheel, bumper, or an energy absorption device is provided to show compliance with Sec. 23.925(b), the following apply: | 新增 |
| (1) | Suitable design loads must be established for the tail wheel, bumper, or energy absorption device; and | 修订 |
| (2) | The supporting structure of the tail wheel, bumper, or energy absorption device must be designed to withstand the loads established in paragraph (c)(1) of this section. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (b) | ● 新增 | ○ 无 |
| (1) | ● 新增 | △ 修订 |
| (2) | ● 新增 | △ 修订 |
| (3) | ● 新增 | ○ 无 |
| (c) | ○ 无 | ● 新增 |

## 4　§23.497 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 2 个历史版本：初始版, Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [4] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [5] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [6] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [7] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.497 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 9886a654-7063-4788-8a4d-f1d2c07b5743 |
| Amdt 23-48 | 1996-03-11 | 3b45b4b8-fe9f-41c3-b134-a4cdc563ea6f |

# § 23.499　前轮的补充情况（Supplementary conditions for nose wheels.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 2 个历史版本

## 1　§23.499 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.499 |
| 条款标题（英文原文） | Supplementary conditions for nose wheels. |
| 中文名称 | 前轮的补充情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.499 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增段落：(d) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1996 23-48　新增段落：(d)

## 2　§23.499 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：前轮的补充情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-48（1996-03-11 生效）：对 §23.499 条进行修订

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：22. 对 §23.499 条进行修订，增加新的 (d) 段和 (e) 段，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局提议在第23.499条中新增(d)和(e)段，以规定由液压或其他动力控制的可转向前轮飞机以及具有与方向舵踏板直接机械连接的可转向前轮飞机的前轮条件。

英文原文：

The FAA proposed to add new paragraphs (d) and (e) to Sec. 23.499 to establish nose wheel conditions for airplanes with a steerable nose wheel controlled by hydraulic or other power and for airplanes with a steerable wheel that has a direct mechanical connection to the rudder pedals.

修订指令英文原文（供核对）：

22. Section 23.499 is amended by adding new paragraphs (d) and (e) to read as follows:

▪ 本版重印条文段落：(d)、(e)

## 3　§23.499 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | For aft loads, the limit force components at the axle must be-- | 新增 |
| (1) | A vertical component of 2.25 times the static load on the wheel; and | 新增 |
| (2) | A drag component of 0.8 times the vertical load. | 新增 |
| (b) | For forward loads, the limit force components at the axle must be-- | 新增 |
| (1) | A vertical component of 2.25 times the static load on the wheel; and | 新增 |
| (2) | A forward component of 0.4 times the vertical load. | 新增 |
| (c) | For side loads, the limit force components at ground contact must be-- | 新增 |
| (1) | A vertical component of 2.25 times the static load on the wheel; and | 新增 |
| (2) | A side component of 0.7 times the vertical load. | 新增 |

### 3.2　版本2：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

新增段落：(d)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (d) | For airplanes with a steerable nose wheel that is controlled by hydraulic or other power, at design takeoff weight with the nose wheel in any steerable position, the application of 1.33 times the full steering torque combined with a vertical reaction equal to 1.33 times the maximum static reaction on the nose gear must be assumed. However, if a torque limiting device is installed, the steering torque can be reduced to the maximum value allowed by that device. | 新增 |
| (e) | For airplanes with a steerable nose wheel that has a direct mechanical connection to the rudder pedals, the mechanism must be designed to withstand the steering torque for the maximum pilot forces specified in Sec. 23.397(b). | 新增 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (1) | ● 新增 | ○ 无 |
| (2) | ● 新增 | ○ 无 |
| (b) | ● 新增 | ○ 无 |
| (c) | ● 新增 | ○ 无 |
| (d) | ○ 无 | ● 新增 |
| (e) | ○ 无 | ● 新增 |

## 4　§23.499 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 2 个历史版本：初始版, Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [4] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [5] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [6] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [7] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.499 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 866fa8e0-16d7-42e1-b8d4-a7dba8df1f81 |
| Amdt 23-48 | 1996-03-11 | 8443182c-e391-42ea-8cc9-6ca1fc38944f |

# § 23.505　滑橇式飞机的补充情况（Supplementary conditions for skiplanes.）

1965-02-01 初版　｜　1969-09-14 Amdt 23-7　｜　共 2 个历史版本

## 1　§23.505 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.505 |
| 条款标题（英文原文） | Supplementary conditions for skiplanes. |
| 中文名称 | 滑橇式飞机的补充情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1969-09-14 （约 4 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.505 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订 |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订

## 2　§23.505 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：滑橇式飞机的补充情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：第 23.505 条修订后内容如下

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：38. 第 23.505 条修订后内容如下：

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案15. 滑橇式飞机的极限侧力情况（§23.505）。第23.505条将修改为如下内容：§23.505 滑橇式飞机的极限侧力情况。在下列情况下，滑橇式飞机必须按限制载荷因数设计：(a) 在粗糙冰面上以最大滑行速度滑行时，侧向力因数达到0.8；(b) 在粗糙冰面上以最大滑行速度滑行时，纵向力因数达到0.8；(c) 在粗糙冰面上以最大滑行速度滑行时，垂直力因数达到1.0。

英文原文：

Proposal 15. Limit side force condition for skiplanes (Sec. 23.505). Section 23.505 would be amended to read as follows:

修订指令英文原文（供核对）：

38. Section 23.505 is amended to read as follows:

## 3　§23.505 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | P being the static ground reaction on the tail ski; and | 新增 |
| (b) | A factor of safety of 1.0. Water Loads | 新增 |

### 3.2　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | 初始版 (1965-02-01) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.505 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 2 个历史版本：初始版, Amdt 23-7。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [6] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.505 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 1d0d6d1b-210e-4d87-94ba-e9bcd42ba5f9 |
| Amdt 23-7 | 1969-09-14 | 0adf6a64-b354-4fc4-acaa-e4a6e280cfb2 |

# § 23.507　顶升载荷（Jacking loads.）

1973-12-20 初版　｜　1973-12-20 Amdt 23-14　｜　共 1 个历史版本

## 1　§23.507 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.507 |
| 条款标题（英文原文） | Jacking loads. |
| 中文名称 | 顶升载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1973-12-20 初始版） |
| 时间跨度 | 1973-12-20 → 1973-12-20 （约 0 年） |
| 初版来源 | Docket 11011，1973-11-07 签发，38 FR 31816 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.507 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-14 | 1973-12-20 | Notice 71-13，1971-04-27 签发，36 FR 8398 | Docket 11011，1973-11-07 签发，38 FR 31816 | 新增条款 |

### 1.3　修订脉络一览

1973 初版　新增条款

## 2　§23.507 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1973-12-20，Amdt 23-14）：顶升载荷

▪ 修订沿革：本版由 NPRM　Notice 71-13，1971-04-27 签发，36 FR 8398　提出；最终规章 Docket 11011，1973-11-07 签发，38 FR 31816，1973-12-20 生效。

▪ 修订指令（中文译文）：15. 新增 §23.507 条，内容如下：　16. 将新增 §23.507 条，内容如下：

来源：Final Rule｜38 FR 31816｜FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt

▪ 背景与评论处置（1）：针对收到
该评论超出了本通告的范围，且按建议对该条进行修订，将对航空器设计施加不必要的限制。

英文原文：

In response to a comment received, Secs. 23.507(a) and 23.509(c)(1) have been editorially revised for greater clarity. In addition, proposed Sec. 23.507(b) has been revised to state more clearly the requirement of that paragraph. One commentator objected to proposed Sec. 23.507(a)(2)(ii) as being too severe and suggested that 0.2 times the vertical static reactions be used for the fore, aft, and lateral load factors. The FAA does not agree. The commentator has not submitted, and the FAA is unaware of, any data to substantiate the claim of severity.

The notice explained that the purpose of the proposed amendment to Sec. 23.572 (a)(1) was to make it clear that, for a fatigue evaluation, the use of analysis alone is acceptable only under certain specified circumstances. The word "only" has accordingly been inserted in that section to make the requirement clear. One commentator suggested that the section be revised to require fail safe structure in fatigue critical areas. The FAA does not agree. The comment is beyond the scope of the notice and a revision of the section, as suggested, would place an unnecessary restriction on aircraft design.

修订指令英文原文（供核对）：

15. A new Sec. 23.507 is added to read as follows:

16. A new Sec. 23.507 would be added to read as follows:

## 3　§23.507 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1973-12-20 生效）　来源：Docket 11011，1973-11-07 签发，38 FR 31816

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The airplane must be designed for the loads developed when the aircraft is supported on jacks at the design maximum weight assuming the following load factors: | 新增 |
| (1) | For landing gear jacking points at a three-point attitude- | 新增 |
| (i) | Vertical-load factor of 1.35 times the static reactions; and | 新增 |
| (ii) | Fore, aft and lateral load factor of 0.4 times the vertical static reactions; and | 新增 |
| (2) | For primary flight structure jacking points in the level attitude- | 新增 |
| (i) | Vertical load factor of 1.35 times the static reactions; and | 新增 |
| (ii) | Fore, aft and lateral load factor of 0.4 times the vertical static reactions. | 新增 |
| (b) | The horizontal loads at the jack points must be reacted by inertia forces as to result in no change in the vertical loads at the jack points. | 新增 |
| (c) | The horizontal loads must be considered in all combinations with the vertical load. Explanation: Since Part 23 airplanes are being designed with provisions for jacking for maintenance purposes, the regulations should provide design loads for such conditions. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-14 (1973-12-20) |
| --- | --- |
| (a) | ● 新增 |
| (1) | ● 新增 |
| (i) | ● 新增 |
| (ii) | ● 新增 |
| (2) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |

## 4　§23.507 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 澄清性与编辑性修订 | omment received, Secs. 23.507(a) and 23.509(c)(1) have been editorially revised for greater clarity. In addition, proposed Sec. 23.507(b) has been revised to state more clearly the requirement of that paragraph. One comm… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-14。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 11011 | 38 FR 31816 | FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt |
| [2] | NPRM（提案） | Notice 71-13 / Docket 11011 | 36 FR 8398 | NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.507 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-14 | 1973-12-20 | fba9173e-3045-417f-a286-18b422bb6e91 |

# § 23.509　牵引载荷（Towing loads.）

1973-12-20 初版　｜　1973-12-20 Amdt 23-14　｜　共 1 个历史版本

## 1　§23.509 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.509 |
| 条款标题（英文原文） | Towing loads. |
| 中文名称 | 牵引载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1973-12-20 初始版） |
| 时间跨度 | 1973-12-20 → 1973-12-20 （约 0 年） |
| 初版来源 | Docket 11011，1973-11-07 签发，38 FR 31816 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.509 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-14 | 1973-12-20 | Notice 71-13，1971-04-27 签发，36 FR 8398 | Docket 11011，1973-11-07 签发，38 FR 31816 | 新增条款：(d) |

### 1.3　修订脉络一览

1973 初版　新增条款：(d)

## 2　§23.509 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1973-12-20，Amdt 23-14）：牵引载荷

▪ 修订沿革：本版由 NPRM　Notice 71-13，1971-04-27 签发，36 FR 8398　提出；最终规章 Docket 11011，1973-11-07 签发，38 FR 31816，1973-12-20 生效。

▪ 修订指令（中文译文）：16. 新增第 23.509 条，内容如下：　17. 新增一条 §23.509，内容如下：

修订指令英文原文（供核对）：

16. A new Sec. 23.509 is added to read as follows:

17. A new Sec. 23.509 would be added to read as follows:

## 3　§23.509 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1973-12-20 生效）　来源：Docket 11011，1973-11-07 签发，38 FR 31816

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The towing loads specified in paragraph (d) of this section must be considered separately. These loads must be applied at the towing fittings and must act parallel to the ground. In addition; | 新增 |
| (1) | A vertical load factor equal to 1.0 must be considered acting at the center of gravity; and | 新增 |
| (2) | The shock struts and tires must be in there static positions. | 新增 |
| (b) | For towing points not on the landing gear but near the plane of symmetry of the airplane, the drag and side tow load components specified for the auxiliary gear apply. For towing points located outboard of the main gear, the drag and side tow load components specified for the main gear apply. Where the specified angle of swivel cannot be reached, the maximum obtainable angle must be used. | 新增 |
| (c) | The towing loads specified in paragraph (d) of this section must be reacted as follows: | 新增 |
| (1) | The side component of the towing load at the main gear must be reacted by a side force at the static ground line of the wheel to which the load is applied. | 新增 |
| (2) | The towing loads at the auxiliary gear and the drag components of the towing loads at the main gear must be reacted as follows: | 新增 |
| (i) | A reaction with a maximum value equal tot he vertical reaction must be applied at the axle of the wheel to which the load is applied. Enough airplane inertia to achieve equilibrium must be applied. | 新增 |
| (ii) | The loads must be reacted by airplane inertia. | 新增 |
| (d) | The prescribed towing loads are as follows, where W is the design maximum weight: Tow point Position Load Magnitude No. Direction Main gear 0.223W per main gear unit. 1................................ 2................................ 3................................ 4................................ Forward, parallel to drag axis. Forward, at 30° to drag axis. Aft, parallel to drag axis. Aft, at 30° to drag axis. Auxiliary Gear Swiveled forward.. 0.3W | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-14 (1973-12-20) |
| --- | --- |
| (a) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |
| (i) | ● 新增 |
| (ii) | ● 新增 |
| (d) | ● 新增 |

## 4　§23.509 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-14。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 11011 | 38 FR 31816 | FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt |
| [2] | NPRM（提案） | Notice 71-13 / Docket 11011 | 36 FR 8398 | NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.509 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-14 | 1973-12-20 | 63869f3c-9a69-46d4-bc54-777d0bc312ec |

# § 23.511　地面载荷；多轮装置的不对称载荷（Ground load; unsymmetrical loads on multiple-wheel units.）

1969-09-14 初版　｜　1969-09-14 Amdt 23-7　｜　共 1 个历史版本

## 1　§23.511 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.511 |
| 条款标题（英文原文） | Ground load; unsymmetrical loads on multiple-wheel units. |
| 中文名称 | 地面载荷；多轮装置的不对称载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1969-09-14 初始版） |
| 时间跨度 | 1969-09-14 → 1969-09-14 （约 0 年） |
| 初版来源 | Docket 8083，1969-08-01 签发，34 FR 13078 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.511 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 新增条款：(1) |

### 1.3　修订脉络一览

1969 初版　新增条款：(1)

## 2　§23.511 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1969-09-14，Amdt 23-7）：地面载荷；多轮装置的不对称载荷

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：39. 新增§23.511，内容如下：

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案 16。多轮起落架组件的证实（新的第 23.511 条）。将增加新的第 23.511 条，内容如下：

英文原文：

Proposal 16. Substantiation of multiple-wheel landing gear units (new Sec. 23.511). A new Sec. 23.511 would be added to read as follows:

修订指令英文原文（供核对）：

39. A new Sec. 23.511 is added to read as follows:

## 3　§23.511 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Pivoting loads. The airplane is assumed to pivot about on side of the main gear with - | 新增 |
| (1) | The brakes on the pivoting unit locked; and | 新增 |
| (2) | Loads corresponding to a limit vertical load factor of 1, and coefficient of friction of 0.8, applied to the main gear and its supporting structure. | 新增 |
| (b) | Unequal tire loads. The loads established under Secs. 23.471 through 23.483 must be applied in turn, in a 60/40 percent distribution, to the dual wheels and tires in each dual wheel landing gear unit. | 新增 |
| (c) | Deflated tire loads. For the deflated tire condition -. | 新增 |
| (1) | 60 percent of the loads established under Secs. 23.471 through 23.483 must be applied in turn to each wheel in a landing gear unit; and | 新增 |
| (2) | 60 percent of the limit drag and side loads, and 100 percent of the limit vertical load established under Sec. 23.485 and 23.493 or lesser vertical load obtained under subparagraph (1) of this paragraph, must be applied in turn to each wheel in the dual wheel landing gear unit. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-7 (1969-09-14) |
| --- | --- |
| (a) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |

## 4　§23.511 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-7。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [2] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.511 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-7 | 1969-09-14 | 53970fcc-7a3d-4761-8974-00aa91dcd928 |

# § 23.521　水上载荷情况（Water load conditions.）

1965-02-01 初版　｜　1996-03-11 Amdt 23-48　｜　共 3 个历史版本

## 1　§23.521 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.521 |
| 条款标题（英文原文） | Water load conditions. |
| 中文名称 | 水上载荷情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 3 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 1996-03-11 （约 31 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.521 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订段落：(b)、(a) |
| 3 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 删除段落：(c) |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1993 23-45　修订段落：(b)、(a)　→　1996 23-48　删除段落：(c)

## 2　§23.521 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：水上载荷情况

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-45（1993-09-07 生效）：第23.521条经修订

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：第23.521条经修订，将(b)和(c)段修改为以下内容：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订第23.521条，并新增第23.523、23.525、23.527、23.529、23.531、23.533、23.535、23.537条以及一个新的附录H，以提供一套完整的新水上载荷要求。现行第23部引用《空军—海军—民用（ANC-3）》，并通过引用纳入第25部的水上载荷条款。由于收到的来自欧洲联合航空局（JAA）的唯一评论意见同意该提案，因此所提议的修订、新条款和附录H按提案予以采纳。

英文原文：

The FAA proposed to amend Section 23.521 and to add new Section 23.523, 23.525, 23.527, 23.529, 23.531, 23.533, 23.535, 23.537, and a new appendix H, to provide a complete new set of water load requirements. Present part 23 refers to Air Force-Navy-Civil (ANC-3) and incorporates by reference the water loads sections of part 25. Since the one comment received, from the JAA, agrees with the proposal, the proposed amendment, new sections and appendix H are adopted as proposed.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（2）：美国联邦航空局提议新增一个附录 H，以支持经修订的第 23.521 条。

英文原文：

The FAA proposed to add a new appendix H that supports amended Section 23.521.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：说明：参见第23.521条的提案。

英文原文：

Explanation: See proposal for Section 23.521.

修订指令英文原文（供核对）：

38. Section 23.521 is amended by revising paragraphs (b) and (c) to read as follows:

▪ 本版重印条文段落：(b)、(c)

### 2.3　Amdt 23-48（1996-03-11 生效）：对 §23.521 条进行修订

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：23. 对 §23.521 条进行修订，删除 (c) 段。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议修订§23.521，删除(c)段，该段涉及先前批准的浮筒，因为美国联邦航空局同意欧洲联合航空局（JAA）的意见，即(c)段的要求已包含在(a)段的一般要求中。

英文原文：

The FAA proposed to amend Sec. 23.521 by deleting paragraph (c), which deals with previously approved floats, because the FAA agreed with the JAA that the requirements of paragraph (c) are covered by the general requirements of paragraph (a).

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：本提案拟修订 §23.521 条，删除由第 23-45 号修正案增加的 (c) 段。欧洲联合航空局（JAA）指出，(c) 段包含的要求已在 (a) 段中涵盖。美国联邦航空局（FAA）表示同意，并提议删除 (c) 段。

英文原文：

This proposal would amend Sec. 23.521 by deleting paragraph (c), which was added by Amendment No. 23-45. The JAA pointed out that paragraph (c) contains requirements already covered in paragraph (a). The FAA agrees, and proposes to delete paragraph (c).

修订指令英文原文（供核对）：

23. Section 23.521 is amended by removing paragraph (c).

## 3　§23.521 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The structure of seaplanes and amphibians must be designed for water loads developed during takeoff and landing with the seaplane in any attitude likely to occur in normal operation at appropriate forward and sinking velocities under the most severe sea conditions likely to be encountered. | 新增 |
| (b) | Unless the applicant makes a rational analysis of the water loads, or uses the standards in ANC-3, Secs. 25.523 through 25.537 of this chapter apply. | 新增 |
| (c) | Floats certificated under Part 4a of this chapter before November 9, 1945, may be installed on airplanes that are designed under this part. Emergency Landing Conditions | 新增 |

### 3.2　版本2：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

修订段落：(b)、(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | Unless the applicant makes a rational analysis of the water loads, Sections 23.523 through 23.537 apply. | 修订 |
| (c) | Floats previously approved by the FAA may be installed on airplanes that are certificated under this part, provided that the floats meet the criteria of paragraph (a) of this section. | 修订 |

### 3.3　段落结构演变矩阵（横向对照 3 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-45 (1993-09-07) |
| --- | --- | --- |
| (a) | ● 新增 | ○ 无 |
| (b) | ● 新增 | △ 修订 |
| (c) | ● 新增 | △ 修订 |

## 4　§23.521 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | ctions of part 25. Since the one comment received, from the JAA, agrees with the proposal, the proposed amendment, new sections and appendix H are adopted as proposed. The FAA proposed to add a new appendix H that suppor… | Amdt 23-45 |
| 国际协调 | previously approved floats, because the FAA agreed with the JAA that the requirements of paragraph (c) are covered by the general requirements of paragraph (a). This proposal would amend Sec. 23.521 by deleting paragraph… | Amdt 23-48 |

本条取证提示 共 3 个历史版本：初始版, Amdt 23-45, Amdt 23-48。 其中 2 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [4] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [5] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [6] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [7] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [8] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [9] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.521 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | 5d1cdf0d-4228-46c8-909c-0de6e7d1f8e9 |
| Amdt 23-45 | 1993-09-07 | 4f40a9b1-7aa0-456c-81d4-5efb430d5bcc |
| Amdt 23-48 | 1996-03-11 | 13d2ca48-c22b-46c9-b1f1-66cc2576686e |

# § 23.523　设计重量与重心位置（Design weights and center of gravity positions.）

1993-09-07 初版　｜　1993-09-07 Amdt 23-45　｜　共 1 个历史版本

## 1　§23.523 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.523 |
| 条款标题（英文原文） | Design weights and center of gravity positions. |
| 中文名称 | 设计重量与重心位置 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 1993-09-07 （约 0 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.523 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订 |

### 1.3　修订脉络一览

1993 初版　修订

## 2　§23.523 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：设计重量与重心位置

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：39. 在“水载荷”标题下新增一条 §23.523，内容如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：美国联邦航空局提议修订第23.521条，并新增第23.523、23.525、23.527、23.529、23.531、23.533、23.535、23.537条以及一个新的附录H，以提供一套完整的新水上载荷要求。现行第23部引用《空军—海军—民用（ANC-3）》，并通过引用纳入第25部的水上载荷条款。由于收到的来自欧洲联合航空局（JAA）的唯一评论意见同意该提案，因此所提议的修订、新条款和附录H按提案予以采纳。

英文原文：

The FAA proposed to amend Section 23.521 and to add new Section 23.523, 23.525, 23.527, 23.529, 23.531, 23.533, 23.535, 23.537, and a new appendix H, to provide a complete new set of water load requirements. Present part 23 refers to Air Force-Navy-Civil (ANC-3) and incorporates by reference the water loads sections of part 25. Since the one comment received, from the JAA, agrees with the proposal, the proposed amendment, new sections and appendix H are adopted as proposed.

修订指令英文原文（供核对）：

39. A new Section 23.523 is added under the heading "Water Loads" to read as follows:

## 3　§23.523 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Design weights. The water load requirements must be met at each operating weight up to the design landing weight except that, for the takeoff condition prescribed in Section 23.531, the design water takeoff weight (the maximum weight for water taxi and takeoff run) must be used. | 新增 |
| (b) | Center of gravity positions. The critical centers of gravity within the limits for which certification is requested must be considered to reach maximum design loads for each part of the seaplane structure. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-45 (1993-09-07) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.523 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | ctions of part 25. Since the one comment received, from the JAA, agrees with the proposal, the proposed amendment, new sections and appendix H are adopted as proposed. 39. A new Section 23.523 is added under the heading… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.523 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | ac794cfc-db55-497d-aaec-6634df6cec14 |

# § 23.525　载荷的施加（(Application of loads.)）

1993-09-07 初版　｜　1993-09-07 Amdt 23-45　｜　共 1 个历史版本

## 1　§23.525 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.525 |
| 条款标题（英文原文） | (Application of loads.) |
| 中文名称 | 载荷的施加 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 1993-09-07 （约 0 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 58 FR 51970 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.525 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订 |

### 1.3　修订脉络一览

1993 初版　修订

## 2　§23.525 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：载荷的施加

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：新增第23.525条，标题为“水载荷”，内容如下：

修订指令英文原文（供核对）：

41. A new Section 23.525 is added under the heading "Water Loads" to read as follows:

### 2.2　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.525 的实际改动 |
| --- | --- | --- | --- |
| 58 FR 51970 | 1993-10-05 | 更正通告 (CFR 溯源注) | 32 条印刷更正；含 appendix H→appendix I、§23.613/23.865/23.23 等编号错植 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.525 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Unless otherwise prescribed, the seaplane as a whole is assumed to be subjected to the loads corresponding to the load factors specified in Section 23.527. | 新增 |
| (b) | In applying the loads resulting from the load factors prescribed in Section 23.527, the loads may be distributed over the hull or main float bottom (in order to avoid excessive local shear loads and bending moments at the location of water load application) using pressures not less than those prescribed in Section 23.533(b). | 新增 |
| (c) | For twin float seaplanes, each float must be treated as an equivalent hull on a fictitious seaplane with a weight equal to one-half the weight of the twin float seaplane. | 新增 |
| (d) | Except in the takeoff condition of Section 23.531, the aerodynamic lift on the seaplane during the impact is assumed to be 2/3 of the weight of the seaplane. Explanation: See proposal for Section 23.521. Reference: See proposal for Section 23.521. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-45 (1993-09-07) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |
| (d) | ● 新增 |

## 4　§23.525 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：58 FR 51970，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [3] | 更正通告 / 校正 | 58 FR 51970 | 1993-10-05 | Correction_58FR51970_1993-10-05_Amdt23-45.clean.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.525 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | 9d6831e5-279e-40dd-9b57-6c97fd2757e4 |

# § 23.527　船身与主浮筒载荷因数（(Hull and main float load factors.)）

1993-09-07 初版　｜　1993-09-07 Amdt 23-45　｜　共 1 个历史版本

## 1　§23.527 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.527 |
| 条款标题（英文原文） | (Hull and main float load factors.) |
| 中文名称 | 船身与主浮筒载荷因数 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 1993-09-07 （约 0 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 58 FR 51970 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.527 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订 |

### 1.3　修订脉络一览

1993 初版　修订

## 2　§23.527 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：船身与主浮筒载荷因数

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：41. 新增第23.527条，标题为“水载荷”，内容如下：

修订指令英文原文（供核对）：

41. A new Section 23.527 is added under the heading "Water Loads" to read as follows:

### 2.2　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.527 的实际改动 |
| --- | --- | --- | --- |
| 58 FR 51970 | 1993-10-05 | 更正通告 (CFR 溯源注) | 32 条印刷更正；含 appendix H→appendix I、§23.613/23.865/23.23 等编号错植 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.527 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Water reaction load factors nw must be computed in the following manner: | 新增 |
| (1) | For the step landing case | 新增 |
| (2) | For the bow and stem landing cases | 新增 |
| (b) | The following values are used: | 新增 |
| (1) | nw =water reaction load factor (that is, the water reaction divided by seaplane weight). | 新增 |
| (2) | C1 =empirical seaplane operations factor equal to 0.012 (except that this factor may not be less than that necessary to obtain the minimum value of step load factor of 2.33). | 新增 |
| (3) | VSO=seaplane stalling speed in knots with flaps extended in the appropriate landing position and with no slipstream effect. | 新增 |
| (4) | 9=Angle of dead rise at the longitudinal station at which the load factor is being determined in accordance with figure 1 of appendix H of this part. | 新增 |
| (5) | W=seaplane landing weight in pounds. | 新增 |
| (6) | K1=empirical hull station weighting factor, in accordance with figure 2 of appendix H of this part. | 新增 |
| (7) | rx=ratio of distance, measured parallel to hull reference axis, from the center of gravity of the seaplane to the hull longitudinal station at which the load factor is being computed to the radius of gyration in pitch of the seaplane, the hull reference axis being a straight line, in the plane of symmetry, tangential to the keel at the main step. | 新增 |
| (c) | For a twin float seaplane, because of the effect of flexibility of the attachment of the floats to the seaplane, the factor K1 may be reduced at the bow and stern to 0.8 of the value shown in figure 2 of appendix H of this part. This reduction applies only to the design of the carrythrough and seaplane structure. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-45 (1993-09-07) |
| --- | --- |
| (a) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (b) | ● 新增 |
| (3) | ● 新增 |
| (4) | ● 新增 |
| (5) | ● 新增 |
| (6) | ● 新增 |
| (7) | ● 新增 |
| (c) | ● 新增 |

## 4　§23.527 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 从处方性走向性能化 | ep. (c) For a twin float seaplane, because of the effect of flexibility of the attachment of the floats to the seaplane, the factor K1 may be reduced at the bow and stern to 0.8 of the value shown in figure 2 of appendix… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：58 FR 51970，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [3] | 更正通告 / 校正 | 58 FR 51970 | 1993-10-05 | Correction_58FR51970_1993-10-05_Amdt23-45.clean.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.527 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | 32ec4645-c40d-41f7-bc69-572427b55ab8 |

# § 23.529　船身与主浮筒着陆情况（(Hull and main float landing conditions.)）

1993-09-07 初版　｜　1993-09-07 Amdt 23-45　｜　共 1 个历史版本

## 1　§23.529 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.529 |
| 条款标题（英文原文） | (Hull and main float landing conditions.) |
| 中文名称 | 船身与主浮筒着陆情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 1993-09-07 （约 0 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.529 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订 |

### 1.3　修订脉络一览

1993 初版　修订

## 2　§23.529 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：船身与主浮筒着陆情况

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：42. 在“水载荷”标题下新增第23.529条，内容如下：

修订指令英文原文（供核对）：

42. A new Section 23.529 is added under the heading "Water Loads" to read as follows:

## 3　§23.529 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Symmetrical step, bow, and stern landing. For symmetrical step, bow, and stern landings, the limit water reaction load factors are those computed under Section 23.527. In addition -- | 新增 |
| (1) | For symmetrical step landings, the resultant water load must be applied at the keel, through the center of gravity, and must be directed perpendicularly to the keel line; and | 新增 |
| (2) | For symmetrical bow landings, the resultant water load must be applied at the keel, one-fifth of the longitudinal distance from the bow to the step, and must be directed perpendicularly to the keel line; and | 新增 |
| (3) | For symmetrical stern landings, the resultant water load must be applied at the keel, at a point 85 percent of the longitudinal distance from the step to the stern post, and must be directed perpendicularly to the keel line. | 新增 |
| (b) | Unsymmetrical landing for hull and single float seaplanes. Unsymmetrical step, bow, and stern landing conditions must be investigated. In addition-- | 新增 |
| (1) | The loading for each condition consists of an upward component and a side component equal, respectively, to 0.75 and 0.25 tan times the resultant load in the corresponding symmetrical landing condition; and | 新增 |
| (2) | The point of application and direction of the upward component of the load is the same as that in the symmetrical condition, and the point of application of the side component is at the same longitudinal station as the upward component but is directed inward perpendicularly to the plane of symmetry at a point midway between the keel and chine lines. | 新增 |
| (c) | Unsymmetrical landing; twin float seaplanes. The unsymmetrical loading consists of an upward load at the step of each float of 0.75 and a side load of 0.25 tan at one float times the step landing load reached under Section 23. 527. The side load is directed inboard, perpendicularly to the plane of symmetry midway between the keel and chine lines of the float, at the same longitudinal station as the upward load. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-45 (1993-09-07) |
| --- | --- |
| (a) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (3) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |

## 4　§23.529 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.529 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | 9c8702f8-3985-4a18-8564-1c0cdeb6c23d |

# § 23.531　船身与主浮筒起飞情况（(Hull and main float takeoff condition.)）

1993-09-07 初版　｜　1993-09-07 Amdt 23-45　｜　共 1 个历史版本

## 1　§23.531 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.531 |
| 条款标题（英文原文） | (Hull and main float takeoff condition.) |
| 中文名称 | 船身与主浮筒起飞情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 1993-09-07 （约 0 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.531 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订 |

### 1.3　修订脉络一览

1993 初版　修订

## 2　§23.531 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：船身与主浮筒起飞情况

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：43. 新增第23.531条，标题为"水载荷"，内容如下：　44. 在标题“水载荷”下新增第23.531条，内容如下：

修订指令英文原文（供核对）：

43. A new Section 23.531 is added under the heading "Water Loads" to read as follows:

44. A new Section 23.531 is added under heading "Water Loads" to read as follows:

## 3　§23.531 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The aerodynamic wing lift is assumed to be zero; and | 新增 |
| (b) | A downward inertia load, corresponding to a load factor computed from the following formula, must be applied: Where-- n=inertia load factor; CTO=empirical seaplane operations factor equal to 0.004; VS1=seaplane stalling speed (knots) at the design takeoff weight with the flaps extended in the appropriate takeoff position; =angle of dead rise at the main step (degrees); and W=design water takeoff weight in pounds. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-45 (1993-09-07) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.531 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.531 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | e878de89-c14d-4586-af9a-32c9e3163b9c |

# § 23.533　船身与主浮筒底部压力（(Hull and main float bottom pressures.)）

1993-09-07 初版　｜　1993-09-07 Amdt 23-45　｜　共 1 个历史版本

## 1　§23.533 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.533 |
| 条款标题（英文原文） | (Hull and main float bottom pressures.) |
| 中文名称 | 船身与主浮筒底部压力 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 1993-09-07 （约 0 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 58 FR 51970 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.533 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订：(b)、(c) |

### 1.3　修订脉络一览

1993 初版　修订：(b)、(c)

## 2　§23.533 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：船身与主浮筒底部压力

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：44. 在“水载荷”标题下新增第23.533条，内容如下：

修订指令英文原文（供核对）：

44. A new Section 23.533 is added under the heading "Water Loads" to read as follows:

### 2.2　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.533 的实际改动 |
| --- | --- | --- | --- |
| 58 FR 51970 | 1993-10-05 | 更正通告 (CFR 溯源注) | 32 条印刷更正；含 appendix H→appendix I、§23.613/23.865/23.23 等编号错植 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.533 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | General. The hull and main float structure, including frames and bulkheads, stringers, and bottom plating, must be designed under this section. | 新增 |
| (b) | Local pressures. For the design of the bottom plating and stringers and their attachments to the supporting structure, the following pressure distributions must be applied: | 新增 |
| (1) | For an unflared bottom, the pressure at the chine is 0.75 times the pressure at the keel, and the pressures between the keel and chine vary linearly, in accordance with figure 3 of appendix H of this part. The pressure at the keel (p.s.i.) is computed as follows: where-- PK=pressure (p.s.i.) at the keel; C2=0.00213; K2=hull station weighing factor, in accordance with figure 2 of appendix H of this part; VS1=seaplane stalling speed (knots) at the design water takeoff weight with flaps extended in the appropriate takeoff position; and x=angle of dead rise at keel, in accordance with figure 1 of appendix H of this part. | 新增 |
| (2) | For a flared bottom, the pressure at the beginning of the flare is the same as that for an unflared bottom, and the pressure between the chine and the beginning of the flare varies linearly, in accordance with figure 3 of appendix H of this part. The pressure distribution is the same as that prescribed in paragraph (b)(1) of this section for an unflared bottom except that the pressure at the chine is computed as follows: where-- Pch=pressure (p.s.i.) at the chine; C3=0.0016; K2=hull station weighing factor, in accordance with figure 2 of appendix H of this part; VS1=seaplane stalling speed (knots) at the design water takeoff weight with flaps extended in the appropriate takeoff position; and =angle of dead rise at appropriate station. The area over which these pressures are applied must simulate pressures occurring during high localized impacts on the hull or float, but need not extend over an area that would induce critical stresses in the frames or in the overall structure. | 新增 |
| (c) | Distributed pressures. For the design of the frames, keel, and chine structure, the following pressure distributions apply: | 新增 |
| (1) | Symmetrical pressures are computed as follows: where-- P=pressure (p.s.i.); C4=0.078 C1 (with C1 computed under Section 23.527); K2=hull station weighing factor, determined in accordance with figure 2 of appendix H of this part; VSO=seaplane stalling speed (knots) with landing flaps extended in the appropriate position and with no slipstream effect; and =angle of dead rise at appropriate station. | 新增 |
| (2) | The unsymmetrical pressure distribution consists of the pressures prescribed in paragraph (c)(1) of this section on one side of the hull or main float centerline and one-half of that pressure on the other side of the hull or main float centerline, in accordance with figure 3 of appendix H of this part. | 新增 |
| (3) | These pressures are uniform and must be applied simultaneously over the entire hull or main float bottom. The loads obtained must be carried into the sidewall structure of the hull proper, but need not be transmitted in a fore and aft direction as shear and bending loads. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-45 (1993-09-07) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |
| (1) | ● 新增 |
| (2) | ● 新增 |
| (c) | ● 新增 |
| (3) | ● 新增 |

## 4　§23.533 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：58 FR 51970，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [3] | 更正通告 / 校正 | 58 FR 51970 | 1993-10-05 | Correction_58FR51970_1993-10-05_Amdt23-45.clean.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.533 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | 746af4b3-12cc-41f1-9c82-7437ae0c180d |

# § 23.535　辅助浮筒载荷（(Auxiliary float loads.)）

1993-09-07 初版　｜　1993-09-07 Amdt 23-45　｜　共 1 个历史版本

## 1　§23.535 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.535 |
| 条款标题（英文原文） | (Auxiliary float loads.) |
| 中文名称 | 辅助浮筒载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 1993-09-07 （约 0 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 58 FR 51970 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.535 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订：(b)、(g)、(a)、(c) |

### 1.3　修订脉络一览

1993 初版　修订：(b)、(g)、(a)、(c)

## 2　§23.535 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：辅助浮筒载荷

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：45. 在标题“水载荷”下新增第 23.535 条，内容如下：

修订指令英文原文（供核对）：

45. A new Section 23.535 is added under the heading "Water Loads" to read as follows:

### 2.2　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.535 的实际改动 |
| --- | --- | --- | --- |
| 58 FR 51970 | 1993-10-05 | 更正通告 (CFR 溯源注) | 32 条印刷更正；含 appendix H→appendix I、§23.613/23.865/23.23 等编号错植 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.535 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | General. Auxiliary floats and their attachments and supporting structures must be designed for the conditions prescribed in this section. In the cases specified in paragraphs (b) through (e) of this section, the prescribed water loads may be distributed over the float bottom to avoid excessive local loads, using bottom pressures not less than those prescribed in paragraph (g) of this section. | 新增 |
| (b) | Step loading. The resultant water load must be applied in the plane of symmetry of the float at a point three-fourths of the distance from the bow to the step and must be perpendicular to the keel. The resultant limit load is computed as follows, except that the value of L need not exceed three times the weight of the displaced water when the float is completely submerged: where-- L=limit load (lbs.); C5=0.0053; VSO=seaplane stalling speed (knots) with landing flaps extended in the appropriate position and with no slipstream effect; W=seaplane design landing weight in pounds; 5=angle of dead rise at a station 3/4 of the distance from the bow to the step, but need not be less than 15 degrees; and ry=ratio of the lateral distance between the center of gravity and the plane of symmetry of the float to the radios of gyration in roll. | 新增 |
| (c) | Bow loading. The resultant limit load must be applied in the plane of symmetry of the float at a point one-fourth of the distance from the bow to the step and must be perpendicular to the tangent to the keel line at that point. The magnitude of the resultant load is that specified in paragraph (b) of this section. | 新增 |
| (d) | Unsymmetrical step loading. The resultant water load consists of a component equal to 0.75 times the load specified in paragraph (a) of this section and a side component equal to 3.25 tan times the load specified in paragraph (b) of this section. The side load must be applied perpendicularly to the plane of symmetry of the float at a point midway between the keel and the chine. | 新增 |
| (e) | Unsymmetrical bow loading. The resultant water load consists of a component equal to 0.75 times the load specified in paragraph (b) of this section and a side component equal to 0.25 tan times the load specified in paragraph (c) of this section. The side load must be applied perpendicularly to the plane of symmetry at a point midway between the keel and the chine. | 新增 |
| (f) | Immersed float condition. The resultant load must be applied at the centroid of the cross section of the float at a point one-third of the distance from the bow to the step. The limit load components are as follows: where-- P=mass density of water (slugs/ft.3) V=volume of float (ft.3); Cx=coefficient of drag force, equal to 0.1333; Cy=coefficient of side force, equal to 0.106; K=0.8, except that lower values may be used if it is shown that the floats are incapable of submerging at a speed of 0.8 VSO in normal operations; VSO=seaplane stalling speed (knots) with landing flaps extended in the appropriate position and with no slipstream effect; and g=acceleration due to gravity (ft/sec2). | 新增 |
| (g) | Float bottom pressures. The float bottom pressures must be established under Section 23.533, except that the value of K2 in the formulas may be taken as 1.0. The angle of dead rise to be used in determining the float bottom pressures is set forth in paragraph (b) of this section. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-45 (1993-09-07) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |
| (c) | ● 新增 |
| (d) | ● 新增 |
| (e) | ● 新增 |
| (f) | ● 新增 |
| (g) | ● 新增 |

## 4　§23.535 总结

本条各版本的修订动因未在 NPRM/Final Rule 前言中展开说明，多为随主修订案一并调整的编辑性或交叉引用性修改。

本条取证提示 共 1 个历史版本：Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：58 FR 51970，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [3] | 更正通告 / 校正 | 58 FR 51970 | 1993-10-05 | Correction_58FR51970_1993-10-05_Amdt23-45.clean.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.535 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | e1b85f54-9daf-4d2f-be1f-6c32f284a751 |

# § 23.537　海翼载荷（Seawing loads.）

1993-09-07 初版　｜　1993-09-07 Amdt 23-45　｜　共 1 个历史版本

## 1　§23.537 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.537 |
| 条款标题（英文原文） | Seawing loads. |
| 中文名称 | 海翼载荷 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 1993-09-07 （约 0 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.537 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 修订 |

### 1.3　修订脉络一览

1993 初版　修订

## 2　§23.537 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：海翼载荷

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：46. 新增第23.537条，标题为“水载荷”，内容如下：

修订指令英文原文（供核对）：

46. A new Section 23.537 is added under the heading "Water Loads" to read as follows:

## 3　§23.537 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.537 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | wing loads Seawing design loads must be based on applicable test data.… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-45。 其中 1 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.537 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | ad67048c-acc4-48e1-a7b5-35bfc7baaa34 |

# § 23.561　总则（应急着陆）（General.）

1965-02-01 初版　｜　2012-01-31 Amdt 23-62　｜　共 7 个历史版本

## 1　§23.561 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.561 |
| 条款标题（英文原文） | General. |
| 中文名称 | 总则（应急着陆） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 7 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 2012-01-31 （约 47 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 52 FR 34745、72 FR 59939、72 FR 72915 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.561 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 修订：(c)、(d) |
| 3 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 条文未改动（本版为交叉引用或其他条款修订所致） |
| 4 | Amdt 23-36 | 1988-09-14 | Notice 86-19，1986-11-28 签发，51 FR 44878 | Docket 25147，1988-08-08 签发，53 FR 30802 | 修订段落：(b) |
| 5 | Amdt 23-46 | 1994-06-16 | Notice 90-20，1990-08-23 签发，55 FR 35544 | Docket 26324，1994-05-11 签发，59 FR 25766 | 新增段落：(b) |
| 6 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增段落：(b)、(d)、(e) |
| 7 | Amdt 23-62 | 2012-01-31 | — | — | 条文未改动（本版为交叉引用或其他条款修订所致） |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1969 23-7　修订：(c)、(d)　→　1987 23-34　条文未改动（本版为交叉引用或其他条款修订所致　→　1988 23-36　修订段落：(b)　→　1994 23-46　新增段落：(b)　→　1996 23-48　新增段落：(b)、(d)、(e)　→　2012 23-62　条文未改动（本版为交叉引用或其他条款修订所致

## 2　§23.561 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：总则（应急着陆）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-7（1969-09-14 生效）：修订第23.561条

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：40. 修订第23.561条，将(c)(3)段和(d)段的引言文字修改为如下：

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（1）：提案 17。应急着陆情况（§23.561 条）。1. §23.561(c)(3) 和 (d) 段（引言文字）将通过在“假设”一词之后增加一个逗号和“在没有更合理的分析的情况下——”这些词来修订。

英文原文：

Proposal 17. Emergency landing conditions (Sec. 23.561). 1. Section 23.561 (c) (3) and (d) (introductory text) would be amended by adding a comma and the words "in the absence of a more rational analysis -" after the word "assuming."

来源：NPRM（提案）｜32 FR 5791｜NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt

▪ 背景与评论处置（2）：解释。

英文原文：

Explanation. change 1 would reflect present intent with respect to allowing the use of rationally determined values of ultimate inertia force and coefficient of friction instead of the specified values. Change 2 is necessary because the term "vertical ultimate acceleration" in the note following former CAR Sec. 3.386 (c) was intended to refer to inertia forces that are downward, with respect to the airline, in the wheels-retracted emergency landing condition. For the turnover condition of Sec. 23.561 (d), the CAR language "vertical ultimate acceleration" is properly recodified as "upward," again with respect to the airplane.

修订指令英文原文（供核对）：

40. Section 23.561 is amended by amending paragraph (c)(3) and the introductory text of paragraph (d) to read as follows:

▪ 本版重印条文段落：(c)、(3)、(i)、(ii)、(d)

### 2.3　Amdt 23-34（1987-02-17 生效）：条文未改动（本版为交叉引用或其他条款修订所致）

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 本版对条文的处置：条文未改动（本版为交叉引用或其他条款修订所致）

来源：Final Rule｜52 FR 1806｜FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt

▪ 背景与评论处置（1）：对§23.561条的评论。对于通勤类飞机，一名评论者指出，没有提出任何提案来加强§23.561条中与应急着陆情况相关的最大载荷因数。该评论者主张，§23.561条中包含的极限惯性力远低于人体能够承受的水平。该评论者引用美国国家运输安全委员会（NTSB）1981年题为“运输类飞机座舱安全”的报告作为此信息的来源。针对此评论，该提案的目的并非在此时重新评估根据第23部取得型号合格证的所有飞机与应急着陆情况相关的最大载荷因数。美国联邦航空局（FAA）正在考虑在第23部适航审查框架内修订§23.561条的要求。该评论超出了第83-17号通告的范围，无法作为本次规章制定的一部分予以处理。

英文原文：

Comment to Sec. 23.561. For commuter category airplanes, one commenter notes that no proposal has been made to strengthen the maximum load factors of Sec. 23.561 associated with emergency landing conditions. This commenter contends that the ultimate inertia forces contained in Sec. 23.561 are far below the level the human body is capable of withstanding. The commenter cites the National Transportation Safety Board (NTSB) 1981 report entitled "Cabin Safety in Transport Category Aircraft" as the source for this information. In response to this comment, the purpose of the proposal was not to reevaluate the maximum load factors associated with emergency landing conditions for all airplanes to be type certificated pursuant to Part 23 at this time. The FAA is considering a revision to the requirements of Sec. 23.561 within the framework of the Part 23 Airworthiness Review. The comment is beyond the scope of Notice No. 83-17 and cannot be acted on as a part of this rulemaking.

### 2.4　Amdt 23-36（1988-09-14 生效）：修订第23.561条

▪ 修订沿革：本版由 NPRM　Notice 86-19，1986-11-28 签发，51 FR 44878　提出；最终规章 Docket 25147，1988-08-08 签发，53 FR 30802，1988-09-14 生效。

▪ 修订指令（中文译文）：3. 修订第23.561条，将段(b)和段(d)修改如下：　1-2. 通过删除 (c) 段来修订第 23.561 条；修订 (b) 段；将 (d) 段和 (e) 段分别重新编号为 (c) 段和 (d) 段；并修订重新编号后的 (c) 段和 (d) 段，内容如下：

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（1）：提案 1-2。本提案将重新组织第 23.561 条，引用新的第 23.562 条，增加正常类、实用类和特技类飞机中质量块固定的强度要求，并以客观术语表述翻转要求，其中包括在确定应急着陆情况下发生翻转的可能性时应考虑的变量。本提案是根据小型飞机适航审查会议上提出的第 218、219、220、221、222 和 518 号提案作出的。

英文原文：

Proposal 1-2. This proposal would reorganize Section 23.561, make reference to a new Section 23.562, add strength requirements for the retention of items of mass in normal, utility, and acrobatic category airplanes, and state turnover requirements in objective terms which include the variables to be considered in determining the likelihood of a turnover during emergency landing conditions. This proposal was made as a result of proposals 218, 219, 220, 221, 222, and 518 presented at the Small Airplane Airworthiness Review Conference.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（2）：一名评论者提议，将拟议的 §23.561(b) 条中的术语“轻微坠撞着陆”替换为“应急着陆情况”，以增加一致性。该评论者还建议将“避免严重伤害”一词替换为“设计用于保护每位乘员”。美国联邦航空局（FAA）表示同意，并在 §23.561(b) 条中采用了该评论者提议的措辞。此外，由于上述评论，本最终规章的 §23.561(b) 条规定了第 23 部飞机所有类别的应急着陆静载荷因数。

英文原文：

One commenter proposes replacing the term "minor crash landing" in proposed Section 23.561(b) with the term "emergency landing conditions" to add consistency. The same commenter suggests replacing the words "escape serious injury" with the words "designed to protect each occupant." The FAA agrees, and the wording proposed by the commenter is used in Section 23.561(b). Also, as a result of the previous comment, emergency landing static load factors for all categories of Part 23 airplanes are set forth in Section 23.561(b) of this final rule.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（3）：一位评论者建议将拟议的第23.561(d)条中的极限静载荷因数修改为：向上载荷因数4.5g，侧向载荷因数4.5g，并增加向后的载荷因数1.5g，同时建议这些载荷因数适用于所有第23部飞机。该评论者并未提供支持这些载荷因数的技术依据。由于该评论者的提议超出通告范围且未获支撑，因此不予纳入本最终规章。

英文原文：

One commenter proposes that the ultimate static load factors of proposed Section 23.561(d) be changed so that the upward load factor is 4.5g, the sideward load factor is 4.5g, and a rearward load factor of 1.5g is added, and suggests these load factors apply to all Part 23 airplanes. The commenter offers no technical basis for support of these load factors. Since the commenter's proposals go beyond the scope of the notice and are unsupported, they are not incorporated in this final rule.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（4）：若干评论针对拟议的 §23.561(b)(3) 条，该条规定了座舱内质量块的约束要求。其中部分评论还涉及拟议的 §23.561(d) 条中所示质量块的具体载荷因数。一位评论者建议，将拟议的 §23.561(b)(3) 条所引用的载荷因数修改为与 1986 年 7 月 17 日在《联邦公报》上发布的第 86-11 号通告（51 FR 25982）中为运输类飞机拟议的载荷因数相一致。针对 §23.561(b)(3) 条所拟议的质量块静载荷要求的制定，是 GASP 座椅/约束系统动力试验提案制定工作的一部分。尽管 GASP 最初建议对质量块进行动力试验，但美国联邦航空局（FAA）和 GASP 随后

英文原文：

Several comments address proposed Section 23.561(b)(3), which stated restraint requirements for items of mass in the cabin. Some of these comments also address the specific load factors for items of mass shown in proposed Section 23.561(d). One commenter recommends that the load factors referenced by proposed Section 23.561(b)(3) be changed to match those proposed for transport airplanes in Notice No. 86-11, which was published in the FEDERAL REGISTER on July 17, 1986 (51 FR 25982). The development of the proposed static load requirements for items of mass, as proposed in Section 23.561(b)(3), was a part of the development of the seat/restraint system dynamic tests proposal of the GASP. Although the GASP originally recommended that items of mass be dynamically tested, the FAA and the GASP subsequently

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（5）：另一位评论者同意质量块的 4.5g 侧向载荷因数，但不同意拟议的 18g 向前载荷因数。该评论者表示，经验表明现有的 9g 向前载荷要求已产生足够的静力强度。美国联邦航空局（FAA）认为 9g 载荷因数不足以满足第 86-19 号通告中拟议的座舱安全水平。在美国联邦航空局（FAA）/美国国家航空航天局（NASA）联合项目中进行的实际坠撞演示试验提供的数据表明，在模拟的可生存坠撞期间，纵轴上的撞击载荷可能超过 9g。日期为 1982 年 11 月、题为“撞击时结构飞机坠撞数据
正如GASP所提议，拟议的静载荷因数的目的在于，考虑到本最终规章其他条所增强的座舱安全性——包括第23.562条中关于座椅/约束系统动力试验的新要求——确保座舱内质量块的连接得到充分约束。该评论者建议修改§23.561(b)(3)段，要求设备及其他大质量块的安放位置应做到，若其松脱，在应急着陆后不太可能导致乘员受伤、刺穿燃油管路或堵塞出口。美国联邦航空局不同意。对许多小型飞机设计而言，将所有具有显著质量的物品（特别是货物）置于在撞击中能确保不会造成乘员受伤或燃油系统被穿透的座舱位置这一构想并不实际。本最终规章中，为质量块设定较高的载荷因数是必要的，以确保在§23.562(b)条中座椅/约束系统的动力载荷情况下，座舱内质量块的连接强度足够。
较高的静力载荷因数预计将防止在应急着陆地面撞击期间因质量块松脱而可能导致的伤害。

英文原文：

Another commenter agrees with the 4.5g sideward load factor for items of mass, but disagrees with the proposed 18g forward load factor. The commenter states that experience indicates that the existing 9g forward load requirement has resulted in adequate static strength. The FAA does not consider the 9g load factor sufficient for the level of cabin safety proposed in Notice No. 86-19. Actual crash demonstration tests conducted in a joint FAA/NASA project provided data which showed that impact loads during simulated survivable crashes could exceed 9g in the longitudinal axis. NASA Technical Paper 2083, dated November 1982, titled, "Correlation and Assessment of Structural Airplane Crash Data With Flight Parameters at Impact" is one of several reports which evaluate the results of the FAA/NASA small airplane crash test project. As proposed by the GASP, the purpose of the proposed static load factors is to ensure that the attachments of items of mass in the cabin are adequately restrained considering the enhanced cabin safety provided by other sections of this final rule, including the new requirements in Section 23.562 for dynamic tests of seat/restraint systems. This commenter proposes changing Section 23.561(b)(3) to require that equipment and other large mass items be positioned so that, if they break loose, they are unlikely to cause injury to the occupants, penetrate fuel lines, or block exits after an emergency landing. The FAA disagrees. For many small airplane designs, the concept of locating all items of significant mass, particularly cargo, at a location in the cabin which will ensure no occupant injury or fuel system penetration during impact is impractical. The higher load factors for items of mass are necessary in this final rule to ensure that the attachment strength for mass items in the cabin is adequate with the seat/restraint system dynamic load conditions of Section 23.562(b). The higher static load factors are expected to prevent injuries which could result from mass items coming loose during an emergency landing ground impact.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（6）：关于建议的§23.561(c)段收到了若干评论，该段规定了乘员翻转保护的各项要求。一名评论者要求对此建议进行澄清，并得出结论认为该...

英文原文：

Several comments were received regarding the proposed Section 23.561(c) which stated requirements for occupant protection from turnover. One commenter asks for clarification of this proposal and concludes that the intent of this

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（7）：要求是将翻转情况作为静力情况而非动力情况进行评定。美国联邦航空局（FAA）同意这一解释，美国联邦航空局（FAA）的意图并非将翻转防护作为动力情况进行评定，例如第 23.562 条的座椅/约束系统动力试验要求。该评论者还得出结论认为，使用拟议的第 23.561(c) 条中确定翻转可能性的拟议准则，几乎所有前三点式起落架

英文原文：

requirement is to evaluate the turnover case as a static rather than a dynamic condition. The FAA agrees with this interpretation, and it is not the intent of the FAA for turnover protection to be evaluated as a dynamic condition such as the seat/restraint system dynamic tests requirements of Section 23.562. This commenter also concludes that, using the proposed criteria for establishing the likelihood of a turnover in proposed Section 23.561(c), almost all tricycle landing gear

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（8）：装有前三点式起落架的飞机在正常运行期间本身具有抗翻转能力，美国联邦航空局（FAA）事故/事件数据表明，前三点式起落架小型飞机在应急着陆情况（包括未达跑道、越过跑道、失去方向控制等事件）下发生了翻转。这些数据显示，上单翼前三点式起落架飞机的翻转频率高于下单翼前三点式起落架飞机；然而，数据表明，下单翼前三点式起落架小型飞机在应急着陆情况下也会以足够高的频率发生翻转，因此也应当调查这些飞机发生翻转的可能性。一些下单翼前三点式起落架小型飞机的近期设计采用了滑动式透明座舱盖，这在翻转时提供的乘员保护非常有限。
建议的用于评估翻转的条件与 §23.561 条其他要求所提供的安全水平相容，且美国联邦航空局（FAA）认为建议的 §23.561(c) 段要求是评估应急着陆情况下翻转可能性的合理准则。

英文原文：

with tricycle landing gear are inherently resistant to turnover during normal operations, FAA accident/incident data showed that tricycle gear small airplanes have turned over during emergency landing conditions (including incidents of undershoot, overshoot, loss of directional control, etc.). These data revealed that the frequency of turnover of high-wing, tricycle gear airplanes is higher than for low-wing, tricycle gear airplanes; however, the data showed that low wing tricycle gear small airplanes also turn over during emergency landing conditions with sufficient frequency that the likelihood of turnover should be investigated for these airplanes also. Some recent designs of low-wing tricycle gear small airplanes feature a sliding transparent canopy which provides very little occupant protection in turnover. The proposed conditions for evaluating turnover are compatible with the level of safety provided by the other requirements of Section 23.561, and the FAA considers the requirements of proposed Section 23.561(c) to be rational criteria for evaluating the likelihood of turnover during emergency landing conditions.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（9）：一位

英文原文：

One commenter suggests that the wording of proposed Section 23.561(c)(1) and (2) be changed to require turnover evaluation in the most adverse combination of all-up-weight and center-of-gravity position. The FAA disagrees since all-up-weight is not a defined term used in Part 23. The forward center-of-gravity position is usually the most critical for turnover evaluation. Another commenter suggests that proposed Section 23.561(c)(6) be designated as Section 23.561(d) and that proposed Section 23.561(d) be designated Section 23.561(e). The FAA agrees that, while paragraphs (1) through (5) of proposed Section 23.561(c) provide criteria for determining the likelihood of a turnover, paragraph (6) defines requirements for the loads to be applied to the inverted airplane after a turnover, for evaluating the occupant protection provided by the airframe. In response to this comment and other editorial changes made to Section 23.561, proposed Section 23.561(c) is designated Section 23.561(d) and reformatted for clarity in this final rule and present Section 23.561(c) is retained.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（10）：该评论超出了该规章制定提案通告的范围。将在动力试验期间在刚性试验夹具上测得的载荷作为静载荷施加于刚性较小的机体结构是不切实际的。在应急着陆撞击期间，飞机地板结构通常会在严重的局部载荷下屈服，这往往会改变座椅/约束系统与机体结构交界面处的连接载荷。在拟议的第 23.562 条的动力试验中纳入包含机体结构的程序将大大增加这些试验的复杂性和成本。支撑座椅/约束系统的机体结构必须满足第 23.561 条及第 23 部其他条款的静强度要求。因此，本最终规章不要求测量连接载荷，尽管此类测量可能提供有用的设计信息。

英文原文：

This comment goes beyond the scope of the notice. It is unrealistic to apply loads, measured on a rigid test fixture during dynamic tests, as static loads to less rigid airframe structure. During an emergency landing impact, the airplane floor structure will typically yield under severe local loads, and this will tend to alter the attachment loads at the seat/restraint system interface to the airframe structure. Incorporating procedures to include airframe structure in the dynamic tests of proposed Section 23.562 would greatly increase the complexity and costs of these tests. Airframe structure supporting the seat/restraint system must meet the static strength requirements of Section 23.561 and other sections of Part 23. Therefore, this final rule does not require measurement of attachment loads, although such measurements may provide useful design information.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（11）：一位评论者建议将拟议的第 23.785 条(b) 段中的术语“肩部束缚装置”替换为“安全带和肩部束缚装置”，另外两位评论者建议删除该段中对第 23.561 条的引用。经审查拟议的第 23.785 条(b) 段后，美国联邦航空局认为这些要求与本最终规章第 23.785 条和第 23.562 条的其他要求重复；因此，拟议的第 23.785 条(b) 段未予采纳。

英文原文：

One commenter suggests replacing the term "shoulder harness" in proposed Section 23.785(b) by the term "seat belt and shoulder harness," and two other commenters suggest removing the reference to Section 23.561 for this section. After reviewing proposed Section 23.785(b) the FAA concludes that these requirements are redundant with other requirements of Sections 23.785 and 23.562 of this final rule; therefore, proposed Section 23.785(b) is not adopted.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（12）：提案1-6。第86-19号通告提议修订第23.787条，以提高货舱的静强度要求，该提案源自小型飞机适航性评审会议的提案288、289、518和524。会议达成共识，货物约束装置应按照第23.561条的极限静载荷因数进行设计，以在应急着陆情况下保护货舱前方乘员的安全。对于在同一舱室内搭载货物和乘员的设计，GASP提议这些舱室应具有足够的强度，以在可能导致乘员暴露于拟议第23.562(b)(2)段动力情况的条件下的情况下固定货物。由于座椅的动力试验评估了特定的乘员伤害标准，美国联邦航空局认为对质量块进行动力试验没有必要，因为只有固定这些物品才是安全问题。因此，第23.561(b)(3)段的极限静载荷因数在第23.787(e)段中被引用。

英文原文：

Proposal 1-6. Notice No. 86-19 proposed to amend Section 23.787 to increase the static strength requirements for cargo compartments and resulted from conference proposals 288, 289, 518, and 524 of the Small Airplane Airworthiness Review Conference. It was a consensus at the conference that cargo restraints should be designed to the ultimate static load factors of Section 23.561 to protect occupants forward of the cargo during emergency landing conditions. For designs that carry cargo in the same compartment with occupants, the GASP proposed that these compartments have adequate strength to retain cargo in conditions which could result in occupant exposure to the dynamic conditions of proposed Section 23.562(b)(2). Since dynamic testing of seats evaluates specific occupant injury criteria, the FAA considers dynamic testing of items of mass to be unnecessary because only retention of these items is a safety concern. Therefore, the ultimate static load factors of Section 23.561(b)(3) are referenced in Section 23.787(e).

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（13）：一名评论者建议，建议的 §23.787 条应要求，在考虑 §23.561 条的极限静载荷因数时，构成乘员与货物之间隔板的结构强度须针对最大重量的货物载荷进行证实。美国联邦航空局（FAA）同意，增加对最大货物重量的引用使该规则的意图更加清晰，且最终规章的 (c) 和 (e) 段包含了在货舱最大允许货物重量下进行评估的要求。

英文原文：

One commenter suggests that proposed Section 23.787 require that the strength of the structure forming the partition between the occupants and cargo be substantiated for a cargo load of maximum weight when considering the ultimate static load factors of Section 23.561. The FAA agrees that adding a reference to the maximum cargo weight adds clarity to the intent of the rule, and paragraphs (c) and (e) of the final rule include requirements for evaluation at the maximum allowed cargo weight for the compartment.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（14）：在对第23.787条的拟议修改中，收到了几条关于引用第23.561条各段的编辑性意见。在本最终规章中已对这些意见予以考虑。

英文原文：

Several editorial comments were received with regard to references to the paragraphs of Section 23.561 in proposed changes to Section 23.787. They have been considered in this final rule.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（15）：一位评论者建议，应当将通告中的法规引用与第 23 部的最新修订版本进行交叉核对。经审查《联邦航空规章》，美国联邦航空局确定需对第 23 部中的若干条款进行修改，使其与座椅/约束系统动力试验的新要求相匹配。本次审查所确定的法规修改并非实质性的，而是为了与本修订案所做的其他修改保持一致。审查结果为本最终规章增加了若干内容。修订 §23.2，删除对 §23.785(g) 和 (h) 段的引用，并改为直接阐明此前以引述方式纳入的约束系统乘员保护要求。修订 §23.967，使其 (e)(1) 段引用 §23.561(b)(2) 段。修订 §23.1411，使其 (b)(2) 段引用 §23.561(b)(3) 段。
第23.1413条修订为仅包含要求，即每条安全带和肩带束缚装置均须配备金属对金属锁定装置，因为现有第23.1413条的段(a)的要求与本最终规章第23.785条的段(a)和段(b)的要求重复，且现有第23.1413条的段(b)的要求与第23.785条(m)(2)的要求不兼容。

英文原文：

One commenter suggests that the regulatory references in the notice should be crosschecked with the latest revisions of Part 23. As a result of reviewing the FAR, the FAA identified several sections of Part 23 which need to be changed for compatibility with the new requirements for dynamic testing of seat/restraint systems. The regulatory changes identified by this review are not substantive but provide consistency with the other changes made by this amendment. This review resulted in several additions to this final rule. Section 23.2 is revised to delete the reference to Section 23.785(g) and (h) and, instead, states these previously incorporated-by-reference occupant protection requirements for restraint systems. Section 23.967 is revised so that paragraph (e)(1) refers to Section 23.561(b)(2). Section 23.1411 is revised so that paragraph (b)(2) refers to section 23.561(b)(3). Section 23.1413 is revised to include only the requirement that each safety belt and shoulder harness be equipped with a metal-to-metal latching device since the requirements of paragraph (a) of existing Section 23.1413 are redundant to the requirements in paragraphs (a) and (b) of Section 23.785 of this final rule, and the requirements of paragraph (b) of existing Section 23.1413 are not compatible with the requirements of Section 23.785(m)(2).

来源：NPRM（提案）｜51 FR 44878｜NPRM_Notice 86-19_Docket25147_51FR44878_1986-12-12.txt

▪ 背景与评论处置（16）：尽管如提案所述

英文原文：

Although Section 23.561(b) as proposed, identifies new dynamic requirements, the table of values provided in current Section 23.561(b)(2) is retained because these values are referenced in other sections of Part 23. Therefore, the contents of the table redesignated as ULTIMATE STATIC LOAD FACTORS are being retained in new Section 23.561(d) with an additional set of values for items of mass within the occupiable cabin area to assure retention of such mass items during an impact event.

The FAA has carefully considered each of the conference proposals listed below together with the transcript of the related discussions which occurred during the conference. The FAA has concluded that the proposal for amendment of Section 23.561, when adopted, would significantly improve the cabin safety and occupant protection of small airplanes. This proposal, supported by the FAA Crash Dynamics Engineering and Development Program and the GASP recommendations, meets the intent of other referenced proposals which would only have increased the static load factors.

来源：NPRM（提案）｜51 FR 44878｜NPRM_Notice 86-19_Docket25147_51FR44878_1986-12-12.txt

▪ 背景与评论处置（17）：美国联邦航空局（FAA）已仔细考虑了下列各项会议提案以及会议期间发生的相关讨论的记录。美国联邦航空局（FAA）得出结论认为，修订第 23.561 条的提案一旦被采纳，将显著改善小型飞机的座舱安全和乘员保护。该提案得到了美国联邦航空局（FAA）坠撞动力学工程与开发计划以及 GASP 建议的支持，符合其他被引用提案的意图，而这些提案只会增加静载荷因数。

英文原文：

The FAA has carefully considered each of the conference proposals listed below together with the transcript of the related discussions which occurred during the conference. The FAA has concluded that the proposal for amendment of Section 23.561, when adopted, would significantly improve the cabin safety and occupant protection of small airplanes. This proposal, supported by the FAA Crash Dynamics Engineering and Development Program and the GASP recommendations, meets the intent of other referenced proposals which would only have increased the static load factors.

修订指令英文原文（供核对）：

3. Section 23.561 is amended by revising paragraphs (b) and (d) to read as follows:

1-2. By amending Section 23.561 by removing paragraph (c); by revising paragraph (b); redesignating paragraphs (d) and (e) as paragraphs (c) and (d) respectively; and revising newly redesignated paragraphs (c) and (d), to read as follows:

▪ 本版重印条文段落：(b)、(1)、(2)、(i)、(ii)、(iii)、(3)、(i)、(ii)、(iii)、(d)、(1)、(i)、(ii)、(iii)、(iv)、(v)、(2)

### 2.5　Amdt 23-46（1994-06-16 生效）：修订 §23.561 条

▪ 修订沿革：本版由 NPRM　Notice 90-20，1990-08-23 签发，55 FR 35544　提出；最终规章 Docket 26324，1994-05-11 签发，59 FR 25766，1994-06-16 生效。

▪ 修订指令（中文译文）：2. 修订 §23.561 条，新增 (b)(2)(iv) 段，内容如下：　2. 修订第23.561条，新增段落 (b)(2)(iv)，内容如下：

来源：Final Rule｜59 FR 25766｜FinalRule_Docket 26324_Amdt23-46_59FR25766_1994-05-17_0002.txt

▪ 背景与评论处置（1）：一名评论者建议

英文原文：

One commenter recommends higher downward inertia loads and a correspondingly higher descent velocity, which was proposed as an alternate approach to establishing the static downward load factor. The FAA disagrees. Although downward load factors may be greater than 6g during emergency landing conditions, the FAA intends to make the airworthiness standards for commuter category airplanes consistent with those for small transport airplanes; therefore, the 6g downward load factor will remain as a minimum requirements. Accordingly, that portion of Sec. 23.561(b)(2)(iv) that specifies the use of any lesser force is deleted in the final rule. The removal of this lesser force form the proposed rule further standardizes the alternate cabin safety and emergency exit requirements of part 23 commuter category airplanes with that of part 25 small transport airplanes. The alternative downward force was similarly deleted from part 25. This proposal is adopted with the aforementioned change.

来源：Final Rule｜59 FR 25766｜FinalRule_Docket 26324_Amdt23-46_59FR25766_1994-05-17_0002.txt

▪ 背景与评论处置（2）：美国联邦

英文原文：

The FAA agrees that clarification of Sec. 23.813(b)(4) and Sec. 23.813(b)(5) is needed. Accordingly, after the semicolon following "compartments" in Sec. 23.813(b)(4), the word "and" will be deleted. The remaining part of the paragraph will read, "unless the door has a means to latch it in the open position. The latching means must be able to withstand the loads imposed upon it by the door when the door is subjected to the inertia loads resulting from the ultimate static load factors prescribed in Sec. 23.561(b)(2)". Proposed Sec. 23.813(b)(5) is adopted with the change noted above.

修订指令英文原文（供核对）：

2. Sec. 23.561 is amended by adding a new paragraph (b)(2)(iv) to read as follows:

2. Section 23.561 is amended by adding a new paragraph (b)(2)(iv) to read as follows:

▪ 本版重印条文段落：(b)、(2)、(iv)

### 2.6　Amdt 23-48（1996-03-11 生效）：第 23.561 条通过修订段 (b) 的引导文本；修订段 (d)(

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：24. 第 23.561 条通过修订段 (b) 的引导文本；修订段 (d)(1)；并新增段 (e) 进行修订，内容如下：　24. 第 23.561 条通过修订 (b) 段的引言；修订 (d)(1)(i) 至 (d)(1)(iv) 段；删除 (d)(1)(v) 段；并增加新的 (e) 段予以修订，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局

英文原文：

The FAA proposed to amend Sec. 23.561 by revising paragraphs (b) and (d), and adding a new paragraph (e). These changes simplify, clarify, and "add references * * * to ensure". The FAA proposed to revise paragraph (b), concerning occupant protection, to make it correspond to 14 CFR part 25 and JAR 25 that cover large airplanes. The proposed revision of paragraph (d), concerning turnovers would simplify and clarify the requirements without making substantive changes. The FAA proposed a new paragraph (e) to ensure that items of mass that could injure an occupant are retained by the supporting structure.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：本提案

英文原文：

This proposal would amend Secs. 23.561 (b), (d), and (e) by revising the existing requirements to harmonize with JAR 23. Revised paragraph (b), concerning occupant protection, proposes language similar to part 25/JAR 25. Paragraph (d), concerning turnovers, would be revised to simplify and clarify the requirements without making substantive changes. Proposed new paragraph (e), concerning supporting structure, would be revised to add references to Sec. 23.561 (b)(3) and Sec. 23.787 (c) to ensure that items of mass are retained to higher accelerations than the occupant for occupant protection.

修订指令英文原文（供核对）：

24. Section 23.561 is amended by revising paragraph (b) introductory text; by revising paragraphs (d)(1); and by adding a new paragraph (e) to read as follows:

24. Section 23.561 is amended by revising the introductory text of paragraph (b); by revising paragraphs (d)(1)(i) through (d)(1)(iv); by removing paragraph (d)(1)(v); and by adding a new paragraph (e) to read as follows:

▪ 本版重印条文段落：(b)、(d)、(1)、(i)、(ii)、(iii)、(iv)、(e)

### 2.7　Amdt 23-62（2012-01-31 生效）：条文未改动（本版为交叉引用或其他条款修订所致）

▪ 修订沿革：本版由 NPRM　—　提出；最终规章 —，2012-01-31 生效。

▪ 本版对条文的处置：条文未改动（本版为交叉引用或其他条款修订所致）

### 2.8　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.561 的实际改动 |
| --- | --- | --- | --- |
| 52 FR 34745 | 1987-09-14 | 更正通告 (CFR 溯源注) | 18 项印刷更正；其中第 9 项新增修订指令 25-1：§23.561(b)(2) 表格第一栏标题 'Normal and utility categories'→'Normal, utility, and commuter categories'。⚠ 本件 FR 抬头把 Amdt Nos. 误印为 23-24（正文通篇写 23-34），CFR 溯源注因此跟着写错 —— DRS 归 Amdt 23-34 是对的 ¦ CFR Correction——删除重复的第二组 (d)(1)(i)~(iv) 及 (d)(1)(v) ¦ CFR Correction（第二次，修正上一条表述）——删除第二段起的五段 |
| 72 FR 59939 | 2007-10-23 | CFR 编纂校正 (OFR CFR Correction) | 18 项印刷更正；其中第 9 项新增修订指令 25-1：§23.561(b)(2) 表格第一栏标题 'Normal and utility categories'→'Normal, utility, and commuter categories'。⚠ 本件 FR 抬头把 Amdt Nos. 误印为 23-24（正文通篇写 23-34），CFR 溯源注因此跟着写错 —— DRS 归 Amdt 23-34 是对的 ¦ CFR Correction——删除重复的第二组 (d)(1)(i)~(iv) 及 (d)(1)(v) ¦ CFR Correction（第二次，修正上一条表述）——删除第二段起的五段 |
| 72 FR 72915 | 2007-12-26 | CFR 编纂校正 (OFR CFR Correction) | 18 项印刷更正；其中第 9 项新增修订指令 25-1：§23.561(b)(2) 表格第一栏标题 'Normal and utility categories'→'Normal, utility, and commuter categories'。⚠ 本件 FR 抬头把 Amdt Nos. 误印为 23-24（正文通篇写 23-34），CFR 溯源注因此跟着写错 —— DRS 归 Amdt 23-34 是对的 ¦ CFR Correction——删除重复的第二组 (d)(1)(i)~(iv) 及 (d)(1)(v) ¦ CFR Correction（第二次，修正上一条表述）——删除第二段起的五段 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.561 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The airplane, although it may be damaged in emergency landing conditions, must be designed as prescribed in this section to protect each occupant under those conditions. | 新增 |
| (b) | The structure must be designed to give each occupant every reasonable chance of escaping serious injury in a minor crash landing when-- | 新增 |
| (1) | Proper use is made of belts or harnesses provided for in the design; and | 新增 |
| (2) | The occupant experiences the ultimate inertia forces shown in the following table: Ultimate Inertia Forces Normal and utility categories Acrobatic category Upward---------------------------- 3.0g 4.5g Forward---------------------------- 9.0g 9.0g Sideward-------------------------- 1.5g 1.5g | 新增 |
| (c) | Each airplane with retractable landing gear must be designed to protect each occupant in a landing-- | 新增 |
| (1) | With the wheels retracted; | 新增 |
| (2) | With moderate descent velocity; and | 新增 |
| (3) | Assuming-- | 新增 |
| (i) | An upward ultimate inertia force of 3 g; and | 新增 |
| (ii) | A coefficient of friction of 0.5 at the ground. | 新增 |
| (d) | If a turnover is reasonably probable, the structure must be designed to protect the occupants in a complete turnover, assuming-- | 新增 |
| (1) | An upward ultimate inertia force of 3 g; and | 新增 |
| (2) | A coefficient of friction of 0.5 at the ground. | 新增 |
| (e) | Except as provided in Sec. 23.787 the supporting structure must be designed to restrain, under loads up to those specified in paragraph (b)(2) of this section, each item of mass that could injure an occupant if it came loose in a minor crash landing. Fatigue Evaluation | 新增 |

### 3.2　版本2：Amdt 23-7（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

修订：(c)、(d)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | * * * | 修订 |
| (3) | Assuming, in the absence of a more rational analysis - | 修订 |
| (i) | A downward ultimate inertia force of 3g; and | 修订 |
| (ii) | A coefficient of friction of 0.5 at the ground. | —（继承上版） |
| (d) | If a turnover is reasonably probable, the structure must be designed to protect the occupants in a complete turnover, assuming, in the absence of a more rational analysis - | 修订 |

### 3.3　版本3：Amdt 23-36（1988-09-14 生效）　来源：Docket 25147，1988-08-08 签发，53 FR 30802

修订段落：(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | The structure must be designed to protect each occupant during emergency landing conditions when: | 新增 |
| (1) | Proper use is made of seats, safety belts, and shoulder harnesses provided for in the design; | 新增 |
| (2) | The occupant experiences the static inertia loads corresponding to the following ultimate load factors-- | 新增 |
| (i) | Upward, 3.0g for normal, utility, and commuter category airplanes, or 4.5g for acrobatic category airplanes; | 修订 |
| (ii) | Forward, 9.0g; | 修订 |
| (iii) | Sideward, 1.5g; and | 新增 |
| (3) | The items of mass within the cabin, that could injure an occupant, experience the static inertia loads corresponding to the following ultimate load factors-- | 修订 |
| (i) | Upward, 3.0g; | 修订 |
| (ii) | Forward, 18.0g; and | 修订 |
| (iii) | Sideward, 4.5g | 新增 |
| (d) | If it is not established that a turnover is unlikely during an emergency landing, the structure must be designed to protect the occupants in a complete turnover as follows: | 修订 |
| (1) | The likelihood of a turnover may be shown by an analysis assuming the following conditions-- | 新增 |
| (i) | Maximum weight; | 修订 |
| (ii) | Most forward center of gravity position; | 修订 |
| (iii) | Longitudinal load factor of 9.0g; | 新增 |
| (iv) | Vertical load factor of 1.0g; and | 新增 |
| (v) | For airplanes with tricycle landing gear, the nose wheel strut failed with the nose contacting the ground. | 新增 |
| (2) | For determining the loads to be applied to the inverted airplane after a turnover, an upward ultimate inertia load factor of 3.0g and a coefficient of friction with the ground of 0.5 must be used. | 新增 |

### 3.4　版本4：Amdt 23-46（1994-06-16 生效）　来源：Docket 26324，1994-05-11 签发，59 FR 25766

新增段落：(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | * * * | 修订 |
| (2) | * * * | 修订 |
| (iv) | Downward, 6.0g when certification to the emergency exit provisions of Sec. 23.807(d)(4) is requested; and | 修订 |

### 3.5　版本5：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

新增段落：(b)、(d)、(e)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | The structure must be designed to give each occupant every reasonable chance of escaping serious injury when-- | 修订 |
| (d) | * * * | 新增 |
| (1) | * * * | 新增 |
| (i) | The most adverse combination of weight and center of gravity position; | 新增 |
| (ii) | Longitudinal load factor of 9.0g; | 新增 |
| (iii) | Vertical load factor of 1.0g; and | 新增 |
| (iv) | For airplanes with tricycle landing gear, the nose wheel strut failed with the nose contacting the ground. | 修订 |
| (e) | Except as provided in Sec. 23.787(c), the supporting structure must be designed to restrain, under loads up to those specified in paragraph (b)(3) of this section, each item of mass that could injure an occupant if it came loose in a minor crash landing. | 新增 |

### 3.6　段落结构演变矩阵（横向对照 7 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-7 (1969-09-14) | Amdt 23-36 (1988-09-14) | Amdt 23-46 (1994-06-16) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- | --- |
| (a) | ● 新增 | ○ 无 | ○ 无 | ○ 无 | ○ 无 |
| (b) | ● 新增 | ○ 无 | ● 新增 | △ 修订 | △ 修订 |
| (1) | ● 新增 | ○ 无 | ● 新增 | ○ 无 | ● 新增 |
| (2) | ● 新增 | ○ 无 | ● 新增 | △ 修订 | ○ 无 |
| (c) | ● 新增 | △ 修订 | ○ 无 | ○ 无 | ○ 无 |
| (3) | ● 新增 | △ 修订 | △ 修订 | ○ 无 | ○ 无 |
| (i) | ● 新增 | △ 修订 | △ 修订 | ○ 无 | ● 新增 |
| (ii) | ● 新增 | ● 有 | △ 修订 | ○ 无 | ● 新增 |
| (d) | ● 新增 | △ 修订 | △ 修订 | ○ 无 | ● 新增 |
| (e) | ● 新增 | ○ 无 | ○ 无 | ○ 无 | ● 新增 |
| (iii) | ○ 无 | ○ 无 | ● 新增 | ○ 无 | ● 新增 |
| (iv) | ○ 无 | ○ 无 | ● 新增 | △ 修订 | △ 修订 |
| (v) | ○ 无 | ○ 无 | ● 新增 | ○ 无 | ○ 无 |

## 4　§23.561 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | e commenter cites the National Transportation Safety Board (NTSB) 1981 report entitled "Cabin Safety in Transport Category Aircraft" as the source for this information. In response to this comment, the purpose of the pro… | Amdt 23-34 |
| 事故调查与研究驱动 | onference. One commenter proposes replacing the term "minor crash landing" in proposed Section 23.561(b) with the term "emergency landing conditions" to add consistency. The same commenter suggests replacing the words "e… | Amdt 23-36 |
| 澄清性与编辑性修订 | adopted with the aforementioned change. The FAA agrees that clarification of Sec. 23.813(b)(4) and Sec. 23.813(b)(5) is needed. Accordingly, after the semicolon following "compartments" in Sec. 23.813(b)(4), the word "an… | Amdt 23-46 |
| 事故调查与研究驱动 | s that could injure an occupant if it came loose in a minor crash landing. 24. Section 23.561 is amended by revising the introductory text of paragraph (b); by revising paragraphs (d)(1)(i) through (d)(1)(iv); by removin… | Amdt 23-48 |

本条取证提示 共 7 个历史版本：初始版, Amdt 23-7, Amdt 23-34, Amdt 23-36, Amdt 23-46, Amdt 23-48, Amdt 23-62。 其中 4 个版本在本地文献中检索到明确修订指令原文；5 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：52 FR 34745、72 FR 59939、72 FR 72915，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [4] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [5] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [6] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [7] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [8] | Final Rule（颁行） | Docket 25147 | 53 FR 30802 | FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt |
| [9] | NPRM（提案） | Notice 86-19 / Docket 25147 | 51 FR 44878 | NPRM_Notice 86-19_Docket25147_51FR44878_1986-12-12.txt |
| [10] | Final Rule（颁行） | Docket 26324 | 59 FR 25766 | FinalRule_Docket 26324_Amdt23-46_59FR25766_1994-05-17_0002.txt |
| [11] | NPRM（提案） | Notice 90-20 / Docket 26324 | 55 FR 35544 | NPRM_Notice 90-20_Docket26324_55FR35544_1990-08-30.txt |
| [12] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [13] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [14] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [15] | 更正通告 / 校正 | 52 FR 34745 | 1987-09-14 | Correction_52FR34745_1987-09-14.clean.txt |
| [16] | 更正通告 / 校正 | 72 FR 59939 | 2007-10-23 | CFRCorrection_72FR59939_2007-10-23_07-55519.clean.txt |
| [17] | 更正通告 / 校正 | 72 FR 72915 | 2007-12-26 | CFRCorrection_72FR72915_2007-12-26_07-55522.clean.txt |
| [18] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [19] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.561 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | c1b8f6a9-56f2-41c5-8f4e-5847a836a3ee |
| Amdt 23-7 | 1969-09-14 | 70523e6c-650a-4940-95d9-d0c5777c0300 |
| Amdt 23-34 | 1987-02-17 | 564de8fe-3467-4abb-b430-266d540577e0 |
| Amdt 23-36 | 1988-09-14 | 024246fb-52f0-4063-a39a-19e8642ee44d |
| Amdt 23-46 | 1994-06-16 | 1d488d69-dc82-4181-9237-96857e36471c |
| Amdt 23-48 | 1996-03-11 | 4e7946eb-090b-4845-81db-697548c2aa83 |
| Amdt 23-62 | 2012-01-31 | e3bb01c4-2726-4f09-bf08-7af03e7fae95 |

# § 23.562　应急着陆动力情况（(Emergency landing dynamic conditions.)）

1988-09-14 初版　｜　2012-01-31 Amdt 23-62　｜　共 4 个历史版本

## 1　§23.562 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.562 |
| 条款标题（英文原文） | (Emergency landing dynamic conditions.) |
| 中文名称 | 应急着陆动力情况 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 4 版（含 1988-09-14 初始版） |
| 时间跨度 | 1988-09-14 → 2012-01-31 （约 24 年） |
| 初版来源 | Docket 25147，1988-08-08 签发，53 FR 30802 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.562 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-36 | 1988-09-14 | Notice 86-19，1986-11-28 签发，51 FR 44878 | Docket 25147，1988-08-08 签发，53 FR 30802 | 新增条款：(b) |
| 2 | Amdt 23-44 | 1993-08-18 | Notice 91-12，1990-04-02 签发，56 FR 22070 | Docket 23746，1993-07-07 签发，58 FR 38634 | 新增段落：(b)、(d)、(e) |
| 3 | Amdt 23-50 | 1996-03-11 | Notice 94-22，1994-07-19 签发，59 FR 37878 | Docket 27807，1996-01-29 签发，61 FR 5171 | 修订 |
| 4 | Amdt 23-62 | 2012-01-31 | — | — | 条文未改动（本版为交叉引用或其他条款修订所致） |

### 1.3　修订脉络一览

1988 初版　新增条款：(b)　→　1993 23-44　新增段落：(b)、(d)、(e)　→　1996 23-50　修订　→　2012 23-62　条文未改动（本版为交叉引用或其他条款修订所致

## 2　§23.562 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1988-09-14，Amdt 23-36）：应急着陆动力情况

▪ 修订沿革：本版由 NPRM　Notice 86-19，1986-11-28 签发，51 FR 44878　提出；最终规章 Docket 25147，1988-08-08 签发，53 FR 30802，1988-09-14 生效。

▪ 修订指令（中文译文）：4. 新增 §23.562，内容如下：　1-3. 增加新的第 23.562 条，内容如下：

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（1）：在审查了在会议上收到的提案和公众意见后，美国联邦航空局（FAA）的首个相关规章制定行动集中于更新与座舱安全和改进耐坠撞性相关的安全标准。1986 年 12 月 12 日，美国联邦航空局（FAA）在《联邦公报》（51 FR 44878）上发布了题为“小型飞机适航审查计划第 1 号通告”的第 86-19 号通告。第 86-19 号通告提议对第 23.561、23.783、23.785、23.787 和 23.807 条进行修订，并包含新的第 23.562 条，该条提议了针对小型飞机座椅/约束系统的动力试验和评估要求。

英文原文：

After reviewing the proposals and public comments received at the conference, the FAA's first related rulemaking action concentrated on updating safety standards related to cabin safety and improved crashworthiness. On December 12, 1986, the FAA published Notice No. 86-19, titled "Small Airplane Airworthiness Review Program Notice No. 1" in the FEDERAL REGISTER (51 FR 44878). Notice No. 86-19 proposed changes to Sections 23.561, 23.783, 23.785, 23.787, and 23.807, and included a new Section 23.562, which proposed dynamic testing and evaluation requirements for the seat/restraint systems of small airplanes.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（2）：提案 1-2。本提案将重新组织第 23.561 条，引用新的第 23.562 条，增加正常类、实用类和特技类飞机中质量块固定的强度要求，并以客观术语表述翻转要求，其中包括在确定应急着陆情况下发生翻转的可能性时应考虑的变量。本提案是根据小型飞机适航审查会议上提出的第 218、219、220、221、222 和 518 号提案作出的。

英文原文：

Proposal 1-2. This proposal would reorganize Section 23.561, make reference to a new Section 23.562, add strength requirements for the retention of items of mass in normal, utility, and acrobatic category airplanes, and state turnover requirements in objective terms which include the variables to be considered in determining the likelihood of a turnover during emergency landing conditions. This proposal was made as a result of proposals 218, 219, 220, 221, 222, and 518 presented at the Small Airplane Airworthiness Review Conference.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（3）：另一位评论者同意质量块的 4.5g 侧向载荷因数，但不同意拟议的 18g 向前载荷因数。该评论者表示，经验表明现有的 9g 向前载荷要求已产生足够的静力强度。美国联邦航空局（FAA）认为 9g 载荷因数不足以满足第 86-19 号通告中拟议的座舱安全水平。在美国联邦航空局（FAA）/美国国家航空航天局（NASA）联合项目中进行的实际坠撞演示试验提供的数据表明，在模拟的可生存坠撞期间，纵轴上的撞击载荷可能超过 9g。日期为 1982 年 11 月、题为“撞击时结构飞机坠撞数据
正如GASP所提议，拟议的静载荷因数的目的在于，考虑到本最终规章其他条所增强的座舱安全性——包括第23.562条中关于座椅/约束系统动力试验的新要求——确保座舱内质量块的连接得到充分约束。该评论者建议修改§23.561(b)(3)段，要求设备及其他大质量块的安放位置应做到，若其松脱，在应急着陆后不太可能导致乘员受伤、刺穿燃油管路或堵塞出口。美国联邦航空局不同意。对许多小型飞机设计而言，将所有具有显著质量的物品（特别是货物）置于在撞击中能确保不会造成乘员受伤或燃油系统被穿透的座舱位置这一构想并不实际。本最终规章中，为质量块设定较高的载荷因数是必要的，以确保在§23.562(b)条中座椅/约束系统的动力载荷情况下，座舱内质量块的连接强度足够。
较高的静力载荷因数预计将防止在应急着陆地面撞击期间因质量块松脱而可能导致的伤害。

英文原文：

Another commenter agrees with the 4.5g sideward load factor for items of mass, but disagrees with the proposed 18g forward load factor. The commenter states that experience indicates that the existing 9g forward load requirement has resulted in adequate static strength. The FAA does not consider the 9g load factor sufficient for the level of cabin safety proposed in Notice No. 86-19. Actual crash demonstration tests conducted in a joint FAA/NASA project provided data which showed that impact loads during simulated survivable crashes could exceed 9g in the longitudinal axis. NASA Technical Paper 2083, dated November 1982, titled, "Correlation and Assessment of Structural Airplane Crash Data With Flight Parameters at Impact" is one of several reports which evaluate the results of the FAA/NASA small airplane crash test project. As proposed by the GASP, the purpose of the proposed static load factors is to ensure that the attachments of items of mass in the cabin are adequately restrained considering the enhanced cabin safety provided by other sections of this final rule, including the new requirements in Section 23.562 for dynamic tests of seat/restraint systems. This commenter proposes changing Section 23.561(b)(3) to require that equipment and other large mass items be positioned so that, if they break loose, they are unlikely to cause injury to the occupants, penetrate fuel lines, or block exits after an emergency landing. The FAA disagrees. For many small airplane designs, the concept of locating all items of significant mass, particularly cargo, at a location in the cabin which will ensure no occupant injury or fuel system penetration during impact is impractical. The higher load factors for items of mass are necessary in this final rule to ensure that the attachment strength for mass items in the cabin is adequate with the seat/restraint system dynamic load conditions of Section 23.562(b). The higher static load factors are expected to prevent injuries which could result from mass items coming loose during an emergency landing ground impact.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（4）：要求是将翻转情况作为静力情况而非动力情况进行评定。美国联邦航空局（FAA）同意这一解释，美国联邦航空局（FAA）的意图并非将翻转防护作为动力情况进行评定，例如第 23.562 条的座椅/约束系统动力试验要求。该评论者还得出结论认为，使用拟议的第 23.561(c) 条中确定翻转可能性的拟议准则，几乎所有前三点式起落架

英文原文：

requirement is to evaluate the turnover case as a static rather than a dynamic condition. The FAA agrees with this interpretation, and it is not the intent of the FAA for turnover protection to be evaluated as a dynamic condition such as the seat/restraint system dynamic tests requirements of Section 23.562. This commenter also concludes that, using the proposed criteria for establishing the likelihood of a turnover in proposed Section 23.561(c), almost all tricycle landing gear

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（5）：提案 1-3。拟议的第 23.562 条"应急着陆动力情况"将增加适用于正常类、实用类和特技类飞机所用座椅/约束系统的动力试验要求，并包含具体的试验参数值和通过/失效准则。就这些要求而言，座椅/约束系统包括座椅及其与座椅滑轨或机身的直接连接件、安全带和肩带，以及用于将其连接到座椅或机身的任何接头或组件。该提案是 GASP 向小型飞机适航审查会议提交的成果（提案 518），基于最新技术水平，并得到出席会议的通用航空界的共识支持。
§23.562 条中的具体动力试验条件得到了几项研究工作的支持，其中包括：（1）由美国国家航空航天局（NASA）进行的一系列全尺寸坠撞冲击“落震试验”，（2）来自美国国家运输安全委员会（NTSB）的事故评估信息，（3）由美国联邦航空局（FAA）民用航空医学研究所（CAMI）进行的动力试验，以及（4）由军方及其他来源开发的公认的人体冲击损伤准则。

英文原文：

Proposal 1-3. Proposed Section 23.562, Emergency landing dynamic conditions, would add dynamic test requirements to be applied to seat/restraint systems used in normal, utility, and acrobatic category airplanes, and contain specific test parameter values and pass/fail criteria. For the purpose of these requirements, the seat/restraint system includes the seat and its immediate attachments to seat rails or the airframe, the safety belt and shoulder harness, and any fittings or assemblies used to attach them to the seat or airframe. The proposal was the result of a GASP submittal to the Small Airplane Airworthiness Review Conference (proposal 518), was based on state-of-the-art technology, and was supported by the consensus of the general aviation community represented at the conference.

The specific dynamic test conditions in Section 23.562 are supported by several research studies including: (1) A series of full-scale crash impact "drop tests" conducted by the National Aeronautics and Space Administration (NASA), (2) accident evaluation information from the National Transportation Safety Board (NTSB), (3) dynamic tests conducted by the FAA Civil Aeromedical Institute (CAMI), and (4) generally accepted human impact injury criteria developed by the military and other sources.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（6）：一位评论者建议将拟议的第23.562(b)(2)条和拟议的第23.562(c)(6)条中使用的术语“上部躯干约束系统”改为“肩部束缚装置”。由于这一评论，美国联邦航空局审查了第86-19号通告拟议规章中使用的术语。在本修正案中，与乘员座椅或安全带约束相关的术语用于提高清晰度和标准化。这些术语包括肩部束缚装置（又称上部躯干约束），指躯干约束系统中包括一条或多条绕过肩部并约束乘员上部躯干的带子的部分；以及躯干约束系统，指用于提供躯干约束的系统，包括安全带、如配备的肩部束缚装置，以及用于将系统连接到座椅或机体结构的相关硬件。因此，该评论者提出的更改未被采纳。

英文原文：

One commenter suggests changing the term "upper torso restraint system" used in proposed Section 23.562(b)(2) and proposed Section 23.562(c)(6) to the words "shoulder harness". As a result of this comment, the FAA reviewed the terms used throughout the proposed rules of Notice No. 86-19. In this amendment, terms related to occupant seats or belt restraints, are used to improve clarity and standardization. These terms include the Shoulder Harness, also known as the upper torso restraint, which refers to that portion of the torso restraint system which includes one or more straps that pass over the shoulders and restrains the upper torso of the occupant and the Torso Restraint System, which refers to that system used for providing torso restraint which includes the safety belt, the shoulder harness if provided, and the associated hardware used to attach the system to the seat or airframe structure. Therefore, the change proposed by this commenter is not adopted.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（7）：一位评论者认为拟议的第23.562条的设计准则过于武断，因为每次坠撞情况各不相同。美国联邦航空局（FAA）不同意这一观点。本条中的乘员保护准则基于广泛的研究数据。这些准则旨在为小型飞机使用的座椅/约束系统所提供的乘员保护提供保守但合理的标准。该评论者提议采用一种简化的动力试验，该试验无需使用ATD及其他相关试验设备。评论者提议的试验程序将要求在座椅上安装仪器，以确保试验脉冲满足要求，并基于座椅不发生破坏的变形制定性能准则。美国联邦航空局不同意这种简化方法，因为该方法不能对拟议的第23.562(c)段所要求的全部乘员伤害防护特性进行评估。
美国联邦航空局（FAA）知道，没有简单的动力试验程序能在不使用 ATD 的情况下，在座椅/约束系统上提供类似人体的载荷分布。由于该意见未提供乘员伤害防护评估，也未提供技术数据以证明该简化程序能提供与通知 No. 86-19 中提出的等效安全水平，因此未采用该简化程序。

英文原文：

One commenter considers the design criteria of proposed Section 23.562 to be arbitrary since each crash situation is different. The FAA disagrees. The occupant protection criteria in this section are based on extensive research data. These criteria were intended to provide conservative, but reasonable, standards for the occupant protection provided by seat/restraint systems developed for use in small airplanes. This commenter proposes the use of a simplified dynamic test which would not require the use of an ATD and other associated test equipment. The test procedure proposed by the commenter would require instrumenting the seat to ensure that the test pulse meets the requirements and would require performance criteria based on seat deformation without failure. The FAA does not agree with the simplified approach because it does not provide for the evaluation of all the occupant injury protection features required in proposed Section 23.562(c). The FAA knows of no simple dynamic test procedure which will provide human-like load distributions on the seat/restraint system without using an ATD. Since the comment does not provide for occupant injury protection evaluation and does not provide technical data to show that the simplified procedure would provide an equivalent level of safety to that proposed in Notice No. 86-19, the simplified procedure is not adopted.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（8）：该评论超出了该规章制定提案通告的范围。将在动力试验期间在刚性试验夹具上测得的载荷作为静载荷施加于刚性较小的机体结构是不切实际的。在应急着陆撞击期间，飞机地板结构通常会在严重的局部载荷下屈服，这往往会改变座椅/约束系统与机体结构交界面处的连接载荷。在拟议的第 23.562 条的动力试验中纳入包含机体结构的程序将大大增加这些试验的复杂性和成本。支撑座椅/约束系统的机体结构必须满足第 23.561 条及第 23 部其他条款的静强度要求。因此，本最终规章不要求测量连接载荷，尽管此类测量可能提供有用的设计信息。

英文原文：

This comment goes beyond the scope of the notice. It is unrealistic to apply loads, measured on a rigid test fixture during dynamic tests, as static loads to less rigid airframe structure. During an emergency landing impact, the airplane floor structure will typically yield under severe local loads, and this will tend to alter the attachment loads at the seat/restraint system interface to the airframe structure. Incorporating procedures to include airframe structure in the dynamic tests of proposed Section 23.562 would greatly increase the complexity and costs of these tests. Airframe structure supporting the seat/restraint system must meet the static strength requirements of Section 23.561 and other sections of Part 23. Therefore, this final rule does not require measurement of attachment loads, although such measurements may provide useful design information.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（9）：将本修订作为第23部型号取证基础的一部分。对于已认证飞机的改装，申请人必须遵守该飞机的原始型号取证基础；然而，美国联邦航空局认为，对改装方而言，证明尽可能最高的乘员保护水平符合其最佳利益。第23.562条中的动力试验程序提供了一种证明新座椅/约束系统设计能显著提高乘员保护水平的方法。

英文原文：

include this amendment to Part 23 as part of their type certification basis. For modifications of previously certificated airplanes, the applicant is required to comply with the original type certification basis for the airplane; however, the FAA considers it in the best interest of the modifier to demonstrate the highest level of occupant protection possible. The dynamic test procedures in Section 23.562 provide a method for demonstrating that new seat/restraint system designs offer a significant increase in occupant protection.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（10）：若干评论涉及拟议的

英文原文：

Several comments address specific technical aspects of the proposed Section 23.562. One commenter supports the test parameters of both the vertical and longitudinal tests as proposed and agrees that these proposed requirements will result in a substantial improvement in the dynamic strength capabilities of small airplane seats. This commenter also supports the performance criteria which limit the impact loads for the occupant's head and lumbar spine.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（11）：一位评论者建议将横滚轴方向的地板错位纳入拟议§23.562(b)(2)段的地板翘曲要求，并将这些地板翘曲规定加入§23.562(b)(1)段的要求。美国联邦航空局同意将横滚轴方向的地板变形预载纳入拟议§23.562(b)(2)段规定的纵向试验工况。该段规定了对座椅/约束系统乘员约束强度的试验。地板变形预载是该试验的一个重要方面，应急着陆过程中俯仰轴和横滚轴方向均可能发生地板翘曲。美国联邦航空局不同意将地板变形预载要求纳入拟议§23.562(b)(1)段的试验条件。该段规定的试验参数用于在可重复条件下评定座椅的吸能能力，以表明最低水平的乘员保护。
地板翘曲预载荷将增加该试验的复杂性，可能对座椅/约束系统能量吸收能力的评定产生不利影响。为增加清晰度，并确保地板翘曲被理解为一种预载荷情况、须在实施动力试验前设定，拟议§23.562(b)(2) 段中的地板变形要求被删除，扩展后的要求在本最终规章的§23.562(b)(3) 段中规定。

英文原文：

One commenter suggests that floor misalignment in the roll axis be added to the floor warpage requirements of proposed Section 23.562(b)(2) and that these floor warpage provisions be added to the requirements of Section 23.562(b)(1). The FAA agrees with adding floor deformation preload in the roll axis to the longitudinal test conditions prescribed in proposed Section 23.562(b)(2). This paragraph provides for a test of the occupant retention strength of the seat/restraint system. Floor deformation preloading is an important aspect of this test, and floor warp in both the pitch and roll axes is possible during an emergency landing. The FAA disagrees with adding floor deformation preload requirements to the test conditions of proposed Section 23.562(b)(1). This paragraph provides for test parameters intended to evaluate the energy absorbing capability of the seat under repeatable conditions to demonstrate a minimum level of occupant protection. Floor warp preloads will add complexities to this test which may adversely affect the evaluation of the energy absorbing capability of the seat/restraint system. To add clarity and ensure that floor warpage is understood to be a preload condition, to be set prior to conducting the dynamic test, the floor deformation requirements in proposed Section 23.562(b)(2) are removed, and the expanded requirements are set forth in Section 23.562(b)(3) of this final rule.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（12）：两位评论者建议，§23.562(b)段中的要求应允许使用经过适当验证的分析。拟议的§23.562(d)段将允许申请人使用替代符合性方法满足该规则，前提是该替代方法所产生的安全水平与规则所确立的安全水平相同或更高。除为清晰起见删除了“（c）段的”字样外，拟议的§23.562(d)段已保留在本最终规章中。因此，美国联邦航空局认为，评论者的意图已通过§23.562(d)段得到实现。

英文原文：

Two commenters suggest that the requirements in Section 23.562(b) should allow the use of a properly validated analysis. Proposed Section 23.562(d) would allow the applicant to use an alternate means of compliance to this rule, provided that the alternate method results in a level of safety as great as, or greater than, that established by the rule. Proposed Section 23.562(d) has been retained in this final rule except the words "paragraph (c) of" have been removed for clarity. Therefore, the FAA considers the intent of the comments to be achieved by Section 23.562(d).

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（13）：一位评论者担心，49 CFR 第 572 部 B 子部分所规定的特定 ATD 并非总能获得。所规定的 ATD 多年来一直是汽车行业使用的标准。由于动力试验的结果可能因所用特定 ATD 的不同而受到显著影响，因此美国联邦航空局认为有必要规定一种标准 ATD，用以表明符合 §23.562 的要求。然而，目前已有若干其他 ATD 设计被制造出来，美国联邦航空局正在监测新 ATD 设计的发展。如果申请人希望使用不同的 ATD 构型来表明符合 §23.562 的要求，则必须表明使用替代 ATD 不会显著影响试验结果，并在开展取证试验之前获得美国联邦航空局对使用替代 ATD 设计的批准。

英文原文：

One commenter is concerned that the specific ATD defined by 49 CFR Part 572, Subpart B, will not always be available. The specified ATD has been the standard used by the automotive industry for many years. Because the results of dynamic tests can be significantly affected by the specific ATD used, the FAA considers it necessary to specify a standard ATD for showing compliance to the requirements of Section 23.562. However, several other ATD designs have been manufactured, and the FAA is monitoring the development of new ATD designs. If an applicant desires to use a different ATD configuration for showing compliance with Section 23.562, the applicant must show that use of the substitute ATD will not significantly affect the test results and obtain FAA approval to use an alternate ATD design prior to conducting certification tests.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（14）：收到几条编辑性意见。一位评论者建议该规章要求测试假人处于正常直立坐姿。该评论者还建议在拟议的§23.562(c)(5)段方程式中增加时间和加速度单位。为清晰起见，这些意见已纳入本最终规章。另一位评论者建议在拟议的§23.562(b)(2)段最后一句中"the"和"seating devices"之间增加"seats or"一词。考虑到该意见，并为与其他涉及乘员座椅系统的规章更加一致，本最终规章将§23.562拟议第(1)和(2)段中的"seat, or other seating device"（座椅或其他座椅装置）替换为"seat/restraint system"（座椅/约束系统）。此外，本最终规章通篇使用"seat/restraint system"一词，用以指代在应急着陆情况下用于提供支撑就座乘员并约束乘员的主要手段的任何系统。
一位评论者建议将拟议的§23.562(b)(3)段中的“如果输入脉冲的形状显著偏离对称三角形”改为“以近似对称三角形”。评论者认为“显著偏离”一词过于模糊，无法界定符合性的外部界限。冲击脉冲情况的具体标准基于这样一个概念，即在理想情况下，该脉冲的形状将接近对称三角形，这一概念得到了美国国家航空航天局（NASA）对全尺寸小型飞机坠毁试验数据分析的支持；然而，大多数冲击试验设施将难以产生非对称三角脉冲。尽管美国联邦航空局（FAA）认为对称三角脉冲是最优的，但只要有必要的试验速度，并在规定的时间限制内达到必要的最小减速度水平，许多现有试验设施可产生的冲击脉冲也能满足所需动力试验的目标。因此，本最终规章未规定三角脉冲。
拟议的第23.562条(b)段中的第(3)段被删除，并且如前所述，由一个新的第(3)段取代，该新段包含经澄清的地板翘曲要求。

英文原文：

Several editorial comments were received. One commenter suggests the rule require the test dummy to be seated in the normal upright position. This commenter also suggests that units of time and acceleration be added to the equation in proposed Section 23.562(c)(5). These comments are incorporated in this final rule for clarity. One commenter suggests that the words "seats or" be added between the words "the" and "seating devices" in the last sentence of proposed Section 23.562(b)(2). In consideration of this comment and for more consistency with other rules regarding the occupant seating systems, the words "seat, or other seating device" in proposed paragraphs (1) and (2) of Section 23.562 are replaced with the words "seat/restraint system" in this final rule. In addition, the term "seat/restraint system" is used throughout this final rule to refer to any system used to provide the primary means to support a seated occupant and restrain the occupant during emergency landing conditions. One commenter suggests replacing the phrase "if the shape of the input pulse deviates significantly from a symmetric triangle" in proposed Section 23.562(b)(3), with the phrase "in order to approximate a symmetric triangle." The commenter considers the words "deviates significantly" to be too nebulous to define the outer limits of compliance. The specific criteria of the impact pulse condition is based on the concept that in the ideal case, the shape of this pulse would approach that of a symmetric triangle, which was supported by NASA analysis of the full-scale small airplane crash test data; however, most impact test facilities will have difficulty producing asymmetric triangular pulse. Although the FAA considers the symmetric triangular pulse to be the optimum, the objective of the required dynamic tests can be met by impact pulses producible in many available test facilities, provided the impact pulse has the necessary test velocity, and reaches the necessary minimum deceleration level within a specified time limit. Therefore, this final rule does not specify a triangular pulse. The proposed paragraph (3) of Section 23.562(b) is omitted and, as previously noted, is replaced by a new paragraph (3) which contains clarified floor warpage requirements.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（15）：一名评论者建议对建议的 §23.785(c) 条进行修改，以便对前向或后向座椅以外的座椅朝向进行规定。为了在本最终规章中更加清晰，建议的 §23.785(c) 条被拆分为四个段——即 (b) 段、(d) 段、(e) 段和 (

英文原文：

One commenter suggests changes to proposed Section 23.785(c) so that seat orientations other than forward-facing or aft-facing seats are addressed. Proposed Section 23.785(c), for clarity in this final rule, is separated into four paragraphs - paragraphs (b), (d), (e), and (g). The new paragraph (b) references requirements of Section 23.562 for normal, utility, and acrobatic category airplane seat/restraint systems and includes the existing requirements for Section 23.785(g)(1) relative to seat orientations other than side-facing and aft-facing seats. As no other comments were received relative to proposed Section 23.785(c), the requirements proposed for a single-point release, the need for a crewmember to perform all functions necessary to flight operations with the restraint system fastened, and the means to secure the safety belt and shoulder harness when not in use, are in paragraphs (d), (e), and (g) of this final rule, respectively.

来源：Final Rule｜53 FR 30802｜FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt

▪ 背景与评论处置（16）：提案1-6。第86-19号通告提议修订第23.787条，以提高货舱的静强度要求，该提案源自小型飞机适航性评审会议的提案288、289、518和524。会议达成共识，货物约束装置应按照第23.561条的极限静载荷因数进行设计，以在应急着陆情况下保护货舱前方乘员的安全。对于在同一舱室内搭载货物和乘员的设计，GASP提议这些舱室应具有足够的强度，以在可能导致乘员暴露于拟议第23.562(b)(2)段动力情况的条件下的情况下固定货物。由于座椅的动力试验评估了特定的乘员伤害标准，美国联邦航空局认为对质量块进行动力试验没有必要，因为只有固定这些物品才是安全问题。因此，第23.561(b)(3)段的极限静载荷因数在第23.787(e)段中被引用。

英文原文：

Proposal 1-6. Notice No. 86-19 proposed to amend Section 23.787 to increase the static strength requirements for cargo compartments and resulted from conference proposals 288, 289, 518, and 524 of the Small Airplane Airworthiness Review Conference. It was a consensus at the conference that cargo restraints should be designed to the ultimate static load factors of Section 23.561 to protect occupants forward of the cargo during emergency landing conditions. For designs that carry cargo in the same compartment with occupants, the GASP proposed that these compartments have adequate strength to retain cargo in conditions which could result in occupant exposure to the dynamic conditions of proposed Section 23.562(b)(2). Since dynamic testing of seats evaluates specific occupant injury criteria, the FAA considers dynamic testing of items of mass to be unnecessary because only retention of these items is a safety concern. Therefore, the ultimate static load factors of Section 23.561(b)(3) are referenced in Section 23.787(e).

来源：NPRM（提案）｜51 FR 44878｜NPRM_Notice 86-19_Docket25147_51FR44878_1986-12-12.txt

▪ 背景与评论处置（17）：在会议上经讨论的 GASP 提案规定，每一种座椅设计都必须成功完成动力试验。有人询问，该提案是否排除了为表明对各项要求的符合性而使用分析方法。会议达成的共识（美国联邦航空局（FAA）对此表示同意）是，应当修订 GASP 提案以解决这一关切。因此，美国联邦航空局的提案规定，如果申请人提出的替代方法能够达到所要求的同等或更高水平的乘员保护，并且该替代方法以合理的依据得到证实，则申请人可以偏离所规定的动力情况。然而，会议上有人表达了如下意见：目前尚无足够的数据库来允许以分析代替动力试验以表明对拟议要求的符合性，而美国联邦航空局赞同这一意见。
美国联邦航空局（FAA）认识到，在分析领域正在取得重大进展，以代替动力试验来证实设计，并且第 23.562(d) 条的措辞旨在当分析技术的水平充分发展到可代替这些动力试验予以应用时提供灵活性。

英文原文：

The GASP proposal, which was discussed at the conference, states that each design for the seat must successfully complete dynamic tests. It was asked whether the proposal excluded the use of an analysis for the showing of compliance with the requirements. It was the consensus at the conference, and the FAA agrees, that the GASP proposal should be revised to address this concern. Accordingly, the FAA proposal states that an applicant may depart from the dynamic conditions prescribed if the alternate approach proposed by the applicant achieves an equivalent, or greater, level of occupant protection as required, and the alternate is substantiated on a rationale basis. However, an opinion was expressed at the conference that there is not a sufficient data base at the present time to permit use of an analysis in lieu of dynamic testing to show compliance with the proposed requirements and the FAA concurs with this opinion. The FAA recognizes that considerable advancement is being made in the area of analysis to substantiate designs in lieu of dynamic testing and the language of Section 23.562(d) is intended to provide flexibility when the state of analytical techniques evolves sufficiently for application in lieu of these dynamic tests.

来源：NPRM（提案）｜51 FR 44878｜NPRM_Notice 86-19_Docket25147_51FR44878_1986-12-12.txt

▪ 背景与评论处置（18）：第 23.785(g) 条规定了构成座椅装置系统的各部件。拟议的这一定义是必要的，以避免如果不将各部件视作一个座椅装置系统、如第 23.562 条所拟议的那样在动力环境中正常发挥作用时可能产生的不利后果。

英文原文：

Section 23.785(g) defines the components which comprise the seating device system. This proposed definition is necessary to avoid the adverse consequences that may arise if the various components are not considered as a seating device system to function properly in the dynamic environment as proposed in Section 23.562.

来源：NPRM（提案）｜51 FR 44878｜NPRM_Notice 86-19_Docket25147_51FR44878_1986-12-12.txt

▪ 背景与评论处置（19）：GASP会议提案指出，座椅的每种设计都必须成功完成动力试验，并且有人提出疑问，该提案是否排除了使用分析来表明对要求的符合性。会议与会者达成共识，且美国联邦航空局（FAA）也表示同意，即应当对GASP提案进行修订以解决这一疑虑。因此，建议的 §23.562(d) 段解决了基于以前批准的、经过动力试验验证的设计进行合理分析的问题。然而，会议上有人提出意见认为，目前还没有足够的数据库来使用分析代替动力试验来表明对建议要求的符合性，美国联邦航空局（FAA）同意这一意见。尽管如此，在利用分析代替动力试验以证实设计这一领域正在取得相当大的进展。

英文原文：

The GASP conference proposal stated that each design for the seat must successfully complete dynamic tests and the question was asked if the proposal excluded the use of an analysis for the showing of compliance with the requirements. It was the consensus of the attendees at the conference, and the FAA agrees, that the GASP proposal should be revised to address this concern. Accordingly, proposed Section 23.562(d) addresses the issue of a rational analysis based upon previously approved dynamically tested designs. However, an opinion was expressed at the conference that at the present time, there is not a sufficient database to use an analysis in lieu of dynamic testing to show compliance with the proposed requirements and the FAA agrees with this opinion. However, considerable advancement is being made in the area of analysis to substantiate designs in lieu of dynamic tests.

修订指令英文原文（供核对）：

4. New Sec. 23.562 is added to read as follows:

1-3. By adding a new Section 23.562 to read as follows:

### 2.2　Amdt 23-44（1993-08-18 生效）：§23.562 条修订如下：修订 (b) 段引言文字的第一句

▪ 修订沿革：本版由 NPRM　Notice 91-12，1990-04-02 签发，56 FR 22070　提出；最终规章 Docket 23746，1993-07-07 签发，58 FR 38634，1993-08-18 生效。

▪ 修订指令（中文译文）：4. §23.562 条修订如下：修订 (b) 段引言文字的第一句，将 (d) 段重新编号为 (e) 段，并增加新的 (d) 段，内容如下：　4. 第 23.562 条通过修订 (b) 段的引言、将 (d) 段重新编号为 (e) 段，并增加新的 (d) 段进行修订，内容如下：

来源：Final Rule｜58 FR 38634｜FinalRule_Docket 23746_Amdt23-44_58FR38634_1993-07-19_0002.txt

▪ 背景与评论处置（1）：§23.67 条。本提案将澄清通过第 23-42 号修正案（56 FR 344, 1991 年 1 月 3 日）对 §23.67 条所作的修改。§23.67(b)(1) 段的规定要求，所有失速速度大于 61 节的活塞式发动机驱动多发飞机满足单发不工作爬升梯度要求。需要对 §23.67 条 (b)(1) 段和 (b)(2) 段进行修改，以澄清最大重量小于 6,000 磅、满足 §23.562(d) 段规定的改进的乘员保护要求且失速速度大于 61 节的多发飞机将仅符合 §23.67(b)(2)(i) 段的爬升梯度确定要求。本提案不改变单发

英文原文：

Section 23.67. This proposal would clarify the change made to Section 23.67 by amendment 23-42 (56 FR 344, January 3, 1991). The provisions of Section 23.67(b)(1) require that all reciprocating engine-powered multiengine airplanes with a stall speed of more than 61 knots meet the one-engine-inoperative climb gradient requirements. A change to Section 23.67, paragraphs (b)(1) and (b)(2), is required to clarify that multiengine airplanes of less than 6,000 pounds maximum weight that meet the improved occupant protection requirements prescribed in Section 23.562(d) and have a stall speed greater than 61 knots would comply only with the climb gradient determination requirements of Section 23.67(b)(2)(i). This proposal does not change the one-engine-inoperative climb requirements.

来源：Final Rule｜58 FR 38634｜FinalRule_Docket 23746_Amdt23-44_58FR38634_1993-07-19_0002.txt

▪ 背景与评论处置（2）：第 23.562 条。在制定第 23.562 条时使用的支持性技术数据取自其失速速度不大于 61 节的小型飞机。飞机乘员未暴露于增加水平的动能冲击能量。高于 61 节失速速度基线的动能冲击能量的增加，与飞机在着陆构型下的失速速度的平方成正比。
利用速度比的平方来得出提高的乘员保护要求，与美国陆军《飞机坠毁生存设计指南》（USARTL-TR-79-22C，第III卷——飞机结构坠撞适增性）中的分析方法相一致；该方法论述了伴随掘土现象的飞机撞击所涉及的动量守恒。美国联邦航空局（FAA）将座椅/约束系统动力试验的最大减速度限定为 32g，该值正是美国联邦航空局正在为通勤类飞机另行制定的一份规章制定提案通告（NPRM）中考虑提出的数值。当失速速度（VSO）达到 79 节时即达到 32g 这一限值。失速速度更高时，该最大减速度仍保持恒定的 32g。

英文原文：

Section 23.562. The supporting technical data used in the development of Section 23.562 was obtained from small airplanes whose stall speeds were not greater than 61 knots. Airplane occupants were not exposed to increased levels of kinetic impact energy. The increase in kinetic impact energy, above the 61 knot stall speed baseline, is proportional to the square of the stall speed of the airplane in the landing configuration. To compensate for increased energy levels, additional occupant protection requirements beyond those stated in Section 23.562 are included in this final rule. The emergency landing dynamic conditions express the impact energy level in terms of an impact velocity. The increased occupant protection requirement in this proposal is obtained by multiplying the ultimate load factors of Section 23.561(b) and the peak deceleration of the seat/restraint system test of Section 23.562(b)(1) by the square of the ratio of the increased stall speed to the stall speed of 61 knots. The use of the velocity ratio squared to obtain the increased occupant protection requirement is consistent with an analytical methodology found in the U.S. Army's Aircraft Crash Survival Design Guide, USARTL-TR-79-22C, Volume III---Aircraft Structural Crashworthiness, which addresses the conservation of momentum associated with an aircraft impact that has earth plowing.

The FAA is limiting the maximum deceleration for the seat/restraint system dynamic test to 32g, which is the value that the FAA is considering proposing in a separate NPRM being developed for commuter category airplanes. The 32g limitation will be reached at a stall speed (VSO) of 79 knots. At a higher stall speed, this maximum deceleration remains constant at 32g.

来源：Final Rule｜58 FR 38634｜FinalRule_Docket 23746_Amdt23-44_58FR38634_1993-07-19_0002.txt

▪ 背景与评论处置（3）：一名评论者指出，在过去 50 年里，重量小于 6,000 磅的飞机的耐撞性没有任何改善；因此，如果放宽失速速度要求，将导致乘员出现更多伤亡，因为乘员将吸收因速度增加而产生的额外能量。这是部分正确的。如果其他条件保持不变，失速速度的增加
此外，小型飞机失速速度研究组开展的研究分析了6年期间37,530份事故报告，其结果未能显示乘员生存性与着陆失速速度之间存在明确关联。该评论者补充说，飞机性能在过去50年中并未发生足够的变化，不足以证明拟议变更是合理的。该评论者以自身经验支持这一观点。该评论者随后指出，操作者错误仍然是航空事故的主要原因，并且由于航空器操作者将继续犯错，现有的失速速度要求应予以保留，从而保护操作者不伤害自身。该评论者正确指出，操作者错误是事故的主要原因。但是，操作者错误以及改进飞行员培训的需要并非飞机合格审定问题，并且超出了本规章制定的范围。

英文原文：

One commenter indicates that there has not been any improvement in crashworthiness for airplanes weighing less than 6,000 pounds during the last 50 years; therefore, if stall speed requirements are relaxed, more fatalities and injuries to occupants will result because the occupants will absorb the additional energy generated by the increased speed. This is partially correct. If other conditions are unchanged, an increase in stall speed will probably result in airframes and occupants absorbing more energy on impact. However, with the development and adoption of emergency landing dynamic conditions into Section 23.562 of the FAR by amendment 23-36, the current emergency landing dynamic conditions will be extended to those applicants who choose to design new airplanes with a stall speed greater than 61 knots. The extension of the current emergency landing dynamic requirements will provide crashworthiness standards that address load attenuation to the occupant. Furthermore, the results of the study conducted by the Small Aircraft Stall Speed Study Group, which consisted of the analysis of 37,530 accident reports over a 6-year period, failed to show a clear correlation between occupant survivability and landing stall speed. The commenter adds that airplane performance has not changed sufficiently in the last 50 years to warrant the proposed change. The commenter supports this with the commenter's own experience. The commenter then indicates that operator error is still the leading cause of aviation accidents and, since aircraft operators will continue to make mistakes, the existing stall speed requirement should remain, thereby protecting operators from themselves.

The commenter is correct that operator error is the leading cause of accidents. However, operator error and the need for improved pilot training are not airplane certification issues, and are beyond the scope of this rulemaking.

来源：NPRM（提案）｜56 FR 22070｜NPRM_Notice 91-12_Docket23746_56FR22070_1991-05-13.txt

▪ 背景与评论处置（4）：本提案

英文原文：

This proposal would clarify the change made to Sec. 23.67 by amendment 23-42 (56 FR 344, January 3, 1991). The provisions of Sec. 23.67 (b)(1) require that all airplanes with a stall speed of more than 61 knots meet the one-engine-inoperative climb gradient requirements. Therefore, a change to Secs. 23.67 (b)(1) and (b)(2) is required to clarify that multiengine airplanes of less than 6,000 pounds maximum weight that include the improved occupant protection of Sec. 23.562 (d) and have a stall speed greater than 61 knots would only comply with the climb gradient determination requirements of Sec. 23.67 (b)(2)(i). This proposal does not change the one-engine-inoperative climb requirements.

来源：NPRM（提案）｜56 FR 22070｜NPRM_Notice 91-12_Docket23746_56FR22070_1991-05-13.txt

▪ 背景与评论处置（5）：在制定被纳入第 23-36 号修正案（53 FR 30802, August 15, 1988）的 §23.562 条（应急着陆动力情况）期间，美国国家运输安全委员会（NTSB）提交的支持性技术数据获取自
本提案中增加的乘员保护要求，是通过将§23.561(b) 段的极限载荷因数以及§23.562(b)(1) 段座椅/约束系统试验的峰值减速度，乘以增加的失速速度与61节失速速度之比的平方来获得的。使用速度比的平方
在更高的失速速度下，该最大减速度将保持在 32g 不变。

英文原文：

During the development of Sec. 23.562, Emergency landing dynamic conditions, which was incorporated in amendment 23-36 (53 FR 30802, August 15, 1988), the supporting technical data presented by the NTSB was obtained from data for small airplanes whose stall speeds were not greater than 61 knots. Forced landings of airplanes whose stall speeds are greater than 61 knots will expose the airplanes' occupants to an increase in kinetic impact energy. The increase in kinetic impact energy is proportional to the square of the stall speed of the airplane in the landing configuration. To compensate for this increased energy level, additional occupant protection requirements beyond those stated in Sec. 23.562 are included in this proposal. The emergency landing dynamic conditions express the impact energy level in terms of an impact velocity. The increased occupant protection requirement in this proposal is obtained by multiplying the ultimate load factors of Sec. 23.561 (b) and the peak deceleration of the seat/restraint system test of Sec. 23.562 (b)(1) by the square of the ratio of the increased stall speed to the stall speed of 61 knots. The use of the velocity ratio squared to obtain the increased occupant protection requirement is consistent with an analytical methodology found in the U.S. Army's Aircraft Crash Survival Design Guide, USARTL-TR-79-22C, Volume III-Aircraft Structural Crashworthiness, which addresses the conservation of momentum associated with an aircraft impact that has earth plowing. A copy of volume III of the design guide is available in the docket.

The FAA proposes to limit the maximum deceleration for the seat/restraint system dynamic test to 32g, which is the value that the FAA is considering proposing in a separate NPRM being developed for commuter category airplanes. The 32g limitation will be reached at a stall speed (Vso) of 79 knots. At higher stall speeds, this maximum deceleration will remain constant at 32g.

修订指令英文原文（供核对）：

4. Section 23.562 is amended by revising the first sentence of the introductory text of paragraph (b), by redesignating paragraph (d) as paragraph (e), and by adding a new paragraph (d) to read as follows:

4. Section 23.562 is amended by revising the introductory text of paragraph (b), by redesignating paragraph (d) as (e), and by adding a new paragraph (d) to read as follows:

▪ 本版重印条文段落：(b)、(d)、(1)、(2)、(i)、(ii)、(iii)

### 2.3　Amdt 23-50（1996-03-11 生效）：修订 §23.562(d) 段的引言部分

▪ 修订沿革：本版由 NPRM　Notice 94-22，1994-07-19 签发，59 FR 37878　提出；最终规章 Docket 27807，1996-01-29 签发，61 FR 5171，1996-03-11 生效。

▪ 修订指令（中文译文）：42. 修订 §23.562(d) 段的引言部分，修订后内容如下：　42. 修订第 23.562 条，将段 (d 修改为如下内容：

来源：Final Rule｜61 FR 5171｜FinalRule_Docket 27807_Amdt23-50_61FR5171_1996-02-09.txt

▪ 背景与评论处置（1）：美国联邦航空局

英文原文：

The FAA proposed to change the one engine inoperative climb to remove the reference in Sec. 23.562(d) and to add it to Sec. 23.67(a)(1).

The only comment on this section is a JAA statement acknowledging existing disharmony.

The proposal is adopted as proposed.

来源：NPRM（提案）｜59 FR 37878｜NPRM_Notice 94-22_Docket27807_59FR37878_1994-07-25.txt

▪ 背景与评论处置（2）：该提案将把第 23.562(d) 条中的单发不工作爬升引用更改为第 23.67(a)(1) 条。

英文原文：

The proposal would change the one engine inoperative climb reference in Sec. 23.562 (d) to Sec. 23.67 (a)(1).

修订指令英文原文（供核对）：

42. Section 23.562(d) introductory text is revised to read as follows:

42. Section 23.562 is amended by revising paragraph (d to read as follows:

▪ 本版重印条文段落：(d)

### 2.4　Amdt 23-62（2012-01-31 生效）：条文未改动（本版为交叉引用或其他条款修订所致）

▪ 修订沿革：本版由 NPRM　—　提出；最终规章 —，2012-01-31 生效。

▪ 本版对条文的处置：条文未改动（本版为交叉引用或其他条款修订所致）

## 3　§23.562 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1988-09-14 生效）　来源：Docket 25147，1988-08-08 签发，53 FR 30802

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Each seat/restraint system for use in a normal, utility, or acrobatic category airplane must be designed to protect each occupant during an emergency landing when-- | 新增 |
| (1) | Proper use is made of seats, safety belts, and shoulder harnesses provided for the design; and | 新增 |
| (2) | The occupant is exposed to the loads resulting from the conditions prescribed in this section. | 新增 |
| (b) | Each seat/restraint system, for crew or passenger occupancy in a normal, utility, or acrobatic category airplane, must successfully complete dynamic tests or be demonstrated by rational analysis supported by dynamic tests, in accordance with each of the following conditions. These tests must be conducted with an occupant simulated by an anthropomorphic test dummy (ATD) defined by 49 CFR Part 572, Subpart B, or an FAA-approved equivalent, with a nominal weight of 170 pounds and seated in the normal upright position. | 新增 |
| (1) | For the first test, the change in velocity may not be less than 31 feet per second. The seat/restraint system must be oriented in its nominal position with respect to the airplane and with the horizontal plane of the airplane pitched up 60 degrees, with no yaw, relative to the impact vector. For seat/restraint systems to be installed in the first row of the airplane, peak deceleration must occur in not more than 0.05 seconds after impact and must reach a minimum of 19g. For all other seat/restraint systems, peak deceleration must occur in not more than 0.06 seconds after impact and must reach a minimum of 15g. | 新增 |
| (2) | For the second test, the change in velocity may not be less than 42 feet per second. The seat/restraint system must be oriented in its nominal position with respect to the airplane and with the vertical plane of the airplane yawed 10 degrees, with no pitch, relative to the impact vector in a direction that results in the greatest load on the shoulder harness. For seat/restraint systems to be installed in the first row of the airplane, peak deceleration must occur in not more than 0.05 seconds after impact and must reach a minimum of 26g. For all other seat/restraint systems, peak deceleration must occur in not more than 0.06 seconds after impact and must reach a minimum of 21g. | 新增 |
| (3) | To account for floor warpage, the floor rails of attachment devices used to attach the seat/restraint system to the airframe structure must be preloaded to misalign with respect to each other by at least 10 degrees vertically (i.e., pitch out of parallel) and one of the rails or attachment devices must be preloaded to misalign by 10 degrees in roll prior to conducting the test defined by paragraph (b)(2) of this section. | 新增 |
| (c) | Compliance with the following requirements must be shown during the dynamic tests conducted in accordance with paragraph (b) of this section: | 新增 |
| (1) | The seat/restraint system must restrain the ATD although seat/restraint system components may experience deformation, elongation, displacement, or crushing intended as part of the design. | 新增 |
| (2) | The attachment between the seat/restraint system and the test fixture must remain intact, although the seat structure may have deformed. | 新增 |
| (3) | Each shoulder harness strap must remain on the ATD's shoulder during the impact. | 新增 |
| (4) | The safety belt must remain on the ATD's pelvis during the impact. | 新增 |
| (5) | The results of the dynamic tests must show that the occupant is protected from serious head injury. | 新增 |
| (i) | When contact with adjacent seats, structure, or other items in the cabin can occur, protection must be provided so that the head impact does not exceed a head injury criteria (HIC) of 1,000. | 新增 |
| (ii) | The value of HIC is defined as-- Where: t1 is the initial integration time, expressed in seconds, t2 is the final integration time, expressed in seconds, (t2-t1) is the time duration of the major head impact, expressed in seconds, and a (t is the resultant deceleration at the center of gravity of the head form expressed as a multiple of g (units of gravity). | 新增 |
| (iii) | Compliance with the HIC limit must be demonstrated by measuring the head impact during dynamic testing as prescribed in paragraphs (b)(1) and (b)(2) of this section or by a separate showing of compliance with the head injury criteria using test or analysis procedures. | 新增 |
| (6) | Loads in individual shoulder harness straps may not exceed 1,750 pounds. If dual straps are used for retaining the upper torso, the total strap loads may not exceed 2,000 pounds. | 新增 |
| (7) | The compression load measured between the pelvis and the lumbar spine of the ATD may not exceed 1,500 pounds. | 新增 |
| (d) | An alternate approach that achieves an equivalent, or greater, level of occupant protection to that required by this section may be used if substantiated on a rational basis. | 新增 |

### 3.2　版本2：Amdt 23-44（1993-08-18 生效）　来源：Docket 23746，1993-07-07 签发，58 FR 38634

新增段落：(b)、(d)、(e)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | Except for those seat/restraint systems that are required to meet paragraph (d) of this section, each seat/restraint system for crew or passenger occupancy in a normal, utility, or acrobatic category airplane, must successfully complete dynamic tests or be demonstrated by rational analysis supported by dynamic tests, in accordance with each of the following conditions. * * * | 修订 |
| (d) | For all single-engine airplanes with a VSO of more than 61 knots at maximum weight, and those multiengine airplanes of 6,000 pounds or less maximum weight with a VSO of more than 61 knots at maximum weight that do not comply with Section 23.67(b)(2)(i): | 修订 |
| (1) | The ultimate load factors of Section 23.561(b) must be increased by multiplying the load factors by the square of the ratio of the increased stall speed to 61 knots. The increased ultimate load factors need not exceed the values reached at a VSO of 79 knots. The upward ultimate load factor for acrobatic category airplanes need not exceed 5.0g. | 修订 |
| (2) | The seat/restraint system test required by paragraph (b)(1) of this section must be conducted in accordance with the following criteria: | 修订 |
| (i) | The change in velocity may not be less than 31 feet per second. | 修订 |
| (ii) | (A) The peak deceleration (gp) of 19g and 15g must be increased and multiplied by the square of the ratio of the increased stall speed to 61 knots: gp = 19.0 (VSO/61)2 or gp = 15.0 (VSO/61)2 (B) The peak deceleration need not exceed the value reached at a VSO of 79 knots. | 修订 |
| (iii) | The peak deceleration must occur in not more than time (tg), which must be computed as follows: where- gp = The peak deceleration calculated in accordance with paragraph (d)(2)(ii) of this section tg = The rise time (in seconds) to the peak deceleration. FOOTER: Issued in Washington, DC. on July 7, 1993. Joseph M. Del Balso, Acting Administrator. [FR Doc. 93-16917 Filed 7-16-93; 8:45 am] BILLING CODE 4910-13-M NPRM ACTIONS: Notice of Proposed Rulemaking. Notice No. 91-12; Issued on 04/02/90. FINAL RULE ACTIONS: Not Applicable. | 修订 |

### 3.3　版本3：Amdt 23-50（1996-03-11 生效）　来源：Docket 27807，1996-01-29 签发，61 FR 5171

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (d) | For all single-engine airplanes with a V SO of more than 61 knots at maximum weight, and those multiengine airplanes of 6,000 pounds or less maximum weight with a V SO of more than 61 knots at maximum weight that do not comply with Sec. 23.67(a)(1); | 修订 |

### 3.4　段落结构演变矩阵（横向对照 4 个版本）

| 段落 | Amdt 23-36 (1988-09-14) | Amdt 23-44 (1993-08-18) | Amdt 23-50 (1996-03-11) |
| --- | --- | --- | --- |
| (a) | ● 新增 | ○ 无 | ○ 无 |
| (1) | ● 新增 | △ 修订 | ○ 无 |
| (2) | ● 新增 | △ 修订 | ○ 无 |
| (b) | ● 新增 | △ 修订 | ○ 无 |
| (3) | ● 新增 | ○ 无 | ○ 无 |
| (c) | ● 新增 | ○ 无 | ○ 无 |
| (4) | ● 新增 | ○ 无 | ○ 无 |
| (5) | ● 新增 | ○ 无 | ○ 无 |
| (i) | ● 新增 | △ 修订 | ○ 无 |
| (ii) | ● 新增 | △ 修订 | ○ 无 |
| (iii) | ● 新增 | △ 修订 | ○ 无 |
| (6) | ● 新增 | ○ 无 | ○ 无 |
| (7) | ● 新增 | ○ 无 | ○ 无 |
| (d) | ● 新增 | △ 修订 | △ 修订 |

## 4　§23.562 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | ating safety standards related to cabin safety and improved crashworthiness. On December 12, 1986, the FAA published Notice No. 86-19, titled "Small Airplane Airworthiness Review Program Notice No. 1" in the FEDERAL REGI… | 初始版 |
| 事故调查与研究驱动 | an analytical methodology found in the U.S. Army's Aircraft Crash Survival Design Guide, USARTL-TR-79-22C, Volume III---Aircraft Structural Crashworthiness, which addresses the conservation of momentum associated with an… | Amdt 23-44 |
| 国际协调 | to Sec. 23.67(a)(1). The only comment on this section is a JAA statement acknowledging existing disharmony. The proposal is adopted as proposed. The proposal would change the one engine inoperative climb reference in Se… | Amdt 23-50 |

本条取证提示 共 4 个历史版本：Amdt 23-36, Amdt 23-44, Amdt 23-50, Amdt 23-62。 其中 3 个版本在本地文献中检索到明确修订指令原文；3 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 25147 | 53 FR 30802 | FinalRule_Docket 25147_Amdt23-36_53FR30802_1988-08-15_0002.txt |
| [2] | NPRM（提案） | Notice 86-19 / Docket 25147 | 51 FR 44878 | NPRM_Notice 86-19_Docket25147_51FR44878_1986-12-12.txt |
| [3] | Final Rule（颁行） | Docket 23746 | 58 FR 38634 | FinalRule_Docket 23746_Amdt23-44_58FR38634_1993-07-19_0002.txt |
| [4] | NPRM（提案） | Notice 91-12 / Docket 23746 | 56 FR 22070 | NPRM_Notice 91-12_Docket23746_56FR22070_1991-05-13.txt |
| [5] | Final Rule（颁行） | Docket 27807 | 61 FR 5171 | FinalRule_Docket 27807_Amdt23-50_61FR5171_1996-02-09.txt |
| [6] | Final Rule（颁行） | Docket 27807 | 61 FR 0 | FinalRule_Docket 27807_Amdt1-43_23-50_61FR_1996-02-09_0002.txt |
| [7] | NPRM（提案） | Notice 94-22 / Docket 27807 | 59 FR 37878 | NPRM_Notice 94-22_Docket27807_59FR37878_1994-07-25.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.562 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-36 | 1988-09-14 | b3631d1e-7c48-41fc-912f-a1713a42162b |
| Amdt 23-44 | 1993-08-18 | c8ce6c54-c49e-4f75-8062-5020e1403615 |
| Amdt 23-50 | 1996-03-11 | 42dba18f-5200-4615-bc02-63e342571a64 |
| Amdt 23-62 | 2012-01-31 | 493c88f7-45e1-4ff5-8bcf-7e47cd87a51c |

# § 23.571　增压座舱（疲劳评定）（Pressurized cabin.）

1965-02-01 初版　｜　2012-01-31 Amdt 23-62　｜　共 6 个历史版本

## 1　§23.571 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.571 |
| 条款标题（英文原文） | Pressurized cabin. |
| 中文名称 | 增压座舱（疲劳评定） |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 6 版（含 1965-02-01 初始版） |
| 时间跨度 | 1965-02-01 → 2012-01-31 （约 47 年） |
| 初版来源 | Docket 4080，1964-09-28 签发，29 FR 17945 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 73 FR 19746 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.571 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | 初始版 | 1965-02-01 | Notice 64-17，1964-03-25 签发，29 FR 5111 | Docket 4080，1964-09-28 签发 | 初始颁行，确立本条条文 |
| 2 | Amdt 23-14 | 1973-12-20 | Notice 71-13，1971-04-27 签发，36 FR 8398 | Docket 11011，1973-11-07 签发，38 FR 31816 | 修订 |
| 3 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 条文未改动（本版为交叉引用或其他条款修订所致） |
| 4 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 新增段落：(c) |
| 5 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订：(a) |
| 6 | Amdt 23-62 | 2012-01-31 | — | — | 条文未改动（本版为交叉引用或其他条款修订所致） |

### 1.3　修订脉络一览

1965 初版　初始颁行，确立本条条文　→　1973 23-14　修订　→　1987 23-34　条文未改动（本版为交叉引用或其他条款修订所致　→　1993 23-45　新增段落：(c)　→　1996 23-48　修订：(a)　→　2012 23-62　条文未改动（本版为交叉引用或其他条款修订所致

## 2　§23.571 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1965-02-01，初始版）：增压座舱（疲劳评定）

▪ 修订沿革：本版由 NPRM　Notice 64-17，1964-03-25 签发，29 FR 5111　提出；最终规章 Docket 4080，1964-09-28 签发，1965-02-01 生效。

▪ 本版对条文的处置：初始颁行，确立本条条文

### 2.2　Amdt 23-14（1973-12-20 生效）：§23.571 条的 (a) 段修订为如下内容

▪ 修订沿革：本版由 NPRM　Notice 71-13，1971-04-27 签发，36 FR 8398　提出；最终规章 Docket 11011，1973-11-07 签发，38 FR 31816，1973-12-20 生效。

▪ 修订指令（中文译文）：17. §23.571 条的 (a) 段修订为如下内容：　18. §23.571(a) 段将修订如下：

修订指令英文原文（供核对）：

17. Paragraph (a) of Sec. 23.571 is amended to read as follows:

18. Paragraph (a) of Sec. 23.571 would be amended to read as follows:

▪ 本版重印条文段落：(a)

### 2.3　Amdt 23-34（1987-02-17 生效）：条文未改动（本版为交叉引用或其他条款修订所致）

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 本版对条文的处置：条文未改动（本版为交叉引用或其他条款修订所致）

### 2.4　Amdt 23-45（1993-09-07 生效）：§23.571 修订如下：删除引言段中的"either"一词

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：47. §23.571 修订如下：删除引言段中的"either"一词，替换为"one"，并新增段(c)，内容如下：

修订指令英文原文（供核对）：

47. Section 23.571 is amended by removing the word "either" from the introductory paragraph and replacing it with the word "one" and by adding a new paragraph (c) to read as follows:

▪ 本版重印条文段落：(c)

### 2.5　Amdt 23-48（1996-03-11 生效）：通过修改标题、引言文字和段 (a) 对第 23.571 条进行修订

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：25. 通过修改标题、引言文字和段 (a) 对第 23.571 条进行修订，内容如下：　27. 修订§23.573 条，方法是：删除 (b) 段中引用的“§23.571(c) 段”并在其位置添加引用“§23.571(a)(3) 段”；删除 (c) 段；并修改 (a)(5) 段的导语，使其内容如下：　27. 修订第 23.573 条：删除 (b) 段中的引用“§23.571(c) 段”并代之以“§23.571(a)(3) 段”；删除 (c) 段；并修改 (a)(5) 段的导言部分，使其改读如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议修订§23.571，将其标题从"增压座舱"改为"金属增压座舱结构"，因为非金属结构在§23.573(a)中另有规定。美国联邦航空局（FAA）提议修订引言部分，将适用范围限定为正常类、实用类和特技类，因为通勤类飞机另有规定。美国联邦航空局（FAA）提议修订(a)段，要求疲劳强度调查表明结构能够承受运行中预期的可变幅值重复载荷。

英文原文：

The FAA proposed to revise Sec. 23.571 by changing the heading from "Pressurized cabin" to "Metallic pressurized cabin structures" because nonmetallic structure is addressed in Sec. 23.573(a). The FAA proposed to revise the introductory text to limit the applicability to normal, utility, and acrobatic categories because commuter category airplanes are addressed separately. The FAA proposed to revise paragraph (a) to require the fatigue strength investigation to show that the structure can withstand repeated loads of variable magnitude expected in service.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局提议修订条标题以增加“金属的”一词，修订(a)段以将适用范围限制在正常类、实用类和特技类飞机，并进行小幅文字修改。(a)(1)段拟作修订，以与欧洲联合航空要求第23部（JAR 23）协调统一，要求按本前言中§23.571条所述进行试验，或进行有试验证据支持的分析。
关于本条收到的唯一评议来自Kal-Aero，该评议适用于本条和§23.571条。该评议已在§23.571条下讨论。
所提议案按建议通过。

英文原文：

The FAA proposed to revise the section heading to add the word "metallic," to revise paragraph (a) to limit the applicability to normal, utility, and acrobatic category airplanes, and to make minor editorial changes. Paragraph (a)(1) would be revised to harmonize with JAR 23 by requiring tests, or analysis supported by test evidence, as discussed under Sec. 23.571 of this preamble.

The only comment received on this section is from Kal-Aero, and applies to this section and to Sec. 23.571. The comment was discussed under Sec. 23.571.

The proposals are adopted as proposed.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（3）：针对本条收到的唯一评论来自 Kal-Aero，且适用于本条和 §23.571 条。该评论已在 §23.571 条下进行了讨论。提案按提案予以采纳。

英文原文：

The only comment received on this section is from Kal-Aero, and applies to this section and to Sec. 23.571. The comment was discussed under Sec. 23.571.

The proposals are adopted as proposed.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（4）：收到两条关于提案经济影响的评论；一条涉及 §23.571 金属增压座舱结构，一条涉及 §23.572 金属机翼、尾翼及关联结构。这两条评论及美国联邦航空局（FAA）的答复均列入"修订讨论"部分。

英文原文：

Two comments were received regarding the economic impact of the proposals; one concerning Sec. 23.571, Metallic pressurized cabin structures, and one concerning Sec. 23.572, Metallic wing, empennage, and associated structures. Both of these comments, as well as the FAA's responses, are included in the section "Discussion of Amendments".

来源：Final Rule｜—｜FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt

▪ 背景与评论处置（5）：本次技术修订将修正§23.573(b)段，以正确引用§23.571(c)段。

英文原文：

This technical amendment will correct Sec. 23.573(b) to properly reference Sec. 23.571(c).

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（6）：将修订第23.571条，将标题从"Pressurized cabin"修改为"Metallic pressurized cabin structure"，因为非金属结构在第23.573(a)条中述及。将修订引言文字，将适用范围限于正常类、实用类和特技类，因为通勤类飞机另行述及。将修订(a)段，要求疲劳强度调查表明结构能够承受使用中预计出现的量值变化的重复载荷。目前，疲劳强度可通过试验或分析或两者并用表明。根据拟议修订，结构强度必须通过试验或由试验证据支持的分析表明。

英文原文：

Section 23.571 would be revised by changing the heading from "Pressurized cabin" to "Metallic pressurized cabin structure" because nonmetallic structure is addressed in Sec. 23.573 (a). The introductory text would be revised to limit the applicability to normal, utility, and acrobatic category only because commuter category airplanes are addressed separately. Paragraph (a) would be revised to require the fatigue strength investigation to show that the structure can withstand repeated loads of variable magnitude expected in service. Currently, fatigue strength may be shown by tests or analysis or both. Under the proposed revision, structural strength must be shown by tests or by analysis supported by test evidence.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（7）：本提案将修订标题以增添“金属”一词，并修订 §23.572(a) 段，将适用范围限定为正常类、实用类和特技类飞机，以及进行次要编辑性修改。为与欧洲联合航空要求（JAR）23 协调统一，(a)(1) 段经修订要求进行试验或有试验证据支持的分析，如前言 §23.571 条所述。

英文原文：

This proposal would revise the heading to add the word "metallic" and revise Sec. 23.572 (a) to limit the applicability to normal, utility, and acrobatic category airplanes and to make minor editorial changes. Paragraph (a)(1) is revised to harmonize with JAR 23 by requiring tests or analysis supported by test evidence, as discussed under Sec. 23.571 of this preamble.

修订指令英文原文（供核对）：

25. Section 23.571 is amended by revising the heading, the introductory text, and paragraph (a), to read as follows:

27. Section 23.573 is amended by removing the reference in paragraph (b) "Sec. 23.571(c)" and adding the reference "Sec. 23.571(a)(3)" in its place; by removing paragraph (c); and by revising the introductory text of paragraph (a)(5) to read as follows:

27. Section 23.573 is amended by removing the reference in paragraph(b) "Sec. 23.571(c)" and adding in its place "Sec. 23.571(a)(3)"; by removing paragraph (c); and by revising the introductory text of paragraph (a)(5) to read as follows:

▪ 本版重印条文段落：(a)

### 2.6　Amdt 23-62（2012-01-31 生效）：条文未改动（本版为交叉引用或其他条款修订所致）

▪ 修订沿革：本版由 NPRM　—　提出；最终规章 —，2012-01-31 生效。

▪ 本版对条文的处置：条文未改动（本版为交叉引用或其他条款修订所致）

### 2.7　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.571 的实际改动 |
| --- | --- | --- | --- |
| 73 FR 19746 | 2008-04-11 | 技术性修订 (Technical Amendment) | 重写 §23.573(b) 首句——损伤容限评定须含疲劳/腐蚀/意外损伤位置与模式 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.571 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1965-02-01 生效）　来源：Docket 4080，1964-09-28 签发，29 FR 17945

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | A fatigue strength investigation, in which the structure is shown by analysis, tests, or both to be able to withstand the repeated loads of variable magnitude expected in service. | 新增 |
| (b) | A fail safe strength investigation, in which it is shown by analysis, tests, or both that catastrophic failure of the structure is not probable after fatigue failure, or obvious partial failure, of a principal structural element, and that the remaining structures are able to withstand a static ultimate load factor of 75 percent of the limit load factor at VC, considering the combined effects of normal operating pressures, expected external aerodynamic pressures, and flight loads. These loads must be multiplied by a factor of 1.15 unless the dynamic effects of failure under static load are otherwise considered. | 新增 |

### 3.2　版本2：Amdt 23-14（1973-12-20 生效）　来源：Docket 11011，1973-11-07 签发，38 FR 31816

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | A fatigue strength investigation, in which the structure is shown by analysis, test, or both to be able to withstand the repeated loads of variable magnitude expected in service. Analysis alone is considered acceptable only when it is conservative and applied to simple structures. | 修订 |

### 3.3　版本3：Amdt 23-45（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

新增段落：(c)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (c) | The damage tolerance evaluation of Section 23. 573(b). | 新增 |

### 3.4　版本4：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | A fatigue strength investigation in which the structure is shown by tests, or by analysis supported by test evidence, to be able to withstand the repeated loads of variable magnitude expected in service; or | 新增 |

### 3.5　段落结构演变矩阵（横向对照 6 个版本）

| 段落 | 初始版 (1965-02-01) | Amdt 23-14 (1973-12-20) | Amdt 23-45 (1993-09-07) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | ○ 无 | ● 新增 |
| (b) | ● 新增 | ○ 无 | ○ 无 | ○ 无 |
| (c) | ○ 无 | ○ 无 | ● 新增 | ○ 无 |

## 4　§23.571 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | 23.571 Pressurized cabin. * * * * * (a) A fatigue strength investigation, in which the structure is shown by analysis, test, or both to be able to withstand the repeated loads of variable magnitude expected in service.… | Amdt 23-14 |
| 事故调查与研究驱动 | sed to revise paragraph (a) to require the fatigue strength investigation to show that the structure can withstand repeated loads of variable magnitude expected in service. The FAA proposed to revise the section heading… | Amdt 23-48 |

本条取证提示 共 6 个历史版本：初始版, Amdt 23-14, Amdt 23-34, Amdt 23-45, Amdt 23-48, Amdt 23-62。 其中 3 个版本在本地文献中检索到明确修订指令原文；4 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：73 FR 19746，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 4080 | （FR 引证未在文本中标注） | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |
| [2] | NPRM（提案） | Notice 64-17 / Docket 4080 | 29 FR 5111 | NPRM_Notice 64-17_Docket4080_29FR5111_1964-04-14.txt |
| [3] | Final Rule（颁行） | Docket 11011 | 38 FR 31816 | FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt |
| [4] | NPRM（提案） | Notice 71-13 / Docket 11011 | 36 FR 8398 | NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt |
| [5] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [6] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [7] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [8] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [9] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [10] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [11] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [12] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [13] | 更正通告 / 校正 | 73 FR 19746 | 2008-04-11 | TechAmend_73FR19746_2008-04-11_E8-7649.clean.txt |
| [14] | 修订记录 | FAA 动态法规系统（DRS）条款修订版索引 | Part 23 @ Amdt 23-63 as of 08/29/2017 and earlier | FAR23_Amdt23-63_条款清单.csv（本地） |
| [15] | 条文校对 | 1965 年重编原始规章（初始条文） | 29 FR 17955，1964-12-18 | FinalRule_Docket 4080_Amdt23-0_1964-12-18_0002.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.571 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| 初始版 | 1965-02-01 | ab506415-cc0c-4c4d-9667-190c04c67add |
| Amdt 23-14 | 1973-12-20 | e0fdd96f-327e-49e5-98a8-2fbd49b01913 |
| Amdt 23-34 | 1987-02-17 | efdef920-4515-46af-8d8e-561888015283 |
| Amdt 23-45 | 1993-09-07 | d20ec710-2b61-4e29-bae2-16d1269e2ab1 |
| Amdt 23-48 | 1996-03-11 | 1abfd938-400e-46d3-b553-a8428258a544 |
| Amdt 23-62 | 2012-01-31 | 45a21af3-8a8d-4fd4-a355-5749e67ab4c9 |

# § 23.572　机翼及相关结构（Wing, empennage, and associated structures.）

1969-09-14 初版　｜　1996-03-11 Amdt 23-48　｜　共 6 个历史版本

## 1　§23.572 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.572 |
| 条款标题（英文原文） | Wing, empennage, and associated structures. |
| 中文名称 | 机翼及相关结构 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 6 版（含 1969-09-14 初始版） |
| 时间跨度 | 1969-09-14 → 1996-03-11 （约 27 年） |
| 初版来源 | Docket 8083，1969-08-01 签发，34 FR 13078 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 73 FR 19746 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.572 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-7 | 1969-09-14 | Notice 67-14，1967-04-05 签发，32 FR 5791 | Docket 8083，1969-08-01 签发，34 FR 13078 | 新增条款 |
| 2 | Amdt 23-14 | 1973-12-20 | Notice 71-13，1971-04-27 签发，36 FR 8398 | Docket 11011，1973-11-07 签发，38 FR 31816 | 修订 |
| 3 | Amdt 23-34 | 1987-02-17 | Notice 83-17，1983-09-23 签发，48 FR 52010 | Docket 23516，1987-01-08 签发，52 FR 1806 | 新增段落：(a)、(b) |
| 4 | Amdt 23-38 | 1989-10-26 | Notice 86-14，1986-09-11 签发，51 FR 33700 | Docket 25086，1989-09-21 签发 | 修订：(a) |
| 5 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 条文未改动（本版为交叉引用或其他条款修订所致） |
| 6 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 修订段落：(a) |

### 1.3　修订脉络一览

1969 初版　新增条款　→　1973 23-14　修订　→　1987 23-34　新增段落：(a)、(b)　→　1989 23-38　修订：(a)　→　1993 23-45　条文未改动（本版为交叉引用或其他条款修订所致　→　1996 23-48　修订段落：(a)

## 2　§23.572 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1969-09-14，Amdt 23-7）：机翼及相关结构

▪ 修订沿革：本版由 NPRM　Notice 67-14，1967-04-05 签发，32 FR 5791　提出；最终规章 Docket 8083，1969-08-01 签发，34 FR 13078，1969-09-14 生效。

▪ 修订指令（中文译文）：41. 新增 §23.572 条，内容如下：

来源：Final Rule｜34 FR 13078｜FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt

▪ 背景与评论处置（1）：该通告提议修订§23.571条，要求对机翼及相关结构进行有效的疲劳评定。鉴于现行§23.571条的标题就这些要求而言可能会引起误导，该提案已作为新的§23.572条被采纳。收到了大量反对该提案的意见。对此，有意见指出：(1) 现行的强度要求和设计实践是保守的，足以防止严重的疲劳问题；(2) 腐蚀、先前的滥用
腐蚀、既往不当使用和特殊用途运行可能促成疲劳破坏，但此类破坏的主要原因在于强度不足。由于疲劳破坏具有时间依赖性，老旧飞机出现更高的破坏率是可预见的；然而，近年来取证的飞机中也已出现疲劳问题。无论是疲劳证实还是更好的维修方案，都无法消除所有裂纹。拟议规章的目的是防止灾难性破坏。此外，第23部飞机并非基于冗余结构、破损安全原则设计，仅靠维修是不足以应对的。疲劳证实与良好的维修方案两者均属必需。根据现有的广泛阵风数据，已经能够且已经建立了合理的载荷谱，同时合理且可接受的符合性方法也已确立并广泛公布。
然而，虽然已确定疲劳验证是必要的，但美国联邦航空局不同意一位评论者提出的建议，即拟议要求应扩展为完整的限制载荷破损安全要求，设计目标至少为10,000小时，试验寿命为30,000次飞行。这种要求比运输类飞机的要求更为严格，无法基于使用经验来证明其合理性。该修订按拟议方式采纳，但作为新的§23.572条。该通告拟修订§23.607条，以允许在操作中旋转的螺栓上使用自锁螺母，前提是提供了额外的锁定装置。在通告发布之后，已采纳了第27-4号和第29-5号修订案，规定只有在额外的锁定装置是非摩擦锁定装置时，才允许在旋翼机上使用自锁螺母。这些修订案还增加了对可拆卸螺栓、螺钉、螺母、销钉和其他紧固件的要求，如果这些紧固件丢失，可能会危及旋翼机的安全运行。
第 68-18 号通告中已为运输类飞机提出了类似要求。鉴于就第 27-4 号和第 29-5 号修正案以及第 68-18 号通告中的提案所收到的评论意见，对 §23.607 条的拟议修正案予以撤回，有待进一步研究。该通告提议修订 §23.611 条，要求零部件的可接近性足以满足必要维护中修理和更换方面的需要。反对该提案的评论意见称，该提案没有必要，记录符合性属于多余负担，并且维护、检查和润滑的充分接近途径将在飞行试验大纲期间提供。美国联邦航空局（FAA）不同意此意见。使用经验表明，并非总能提供充分的接近途径，且已发布适航指令，要求进行改装以提供接近途径。
拟议的修正案已作出更改，以更清楚地说明通告中所解释的提案意图，即必须提供可达性，以便进行由检查所发现必要的修理或更换，以及勤务、润滑和调整。

英文原文：

The notice proposed to amend Sec. 23.571 to require an effective fatigue evaluation of the wing and associated structures. Inasmuch as the title of current Sec. 23.571 might be misleading insofar as these requirements are concerned, the proposal has been adopted as new Sec. 23.572. Numerous comments were received objecting to this proposal. In this connection, it was stated that (1) the current strength requirements and design practices are conservative and adequate to prevent serious fatigue problems; (2) corrosion, prior abuse, and special purpose operations cause fatigue failure, rather then lack of design fatigue strength; (3) fatigue substantiation will not eliminate cracks, and a better maintenance program would be more effective; and (4) sufficient data is not available to establish load spectra for fatigue substantiation. The FAA does not agree. Service experience and discussions with industry of designs which have sustained fatigue failures indicate that present design practices do not adequately account for fatigue. Corrosion, prior abuse, and special purpose operations may contribute to fatigue failure, but the primary reason for such failures is lack of strength. Since fatigue failures are time-dependent, a higher failure rate among older airplanes is to be expected; however, fatigue problems have arisen in airplanes certificated in recent years. Neither fatigue substantiation nor better maintenance programs will eliminate all cracks. The purpose of the proposed rule is to prevent catastrophic failures. Furthermore, Part 23 airplanes are not designed on a redundant-structure, fail-safe basis, for which maintenance alone would be sufficient. Both fatigue substantiation and a good maintenance program are needed. Reasonable load spectra can and have been established from the extensive gust data which is available, and reasonable and acceptable methods of compliance have been established and widely publicized. However, while it has been determined that fatigue substantiation is necessary, the FAA does not agree with one comment which suggested that the proposed requirement should be expanded to a full limit load fail-safe requirement with a design objective of at least 10,000 hours and a test life of 30,000 flights. Such a requirement would be more severe than that required for transport category airplanes and cannot be justified on the basis of service experience. The amendment is adopted as proposed, except that it is adopted as new Sec. 23.572.

The notice proposed to amend Sec. 23.607 to permit the use of self-locking nuts on bolts subject to rotation in operation if additional locking means were provided. Subsequent to the issuance of the notice, Amendments 27-4 and 29-5 have been adopted which permit self-locking nuts to be used on rotorcraft only if the additional locking means is a nonfriction locking device. Those amendments also added requirements for removable bolts, screws, nuts, pins, and other fasteners whose loss could jeopardize the safe operation of the rotorcraft. Similar requirements have been proposed for transport category airplanes in Notice 68-18. In view of comments received in connection with Amendments 27-4 and 29-5 and the proposal in Notice 68-18, the proposed amendment to Sec. 23.607 is withdrawn pending further study.

The notice proposed to amend Sec. 23.611 to require that accessibility to parts be adequate for the repair and replacement aspects of necessary maintenance. Comments objecting to the proposal stated that the proposal is unnecessary, that documenting compliance is a superfluous burden, and that adequate access for maintenance, inspection, and lubrication will be provided during flight test program. The FAA does not agree. Service experience has shown that adequate access means are not always provided, and Airworthiness Directives have been issued in which modifications to provide access were required. The proposed amendment has been changed to more clearly state the intent of the proposal as explained in the notice, that accessibility must be provided for the purpose of making a repair or replacement found necessary by the inspections, as well as for servicing, lubrication, and adjustments.

修订指令英文原文（供核对）：

41. A new Sec. 23.572 is added to read as follows:

### 2.2　Amdt 23-14（1973-12-20 生效）：将§23.572的(a)(1)段修订如下

▪ 修订沿革：本版由 NPRM　Notice 71-13，1971-04-27 签发，36 FR 8398　提出；最终规章 Docket 11011，1973-11-07 签发，38 FR 31816，1973-12-20 生效。

▪ 修订指令（中文译文）：18. 将§23.572的(a)(1)段修订如下：　19. §23.572 条 (a)(1) 段将修订如下：

来源：Final Rule｜38 FR 31816｜FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt

▪ 背景与评论处置（1）：该通告解释说，对§23.572(a)(1)段拟议修订的目的在于阐明，对于疲劳评定，仅在某些指定的情况下，方可接受单独使用分析。为此，该条中插入了"only"一词，以使要求更加明确。一位评论者建议修订该条，要求在疲劳关键区域采用破损安全结构。美国联邦航空局不同意。该意见超出了通告的范围，且如建议那样修订该条，会对飞机设计施加不必要的限制。

英文原文：

The notice explained that the purpose of the proposed amendment to Sec. 23.572 (a)(1) was to make it clear that, for a fatigue evaluation, the use of analysis alone is acceptable only under certain specified circumstances. The word "only" has accordingly been inserted in that section to make the requirement clear. One commentator suggested that the section be revised to require fail safe structure in fatigue critical areas. The FAA does not agree. The comment is beyond the scope of the notice and a revision of the section, as suggested, would place an unnecessary restriction on aircraft design.

修订指令英文原文（供核对）：

18. Paragraph (a)(1) of Sec. 23.572 is amended to read as follows:

19. Paragraph (a)(1) of Sec. 23.572 would be amended to read as follows:

▪ 本版重印条文段落：(a)、(1)

### 2.3　Amdt 23-34（1987-02-17 生效）：修订第 23.572 条

▪ 修订沿革：本版由 NPRM　Notice 83-17，1983-09-23 签发，48 FR 52010　提出；最终规章 Docket 23516，1987-01-08 签发，52 FR 1806，1987-02-17 生效。

▪ 修订指令（中文译文）：26. 修订第 23.572 条，将该条标题从“机翼及相关结构”改为“飞行结构”；在(a)段第一个词前插入“对于正常类、实用类和特技类飞机，”；将(a)段中的“The”改为“the”；并新增(b)段，内容如下：　24. 通过修订§23.572条的条标题，在段(a)的第一个词之前插入"对于正常类、实用类和特技类飞机，"；将段(a)中的单词"The"更改为"the"；并增加新段(b)，如下：

修订指令英文原文（供核对）：

26. By amending Sec. 23.572 by revising the title of the section from "Wing and associated structure" to "Flight structure"; by inserting the words "For normal, utility, and acrobatic category airplanes," before the first word in paragraph(a); by changing the word "The" to "the" in paragraph (a); and by adding a new paragraph (b) to read as follows:

24. By amending Sec. 23.572 by revising the section heading, by inserting the words "For normal, utility, and acrobatic category airplanes," before the first word in paragraph (a); by changing the word "The" to "the" in paragraph (a); and by adding a new paragraph (b) to read as follows:

▪ 本版重印条文段落：(b)、(1)、(2)

### 2.4　Amdt 23-38（1989-10-26 生效）：§23.572 条通过修订标题、(a) 段的引言文本以及 (a)(1

▪ 修订沿革：本版由 NPRM　Notice 86-14，1986-09-11 签发，51 FR 33700　提出；最终规章 Docket 25086，1989-09-21 签发，1989-10-26 生效。

▪ 修订指令（中文译文）：2. §23.572 条通过修订标题、(a) 段的引言文本以及 (a)(1) 段和 (b) 段，修改如下：　这位评论者还建议将 86-14 号通告中拟议的第 23.572(b) 条修订为两条，内容如下：　2. §23.572修订如下：修订标题；修订(a)段引言文字；并新增(b)段，内容如下：

来源：Final Rule｜—｜FinalRule_Docket 25086_Amdt23-38_1989-09-26_0002.txt

▪ 背景与评论处置（1）：一位评论者表示，对美国联邦航空局（FAA）针对第 23.572(b) 条提案的“升力面尾流的影响”的含义感到担忧。该评论者建议美国联邦航空局（FAA）解释所指代的是什么，以及“如何进行此类评定。”美国联邦航空局（FAA）使用“升力面尾流的影响”这一短语的目的是为了包含被评定结构上的任何载荷，这些载荷是飞机上另一个升力面的气动效应的结果，并且对被评定结构的疲劳载荷有重大影响。举例来说，升力面尾流的影响可能包括：(1) 相互气动效应（下洗效应），该效应可以改变其他升力面上的稳态展向载荷分布；以及 (2) 来自前方升力面的涡流效应，该效应会在后方翼面上诱发振动载荷。
最终

英文原文：

One commenter states concern about the FAA meaning of "the effects of lifting surface wakes" as proposed for Section 23.572(b). This commenter proposes that the FAA explain what is being alluded to and "how one goes about such an evaluation." The intent of the FAA in using the phrase, "the effects of lifting surface wakes" was to include any loading on the structure being evaluated which is the result of the aerodynamic effects from another lifting surface on the airplane and which has a significant effect on the fatigue loading of the structure being evaluated. As examples, lifting surface wake effects may include: (1) mutual aerodynamic effects (downwash effects) which can alter the steady-state spanwise load distribution on other lifting surfaces, and (2) vortex effects from a forward-lifting surface which induce vibratory loads on the trailing surfaces. The final rule replaces the term "lifting surface wakes" with the more definitive terms "mutual influence of aerodynamic surfaces" and "buffet from vortex impingements." The subject of lifting surface wake effects was discussed at the Part 23 Review Conference in October 1984, and at that time, the FAA received recommendations to incorporate type certification requirements to account for structural loading from these effects. Although regulatory actions based on these recommendations are still under consideration, the FAA considers the influence of these loading effects to be an essential part of the fatigue evaluation of certain airframe designs.

This commenter also suggests that the proposed Section 23.572(b) in notice 86-14 be revised as two sections to read as follows:

来源：Final Rule｜—｜FinalRule_Docket 25086_Amdt23-38_1989-09-26_0002.txt

▪ 背景与评论处置（2）：“（c）在符合本条要求时，必须考虑螺旋桨尾流引起的振动的任何显著贡献。”该评论者认为，这是必要的，因为拟议的 §23.572(b) 段的三重范围不便于纳入有限载荷谱。美国联邦航空局（FAA）同意该评论者表达的担忧。在 §23.572 条要求的评定中所使用的有限载荷谱，必须包括对疲劳损伤有显著贡献的该型号飞机的典型载荷谱。在某些设计案例中，独特的载荷源（例如螺旋桨滑流载荷、气动表面的相互影响或涡流撞击引起的抖振）可能会对各种结构部件的疲劳损伤产生贡献，这取决于飞机设计。这些载荷源中的任何一个，如果被确定对结构部件的疲劳损伤有显著影响，都应当在疲劳评定中予以考虑。
然而，在许多情况下，由螺旋桨滑流效应、气动表面的相互影响或涡冲击引起的气动抖动所导致的疲劳损伤，均可通过可接受的设计细节予以避免。因此，重述 (b) 段以确保更清晰地理解该规则。与 86-14 号通告中的措辞相比，对 §23.572(b) 条所作的更改旨在阐明规则意图，且不导致要求发生实质性变更，这与 86-14 号通告中的提议一致。

英文原文：

"(c) Account must be taken of any significant contribution of propeller wake-induced vibrations when complying with this section." This commenter contends that this is necessary because the threefold scope of the proposed Section 23.572(b) does not lend itself to inclusion of finite load spectra. The FAA agrees with the concerns expressed by the commenter. The finite loading spectra used in the evaluation required by Section 23.572 must include typical loading spectra for the type airplane which significantly contribute to the fatigue damage. In certain design cases, the unique sources of loading, such as propeller slipstream loading, the mutual influence of aerodynamic surfaces, or the buffet from vortex impingement, may contribute to the fatigue damage of various structural components depending on the airplane design. Any of these sources of loading, which is determined to have a significant effect on the fatigue damage of a structural component, should be considered in the fatigue evaluation. However, in many cases, fatigue damage resulting from the effects of the propeller slipstream, the mutual influence of the aerodynamic surfaces, or buffet due to vortex impingement can be avoided by acceptable design detail. Therefore, paragraph (b) is restated to ensure a clearer understanding of the rule. The changes to Section 23.572(b) from the wording in notice 86-14 are made to clarify the intent of the rule and result in no substantive changes to the requirements, as proposed in notice 86-14.

修订指令英文原文（供核对）：

2. Section 23.572 is amended by revising the title, the introductory text of paragraph (a), and paragraphs (a)(1), and (b) to read as follows:

This commenter also suggests that the proposed Section 23.572(b) in notice 86-14 be revised as two sections to read as follows:

2. Section 23.572 is amended by revising the title; by revising paragraph (a) introductory text; and by adding a new paragraph (b) to read as follows:

▪ 本版重印条文段落：(a)、(1)、(b)、(1)、(2)、(3)

### 2.5　Amdt 23-45（1993-09-07 生效）：条文未改动（本版为交叉引用或其他条款修订所致）

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 本版对条文的处置：条文未改动（本版为交叉引用或其他条款修订所致）

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（1）：说明：有三项会议提案针对 §23.345 条。会议提案 191 建议增加机翼襟翼及其支撑结构的设计限制载荷因数和极限载荷因数，以考虑滑流效应，并提供最低静强度和疲劳强度能力。会议之后，美国联邦航空局（FAA）发布了修正案 23.38（54 FR 39508），该修正案修订 §23.572 条，以解决失效将导致灾难性后果的机翼部件的疲劳要求。美国联邦航空局认定，此类机翼部件包括襟翼以及螺旋桨滑流对这些襟翼的冲击效应。因此，基于会议提案 191，不建议修改 §23.345 条。

英文原文：

Explanation: There are three conference proposals directed at Section 23.345. Conference proposal 191 recommends increasing the design limit and ultimate load factors for wing flaps and their supporting structure to account for slipstream effects and to provide a minimum static and fatigue strength capability. Subsequent to the conference, the FAA issued amendment 23.38 (54 FR 39508), which amends Section 23.572 to address fatigue requirements for those parts of the wing whose failure would be catastrophic. The FAA interprets such parts of the wing to include flaps and the effects of propeller slipstream impingement on those flaps. As such, no change to Section 23.345 is recommended as a result of conference proposal 191.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（2）：说明：共有五项会议提案针对第23.572条，另有一项提议新增第23.573条。

英文原文：

Explanation: There are five conference proposals directed at Section 23.572 and one that proposes a new Section 23.573.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（3）：会议提案 226 和 229 建议将现有疲劳要求扩展至尾翼，具体做法是在现有第 23.572 条中纳入"尾翼"一词，或设立一个题为"尾翼及其相关结构"的新条款。会议提案 225 建议第 23.572 条适用于鸭式布局和串列翼布局以及主翼。会议提案 228 建议将重量低于 1500 磅的飞机排除在第 23.572 条的要求之外。会议提案 227 和 515 建议要求对飞机主要结构中失效会造成灾难性后果的任何部件进行疲劳强度或破损安全验证。最后，会议提案 227 还建议专门处理螺旋桨尾流引起的振动所产生的载荷，会议提案 515 建议要求将破损安全准则作为重量超过 6000 磅的飞机的主要验证方法。

英文原文：

Conference proposals 226 and 229 recommended extending the existing fatigue requirements to the empennage by either including the term "empennage" in existing Section 23.572 or by establishing a new section entitled "Empennage and associated structure". Conference proposal 225 recommends that Section 23.572 apply to canard and tandem wing configurations as well as the main wing. Conference proposal 228 recommends excluding airplanes of less than 1500 pounds from the requirements of Section 23.572 Conference proposals 227 and 515 recommend requiring fatigue strength or fail-safe substantiation for any part of the airplane primary structure whose failure would be catastrophic. Finally, conference proposal 227 also recommends that the loads resulting from propeller wake-induced vibrations be specifically addressed, and conference proposal 515 recommends requiring fail-safe criteria as the primary method of substantiation for airplanes above 6,000 pounds.

### 2.6　Amdt 23-48（1996-03-11 生效）：修订 §23.572 条

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：26. 修订 §23.572 条，修改标题；修改段(a)引导文本；并将段(a)(1)修改如下：　26. 条 23.572 修订如下：修订标题，以及修订段(a)引言文字和段(a)(1)。

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：收到两条关于提案经济影响的评论；一条涉及 §23.571 金属增压座舱结构，一条涉及 §23.572 金属机翼、尾翼及关联结构。这两条评论及美国联邦航空局（FAA）的答复均列入"修订讨论"部分。

英文原文：

Two comments were received regarding the economic impact of the proposals; one concerning Sec. 23.571, Metallic pressurized cabin structures, and one concerning Sec. 23.572, Metallic wing, empennage, and associated structures. Both of these comments, as well as the FAA's responses, are included in the section "Discussion of Amendments".

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：本提案将修订标题以增添“金属”一词，并修订 §23.572(a) 段，将适用范围限定为正常类、实用类和特技类飞机，以及进行次要编辑性修改。为与欧洲联合航空要求（JAR）23 协调统一，(a)(1) 段经修订要求进行试验或有试验证据支持的分析，如前言 §23.571 条所述。

英文原文：

This proposal would revise the heading to add the word "metallic" and revise Sec. 23.572 (a) to limit the applicability to normal, utility, and acrobatic category airplanes and to make minor editorial changes. Paragraph (a)(1) is revised to harmonize with JAR 23 by requiring tests or analysis supported by test evidence, as discussed under Sec. 23.571 of this preamble.

修订指令英文原文（供核对）：

26. Section 23.572 is amended by revising the heading; by revising paragraph (a) introductory text; and by revising paragraph (a)(1) to read as follows:

26. Section 23.572 is amended by revising the heading and by revising paragraphs (a) introductory text and (a)(1) to read as follows:

▪ 本版重印条文段落：(a)、(1)

### 2.7　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.572 的实际改动 |
| --- | --- | --- | --- |
| 73 FR 19746 | 2008-04-11 | 技术性修订 (Technical Amendment) | 重写 §23.573(b) 首句——损伤容限评定须含疲劳/腐蚀/意外损伤位置与模式 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.572 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1969-09-14 生效）　来源：Docket 8083，1969-08-01 签发，34 FR 13078

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The strength, detail design, and fabrication of those parts of the wing, wing carrythrough and attaching structure whose failure would be catastrophic must be evaluated under either of the following unless it is shown that the structure, operating stress level, materials, and expected use are comparable, from a fatigue standpoint, to a similar design that has had extensive satisfactory service experience: | 新增 |
| (1) | A fatigue strength investigation, in which the structure is shown by analysis, tests, or both, to be able to withstand the repeated loads of variable magnitude expected in service. | 新增 |
| (2) | A fail safe strength investigation in which it is shown by analysis, tests, or both, that catastrophic failure of the structure is not probable after fatigue failure, or obvious partial failure, of a principal structural element, and that the remaining structure is able to withstand a static ultimate load factor of 75 percent of the critical limit load factor at Vc. These loads must be multiplied by a factor of 1.15 unless the dynamic effects of failure under static load are otherwise considered. | 新增 |

### 3.2　版本2：Amdt 23-14（1973-12-20 生效）　来源：Docket 11011，1973-11-07 签发，38 FR 31816

修订

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (1) | A fatigue strength investigation, in which the structure is shown by analysis, tests, or both, to be able to withstand the repeated loads of variable magnitude expected in service. Analysis alone is acceptable only when it is conservative and applied to simple structures. | 修订 |

### 3.3　版本3：Amdt 23-34（1987-02-17 生效）　来源：Docket 23516，1987-01-08 签发，52 FR 1806

新增段落：(a)、(b)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (b) | For commuter category airplanes, unless it is shown that the structure, operating stress levels, materials, and expected use are comparable from a fatigue standpoint to similar design which has had a substantial satisfactory service experience, the strength, detail design, and the fabrication of those parts of the wing, wing carrythrough, vertical fin, horizontal stabilizer, and attaching structure whose failure would be catastrophic must be evaluated under either-- | 新增 |
| (1) | A fatigue strength investigation, in which the structure is shown by analysis, tests, or both, to be able to withstand the repeated loads of variable magnitude expected in service. Analysis alone is acceptable only when it is conservative and applied to simple structures; or | 修订 |
| (2) | A fail-safe strength investigation in which it is shown by analysis, tests, or both, that catastrophic failure of the structure is not probable after fatigue failure, or obvious partial failure, of a principal structural element, and that the remaining structure is able to withstand a static ultimate load factor of 75 percent of the critical limit load at VC. These loads must be multiplied by a factor of 1.15 unless the dynamic effects of failure under static load are otherwise considered. Source: SFAR 41, Sec. 5(d). | 新增 |

### 3.4　版本4：Amdt 23-38（1989-10-26 生效）　来源：Docket 25086，1989-09-21 签发

修订：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | The strength, detail design, and fabrication of those parts of the wings (including canards, tandem wings, and winglets/tip fins), empennage, their carry-through and attaching structures, whose failure would be catastrophic, must be evaluated under either of the following unless it is shown that the structure, operating stress level, materials, and expected uses are comparable, from a fatigue standpoint, to a similar design that has had extensive satisfactory service experience: | 新增 |
| (1) | A fatigue strength investigation in which the structure is shown by analysis, tests, or both to be able to withstand, the repeated loads of variable magnitude expected in service. Analysis alone is acceptable only when it is conservative and applied to simple structures; or | 修订 |
| (b) | Each evaluation required by this section must - | 修订 |
| (1) | Include typical loading spectra (e.g. taxi, ground-air-ground cycles, maneuver, gust); | 修订 |
| (2) | Account for any significant effects due to the mutual influence of aerodynamic surfaces; and | 修订 |
| (3) | Consider any significant effects from propeller slipstream loading, and buffet from vortex impingements. FOOTER: Issued in Washington, DC, on September 21, 1989. James B. Busey, Administrator. [FR Doc. 89-22707 Filed 9-25-89; 8:45 am] BILLING CODE 4010-13-M NPRM ACTIONS: Notice of Proposed Rulemaking. Notice No. 86-14; Issued on 09/11/86. FINAL RULE ACTIONS: Not Applicable. | 新增 |

### 3.5　版本5：Amdt 23-48（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

修订段落：(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | For normal, utility, and acrobatic category airplanes, the strength, detail design, and fabrication of those parts of the airframe structure whose failure would be catastrophic must be evaluated under one of the following unless it is shown that the structure, operating stress level, materials and expected uses are comparable, from a fatigue standpoint, to a similar design that has had extensive satisfactory service experience: | 修订 |
| (1) | A fatigue strength investigation in which the structure is shown by tests, or by analysis supported by test evidence, to be able to withstand the repeated loads of variable magnitude expected in service; or | 修订 |

### 3.6　段落结构演变矩阵（横向对照 6 个版本）

| 段落 | Amdt 23-7 (1969-09-14) | Amdt 23-14 (1973-12-20) | Amdt 23-34 (1987-02-17) | Amdt 23-38 (1989-10-26) | Amdt 23-48 (1996-03-11) |
| --- | --- | --- | --- | --- | --- |
| (a) | ● 新增 | △ 修订 | ○ 无 | ● 新增 | △ 修订 |
| (1) | ● 新增 | △ 修订 | △ 修订 | △ 修订 | △ 修订 |
| (2) | ● 新增 | ○ 无 | ● 新增 | △ 修订 | ○ 无 |
| (b) | ○ 无 | ○ 无 | ● 新增 | △ 修订 | ○ 无 |
| (3) | ○ 无 | ○ 无 | ○ 无 | ● 新增 | ○ 无 |

## 4　§23.572 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | posed amendment to Sec. 23.607 is withdrawn pending further study. The notice proposed to amend Sec. 23.611 to require that accessibility to parts be adequate for the repair and replacement aspects of necessary maintenan… | 初始版 |
| 事故调查与研究驱动 | and associated structure. (a) * * * (1) A fatigue strength investigation, in which the structure is shown by analysis, tests, or both, to be able to withstand the repeated loads of variable magnitude expected in service… | Amdt 23-14 |
| 事故调查与研究驱动 | hic must be evaluated under either-- (1) A fatigue strength investigation, in which the structure is shown by analysis tests, or both, to be able to withstand the repeated loads of variable magnitude expected in service.… | Amdt 23-34 |
| 事故调查与研究驱动 | ive satisfactory service experience: (1) A fatigue strength investigation in which the structure is shown by analysis, tests, or both to be able to withstand, the repeated loads of variable magnitude expected in service.… | Amdt 23-38 |
| 新技术 / 新构型 | erence proposal 225 recommends that Section 23.572 apply to canard and tandem wing configurations as well as the main wing. Conference proposal 228 recommends excluding airplanes of less than 1500 pounds from the require… | Amdt 23-45 |
| 事故调查与研究驱动 | ive satisfactory service experience: (1) A fatigue strength investigation in which the structure is shown by tests, or by analysis supported by test evidence, to be able to withstand the repeated loads of variable magnit… | Amdt 23-48 |

本条取证提示 共 6 个历史版本：Amdt 23-7, Amdt 23-14, Amdt 23-34, Amdt 23-38, Amdt 23-45, Amdt 23-48。 其中 5 个版本在本地文献中检索到明确修订指令原文；5 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：73 FR 19746，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 8083 | 34 FR 13078 | FinalRule_Docket 8083_Amdt23-7_34FR13078_1969-08-13_0002.txt |
| [2] | NPRM（提案） | Notice 67-14 / Docket 8083 | 32 FR 5791 | NPRM_Notice 67-14_Docket8083_32FR5791_1967-04-11.txt |
| [3] | Final Rule（颁行） | Docket 11011 | 38 FR 31816 | FinalRule_Docket 11011_Amdt23-14_38FR31816_1973-11-19_0002.txt |
| [4] | NPRM（提案） | Notice 71-13 / Docket 11011 | 36 FR 8398 | NPRM_Notice 71-13_Docket11011_36FR8398_1971-05-05.txt |
| [5] | Final Rule（颁行） | Docket 23516 | 52 FR 1806 | FinalRule_Docket 23516_Amdt21-59_23-34_36-13_52FR_1987-01-15.txt |
| [6] | Final Rule（颁行） | Docket 23516 | 55 FR 18570 | FinalRule_Docket 23516_Amdt23-39_55FR18570_1990-05-02_0002.txt |
| [7] | NPRM（提案） | Notice 83-17 / Docket 23516 | 48 FR 52010 | NPRM_Notice 83-17_Docket23516_48FR52010_1983-11-15.txt |
| [8] | Final Rule（颁行） | Docket 25086 | （FR 引证未在文本中标注） | FinalRule_Docket 25086_Amdt23-38_1989-09-26_0002.txt |
| [9] | NPRM（提案） | Notice 86-14 / Docket 25086 | 51 FR 33700 | NPRM_Notice 86-14_Docket25086_51FR33700_1986-09-22.txt |
| [10] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [11] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [12] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [13] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [14] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [15] | 更正通告 / 校正 | 73 FR 19746 | 2008-04-11 | TechAmend_73FR19746_2008-04-11_E8-7649.clean.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.572 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-7 | 1969-09-14 | 4d1def83-5dab-4198-8ec7-64d4b87af982 |
| Amdt 23-14 | 1973-12-20 | 052670b3-3d5e-45bc-a49e-eb1b00b60e45 |
| Amdt 23-34 | 1987-02-17 | 87d7718d-e0ff-44c2-89e6-15183af36976 |
| Amdt 23-38 | 1989-10-26 | 5854d0bc-9172-49b7-9f35-8d687c135eff |
| Amdt 23-45 | 1993-09-07 | 61ec3e1b-0460-414d-b5b4-26fbc7f23eef |
| Amdt 23-48 | 1996-03-11 | 70eb8fa3-db1e-48ec-af55-fcdc2ce98593 |

# § 23.573　结构损伤容限与疲劳评定（Damage tolerance and fatigue evaluation of structure.）

1993-09-07 初版　｜　2008-04-11 Amdt 23-48　｜　共 2 个历史版本

## 1　§23.573 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.573 |
| 条款标题（英文原文） | Damage tolerance and fatigue evaluation of structure. |
| 中文名称 | 结构损伤容限与疲劳评定 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 2 版（含 1993-09-07 初始版） |
| 时间跨度 | 1993-09-07 → 2008-04-11 （约 15 年） |
| 初版来源 | Docket 26269，1993-07-28 签发，58 FR 42136 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 58 FR 51970、73 FR 19746 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.573 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-45 | 1993-09-07 | Notice 90-18，1990-06-15 签发，55 FR 26534 | Docket 26269，1993-07-28 签发，58 FR 42136 | 新增条款：(a) |
| 2 | Amdt 23-48 | 2008-04-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 Docket 27805，1996-01-29 签发，61 FR 5138 Docket 27805，2008-04-07 签发，61 FR 5138 | 新增内容：(b)、(c)、(a) |

### 1.3　修订脉络一览

1993 初版　新增条款：(a)　→　2008 23-48　新增内容：(b)、(c)、(a)

## 2　§23.573 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1993-09-07，Amdt 23-45）：结构损伤容限与疲劳评定

▪ 修订沿革：本版由 NPRM　Notice 90-18，1990-06-15 签发，55 FR 26534　提出；最终规章 Docket 26269，1993-07-28 签发，58 FR 42136，1993-09-07 生效。

▪ 修订指令（中文译文）：49. 在“疲劳评定”标题下，新增第 23.573 条，内容如下：　8. 提议的第 23.573(f) 条在最终规章中被删除。该段描述了在损伤容限评定中必须考虑的载荷谱、载荷截取和损伤类型。其中包含关于试验方法的咨询材料，并未包含任何试验要求。尽管该段被删除，但所确定的主题在任何损伤容限评定中都必须予以考虑并形成文件。　48. 在“水载荷”标题下新增第 23.573 条，内容如下：

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（1）：CAA-Australia 认为，当存在不易立即察觉的制造或服役损伤时，FRP 结构应设计为能够承受极限载荷。这一立场是基于对实际损伤起始、扩展速率、检查困难以及易受不可见意外损伤影响的材料缺乏了解。CAA-Australia 对拟议的第 23.573 条提出以下补充意见。

英文原文：

The CAA-Australia believes that FRP structures should be designed to carry the ultimate load when manufacturing or service damage exists that is not immediately obvious. This position is based on the lack of knowledge about actual damage initiation, propagation rates, inspection difficulties, and material that is vulnerable to invisible accidental damage. The CAA-Australia offers the following additional comments on proposed Section 23.573.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（2）：欧洲联合航空局（JAA）的评论指出，欧洲联合航空要求（JAR）23中关于复合材料的拟议条款同样基于美国联邦航空局（FAA）近期发布的专用条件，因此在技术上与拟议的新§23.573基本实现了协调统一。然而，欧洲联合航空局（JAA）指出了若干关切：

英文原文：

The JAA comment states that the proposed provisions of JAR 23 relating to composites are also based on recently issued FAA special conditions and is therefore largely technically harmonized with the proposed new Section 23.573. However, the JAA notes several concerns:

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（3）：美国联邦航空局审查了上述评论，并总体上表示同意。此前为复合材料飞机颁发的专用条件被用作本拟议新增条的依据。这些专用条件中有许多是在AC 20-107发布之前编制并颁发的。因此，当时没有可用于复合材料结构的指导，这些专用条件中加入可接受的符合性方法是适当的。规章应限于型号合格证申请人须满足的最低适航标准，可接受的符合性方法应纳入咨询通告。美国联邦航空局认为，AC 20-107包含了满足拟议第23.573条要求所需的大部分指导。如有需要，美国联邦航空局将制定并发布补充指导。根据评论意见，美国联邦航空局仔细审查了该提案，并从最终规章中删除了多余材料和指导材料。美国联邦航空局同意并总体接受通用航空制造商协会（GAMA）提交的对拟议第23.573条的全部重写。
通用航空制造商协会（GAMA）的改写文本清楚地提出了复合材料的强制性要求，§23.573(a) 段；金属结构的可选损伤容限设计标准，§23.573(b) 段；以及检查规定，§23.573(c) 段。以下各段应有助于读者理解从 NPRM 到本最终规章的过渡：

英文原文：

The FAA reviewed the above comments and, in general, concurs. Special conditions issued earlier for composite airplanes were used as the basis for this proposed new section. Many of these special conditions were prepared and issued before AC 20-107 was issued. Therefore, there was no guidance available for composite structures and it was appropriate for those special conditions to include acceptable means of compliance. The regulations should be limited to minimum airworthiness standards to be met by an applicant for a type certificate and the acceptable means of compliance should be included in advisory circulars. The FAA finds that AC 20-107 contains much of the guidance needed for compliance with the requirement in proposed Section 23.573. If there is a need, the FAA will develop and issue additional guidance. Based on the comments, the FAA has carefully reviewed the proposal and has deleted the redundant material and the guidance material from the final rule.

The FAA agrees with and generally accepts the complete rewrite of proposed Section 23.573 submitted by the GAMA. The GAMA rewrite clearly presents mandatory requirements for composite materials, Section 23.573(a); optional damage tolerance design standards for metallic structures, Section 23.573(b); and inspection provisions, Section 23.573(c).

The following paragraphs should assist the reader's understanding of the transition from NPRM to this final rule:

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（4）：美国联邦航空局（FAA）同意并总体上接受通用航空制造商协会（GAMA）提交的对拟议第23.573条的全面改写。GAMA的改写清晰呈现了复合材料强制要求（第23.573(a)段）、金属结构可选损伤容限设计标准（第23.573(b)段）以及检查规定（第23.573(c)段）。以下段落应有助于读者理解从规章制定提案通告到本最终规章的过渡：

英文原文：

The FAA agrees with and generally accepts the complete rewrite of proposed Section 23.573 submitted by the GAMA. The GAMA rewrite clearly presents mandatory requirements for composite materials, Section 23.573(a); optional damage tolerance design standards for metallic structures, Section 23.573(b); and inspection provisions, Section 23.573(c).

The following paragraphs should assist the reader's understanding of the transition from NPRM to this final rule:

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（5）：最终规章§23.573(a)段包含规章制定提案通告中引言文字所载的规定。该段最后一句源自通告中的拟议§23.573(j)段。拟议§23.573(j)段包含适用于所有复合材料结构的一般要求。该要求本应也纳入通告引言文字中的一般要求内。§23.573(j)段中的"材料变异性和环境条件"字样涵盖了提案中曾详细列出、并在最终规章中已删除的一系列条件（如温度和湿度）。咨询通告AC 20-107包含有关该主题的信息。

英文原文：

1. Section 23.573(a) of the final rule contains the provisions included in the introductory text in the NPRM. The last sentence of this paragraph comes from proposed Section 23.573(j) in the notice. Proposed Section 23.573(j) contained general requirements applicable to all composite structures. It also should have been included in the general requirements of the introductory text in the notice. The words "material variability and environmental conditions" in Section 23.573(j) cover the list of conditions, such as temperature and humidity, that were spelled out in the proposal and that are removed from the final rule. AC 20-107 contains information about this topic.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（6）：5. §23.573(a)(6) 段源自规章制定提案通告（NPRM）中的 §23.573(k) 段。本段已重写，以与本条其他段保持一致。

英文原文：

5. Section 23.573(a)(6) comes from Section 23.573(k) in the NPRM. This paragraph is rewritten for consistency with the other paragraphs in this section.

来源：Final Rule｜58 FR 42136｜FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt

▪ 背景与评论处置（7）：6. 最终规章在§23.573(b)段中提供了这些要求，而非规章制定提案通告中§23.573(a)段为金属结构提出的复合材料损伤容限要求。

英文原文：

6. Instead of the composite damage tolerance requirements proposed for metallic structures by Section 23.573(a) in the NPRM, the final rule provides these requirements in Section 23.573(b).

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（8）：说明：共有五项会议提案针对第23.572条，另有一项提议新增第23.573条。

英文原文：

Explanation: There are five conference proposals directed at Section 23.572 and one that proposes a new Section 23.573.

来源：NPRM（提案）｜55 FR 26534｜NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt

▪ 背景与评论处置（9）：说明：本提案建议修订第23部，新增第23.573条，适用于复合材料结构，并为申请人提供使用损伤容限设计作为金属结构所需的第23.571条和第23.572条安全寿命/破损安全设计理念的替代方案的机会。该新条建议对复合材料提出强制要求，并为金属结构提供可选的设计理念。
最初的会议提案基本上是重新编纂第25.571条的相关部分，在会议上主要遭到反对，理由是：(1) 将第25部的要求任意插入第23部；(2) 如果申请人选择，更严格的第25部要求可能被用于小型飞机的取证基础；(3) 即使作为可选方案，损伤容限准则也可能日后导致第23部设计理念的改变。

英文原文：

Explanation: This proposal recommends amending part 23 to add a new Section 23.573, applicable to composite structure and to provide the applicant the opportunity to use damage-tolerant design as an alternative to the safe-life/fail-safe design philosophies required by Sections 23.571 and 23.572 for metallic structure. This new section proposes a mandatory requirement for composite materials and offers an optional design philosophy for metallic structure.

The initial conference proposal on this subject was essentially a recodification of the pertinent parts of Section 25.571 and was predominantly opposed at the conference on the basis that (1) it was an arbitrary insertion of part 25 requirements into part 23; (2) if chosen by the applicant, more stringent part 25 requirements could be used in the certification basis of a small airplane; and (3) even as an option, the damage-tolerant criteria might later result in a change in part 23 design philosophy.

修订指令英文原文（供核对）：

49. Under the heading "Fatigue Evaluation," a new Section 23.573 is added to read as follows:

8. Proposed Section 23.573(f) is deleted in the final rule. This paragraph described load spectra, load truncation, and types of damage that must be considered in the damage tolerance evaluation. It contained advisory material on testing methods and did not contain any testing requirements. Though this paragraph is removed, the topics identified must be considered and documented in any damage tolerance evaluation.

48. A new Section 23.573 is added under the heading "Water Loads" to read as follows:

### 2.2　Amdt 23-48（2008-04-11 生效）：修订§23.573 条

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196；Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138；Docket 27805，1996-01-29 签发，61 FR 5138；Docket 27805，2008-04-07 签发，61 FR 5138，2008-04-11 生效。

▪ 修订指令（中文译文）：27. 修订§23.573 条，方法是：删除 (b) 段中引用的“§23.571(c) 段”并在其位置添加引用“§23.571(a)(3) 段”；删除 (c) 段；并修改 (a)(5) 段的导语，使其内容如下：　2. 修改 §23.573 条，将 (b) 段导语中的第一句修改为如下内容：　27. 修订第 23.573 条：删除 (b) 段中的引用“§23.571(c) 段”并代之以“§23.571(a)(3) 段”；删除 (c) 段；并修改 (a)(5) 段的导言部分，使其改读如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议修订§23.571，将其标题从"增压座舱"改为"金属增压座舱结构"，因为非金属结构在§23.573(a)中另有规定。美国联邦航空局（FAA）提议修订引言部分，将适用范围限定为正常类、实用类和特技类，因为通勤类飞机另有规定。美国联邦航空局（FAA）提议修订(a)段，要求疲劳强度调查表明结构能够承受运行中预期的可变幅值重复载荷。

英文原文：

The FAA proposed to revise Sec. 23.571 by changing the heading from "Pressurized cabin" to "Metallic pressurized cabin structures" because nonmetallic structure is addressed in Sec. 23.573(a). The FAA proposed to revise the introductory text to limit the applicability to normal, utility, and acrobatic categories because commuter category airplanes are addressed separately. The FAA proposed to revise paragraph (a) to require the fatigue strength investigation to show that the structure can withstand repeated loads of variable magnitude expected in service.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局（FAA）提议修订 §23.573(a)(5) 段，以明确胶接接头的限制载荷能力仅在该胶接接头的失效会导致飞机的灾难性损失时才必须予以证实。美国联邦航空局（FAA）提议删除 §23.573(c) 段，因为其关于检查和其他程序的要求拟移至 §23.575 段。

英文原文：

The FAA proposed to amend Sec. 23.573(a)(5) to make clear that the limit load capacity of a bonded joint must be substantiated only if the failure of the bonded joint would result in catastrophic loss of the airplane.

The FAA proposed to delete Sec. 23.573(c) because its requirements for inspections and other procedures were proposed to be moved to Sec. 23.575.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（3）：美国联邦航空局（FAA）曾提议删除§23.573(c)段，因为其关于检查和其他程序的要求被提议移至§23.575。

英文原文：

The FAA proposed to delete Sec. 23.573(c) because its requirements for inspections and other procedures were proposed to be moved to Sec. 23.575.

来源：Final Rule｜—｜FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt

▪ 背景与评论处置（4）：本次技术修订将修正§23.573(b)段，以正确引用§23.571(c)段。

英文原文：

This technical amendment will correct Sec. 23.573(b) to properly reference Sec. 23.571(c).

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（5）：将修订第23.571条，将标题从"Pressurized cabin"修改为"Metallic pressurized cabin structure"，因为非金属结构在第23.573(a)条中述及。将修订引言文字，将适用范围限于正常类、实用类和特技类，因为通勤类飞机另行述及。将修订(a)段，要求疲劳强度调查表明结构能够承受使用中预计出现的量值变化的重复载荷。目前，疲劳强度可通过试验或分析或两者并用表明。根据拟议修订，结构强度必须通过试验或由试验证据支持的分析表明。

英文原文：

Section 23.571 would be revised by changing the heading from "Pressurized cabin" to "Metallic pressurized cabin structure" because nonmetallic structure is addressed in Sec. 23.573 (a). The introductory text would be revised to limit the applicability to normal, utility, and acrobatic category only because commuter category airplanes are addressed separately. Paragraph (a) would be revised to require the fatigue strength investigation to show that the structure can withstand repeated loads of variable magnitude expected in service. Currently, fatigue strength may be shown by tests or analysis or both. Under the proposed revision, structural strength must be shown by tests or by analysis supported by test evidence.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（6）：本提案拟修订§23.573(a)(5)以澄清该规章，因为按现有表述，该条款容易被误读。改写后的要求使用"any"而非"each"一词，以表明存在另一限制因素。同时调整了各条款的顺序，以避免该规章涉及"限制载荷承载能力的失效"。改写后的文本明确指出，"每个胶接接头均须通过试验予以证实"并非预期结果。
尽管§23.573(b)段的格式与JAR 23.573(b)不完全相同，美国联邦航空局（FAA）并未提议修订该段。虽然现行《联邦规章汇编》第14篇§23.573(b)段包含两个子段，而JAR 23.573(b)（JAR 23-Post Consultation）包含六个子段，但两者在技术上是等同的。
本提案拟删除§23.573(c)。检查及其他程序将移至§23.575，并适用于涉及疲劳评定的四个条款，即§23.571、§23.572、§23.573和§23.574。

英文原文：

This proposal would amend Sec. 23.573 (a)(5) to clarify the regulation because, as written, it could be easily misread. The rewritten requirement uses the word "any" rather than "each" to indicate that another limiting factor exists. It also changes the order of the clauses to prevent the regulation from addressing "failure of the limit load capacity." The rewritten text makes it clear that "Each bonded joint is required to be substantiated by tests" is not the desired result.

The FAA is not proposing a revision to paragraph (b) even though it is not identical in format to JAR 23.573 (b). While current FAR Sec. 23.573 (b) of title 14 contains two subparagraphs and JAR 23.573 (b) (JAR 23-Post Consultation) contains six subparagraphs, the two are technically identical.

This proposal would delete Sec. 23.573 (c). Inspections and other procedures would be moved to Sec. 23.575 and be made applicable to four sections pertaining to fatigue evaluation, namely, Secs. 23.571, 23.572, 23.573 and 23.574.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（7）：美国联邦航空局不提议修订(b)段，尽管其格式与JAR 23.573(b)不完全相同。虽然现行《联邦规章汇编》第14篇的FAR第23.573(b)条包含两个子段，而JAR 23.573(b)（JAR 23—咨询后）包含六个子段，但两者在技术上是相同的。
本提案将删除第23.573(c)条。检查和其他程序将移至第23.575条，并适用于与疲劳评定相关的四个条款，即第23.571、23.572、23.573和23.574条。

英文原文：

The FAA is not proposing a revision to paragraph (b) even though it is not identical in format to JAR 23.573 (b). While current FAR Sec. 23.573 (b) of title 14 contains two subparagraphs and JAR 23.573 (b) (JAR 23-Post Consultation) contains six subparagraphs, the two are technically identical.

This proposal would delete Sec. 23.573 (c). Inspections and other procedures would be moved to Sec. 23.575 and be made applicable to four sections pertaining to fatigue evaluation, namely, Secs. 23.571, 23.572, 23.573 and 23.574.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（8）：本提案将删除 §23.573(c) 段。检查和其他程序将移至 §23.575 条，并适用于涉及疲劳评定的四个条，即 §23.571、§23.572、§23.573 和 §23.574 条。

英文原文：

This proposal would delete Sec. 23.573 (c). Inspections and other procedures would be moved to Sec. 23.575 and be made applicable to four sections pertaining to fatigue evaluation, namely, Secs. 23.571, 23.572, 23.573 and 23.574.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（9）：第 23.573(c) 条将被移至第 23.575 条并进行修订。此次修订包括列出所包含的那些要求，即第 23.571、23.572、23.573 和 23.574 条。这四条针对金属飞机的增压座舱、机翼、尾翼（尾部）及相关结构作出了规定。它们还提供了复合材料和金属飞机结构的损伤容限和疲劳评定标准。拟议的第 23.575 条将在技术上使这些规则与欧洲联合航空要求（JAR）23 协调统一，但采用简单的格式。

英文原文：

Section 23.573 (c) would be moved to Sec. 23.575 and revised. The revision consists of naming those requirements that are included, namely Secs. 23.571, 23.572, 23.573, and 23.574. These four sections address pressurized cabin, wing, empennage (tail), and associated structures for metallic airplanes. They also provide standards for damage tolerance and fatigue evaluations of both composite and metallic airplane structures. Proposed Sec. 23.575 would harmonize these rules with JAR 23 technically, but a simple format.

修订指令英文原文（供核对）：

27. Section 23.573 is amended by removing the reference in paragraph (b) "Sec. 23.571(c)" and adding the reference "Sec. 23.571(a)(3)" in its place; by removing paragraph (c); and by revising the introductory text of paragraph (a)(5) to read as follows:

2. Amend Sec. 23.573 by revising the first sentence in paragraph (b) introductory text to read as follows:

27. Section 23.573 is amended by removing the reference in paragraph(b) "Sec. 23.571(c)" and adding in its place "Sec. 23.571(a)(3)"; by removing paragraph (c); and by revising the introductory text of paragraph (a)(5) to read as follows:

▪ 本版重印条文段落：(a)、(5)

### 2.3　更正通告与 CFR 编纂校正

CFR 官方溯源注通常只登记修订案的 FR 引证，更正通告多被吞并或只留一个裸页码；CFR 编纂校正（OFR 在年度版排印时发出的 Correction）更是不进溯源注。下列文献经官方 CFR 溯源注、federalregister.gov API 与 govinfo 影印件三源交叉核实。

| FR 引证 | 日期 | 性质 | 对 §23.573 的实际改动 |
| --- | --- | --- | --- |
| 58 FR 51970 | 1993-10-05 | 更正通告 (CFR 溯源注) | 32 条印刷更正；含 appendix H→appendix I、§23.613/23.865/23.23 等编号错植 ¦ 重写 §23.573(b) 首句——损伤容限评定须含疲劳/腐蚀/意外损伤位置与模式 |
| 73 FR 19746 | 2008-04-11 | 技术性修订 (Technical Amendment) | 32 条印刷更正；含 appendix H→appendix I、§23.613/23.865/23.23 等编号错植 ¦ 重写 §23.573(b) 首句——损伤容限评定须含疲劳/腐蚀/意外损伤位置与模式 |

引用提示 更正件通常改动拼写、交叉引用编号或表头，不产生新的 Amdt 号，DRS 条款版本索引与 CFR 溯源注都可能漏记；引用时请以现行 CFR 文本为准。

## 3　§23.573 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1993-09-07 生效）　来源：Docket 26269，1993-07-28 签发，58 FR 42136

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Composite airframe structure. Composite airframe structure must be evaluated under this paragraph instead of Sections 23.571 and 23.572. The applicant must evaluate the composite airframe structure, the failure of which would result in catastrophic loss of the airplane, in each wing (including canards, tandem wings, and winglets), empennage, their carry through and attaching structure, moveable control surfaces and their attaching structure fuselage, and pressure cabin using the damage-tolerance criteria prescribed in paragraphs (a)(1) through (a)(4) of this section unless shown to be impractical. If the applicant establishes that damage-tolerance criteria is impractical for a particular structure, the structure must be evaluated in accordance with paragraphs (a)(1) and (a)(6) of this section. Where bonded joints are used, this structure must also be evaluated in accordance with paragraph (a)(5) of this section. The effects of material variability and environmental conditions on the strength and durability properties of the composite materials must be accounted for in the evaluations required by this section. | 新增 |
| (1) | It must be demonstrated by tests, or by analysis supported by tests, that the structure is capable of carrying ultimate load with damage up to the threshold of delectability considering the inspection procedures employed. | 新增 |
| (2) | The growth rate or no-growth of damage that may occur from fatigue, corrosion, manufacturing flaws or impact damage under repeated loads expected in service, must be established by tests or analysis supported by tests. | 新增 |
| (3) | The structure must be shown by residual strength tests, or analysis supported by residual strength tests, to be able to withstand critical limit flight loads, considered as ultimate loads, with the extent of detectable damage consistent with the results of the damage tolerance evaluations. For pressurized cabins, the following loads must be withstood; | 新增 |
| (i) | Critical limit flight loads with the combined effects of normal operating pressure and expected external aerodynamic pressures. | 新增 |
| (ii) | The expected external aerodynamic pressures in 1g flight combined with a cabin differential pressure equal to 1.1 times the normal operating differential pressure without any other load. | 新增 |
| (4) | The damage growth, between initial detectability and the value selected for residual strength demonstrations, factored to obtain inspection intervals, must allow development of an inspection program suitable for application by operation and maintenance personnel. | 新增 |
| (5) | The limit load capacity of each bonded joint must be substantiated by one of the following methods; | 新增 |
| (i) | The maximum disbonds of each bonded joint consistent with the capability to withstand the loads in paragraph (a)(3) of this section must be determined by analysis, tests, or both. Disbonds of each bonded joint greater than this must be prevented by design features; or | 新增 |
| (ii) | Proof testing must be conducted on each production article that will apply the critical limit design load to each critical bonded joint; or | 新增 |
| (iii) | Repeatable and reliable non-destructive inspection techniques must be established that ensure the strength of each joint. | 新增 |
| (6) | Structural components for which the damage tolerance method is shown to be impractical must be shown by component fatigue tests (or analysis supported by tests) to be able to withstand the repeated loads of variable magnitude expected in service. Sufficient component, subcomponent, element, or coupon tests must be done to establish the fatigue scatter factor and the environmental effects. Damage up to the threshold of detectability and ultimate load residual strength capability must be considered in the demonstration. | 新增 |
| (b) | Metallic airframe structure. If the applicant elects to use Section 23.571(c) or Section 23.472(a)(3), then the damage tolerance evaluation must include a determination of the probable locations and modes of damage due to fatigue, corrosion, or accidental damage. The determination must be by analysis supported by test evidence and, if available, service experience. Damage at multiple sites due to fatigue must be included where the design is such that this type of damage can be expected to occur. The evaluation must incorporate repeated load and static analyses supported by test evidence. The extent of damage for residual strength evaluation at any time within the operational life of the airplane must be consistent with the initial detectability and subsequent growth under repeated loads. The residual strength evaluation must show that the remaining structure is able to withstand critical limit flight loads, considered as ultimate, with the extent of detectable damage consistent with the results of the damage tolerance evaluations. For pressurized cabins, the following load must be withstood: | 新增 |
| (1) | The normal operating differential pressure combined with the expected external aerodynamic pressures applied simultaneously with the flight loading conditions specified in this part, and | 新增 |
| (2) | The expected external aerodynamic pressures in 1g flight combined with a cabin differential pressure equal to 1.1 times the normal operating differential pressure without any other load. | 新增 |
| (c) | Inspection. Based on evaluations required by this section, inspections or other procedures must be established as necessary to prevent catastrophic failure and must be included in the Airworthiness Limitations section of the Instructions for Continued Airworthiness required by Section 23.1529. | 新增 |

### 3.2　版本2：Amdt 23-48（2008-04-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138；Docket 27805，1996-01-29 签发，61 FR 5138；Docket 27805，2008-04-07 签发，61 FR 5138

新增内容：(b)、(c)、(a)

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | * * * | 修订 |
| (5) | For any bonded joint, the failure of which would result in catastrophic loss of the airplane, the limit load capacity must be substantiated by one of the following methods-- | 修订 |

### 3.3　段落结构演变矩阵（横向对照 2 个版本）

| 段落 | Amdt 23-45 (1993-09-07) | Amdt 23-48 (2008-04-11) |
| --- | --- | --- |
| (a) | ● 新增 | △ 修订 |
| (1) | ● 新增 | ○ 无 |
| (2) | ● 新增 | ○ 无 |
| (3) | ● 新增 | ○ 无 |
| (i) | ● 新增 | ○ 无 |
| (ii) | ● 新增 | ○ 无 |
| (4) | ● 新增 | ○ 无 |
| (5) | ● 新增 | △ 修订 |
| (iii) | ● 新增 | ○ 无 |
| (6) | ● 新增 | ○ 无 |
| (b) | ● 新增 | ○ 无 |
| (c) | ● 新增 | ○ 无 |

## 4　§23.573 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 事故调查与研究驱动 | difficulties, and material that is vulnerable to invisible accidental damage. The CAA-Australia offers the following additional comments on proposed Section 23.573. The JAA comment states that the proposed provisions of… | 初始版 |
| 事故调查与研究驱动 | sed to revise paragraph (a) to require the fatigue strength investigation to show that the structure can withstand repeated loads of variable magnitude expected in service. The FAA proposed to amend Sec. 23.573(a)(5) to… | Amdt 23-48 |

本条取证提示 共 2 个历史版本：Amdt 23-45, Amdt 23-48。 其中 2 个版本在本地文献中检索到明确修订指令原文；2 个版本检索到重印全文。 本条另有更正通告 / CFR 编纂校正：58 FR 51970、73 FR 19746，DRS 与 CFR 溯源注均可能漏记。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 26269 | 58 FR 42136 | FinalRule_Docket 26269_Amdt23-45_58FR42136_1993-08-06_0003.txt |
| [2] | NPRM（提案） | Notice 90-18 / Docket 26269 | 55 FR 26534 | NPRM_Notice 90-18_Docket26269_55FR26534_1990-06-28.txt |
| [3] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [4] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [5] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |
| [6] | 更正通告 / 校正 | 58 FR 51970 | 1993-10-05 | Correction_58FR51970_1993-10-05_Amdt23-45.clean.txt |
| [7] | 更正通告 / 校正 | 73 FR 19746 | 2008-04-11 | TechAmend_73FR19746_2008-04-11_E8-7649.clean.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.573 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-45 | 1993-09-07 | cc5d4235-d9da-4fc6-b134-a18a2548882c |
| Amdt 23-48 | 2008-04-11 | d67ac3cd-6d99-4644-a3f1-a20680c745bb |

# § 23.574　通勤类飞机金属结构损伤容限与疲劳评定（Metallic damage tolerance and fatigue evaluation of commuter category airplanes.）

1996-03-11 初版　｜　1996-03-11 Amdt 23-48　｜　共 1 个历史版本

## 1　§23.574 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.574 |
| 条款标题（英文原文） | Metallic damage tolerance and fatigue evaluation of commuter category airplanes. |
| 中文名称 | 通勤类飞机金属结构损伤容限与疲劳评定 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1996-03-11 初始版） |
| 时间跨度 | 1996-03-11 → 1996-03-11 （约 0 年） |
| 初版来源 | Docket 27805，1996-01-29 签发，61 FR 5138 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.574 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增条款：(b)、(a) |

### 1.3　修订脉络一览

1996 初版　新增条款：(b)、(a)

## 2　§23.574 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1996-03-11，Amdt 23-48）：通勤类飞机金属结构损伤容限与疲劳评定

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：28. 增设第 23.574 条，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）提议增加新的§23.574条，规定通勤类飞机的损伤容限和疲劳评定要求。如前所述，正在修订§23.571和§23.572条，以明确这些条

英文原文：

The FAA proposed to add a new Sec. 23.574 that addresses damage tolerance and fatigue evaluation requirements for commuter category airplanes. As discussed previously, Secs. 23.571 and 23.572 are being revised to clarify that these sections apply only to normal, utility, and acrobatic category airplanes. Newly type certificated commuter category airplanes would have to meet proposed Sec. 23.574 instead of Secs. 23.571 and 23.572.

The only comment received on this proposed new section is a JAA statement that this change will be considered for JAR 23. The proposal is adopted as proposed.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（2）：本提案将新增一个 §23.574 条，用以具体规定通勤类飞机的损伤容限和疲劳评定要求。英国民用航空局建议修订欧洲联合航空要求（JAR）23.571 和 23.572，以要求通勤类飞机符合这些条中的破损安全条款，从而删除

英文原文：

This proposal would add a new Sec. 23.574 that would delineate the damage tolerance and fatigue evaluation requirement for commuter category airplanes. The United Kingdom Civil Aviation Authority proposed to revise JAR 23.571 and 23.572 to require commuter category airplanes to meet the fail-safe provisions of those sections and, thus, remove the safe-life provisions. The FAA representative agree with the intent of the proposal but could not agree with any specific recommendation because the FAA was in the process of determining requirements for commuter category airplanes in the aging aircraft program. The majority of the subgroup decided they would not recommend the United Kingdom Civil Aviation Authority proposal.

修订指令英文原文（供核对）：

28. A new Sec. 23.574 is added to read as follows:

## 3　§23.574 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

### 3.1　版本1：初始版（1996-03-11 生效）　来源：Docket 27805，1996-01-29 签发，61 FR 5138

1965 年重编版，源自 CAR Part 3 相应条款。

| 段落 | 条文内容（英文原文） | 状态 |
| --- | --- | --- |
| (a) | Metallic damage tolerance. An evaluation of the strength, detail design, and fabrication must show that catastrophic failure due to fatigue, corrosion, defects, or damage will be avoided throughout the operational life of the airplane. This evaluation must be conducted in accordance with the provisions of Sec. 23.573, except as specified in paragraph (b) of this section, for each part of the structure that could contribute to a catastrophic failure. | 新增 |
| (b) | Fatigue (safe-life) evaluation. Compliance with the damage tolerance requirements of paragraph (a) of this section is not required if the applicant establishes that the application of those requirements is impractical for a particular structure. This structure must be shown, by analysis supported by test evidence, to be able to withstand the repeated loads of variable magnitude expected during its service life without detectable cracks. Appropriate safe-life scatter factors must be applied. | 新增 |

### 3.2　段落结构演变矩阵（横向对照 1 个版本）

| 段落 | Amdt 23-48 (1996-03-11) |
| --- | --- |
| (a) | ● 新增 |
| (b) | ● 新增 |

## 4　§23.574 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 国际协调 | The only comment received on this proposed new section is a JAA statement that this change will be considered for JAR 23. The proposal is adopted as proposed. This proposal would add a new Sec. 23.574 that would delineat… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；1 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [2] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [3] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.574 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-48 | 1996-03-11 | 93c505e9-4444-433c-b03a-8071198deb68 |

# § 23.575　检查与其他程序（Inspections and other procedures.）

1996-03-11 初版　｜　1996-03-11 Amdt 23-48　｜　共 1 个历史版本

## 1　§23.575 修订历史汇总

### 1.1　条款基本信息

| 项目 | 内容 |
| --- | --- |
| 条款号 | 14 CFR § 23.575 |
| 条款标题（英文原文） | Inspections and other procedures. |
| 中文名称 | 检查与其他程序 |
| 所属分部 | C 分部 结构 |
| 历史版本数 | 1 版（含 1996-03-11 初始版） |
| 时间跨度 | 1996-03-11 → 1996-03-11 （约 0 年） |
| 初版来源 | Docket 27805，1996-01-29 签发，61 FR 5138 |
| 当前状态 | Historical。2017 年 Amdt 23-64 性能化重构后本条体系已改写，本卷记录 1965–2017 年间的历史沿革。 |
| 更正通告 / CFR 编纂校正 | 无 |

### 1.2　修订沿革总表（按时间排序）

| # | 修订号 | 生效日期 | NPRM（编号 / 签发 / FR 引证） | Final Rule（案号 / 签发 / FR 引证） | 本版对 §23.575 的修订内容 |
| --- | --- | --- | --- | --- | --- |
| 1 | Amdt 23-48 | 1996-03-11 | Notice 94-20，1994-07-05 签发，59 FR 35196 | Docket 27805，1996-01-29 签发，61 FR 5138 | 新增条款 |

### 1.3　修订脉络一览

1996 初版　新增条款

## 2　§23.575 各阶段修订背景与原因深度剖析

本节逐条直译 NPRM 与 Final Rule 中与本条有关的原始论述——提案说明、FAA 对评论的答复、以及修订理由。为便于核对，每段中文之下附该段英文原文（灰色小字）。

### 2.1　初始颁行（1996-03-11，Amdt 23-48）：检查与其他程序

▪ 修订沿革：本版由 NPRM　Notice 94-20，1994-07-05 签发，59 FR 35196　提出；最终规章 Docket 27805，1996-01-29 签发，61 FR 5138，1996-03-11 生效。

▪ 修订指令（中文译文）：29. 新增§23.575条，内容如下：

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（1）：美国联邦航空局（FAA）曾提议删除§23.573(c)段，因为其关于检查和其他程序的要求被提议移至§23.575。

英文原文：

The FAA proposed to delete Sec. 23.573(c) because its requirements for inspections and other procedures were proposed to be moved to Sec. 23.575.

来源：Final Rule｜61 FR 5138｜FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt

▪ 背景与评论处置（2）：美国联邦航空局提议新增§23.575，以澄清当设计获得美国联邦航空局批准时，飞机制造商须提供关于检查频次、位置和方法的建议，且这些项目必须纳入§23.1529要求的持续适航文件的限制章节。
原§23.573(c)的要求将移至§23.575，并使其适用于§23.571、§23.572、§23.573和§23.574。
对该拟议新增条款的唯一意见是欧洲联合航空局（JAA）表示，这一变更将在JAR 23中予以考虑。提案按拟议内容通过。

英文原文：

The FAA proposed to add a new Sec. 23.575 to clarify that airplane manufacturers are required to provide recommendations for inspections frequencies, locations, and methods when a design is approved by the FAA, and that these items must be included in the Limitations Section of the Instructions for Continued Airworthiness required by Sec. 23.1529.

The requirements of Sec. 23.573(c) would be moved to Sec. 23.575 and the requirements are made applicable to Secs. 23.571, 23.572, 23.573 and 23.574.

The only comment on this proposed new section is a JAA statement that this change will be considered for JAR 23. The proposals are adopted as proposed.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（3）：本提案将删除 §23.573(c) 段。检查和其他程序将移至 §23.575 条，并适用于涉及疲劳评定的四个条，即 §23.571、§23.572、§23.573 和 §23.574 条。

英文原文：

This proposal would delete Sec. 23.573 (c). Inspections and other procedures would be moved to Sec. 23.575 and be made applicable to four sections pertaining to fatigue evaluation, namely, Secs. 23.571, 23.572, 23.573 and 23.574.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（4）：从技术角度看，这些措施与欧洲联合航空局（JAA）在欧洲联合航空要求（JAR）23的类似段落中所采取的努力相协调，JAR 23在JAR 23.571(b)、JAR 23.572(c)和JAR 23.573(c)中包含相同的检查要求。美国联邦航空局（FAA）的格式与JAR 23的呈现方式不同。JAR 23使用了三个段落；拟议§23.575使用一条达到相同的最终结果。

英文原文：

Technically, these actions harmonize with the efforts taken by the JAA in similar paragraphs of JAR 23, JAR 23 contains identical inspection requirements in JAR 23.571 (b), JAR 23.572 (c) and JAR 23.573 (c). The FAA format is different from the JAR 23 presentation. JAR 23 uses three paragraphs; proposed Sec. 23.575 uses one section to accomplish the identical end result.

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（5）：本提案将增加一个新的 §23.575 条，以澄清当设计获得美国联邦航空局（FAA）批准时，飞机制造商须提供关于检查频率、位置和方法的建议。过去 20 年来，是否要求这些检查和程序一直不明确。本提案澄清，安全寿命和损伤容限飞机设计均将涉及。此外，复合材料和金属飞机均将包括在内。

英文原文：

This proposal would add a new Sec. 23.575 that would clarify that airplane manufacturers are required to provide recommendations for inspection frequencies, locations and methods when the design is approved by the FAA. Whether these inspections and procedures are required has been unclear for the past 20 years. This proposal clarifies that Both safe-life and damage-tolerant airplane designs would be involved. Also, both composite and metallic airplanes would be included

来源：NPRM（提案）｜59 FR 35196｜NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt

▪ 背景与评论处置（6）：第 23.573(c) 条将被移至第 23.575 条并进行修订。此次修订包括列出所包含的那些要求，即第 23.571、23.572、23.573 和 23.574 条。这四条针对金属飞机的增压座舱、机翼、尾翼（尾部）及相关结构作出了规定。它们还提供了复合材料和金属飞机结构的损伤容限和疲劳评定标准。拟议的第 23.575 条将在技术上使这些规则与欧洲联合航空要求（JAR）23 协调统一，但采用简单的格式。

英文原文：

Section 23.573 (c) would be moved to Sec. 23.575 and revised. The revision consists of naming those requirements that are included, namely Secs. 23.571, 23.572, 23.573, and 23.574. These four sections address pressurized cabin, wing, empennage (tail), and associated structures for metallic airplanes. They also provide standards for damage tolerance and fatigue evaluations of both composite and metallic airplane structures. Proposed Sec. 23.575 would harmonize these rules with JAR 23 technically, but a simple format.

修订指令英文原文（供核对）：

29. A new Sec. 23.575 is added to read as follows:

## 3　§23.575 条款原文演变对比

本节先给出颜色图例，再按版本逐条列出条文（英文原文），最后以段落结构矩阵横向对照各版本。条文表的「状态」列标明该段在本版是新增、修订、删除还是继承未变。

■ 蓝色＝本版新增　　■ 红色＝本版修订/替换　　■ 灰色删除线＝本版删除　　■ 黑色＝继承上版、未变化

（本地文献中未检索到本条各版本的重印全文，请参照 1.2 沿革总表与各版修订指令。）

### 3.1　段落结构演变矩阵（横向对照 1 个版本）

（无可用全文，矩阵略。）

## 4　§23.575 总结

下表按「驱动力」归纳本条各次修订的成因（依据各版 NPRM/Final Rule 原文关键词自动归纳）。

| 驱动力 | 在本条上的体现（原文摘录） | 对应版本 |
| --- | --- | --- |
| 产业豁免压力倒逼 | proposed to add a new Sec. 23.575 to clarify that airplane manufacturers are required to provide recommendations for inspections frequencies, locations, and methods when a design is approved by the FAA, and that these i… | 初始版 |

本条取证提示 共 1 个历史版本：Amdt 23-48。 其中 1 个版本在本地文献中检索到明确修订指令原文；0 个版本检索到重印全文。 本条未见更正通告。

## 5　参考文献

下列文件均已下载并保存于本地目录「FAR 23/Rulemaking Docs」；FR 引证格式为「卷号 FR 起始页码」。文件名保留原名以便按名检索。

| 编号 | 类型 | 文件编号 / 案号 | Federal Register 引证与日期 | 本地文件名 / 来源 |
| --- | --- | --- | --- | --- |
| [1] | Final Rule（颁行） | Docket 27805 | 61 FR 5138 | FinalRule_Docket 27805_Amdt23-48_61FR5138_1996-02-09_0002.txt |
| [2] | Final Rule（颁行） | Docket 27805 | （FR 引证未在文本中标注） | FinalRule_Docket 27805_Amdt23-48_2008-04-11.txt |
| [3] | NPRM（提案） | Notice 94-20 / Docket 27805 | 59 FR 35196 | NPRM_Notice 94-20_Docket27805_59FR35196_1994-07-08.txt |

## 附录　DRS 条款版本记录 ID

FAA 动态法规系统（DRS）中 §23.575 各历史版本的记录 ID，可用于在 DRS 中直接定位相应版本条文。

| 修订号 | 生效日期 | DRS 记录 ID（docUniqueId） |
| --- | --- | --- |
| Amdt 23-48 | 1996-03-11 | 882cd378-5588-41e7-b934-754e40d42bff |